| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Location update |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Giwon Park, Ronny Yongho Kim and Kiseon Ryu LG Electronics | gwpark@lge.com, Ronnykim@lge.com, ksryu@lge.com |
| Re: | Reply to Call for comment (IEEE 80216m-08/052) on SDD 80216m-08/003r6 |  |
| Abstract | Proposal to enable AMS mobility based Location update in 802.16m system |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment of location update

Giwon Park, Ronny Yongho Kim, Kiseon Ryu

LG Electronics

# Introduction

The current SDD document (80216m-08_003r6) includes the following text:

“The network assigns idle mode AMS to a paging group during idle mode entry or location update. The design allows the network to minimize the number of location updates performed by the AMS and the paging signaling overhead caused to the ABSs. The idle mode operation considers user mobility.

ABSs and Idle Mode AMSs may belong to one or multiple paging groups in order to minimize the number of location updates and paging load without increasing average paging delay and without increasing the overhead of transmitting of multiple paging IDs by the ABSs. Idle mode AMSs may be assigned paging groups of different sizes and shapes based on user mobility.”

To support such features this contribution proposes AMS mobility profile based location update.

# SDD Text Proposal

---------------------------------------------------------Start of the text--------------------------------------------------

[Insert following text]

10.5.2.3	Location Update

10.5.2.3.1 Location update trigger condition

An AMS in idle mode shall perform a location update process operation if any of the following location update trigger condition is met.

- Paging group location update

- Timer based location update

- Power down location update

During paging group location update or timer based location update, AMS may update paging cycle and paging offset. AMS may include mobility information in the location update.

----------------------------------------------------------End of the text----------------------------------------------------

# Reference

- IEEE C802.16m-08/003r6, “Project 802.16m System Description Document (SDD)”.