| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC header format in 802.16m SDD |  |
| Date Submitted | 2009-01-13 |  |
| Source(s) | Sungjin Lee, Jungje Son, Rakesh Taori Samsung Electronics Mary Chion, Sean Cai ZTE Sean McBeath Huawei Technologies Mo-Han Fong Nortel Networks David Johnston, Muthaiah venkatachalam Intel Corporation | E-mail:steve.lee@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> E-mail: mchion@zteusa.com E-mail: smcbeath@huawei.com E-mail: mhfong@nortel.com E-mail: dj.johnston@intel.com |
| Re: | 80216m-08/052 - Call for Comments on Project 802.16m System Description Document (SDD) |  |
| Abstract | This contribution proposes a MAC header format |  |
| Purpose | For discussion and approval by IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC header format in 802.16m SDD

Sungjin Lee, Jungje Son and Rakesh Taori

Samsung Electronics

Mary Chion, Sean Cai

ZTE Corporation

Sean McBeath

Huawei Technologies

Mo-Han Fong

Nortel Networks

# Introduction

In current SDD we have multiple sections for a number of MAC headers. However the generic MAC header in current SDD has many unused fields and it is not well optimized. In this contribution we propose to smaller size of MAC header in 2 byte length by including only the essential and most common information from most MPDUs transmitted over the air.

First I deleted HT fields and other subsections following section 10.12.1.1 generic MAC header. We can have one single format of MAC header by removing other formats and the Generic MAC header size reduce to 2byte. With this 2byte size MAC header, we do not need compact header since the MAC header size is small enough.

EKS field is also removed from the MAC header since the EKS is needed only for the encrypted so that it may not needed if the payload is not encrypted. We need to delete the EKS from the MAC header and discuss to find other place to include this information later.

In previous meeting we have decided to have extended header for carrying additional information following generic MAC header. We may be able to carry other information including signaling by using this extended header but this issue is left for the the stage 3 discussion.

# Proposed Text to SDD

[Replace the current section Title of 10.12.1 and section 10.12.1.1 with following Proposed Text]

10.12.1 MAC Header format

10.12.1.1 Generic MAC Header

| EH (1) | FlowID (4) | Length (3) |
| --- | --- | --- |
| Length (8) |  |  |

Figure 22 Generic MAC Header Format

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- FlowID (Flow Identifier): This field indicates the flow that is addressed.. This field is 4bits long.

- Length: Length of the payload. This field is 11bits long

[Delete from the section 10.12.1.2 to 10.12.1.5 as following]