| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for the PHY structure of BRCH |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Chuangming Zheng, Xin Chang, Yunsong Yang, Tao Qian, Yinwei Zhao Huawei Technologies | xin.chang@huawei.com zhengchuangming@huawei.com |
| Re: | IEEE 802.16m-08/052: Call for Comments on Project 802.16m System Description Document (SDD) Section: Bandwidth Request Channel |  |
| Abstract | The contribution proposes the modified text for the PHY structure of Bandwidth Request Channel section. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for the PHY structure of BRCH

Chuangming Zheng, Xin Chang, Yunsong Yang, Tao Qian, Yinwei Zhao

Huawei Technologies

- Introduction

The contribution proposes modified text for the BRCH PHY structure section included in the 802.16m SDD (IEEE 802.16m-08/003r6) [1]. The proposed text is compliant to the 802.16m SRD [2], and it follows the style and format guidelines in [3].

- Definition

BRCH		Bandwidth Request Channel

- Introduction

According SRD requirement, 30 active users/MHz/Sector shall be support for VoIP. For 10MHz bandwidth, 240 active users should be support in a conservative expectation. If the DL:UL ratio is 5:3, the 3 UL subframe is available for transmit bandwidth request. According to active factor 50% in EMD[4], 240 silent users are expected to transmit bandwidth request in these 3 UL subframe. In average, each UL subframe shall support bandwidth request for 80 silent users.

Let’s consider the probability of transmitting bandwidth request for VoIP silent user when transits from silent state to active state. The probability of such transition is calculated by adopting transition probability specified in EMD.

The probability of no transition is: C8000.986780=34.3%;

The probability of single transition is: C8010.986779×0.0133=36.9%;

The probability of two transitions is: C8020.986778×0.01332=19.7%;

The probability of three transitions is: C8030.986777×0.01333=6.9%;

The probability of transition more than three is: 1−34.3%−36.9%−19.7%−6.9%=2.2%.

That means at most case, the concurrent bandwidth requests are no more than 2.2%. Hence, limited number of orthogonal sequences is needed to support contention-based bandwidth request indication. Dedicated manner is resource wasteful. Request delay can be enhanced through piggy-back BW-REQ message with data if BS allocates a small size of UL resource when receiving BR indicator.

- Scheme Description

The proposed quick access procedure for bandwidth request is shown in figure 1.

Figure 1: 3-step quick access procedure for bandwidth request.

A 5-step regular procedure or an optional 3-step quick access procedure may be supported concurrently. The optional 3-step quick access procedure is described as following:

1. MS transmits BR indicator in a contention-based way

2. BS allocate pre-defined size of resources if detects BR indicator successfully, which can at least satisfies the resource requirement of BW-REQ message transmission.

3. MS transmits the UL data and optional further BW-REQ message. If the allocated resource can satisfy the data transmission, the step 4 and 5 can be skipped.

4. If BS receives further BW-REQ message in step 3, BS can transmit UL Grant message to allocate UL resource according to scheduling result.

5. MS transmits the UL data when receiving UL Grant message.

A BW REQ tile is defined as 2 contiguous subcarriers by 6 OFDM symbols. Each BW REQ channel consists of 3 distributed BW-REQ tiles.

CDM allows multiple bandwidth request indicators to be transmitted on the same BW REQ channel. In addition, multiple BW REQ channels may be allocated per subframe using FDM. A BW REQ channel shall be carried by 12-ray orthogonal sequences with OOK modulation mapping to every of 3 distributed BW-REQ tiles. The illustration of PHY structure is shown in Figure 2.

OFDM Symbol

subcarrier

Figure 2  the resource allocated for one BRCH.

An exampled orthogonal sequence is given in Table 1:

| No. | orthogonal Sequence |
| --- | --- |
| 1 | 1 1 1 1 1 1 1 1 1 1 1 1 |
| 2 | 1 0 1 0 1 1 1 0 0 0 1 0 |
| 3 | 1 0 0 1 0 1 1 1 0 0 0 1 |
| 4 | 1 1 0 0 1 0 1 1 1 0 0 0 |
| 5 | 1 0 1 0 0 1 0 1 1 1 0 0 |
| 6 | 1 0 0 1 0 0 1 0 1 1 1 0 |
| 7 | 1 0 0 0 1 0 0 1 0 1 1 1 |
| 8 | 1 1 0 0 0 1 0 0 1 0 1 1 |
| 9 | 1 1 1 0 0 0 1 0 0 1 0 1 |
| 10 | 1 1 1 1 0 0 0 1 0 0 1 0 |
| 11 | 1 0 1 1 1 0 0 0 1 0 0 1 |
| 12 | 1 1 0 1 1 1 0 0 0 1 0 0 |

Table 1: orthogonal sequences for BRCH

- Simulation

The simulation parameter is shown in Table 2.

| Parameter | Value |
| --- | --- |
| Channel Model | PB3 and VA120 |
| Bandwidth | 10MHz |
| FFTSize | 1024 |
| CP Ratio | 1/8 |
| Antenna Configuration | 1Tx * 2Rx |
| Power Control | Ideal |

Table 2: BRCH simulation parameter

The simulation result of BRCH performance for the false alarm is as following figure.

Figure 3a: the performance of BRCH for PB3

Figure 3b: the performance of BRCH for VA30

- References

[1] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[2] IEEE 802.16m-07/002r6, “802.16m System Requirements”

[3] IEEE 802.16m-08/052, “Call for Comments on Project 802.16m System Description Document (SDD)”

[4] IEEE 802.16m-08/004r4, “IEEE 802.16m Evaluation Methodology Document (EMD)”

- Text proposal for the 802.16m SDD

---------------------------------------  Text Start  ---------------------------------------------------

        - PHY structure

A BW REQ tile is defined as 62 contiguous subcarriers by 6 OFDM symbols. Each BW REQ channel consists of 3 distributed BW-REQ tiles.

CDM allows multiple bandwidth request indicators to be transmitted on the same BW REQ channel. In addition, multiple BW REQ channels may be allocated per subframe using FDM. The ranging sequence design and mapping to subcarriers are TBD. A BW REQ channel shall be carried by 12-ary orthogonal sequences with OOK modulation mapping to every of 3 distributed BW-REQ tiles.

---------------------------------------  Text End  ---------------------------------------------------