| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on SDD for simplifying ARQ feedback types |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son Samsung Electronics | yb.chang@samsung.com, anilag@samsung.com |
| Re: | Call for Comments on 802.16m SDD (IEEE 802.16m-08/052) |  |
| Abstract | This contribution is about ARQ feedback types |  |
| Purpose | For adopting proposed text into 16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on SDD for Simplifying ARQ feedback types

Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son

Samsung Electronics

# Problem description

- In current 16e standard, there are five ARQ feedback types (See Rev2/D8, 6.3.4.2.1)

  - 3 standalone types (Cumulative ACK, Selective ACK, Block sequence ACK)

  - 2 combination types (Cumulative + Selective ACK, Cumulative + Block sequence ACK)

- Reason for various ARQ feedback types

  - To support various error pattern:

    - Cumulative ACK: if there is no error in the receiver.

    - Selective ACK: if error pattern is selective

    - Block sequence ACK : if error pattern is contiguous

    - Cumulative + Selective or Block sequence: if some data received correctly and some data has an error (Depending on error pattern)

- These several ARQ feedback types make both transmitter and receiver complex.

# Solution for simplifying ARQ feedback type

- Remove block sequence ACK type. Block sequence ACK type is not needed if,

  - MPDU payload per flow contains one ARQ block and not multiple ARQ blocks

  - When error is detected in the receiver side, receiver sends ARQ feedback.

- Use only cumulative and selective ACK type

  - Cumulative ACK type is used to feedback successful reception of ARQ blocks

  - Selective ACK type is used to feedback the failed reception of an ARQ block.

  - Mixed (Cumulative and selective) ACK type is used to feedback when some portion of ARQ blocks are received successfully and some potion of ARQ blocks have an error.

# Text Proposal

[Insert the following text after line 9 on page 42]

Cumulative and selective ACK types are used by the receiver for sending an ARQ feedback.