| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on the SDD Text on Zone Switch operation |  |
| Date Submitted | 2009-1-5 |  |
| Source(s) | Yanling Lu Kevin Power Rajni Agarwal Masato Okuda Fujitsu. | Yanling.lu@cn.fujitsu.com Kevin.power@uk.fujitsu.com |
| Re: | Comment on IEEE 802.16m System Description Document: Session 10.3.5.2 “Handover from 16e to 16m” |  |
| Abstract | This contribution proposes some detail description on zone switch |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Comments on the SDD Text on Zone Switch

Yanling Lu, Kevin Power, Rajni Agarwal, Masato Okuda

Fujitsu.

# Proposed SDD Text Modification

[Black: original text, blue: new next, red: modified text]
--------------------------------------------------------  Text Start  ------------------------------------------------------------

10.3.5.2 Handover from 16e to 16m

When a 16e-to-16m handover is triggered for a YMS, the YMS handover is from the serving YBS to the LZone of the target ABS using 16e handover signaling and procedures.

When a 16e-to-16m handover is triggered for a AMS, the AMS may handover from the serving YBS to the LZone of the target ABS using 16e handover signaling and procedures. The target YBS/ABS may direct the AMS to switch to 16m zoneMZone. The detail procedure for zone switching is FFS described in 10.3.7.

An AMS may also handover from a YBS to a 16m only ABS or MZone of ABS directly if AMS is able to scan 16m-only ABS or 16m zoneMZone prior to handover. The detail procedure is FFS.

[Add the following text after Section 10.3.5.3]

10.3.7 Zone switch

The AMS may request or be directed by the ABS to switch between LZone and MZone after network entry to the ABS, based on some criteria, such as traffic load. The ABS can inform the AMS of target mode of operation before switching zone. Since the LZone and MZone are synchronized in frequency, during the zone switch procedure and depending on the information exchanged beforehand, it may be unnecessary for the AMS to perform some phases of network re-entry, such as ranging, registration and so on.

The AMS may request or be directed by the ABS to switch between LZone and MZone during network entry to the ABS. The target ABS can inform the AMS zone of switch information, such as MZone location and related parameters, after DL synchronization. The AMS shall then perform the remaining network re-entry procedure in the MZone as per 10.3.2.3.

--------------------------------------------------------  Text End  ------------------------------------------------------------