| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Roshni Srinivasan, Hongmei Sun, Changlong Xu, Yuan Zhu, Jong-Kae (JK) Fwu, Hujun Yin, Qinghua Li, Alexei Davydov, Rath Vannithamby, Sassan Ahmadi, Intel Corporation | E-mail: {roshni.m.srinivasan, hongmei.sun, changlong.xu, yuan.y.zhu}@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16m amendment working document |  |
| Abstract | Proposed text for the PHY structure for UL control in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control

Roshni Srinivasan, Hongmei Sun, Changlong Xu, Yuan Zhu, Jong-Kae (JK) Fwu, Hujun Yin, Qinghua Li, Alexei Davydov, Rath Vannithamby, Sassan Ahmadi

Intel Corporation

Introduction

This contribution provides amendment text for the UL Control Structure in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Amendment Working Document [1]. The text was developed using baseline text from the Draft IEEE 802.16m System Description Document (SDD) [2]. Further details of the PHY structure have been provided for the UL Primary and Secondary Fast Feedback Channels, the HARQ Feedback Channel and Bandwidth Request Channel including tile and pilot structure, coding and modulation.

Performance evaluation of the proposed PHY structures for the UL Primary and Secondary Fast Feedback Channels, the HARQ Feedback Channel and Bandwidth Request Channel are provided in [3][4]and [5] respectively.

References

- IEEE 802.16m Amendment Working Document, IEEE 802.16m-08/050

- IEEE 802.16m System Description Document (SDD) [Draft], IEEE 802.16m-08/003r6.

- Hongmei Sun, Changlong Xu, Jong-Kae (JK) Fwu, Qinghua Li, Eddie Lin, Yuan Zhu, Hujun Yin, Roshni Srinivasan, Rath Vannithamby, Sassan Ahmadi, “Cross Evaluation of Proposed PHY Structures for the IEEE 802.16m UL Primary and Secondary Fast Feedback Channels”, IEEE C802.16m-09/0125, Jan 2009.

- Changlong Xu, Qinghua, Li, Yuan, Zhu, Hongmei Sun , Jong-kae (JK) Fwu, Hujun Yin, Sassan Ahmadi, Roshni Srinivasan”Proposed PHY Structure for the IEEE 802.16m HARQ Feedback Channel”, IEEE C802.16m-09/0169.ppt, January 2009.

- Yuan Zhu, Xiangying , Qinghua Li, Changlong Xu, Jong-kae Fwu, Hujun Yin, ”Proposed PHY Structure for the IEEE 802.16m Bandwidth Request Channel”, IEEE C802.16m-09/0142, January 2009.

Proposed Amendment Text

The following text is proposed for inclusion in Section 15 of the IEEE 802.16m Amendment Working Document [1].

# UL Control Structure

Details of the UL control structure are described in the following sections.

# UL Control Information Classification

The UL control channels carry multiple types of control information to support air interface procedures. Information carried in the control channels is classified as follows.

# Channel quality feedback

Channel quality feedback provides information about channel conditions as seen by the aMS. This information is used by the aBS for link adaptation, resource allocation, power control etc. Channel quality measurement includes narrowband and wideband measurements. CQI feedback overhead reduction is supported through differential feedback or other compression techniques. Examples of CQI include Physical CINR, Effective CINR, band selection, etc. Channel sounding can also be used to measure uplink channel quality.

# MIMO feedback

MIMO feedback provides wideband and/or narrowband spatial characteristics of the channel that are required for MIMO operation. The MIMO mode, precoder matrix index, rank adaptation information, channel covariance matrix elements, power loading factor, eigenvectors and channel sounding are examples of MIMO feedback information.

# HARQ feedback

HARQ feedback (ACK/NACK) is used to acknowledge DL transmissions. Multiple codewords in MIMO transmission can be acknowledged in a single ACK/NACK transmission.

# Synchronization

Uplink synchronization signals are needed to acquire uplink synchronization during initial access or handover and also to periodically maintain synchronization. Reference signals for measuring and adjusting the uplink timing offset are used for these purposes.

# Bandwidth request

Bandwidth requests are used to provide information about the needed uplink bandwidth to the aBS. Bandwidth requests are transmitted through indicators or messages. A bandwidth request indicator notifies the aBS of a UL grant request by the aMS sending the indicator. Bandwidth request messages can include information about the status of queued traffic at the aMS such as buffer size and quality of service, including QoS identifiers.

# E-MBS feedback

E-MBS may employ a common uplink channel which is used by aMSs to transmit feedback. E-MBS feedback transmission through a dedicated channel is FFS. If a predefined feedback condition is met, a NACK is transmitted through a common E-MBS feedback channel. The feedback condition may be configured by either the AaBS or the network.

During E-MBS service initiation, a common feedback channel per E-MBS service may be allocated. The allocation of more than one common E-MBS feedback channel per E-MBS service is FFS. The allocation of the common E-MBS feedback channel may be configured by the AaBS. The allocation of the common E-MBS feedback channel configured by the network is FFS. Other methods for reducing E-MBS feedback overhead are FFS, e.g. probabilistic transmission. The use of the feedback channels for other purposes, (e.g., counting) is FFS.

The termination notification of MBS service is FFS.

# UL Control Channels

The UL subframe size 6 symbols shall be used for transmission of control information.

# UL Fast Feedback Channel

The UL fast feedback channel shall carry channel quality feedback and MIMO feedback. Transmission of BW REQ indicators on the UL fast feedback channel is FFS. There are two types of UL fast feedback control channels defined in the Advanced Air Interface specifications: primary and secondary fast feedback channels.

# UL Primary Fast Feedback Channel (PFBCH)

The UL primary fast feedback control (PFBCH) channel defined by the Advanced Air Interface specifications carries 4 to 6 bits of information, providing wideband channel quality feedback and MIMO feedback. It is used to support robust feedback reports.

# Multiplexing with other control channels and data channels

The PFBCH shall be multiplexed with other UL control and data channels within a UL subframe using FDM.

The PFBCH shall start at a pre-determined location, with the size defined in a DL control message (add TBD reference to appropriate DL control section). PFBCH allocations to an aMS can be periodic and the allocations are configurable.

The PFBCH carries one or more types of fast feedback information.  Multiple PFBCHs for one or more users are multiplexed using FDM.

# PHY structure

A UL feedback mini-tile (FMT) in the Advanced Air Interface specifications is defined as 2 contiguous subcarriers by 6 OFDM symbols. The UL FMT structure for the PFBCH is shown in Figure 1. The PFBCH structure comprises of 3 FMTs, which are chosen from different UL DRUs for frequency diversity as shown in Figure 2. The procedure for allocation of resources for transmission of UL control information and the formation of DRUs for such transmission is TBD.

Figure 1: UL FMT structure for the PFBCH

Figure 2: PFBCH comprising of three distributed 2x6 UL FMTs

Figure 3: Mapping of information in the PFBCH

Each PFBCH carries  bits, where . The number of information bits carried in the PFBCH is FFS. The symbol generation procedure is shown in Figure 3. First, the 4 to 6 information bits   shall be encoded into12 bits b0b1b2⋯b11using the semi-orthogonal sequences s0,n,s1,n,s2,n,...,s11,n (n=0,1,...,2l−1).

- For 4 information bits or l=4, sequences s0,n,s1,n,s2,n,...,s11,n (n=0,1,...,15) listed in Table 1 shall be used.

- For 5 information bits or l=5, sequences s0,n,s1,n,s2,n,...,s11,n (n=0,1,...,31) listed in Table 2 shall be used.

- For 6 information bit or l=6, sequences s0,n,s1,n,s2,n,...,s11,n (n=0,1,...,63) listed in Table 3 shall be used.

Table 1 : Semi-orthogonal sequences for 4bits information in PFBCH

| n | S0,n | S1,n | S2,n | S3,n | S4,n | S5,n |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -0.19-0.24i | 0.28-0.05i | 0.26+0.12i | 0.28-0.08i | 0.26-0.18i | 0.21+0.14i |
| 1 | 0.06+0.28i | -0.03-0.31i | -0.08+0.24i | 0.16+0.23i | 0.13-0.26i | -0.15+0.27i |
| 2 | -0.09-0.29i | -0.25-0.18i | 0.1+0.27i | -0.26-0.06i | 0.27+0.03i | -0.18-0.23i |
| 3 | -0.23-0.19i | -0.3-0.05i | -0.23+0.15i | 0.07-0.28i | -0.11-0.25i | 0.2-0.2i |
| 4 | 0.08+0.29i | 0.26-0.12i | 0.31+0.03i | 0.29+0.09i | -0.04+0.32i | -0.17-0.22i |
| 5 | -0.29+0.05i | -0.04+0.26i | -0.05-0.26i | 0.21-0.19i | -0+0.3i | 0.06+0.3i |
| 6 | 0.23-0.19i | 0.26-0.16i | 0.13+0.18i | 0.2+0.24i | -0.22+0.22i | 0.24+0.16i |
| 7 | 0.25+0.15i | 0.11+0.29i | 0.12-0.27i | -0.02-0.28i | 0.25-0.17i | 0.15-0.23i |
| 8 | 0.27-0.03i | 0.08+0.3i | -0.24+0.17i | 0.24-0.15i | 0.08-0.23i | 0.16+0.23i |
| 9 | -0.07-0.31i | 0.17+0.23i | 0.09-0.3i | -0.21-0.19i | 0.04+0.26i | -0.11+0.28i |
| 10 | -0.22+0.06i | 0.24+0.18i | 0.28-0.04i | -0.28+0.1i | 0.29+0.03i | 0.29+0.01i |
| 11 | 0.18-0.23i | 0.15+0.27i | 0.31-0.02i | -0.07+0.27i | -0.04-0.3i | 0.29-0.02i |
| 12 | -0.15-0.28i | -0.04+0.3i | -0.15+0.2i | 0.22+0.11i | 0.26-0.2i | -0.05-0.26i |
| 13 | -0.01-0.32i | 0.28-0.06i | -0.11-0.29i | -0.04-0.25i | -0.16-0.25i | -0.26+0.15i |
| 14 | -0.27+0.14i | -0.15+0.22i | -0.08-0.29i | 0.22+0.21i | -0.23-0.22i | 0.29-0.01i |
| 15 | -0.07+0.28i | 0.27-0.07i | 0.14+0.27i | 0.05-0.29i | 0.08-0.26i | -0.1+0.25i |
| n | S6,n | S7,n | S8,n | S9,n | S10,n | S11,n |
| 0 | 0.01-0.27i | -0.23+0.18i | -0.04-0.27i | 0.22-0.21i | 0.27-0.09i | 0.28+0.11i |
| 1 | -0.24+0.13i | 0.07+0.27i | 0.3-0.02i | 0.3-0.03i | -0.16-0.25i | 0.21+0.16i |
| 2 | -0.05-0.26i | -0.1-0.3i | 0.25-0.15i | -0.15-0.23i | 0.25-0.14i | 0.05-0.27i |
| 3 | 0.18-0.26i | -0.2+0.14i | -0.28+0.08i | 0.29+0.12i | -0.1+0.26i | -0.1+0.27i |
| 4 | 0.28+0.03i | -0.3+0.03i | -0.21-0.1i | -0.14-0.26i | -0.21+0.17i | 0.28+0.01i |
| 5 | 0.07-0.28i | 0.15-0.25i | 0.17-0.19i | 0.27-0.14i | 0.04-0.29i | -0.08-0.31i |
| 6 | -0.17-0.24i | 0.04+0.23i | -0.29+0.05i | -0.25+0.12i | 0.25-0.15i | -0.31+0.05i |
| 7 | 0.25+0.13i | -0.02+0.3i | 0.07-0.28i | 0.26+0.1i | 0.26-0.07i | -0.29-0.1i |
| 8 | -0.28+0.07i | -0.02-0.3i | -0.16+0.24i | -0.19-0.2i | 0.29-0.08i | 0.3-0.07i |
| 9 | 0.22+0.2i | -0.2+0.22i | -0.08+0.26i | 0.04-0.27i | -0.12-0.24i | 0.26+0.14i |
| 10 | -0.23+0.1i | 0.3-0.1i | -0.22-0.21i | -0.03+0.31i | 0.09+0.28i | 0.23+0.17i |
| 11 | -0.18-0.18i | 0.04+0.27i | 0.15+0.25i | 0.29-0.03i | -0.2+0.23i | -0.03-0.27i |
| 12 | 0.27+0.13i | 0.29+0.05i | -0.13-0.26i | -0.28+0.08i | -0.09-0.25i | 0.3+0i |
| 13 | -0.3+0.03i | -0.27-0.05i | 0.1-0.25i | -0.14+0.23i | -0.08+0.25i | 0.28+0.12i |
| 14 | 0.21-0.19i | -0.28-0.05i | 0.24+0.11i | -0.3-0.1i | 0.25-0.01i | 0.12+0.24i |
| 15 | 0.27+0.13i | 0.25+0.17i | 0.21+0.21i | -0.28-0.06i | 0.23+0.08i | -0.24+0.18i |

Table 2 : Semi-orthogonal sequences for 5bits information in PFBCH

| n | S0,n | S1,n | S2,n | S3,n | S4,n | S5,n |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.22-0.14i | 0.23+0.2i | -0.07+0.14i | 0.27-0.18i | -0.07+0.08i | -0.2-0.25i |
| 1 | -0.3+0.05i | 0.03+0.26i | -0.07-0.32i | 0-0.12i | 0.26-0.07i | 0.05-0.29i |
| 2 | 0.03-0.29i | -0.25-0.03i | -0.1+0.28i | 0.28-0.13i | -0.09+0.08i | 0.28+0.1i |
| 3 | -0.07-0.26i | 0.24+0.21i | -0.1-0.16i | -0.25+0.17i | 0.07+0.31i | 0.19+0.11i |
| 4 | 0.17-0.32i | -0.1+0i | 0.24-0.15i | 0.02+0.09i | 0.19-0.27i | -0.12+0.31i |
| 5 | -0.11+0.17i | 0.16+0.25i | -0.15-0.2i | 0.29-0.18i | 0.3+0.05i | 0.23-0.2i |
| 6 | -0.11-0.34i | 0.12+0.19i | -0.29-0.18i | -0.03+0.3i | -0.23-0.25i | -0.25-0.24i |
| 7 | 0.26-0.19i | 0.26-0.04i | -0.27-0.13i | 0.3-0.03i | -0.01-0.06i | 0.11+0.3i |
| 8 | -0.25+0.01i | -0.08-0.26i | -0.21-0.23i | -0.09-0.3i | 0.2-0.23i | 0.24+0.2i |
| 9 | -0.29-0.07i | 0.02+0.27i | -0.09+0.33i | 0.23-0.24i | 0.16-0.24i | -0-0.23i |
| 10 | 0.29-0.14i | 0.12-0.3i | -0.02-0.33i | 0.2-0.16i | -0.07-0.32i | 0.05-0.13i |
| 11 | -0.26+0.22i | -0.15+0.12i | 0.21-0.17i | 0.03-0.29i | -0.23-0.24i | -0.19-0.27i |
| 12 | -0.28+0.13i | 0.19-0.2i | 0.29-0.09i | 0.35+0.02i | -0.25-0.14i | -0.08+0.05i |
| 13 | -0.28+0.16i | -0.27-0.04i | -0.1+0.28i | -0.15-0.04i | 0.17+0.16i | -0.29+0.12i |
| 14 | -0.29+0.12i | 0.27+0.14i | 0.04+0.31i | 0.26+0.13i | -0.14-0.29i | 0.29+0.1i |
| 15 | 0.1-0.22i | -0.07+0.14i | 0.02-0.13i | 0.27+0.19i | 0.01-0.35i | 0.06+0.28i |
| 16 | 0.28-0.04i | -0.3+0.16i | -0.18-0.22i | 0.33-0.1i | -0.16-0.29i | 0.1+0.16i |
| 17 | 0.06+0.17i | 0.12-0.31i | -0.05+0.3i | -0.32-0.05i | 0.27+0.18i | 0.02+0.32i |
| 18 | -0.3+0.07i | -0.14+0.27i | 0.16-0.23i | 0.01+0.03i | -0.08+0.31i | -0.27+0.07i |
| 19 | 0.1+0.22i | 0.18-0.24i | 0.32+0.11i | 0.29+0.04i | 0.08+0.22i | 0.27-0.12i |
| 20 | -0.27+0.15i | 0.21-0.27i | 0.3-0.06i | -0.29+0.08i | -0.01-0.32i | 0.05-0.01i |
| 21 | -0.21-0.06i | -0.28-0.1i | 0.08+0.15i | 0.3-0.1i | 0.03-0.29i | -0.24+0.24i |
| 22 | 0.19-0.22i | 0.26-0.05i | -0.1+0.25i | -0.22+0.23i | 0.15-0.31i | -0.16-0.28i |
| 23 | -0.01+0.35i | -0.18-0.23i | -0.32+0.01i | 0.28-0.12i | 0.22+0.02i | 0.28+0.07i |
| 24 | 0.17-0.21i | 0.11-0.28i | 0.1+0.33i | -0.21-0.13i | 0.25-0.19i | 0.01+0.26i |
| 25 | 0.27+0.02i | -0.16+0.12i | -0.25+0.13i | -0.11+0.31i | -0.32+0.08i | 0.21+0.27i |
| 26 | 0.22-0.13i | -0.23-0.05i | 0.31+0.02i | -0.22-0.22i | -0.29+0.17i | 0.13-0.32i |
| 27 | 0.27-0.05i | 0.3+0.11i | 0.26-0.2i | 0.07-0.31i | -0.11-0.03i | -0.1+0.27i |
| 28 | -0.27-0.19i | -0.24-0.16i | -0.18-0.13i | 0.01-0.24i | 0.21-0.27i | -0.28-0.12i |
| 29 | -0.15+0.17i | -0.28-0.11i | -0.29-0.1i | 0.29+0.13i | 0.24+0.25i | -0.25-0.2i |
| 30 | 0.25-0.02i | 0.3-0.07i | -0.26+0.04i | 0.27+0.13i | -0.28-0.04i | -0.17-0.11i |
| 31 | 0.23+0.07i | -0.23+0.09i | -0.08+0.19i | 0.31+0.01i | 0.1+0.23i | -0.33+0.07i |
| n | S6,n | S7,n | S8,n | S9,n | S10,n | S11,n |
| 0 | 0.04-0.21i | 0.2-0.2i | 0.05-0.34i | 0.35-0.07i | 0.22-0.25i | 0.18-0.28i |
| 1 | -0.15+0.22i | 0.04+0.3i | 0.17-0.2i | 0.09-0.31i | -0.05-0.3i | 0.06-0.34i |
| 2 | 0.05+0.31i | 0.03-0.32i | -0.17-0.25i | 0.14-0.33i | 0.32+0i | -0.12+0.15i |
| 3 | -0.3-0.07i | 0.1-0.28i | 0.01-0.32i | -0.27-0.09i | 0.26-0.18i | 0.28-0.04i |
| 4 | 0.08+0.32i | -0.17-0.28i | -0.22-0.17i | -0.3+0.16i | -0.04-0.16i | 0.21+0.28i |
| 5 | -0.14-0.3i | 0.26+0.01i | -0.31-0.13i | -0.03-0.24i | 0.26-0.03i | -0.06+0.3i |
| 6 | 0.01-0.31i | 0.07+0.16i | 0.11-0.17i | -0.06-0.19i | 0.21+0.25i | -0.24-0.09i |
| 7 | 0.29-0.04i | 0.2-0.21i | 0.22-0.18i | -0.18-0.29i | -0.32-0.01i | 0.24+0.11i |
| 8 | -0.11+0.21i | 0.15+0.09i | -0.14-0.29i | -0.09+0.33i | 0.21-0.19i | -0.28+0.14i |
| 9 | 0.05-0.13i | -0.19+0.27i | 0.3-0.04i | 0.14+0.13i | 0.25-0.16i | -0.27+0.24i |
| 10 | 0.25+0.25i | 0.05-0.28i | 0.21+0.21i | -0.01+0.17i | 0.27+0.07i | -0.21+0.23i |
| 11 | 0.07+0.22i | 0.3-0.17i | -0.19+0.17i | -0.04+0.26i | -0.01+0.28i | 0.3-0.04i |
| 12 | 0.02+0.29i | -0.24-0.05i | 0.15-0.28i | 0.22+0.25i | 0.33-0.08i | 0.16-0.16i |
| 13 | 0.17+0.24i | 0.06-0.24i | 0.22-0.19i | 0.13+0.32i | 0.23+0.28i | -0.15+0.21i |
| 14 | -0.07-0.2i | -0.31-0.09i | -0.27+0.08i | 0.02-0.16i | -0.09+0.28i | -0.23+0.18i |
| 15 | -0.2-0.27i | 0.26+0.18i | -0.04+0.3i | -0.08+0.3i | 0.18-0.26i | -0.15+0.24i |
| 16 | -0.02+0.07i | -0.24+0.22i | -0.09-0.26i | -0.31-0.14i | -0.15+0.17i | -0.32+0.04i |
| 17 | 0.08-0.17i | 0.16+0.27i | 0.27-0.12i | -0.29+0.05i | 0.01+0.14i | 0.3+0.19i |
| 18 | -0.02+0.34i | -0.16+0.27i | -0.13-0.25i | -0.27-0.02i | 0.22+0.15i | 0.22+0.24i |
| 19 | 0.17-0.29i | -0.08-0.34i | 0-0.17i | -0.23+0.03i | 0.26-0.21i | -0.26-0.14i |
| 20 | 0.29-0.11i | 0.22+0.04i | -0.18-0.15i | 0.34+0i | 0.12+0.25i | -0.3-0.13i |
| 21 | 0.3-0.18i | 0.21+0.2i | -0.06-0.35i | -0.1-0.27i | 0.18+0.04i | 0.22-0.2i |
| 22 | 0.17-0.32i | -0.17+0.02i | -0.21-0.27i | 0.03+0.29i | -0.07-0.16i | 0.15+0.17i |
| 23 | -0.1-0.31i | -0.23-0.05i | 0.23-0.15i | 0.31+0.07i | -0.19+0.12i | 0.13+0.26i |
| 24 | 0.05+0.27i | 0.02+0.22i | -0.1+0.32i | 0.26-0.12i | 0.32-0.14i | -0.09+0.18i |
| 25 | -0.24+0.24i | -0.12+0.31i | -0.26+0.03i | 0.08+0.23i | 0.17-0.21i | 0.09-0.21i |
| 26 | 0.3+0.1i | 0.26-0.18i | -0.08-0.3i | -0+0.2i | -0.14+0.06i | -0.29+0.03i |
| 27 | -0.05+0.06i | -0.24-0.16i | 0.24-0.25i | 0.29+0.17i | -0.3+0.11i | -0.29+0.1i |
| 28 | -0.17-0.15i | 0.32-0.1i | 0.03+0.26i | 0.33-0.06i | 0.34-0.05i | -0.09-0.16i |
| 29 | -0.08+0.21i | 0.28-0.2i | -0.26+0.01i | 0.24-0.24i | -0.3+0.05i | 0.09+0.09i |
| 30 | 0.28+0.15i | -0.3-0.05i | -0.32+0.02i | -0.2-0.15i | 0.34+0.03i | 0.29-0.08i |
| 31 | 0.18-0.17i | 0.18+0.27i | -0.05-0.33i | -0.01+0.3i | -0.24-0.17i | -0.09+0.31i |

Table 3 : Semi-orthogonal sequences for 6bits information in PFBCH

| n | S0,n | S1,n | S2,n | S3,n | S4,n | S5,n |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.1+0.09i | 0.09-0.23i | -0.18+0.32i | 0.05+0.24i | 0.07+0.1i | 0.12+0.08i |
| 1 | -0.17-0.2i | -0.18+0.18i | -0.22-0.1i | -0.31-0.01i | 0.13+0.33i | 0.25+0.19i |
| 2 | -0.01-0.38i | 0.06+0i | -0.05-0.29i | 0.05+0i | -0.22+0.29i | -0.02+0.13i |
| 3 | -0.27-0.27i | 0.24+0.32i | -0.03-0.37i | 0.37+0.12i | -0.22-0.17i | 0.15-0.36i |
| 4 | -0.27-0.24i | -0.15-0.19i | -0.01-0.02i | -0.17+0.22i | -0.19+0.24i | 0.17-0.33i |
| 5 | -0.15+0.05i | -0.19+0.27i | -0.01+0.37i | 0.39+0.04i | 0.14-0.04i | 0.15+0.33i |
| 6 | -0.04+0.16i | -0.41+0.1i | 0.15+0.19i | -0.01+0.32i | -0.06+0.22i | 0.19+0.26i |
| 7 | -0.1+0.22i | 0.36-0.12i | -0.4-0.01i | 0.29+0.23i | -0.11-0.4i | -0.09-0.06i |
| 8 | -0.22-0.13i | -0.08-0.02i | -0.26-0.15i | 0.41+0.05i | 0.07-0.03i | 0.13-0.29i |
| 9 | -0.23-0.17i | 0.21+0.21i | -0.08-0.2i | -0.02-0.33i | 0.05+0.22i | 0.35-0.15i |
| 10 | -0.3-0.1i | -0+0.05i | -0.21+0.12i | 0.36+0.19i | -0.29+0.05i | 0.25+0.19i |
| 11 | -0.27+0.27i | -0.12-0.07i | 0.23+0.25i | 0.2+0.03i | -0.34-0.18i | 0.32+0.13i |
| 12 | 0.2+0.04i | -0.16-0.07i | -0.08-0.29i | -0.13+0.15i | -0.12+0i | 0.26+0.22i |
| 13 | -0.33+0.06i | 0.01-0.3i | -0.09+0.33i | -0.01-0.25i | 0.16+0.28i | 0.17+0.17i |
| 14 | -0.15-0.05i | 0.16+0.18i | -0.15+0.15i | 0.25-0.31i | -0.24-0.01i | -0.16+0.39i |
| 15 | -0.05+0.41i | 0.11+0.31i | -0.05+0.07i | 0.35-0.26i | -0.38-0.05i | -0.04+0.34i |
| 16 | -0.23-0.09i | -0.36+0.12i | 0.05-0.37i | -0-0.17i | -0.3-0.16i | 0.26+0.07i |
| 17 | -0.14+0.3i | 0.22-0.12i | -0.34+0.02i | -0.16-0.02i | 0.01-0.34i | 0.12+0.29i |
| 18 | 0.05-0.33i | -0.33-0.06i | 0+0.24i | 0.14+0i | 0.07+0.17i | 0.19+0.38i |
| 19 | -0.13-0.32i | 0.1-0.27i | -0.25-0.17i | 0.23-0.04i | -0.36-0.12i | 0.13+0.12i |
| 20 | -0.07-0.2i | 0.2-0.31i | 0.03+0.41i | -0.19-0.04i | 0.02-0.14i | -0-0.12i |
| 21 | -0.06+0.1i | -0.32+0.17i | -0.31-0.08i | -0.25+0.17i | 0.21-0.2i | 0.28-0.14i |
| 22 | -0.3+0.11i | -0.3-0.19i | -0.04+0.09i | 0.02-0.18i | -0.23+0.16i | 0.04+0.27i |
| 23 | -0.23-0.07i | 0.03+0.05i | -0.12-0.34i | -0.04+0.32i | -0.24+0i | -0.36+0.1i |
| 24 | -0.35+0.03i | -0.1-0.41i | 0.1-0.3i | 0.05+0.25i | 0.19-0.23i | 0.06+0.02i |
| 25 | 0.21-0.2i | -0.39+0.05i | -0.11-0.04i | -0.12-0.17i | -0.28+0.02i | 0.37-0.06i |
| 26 | -0.27+0.26i | 0.21+0.07i | 0.28-0.07i | 0.16+0.22i | 0.09+0.15i | -0.04-0.04i |
| 27 | -0.27-0.22i | 0.22-0.2i | 0.14+0.02i | 0.28-0.05i | 0.06-0.15i | -0.09+0.32i |
| 28 | -0.09-0.08i | 0.29+0.06i | -0.13+0.33i | 0.18-0.29i | 0.22-0.26i | 0.01+0.06i |
| 29 | -0.04-0.28i | -0.02+0.23i | -0.32+0.22i | -0.03+0.03i | -0.16-0.3i | 0.26+0.17i |
| 30 | -0.26-0.05i | 0.43-0.06i | -0.24+0.35i | -0.03+0.41i | -0.18+0.02i | 0.09-0.13i |
| 31 | -0.23-0.22i | -0.19+0.05i | 0.13-0.03i | 0.05+0.35i | -0.09+0.2i | 0.05+0.29i |
| 32 | -0.13+0.01i | 0.32-0.07i | -0.3+0.04i | -0.22-0.02i | -0.15+0.32i | -0.09+0.13i |
| 33 | -0.3+0.25i | -0.3-0.03i | -0.16-0.08i | 0.18+0.07i | 0.11+0.14i | 0.36-0.05i |
| 34 | 0.01+0.27i | -0.18-0.13i | -0.25+0.05i | 0.06+0.27i | 0.01+0i | 0.07+0.34i |
| 35 | -0.23+0.19i | -0.14+0.11i | -0.01-0.36i | 0.17-0.18i | 0.05+0.16i | -0.19+0.21i |
| 36 | 0.06-0.35i | -0.19+0.12i | 0.06+0.13i | 0.26-0.15i | -0.06+0.13i | 0.2+0.09i |
| 37 | 0.11-0.1i | 0.33-0.04i | 0.36+0.24i | 0.15+0.39i | -0.08-0.3i | 0.01+0.33i |
| 38 | -0.03-0.15i | 0.07-0.31i | -0.02-0.23i | 0.13+0.24i | -0.02+0.32i | 0.19-0.07i |
| 39 | 0.09-0.08i | -0.26-0.14i | -0.37+0.14i | 0.32-0.01i | -0.19-0.08i | 0.17-0.39i |
| 40 | 0.11+0.16i | 0.08+0.05i | -0.04+0.38i | -0.33+0.22i | -0.3+0.02i | 0.14+0.31i |
| 41 | -0.28+0.09i | 0.14-0.03i | 0.09+0.17i | -0.01+0.3i | 0.05+0.39i | -0.09+0.2i |
| 42 | -0.07-0.23i | 0.07-0.2i | -0.25+0.13i | 0.22+0.22i | -0.33+0.01i | 0.18+0.04i |
| 43 | 0.08+0.01i | 0.26-0.15i | 0.26+0.2i | 0.2-0.11i | -0.17-0.31i | 0.16+0i |
| 44 | -0.02-0.34i | 0.02+0.03i | 0.2+0.05i | 0.12+0.31i | 0.16+0.23i | 0.16+0.17i |
| 45 | -0.33-0.02i | -0.38+0.02i | -0.02+0.16i | 0.07+0.14i | 0.05-0.34i | -0.05-0.23i |
| 46 | 0.18+0.23i | -0.18+0.08i | 0.09+0.3i | 0.13+0.26i | -0.23+0.03i | -0.2+0.22i |
| 47 | -0.1+0.31i | 0.17+0.27i | 0.13-0.17i | -0.2+0.01i | -0.16-0.07i | 0.28+0.14i |
| 48 | 0.18-0.28i | -0.06+0.14i | -0.1-0.14i | 0.3-0.24i | 0.08-0.14i | 0.09+0.3i |
| 49 | 0.26-0.06i | -0.11+0.25i | 0.03+0.29i | 0.31-0.02i | -0.4-0.09i | 0.25-0.21i |
| 50 | 0.17-0.36i | 0.09+0.23i | -0.29+0.17i | 0.03+0.04i | 0.04+0.39i | 0.32+0.09i |
| 51 | 0.19-0.27i | -0.08+0.2i | -0.11+0.26i | 0.12-0.35i | -0.31-0.23i | -0.21+0.2i |
| 52 | -0.31-0.17i | -0.11+0.16i | 0.14-0.19i | 0.04-0.26i | 0.12-0.38i | 0.19+0.32i |
| 53 | 0.25+0.08i | 0.01+0.35i | -0.12-0.19i | 0.24+0.21i | -0.18+0.38i | -0.36+0i |
| 54 | 0.17-0.09i | -0.32+0.05i | -0.17-0.19i | 0.17-0.23i | 0.32+0.06i | 0.2-0.12i |
| 55 | 0.16-0.26i | -0.18-0.08i | 0.13+0.35i | -0.13+0.31i | -0.08+0.08i | -0.1-0.16i |
| 56 | -0.35-0.01i | 0.35+0.22i | -0.22-0.01i | 0.17+0.12i | 0.04+0.04i | 0.23+0.27i |
| 57 | 0.15-0.09i | 0.24-0.23i | -0.25-0.32i | 0.32+0.06i | -0.06+0.24i | 0-0.26i |
| 58 | 0.11+0.13i | 0.36-0.13i | -0+0.08i | 0.03+0.26i | -0.12-0.17i | 0.28-0.21i |
| 59 | -0.16+0.26i | 0.37+0.23i | 0-0.25i | 0.04+0.26i | -0.1+0.34i | -0.05+0.2i |
| 60 | -0.22+0.17i | -0.28-0.01i | -0.24-0.04i | 0.19+0.1i | -0.24+0.28i | -0.32-0.11i |
| 61 | 0.05-0.01i | 0.13-0.28i | -0.23-0.06i | 0.31-0.3i | -0.06-0.27i | -0.1+0.14i |
| 62 | -0.15-0.34i | 0.1+0.14i | 0.25+0.15i | 0.33-0.03i | 0.11-0.3i | -0.05-0.17i |
| 63 | 0.32-0.06i | 0.01+0.09i | -0.33-0.02i | 0.01-0.08i | -0.29-0.05i | 0.06-0.31i |
| n | S6,n | S7,n | S8,n | S9,n | S10,n | S11,n |
| 0 | 0.21+0.34i | -0.31-0.26i | 0.09-0.25i | -0.27-0.2i | 0.23+0.21i | 0.31-0.05i |
| 1 | 0.16+0.1i | 0.3+0.3i | 0.11+0.12i | 0.23+0.15i | -0.35+0.11i | 0.14-0.1i |
| 2 | 0.39-0.07i | -0.23+0.15i | 0.24-0.06i | -0.37-0.09i | -0.02+0.29i | 0.26-0.12i |
| 3 | 0.12-0.1i | 0.05+0.1i | 0.03+0i | 0.14-0.17i | -0.19+0.19i | -0.14-0.03i |
| 4 | 0.27+0.19i | 0.13-0.04i | -0.07+0.03i | -0.15-0.39i | 0.09+0.36i | -0.22+0.07i |
| 5 | 0.24-0.21i | 0.08-0.11i | -0.03+0.21i | 0.24+0.32i | -0.21-0.03i | -0.14-0.2i |
| 6 | 0.2+0.03i | 0.07-0.26i | 0.24+0.33i | 0.18-0.21i | 0.04+0.1i | 0.11+0.3i |
| 7 | -0.14+0.19i | -0.13+0.1i | -0.17-0.16i | 0.34+0.05i | 0.13-0.09i | 0.16+0.11i |
| 8 | 0+0.33i | -0.27+0.2i | -0.26-0.08i | -0.32+0.07i | 0.11-0.26i | -0.22+0.2i |
| 9 | -0.19+0.38i | -0.31-0.04i | 0.07-0.19i | 0.09+0.16i | -0.26+0.06i | 0.08+0.2i |
| 10 | 0.28+0.17i | -0.4+0.07i | 0.12-0.02i | 0.17+0.14i | 0.05+0.3i | -0.07+0.26i |
| 11 | -0.22+0.11i | -0.09-0.22i | -0.17-0.24i | -0.02+0.29i | 0.2+0.13i | -0.09-0.24i |
| 12 | -0.16+0.27i | -0.35+0.19i | 0.26-0.18i | 0.35-0.03i | 0.19-0.02i | 0.33+0.22i |
| 13 | 0.21+0.26i | -0.07-0.09i | -0.04+0.08i | -0.14+0.16i | -0.14+0.38i | -0.25-0.23i |
| 14 | 0.19-0.23i | -0.27-0.03i | -0.15-0.33i | -0.21+0.09i | -0.03-0.1i | -0.33-0.07i |
| 15 | -0.02+0.3i | 0.02-0.18i | 0.19+0.21i | -0.08+0.08i | -0.03+0.06i | 0.16+0.09i |
| 16 | -0.1-0.14i | -0.13-0.03i | -0.25+0.25i | 0.14-0.16i | 0.22+0.29i | -0.12+0.26i |
| 17 | 0.06-0.16i | -0.06+0.32i | 0.2+0.2i | 0.27-0.03i | -0.12+0.34i | -0.21+0.07i |
| 18 | -0.25+0.21i | 0.22+0.26i | -0.32-0.22i | 0.04-0.03i | 0.19+0.04i | -0.22-0.17i |
| 19 | -0.36-0.13i | 0.16+0.2i | 0.08-0.09i | -0.12+0.24i | 0.31-0.03i | 0.05+0.28i |
| 20 | -0.19+0.1i | -0.24-0.07i | 0.01-0.36i | 0.37+0.1i | 0.23+0.25i | -0.21+0.18i |
| 21 | 0.33-0.02i | -0.16+0.36i | 0.2+0.11i | -0.37-0.08i | -0.09+0i | -0.06+0.07i |
| 22 | 0.11-0.31i | -0.23-0.25i | 0.24+0.05i | -0.16+0.22i | -0.35-0.01i | 0.3+0.08i |
| 23 | 0.24-0.14i | 0.11+0.33i | -0.03+0.2i | 0.28+0.16i | 0.09+0.14i | 0.25+0.27i |
| 24 | 0.17-0.15i | -0.31+0i | 0.23-0.02i | 0.16-0.03i | 0.19+0.29i | 0.18-0.23i |
| 25 | 0.1-0.01i | 0.02+0.19i | 0.17-0.32i | 0.05+0.33i | 0.03+0.2i | -0.34+0.19i |
| 26 | -0.08+0.27i | -0.16+0.06i | -0.14+0.37i | 0.17-0.22i | -0.04+0.39i | -0.27+0.21i |
| 27 | -0.19+0.14i | -0.02+0.42i | -0.06-0.13i | 0.06-0.27i | -0.33+0.01i | -0.07+0.33i |
| 28 | 0.24+0.19i | -0.35-0.23i | -0.26+0.13i | 0.03-0.32i | -0.11+0.1i | -0.06+0.22i |
| 29 | 0.08-0.06i | 0.14+0.19i | -0.04+0.25i | 0.29-0.15i | -0.13-0.32i | 0.1+0.37i |
| 30 | 0.17-0.27i | -0.21-0.05i | -0.04+0.29i | -0.2+0.12i | 0.15-0.01i | 0.02-0.18i |
| 31 | -0.06-0.3i | -0.31-0.14i | -0.17-0.14i | 0.18+0.27i | 0.38-0.21i | 0.18+0.04i |
| 32 | 0.24+0.11i | -0.29-0.24i | -0.04+0i | 0.31+0.04i | 0.16-0.35i | -0.09+0.35i |
| 33 | -0.32+0.15i | 0.23+0.02i | 0.38+0.09i | -0.16-0.07i | 0.01+0.12i | -0.37+0.13i |
| 34 | -0.11-0.15i | -0.28+0.16i | -0.3+0.29i | -0.24+0.1i | 0.21+0.34i | 0.25+0.13i |
| 35 | 0.38-0.07i | 0.14-0.18i | 0.34-0.16i | 0.13+0.17i | -0.04+0.37i | -0.19+0.14i |
| 36 | 0.18+0.15i | 0.15-0.36i | 0.09-0.34i | 0.18+0.08i | -0.07+0.37i | 0.3-0.19i |
| 37 | 0.06+0.05i | -0.03+0.12i | 0.24+0.02i | 0.03-0.04i | 0.19-0.13i | 0.39+0.1i |
| 38 | -0.2-0.31i | -0.08-0.01i | 0.08-0.22i | 0.4+0.13i | -0.36+0.19i | 0+0.27i |
| 39 | -0.28-0.01i | -0.31-0.04i | 0.35+0.03i | -0.2-0.05i | -0.15-0.05i | 0.21-0.08i |
| 40 | 0.23+0.08i | 0.03+0.25i | -0.17-0.3i | -0.14+0.13i | -0.3+0.13i | 0.26-0.01i |
| 41 | -0.2+0.19i | 0.26+0.07i | 0.14-0.02i | -0.11+0.41i | 0.19+0.34i | 0.07+0.24i |
| 42 | 0.1-0.32i | 0.16-0.23i | -0.24-0.2i | 0.29-0.19i | 0.16-0.1i | -0.36-0.02i |
| 43 | 0.18+0.32i | 0.22+0.13i | 0.06+0.19i | 0.15+0.4i | 0.08+0.18i | -0.23+0.28i |
| 44 | 0.21-0.12i | -0.1+0.28i | -0.06+0.42i | -0.29+0.08i | 0.02-0.18i | -0.1+0.37i |
| 45 | 0.28+0.04i | -0.25-0.02i | -0.04-0.36i | 0.23+0.04i | -0.35-0.02i | 0.23+0.11i |
| 46 | 0.24+0.26i | -0.01+0.05i | 0.21-0.34i | 0.31+0.09i | 0.13-0.28i | -0.23+0.08i |
| 47 | 0.16+0.22i | -0.07+0.1i | -0.01-0.31i | -0.36-0.13i | 0.16+0.35i | -0.33+0.06i |
| 48 | 0.29+0.05i | 0.25+0.1i | 0.15+0.26i | 0.13-0.37i | 0.11+0.23i | -0.21-0.22i |
| 49 | 0.21-0.1i | -0.1-0.22i | -0.1+0.24i | -0.33-0.01i | 0.03+0.28i | -0.16+0.05i |
| 50 | -0.18-0.12i | -0.31-0.04i | 0.02+0.31i | 0.21-0.25i | -0.01+0.24i | -0.02-0.05i |
| 51 | -0.29-0.09i | -0.09-0.05i | 0.03-0.14i | 0.1-0.26i | 0.05+0.31i | 0.14+0.3i |
| 52 | 0.27+0.15i | -0.19+0.04i | 0.02-0.14i | -0.19+0.29i | 0.27-0.04i | 0.2+0.15i |
| 53 | -0.12+0.12i | -0.28+0.06i | -0.08-0.2i | -0.02-0.02i | 0.19+0.24i | 0.03+0.33i |
| 54 | 0.2+0.14i | -0.13-0.16i | 0+0.36i | 0.2+0.31i | -0.01+0.27i | 0.18+0.29i |
| 55 | 0.19-0.08i | -0.14+0.32i | 0.42+0.09i | 0.3+0.04i | -0.17+0.32i | -0.05+0.01i |
| 56 | 0.18+0.22i | 0.32+0.13i | 0.19-0.26i | -0.08+0.12i | -0.05-0.32i | -0.19+0.13i |
| 57 | 0.03+0.27i | -0.17+0.24i | 0.32+0.08i | 0.19+0.3i | 0.1+0.1i | -0.09-0.2i |
| 58 | 0.28+0.2i | -0.29+0.09i | 0.33-0.05i | 0.03-0.33i | -0.07-0.23i | -0.32-0.01i |
| 59 | 0.17+0.07i | -0.2-0.05i | -0.01-0.35i | 0.23-0.05i | -0.2+0.03i | 0.09-0.33i |
| 60 | 0.21-0.02i | -0.39-0.02i | 0.03+0.39i | 0.17-0.15i | -0.07-0.09i | -0.14-0.23i |
| 61 | 0.18-0.23i | -0-0.25i | -0.15+0.23i | 0.07+0.31i | 0.29+0.24i | 0.11-0.27i |
| 62 | -0.31+0.03i | -0.32+0.08i | -0.2+0.13i | 0.13+0.25i | 0.05+0.24i | 0.32-0.08i |
| 63 | -0.2+0.21i | 0.16+0.24i | -0.35+0.12i | 0.03+0.19i | -0.1+0.4i | 0.27+0.05i |

# UL Secondary Fast Feedback Channel (SFBCH)

The UL secondary fast feedback channel (SFBCH) defined by the Advanced Air Interface specifications carries 7 to 24 bits of narrowband channel quality feedback and MIMO feedback information. The SFBCH can be used to support CQI reporting at higher code rate and thus more CQI information bits.

# Multiplexing with other control channels and data channels

The SFBCH shall be multiplexed with other UL control and data channels within a UL subframe using FDM.

The SFBCH shall start at a pre-determined location, with the size defined in a DL control message (add TBD reference to appropriate DL control section). The secondary fast feedback channel can be allocated in a non-periodic manner based on traffic, channel conditions etc.

The SFBCH carries one or more types of fast feedback information.  The control information payload transmitted in the SFBCH has variable size depending on the reporting format of the feedback information. Each SFBCH can support feedback payload information size of up to 24bits. In addition, the code rate can be adapted for different users based on channel conditions.

Multiple SFBCH allocations for one or more users shall be multiplexed using FDM.

# PHY Structure

The UL FMT used for the SFBCH shall consist of a time-frequency block of 2 subcarriers by 6 OFDM symbols and has 2 pilots in fixed locations as shown in Figure 4. The SFBCH shall be constructed from 3 UL FMTs, which are chosen from different UL DRUs as shown in Figure 5. The procedure for allocation of resources for transmission of UL control information and the formation of DRUs for such transmission is TBD.

The pilot sequence p0p1=[1,ejθ] shall be used in each FMT for each user.  is a sector specific phase rotation that is configured to mitigate inter cell interference. The value of  for the three FMTs may be different and the detailed design of  is TBD. The per pilot tone power is x-dB above the per data tone power for each transmit antenna and x is TBD.

Figure 4: UL FMT structure for the SFBCH

Figure 5: SFBCH comprising of three distributed 2x6 UL FMTs

The process of mapping feedback information to the PHY structure in the SFBCH is shown in Figure 6 . First, the SFBCH payload information bits a0a1a2⋯al−1 shall be encoded to 60 bits b0b1b2⋯b59using the linear block codes described in Section 15.x.2.1.2.2.1. When 6<l≤12, information bits a0a1a2⋯al−1 are encoded using the linear block code (60,l). When 12<l≤24, information bits a0a1a2⋯al−1are split into 2 parts: Part A consisting of a0a1a2⋯al2−1, and Part B consisting of al2al2+1al2+2⋯al−1. Part A shall be encoded to 30 bits b0b1b2⋯b29using linear block code (30,l2) and Part B shall be encoded to 30 bits b30b31b32⋯b59 using a linear block code (30,l−l2). The coded sequence b0b1b2⋯b59shall then be modulated to 30 symbols c0c1c2⋯c29using QPSK as described in Section TBD. Here, ci(i=0,1,⋯29)is formed by mapping coded bits b0b1b2⋯b29onto the in-phase component and code bits b30b31b32⋯b59 onto the quadrature component. The modulated symbols c0c1c2⋯c29 and pilot sequence p0p1are then mapped to the data subcarriers of the SFBCH FMTs as shown in Figure 5.

Figure 6: Mapping of information in the SFBCH

For each transmit antenna, power on the pilot subcarriers is boosted by 3dB over that of data subcarriers.

# Channel Coding in the SFBCH

The information bits in the SFBCH shall be  encoded using linear block codes. The codeword lengthshall not be larger than 60. The number of information bits Kcan vary from 6 to 12 bits, denoted by  where . The codeword shall be obtained as a linear combination of the K basis sequences denoted as where i=0,⋯,K−1 in Table 4. These linear block codes shall also used for bandwidth request channel (BRCH) described in Section 15.x.2.5.

Table 4: Basis sequences for (N,K) code

| N | S0,n | S1,n | S2,n | S3,n | S4,n | S5,n | S6,n | S7,n | S8,n | S9,n | S10,n | S11,n |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 2 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| 3 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 4 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| 5 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| 6 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 7 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 9 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 12 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 13 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 14 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 15 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| 16 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 17 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 18 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 19 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 20 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |
| 21 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 22 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 23 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 24 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 25 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 26 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 27 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 28 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 29 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 30 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 31 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 32 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 33 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 34 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 35 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 36 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 |
| 37 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |
| 38 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 39 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 40 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 41 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 42 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 43 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |
| 44 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |
| 45 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 46 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 47 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 48 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 49 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 50 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 51 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 52 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 53 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 54 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 55 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 56 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 57 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 58 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| N−1 | parity check bit |  |  |  |  |  |  |  |  |  |  |  |

If  denotes a codeword with length ofN, any component of the codeword shall be generated as: bn=i=0K−1ai⋅Si,nmod2, where n=0,1,2,⋯N−2. After the N−1components are generated, a parity check bit is appended to the codeword. The parity check bit is set to 1 when the number‘1’ in the codeword appears an odd number of times, otherwise it is set to ‘0’.

# UL HARQ Feedback Channel

This channel shall be used used to carry HARQ feedback information.

# Multiplexing with other control channels and data channels

A UL HARQ feedback channel in the Advanced Air Interface shall start at a pre-determined offset with respect to the corresponding DL transmission.

The UL HARQ feedback channel shall be multiplexed with other control and data channels using FDM. Multiple HARQ feedback channels may be allocated per subframe using FDM.

TDM/FDM shall be used to multiplex multiple HARQ feedback channels.

# PHY structure

The UL HARQ feedback channel in the Advanced Air Interface specifications shall consist of three distributed UL feedback mini-tiles (FMT), where the UL FMT is defined as 2 contiguous subcarriers by 6 OFDM symbols. The procedure for allocation of resources for transmission of UL control information and the formation of DRUs for such transmission is TBD.

Figure 7: TDM/FDM design for HARQ feedback on the UL

The HARQ feedback channel resource of 3 distributed FMTs shall be further divided into 18 units, each having a structure of 1 subcarrier ×2 OFDM symbols. Each HARQ feedback channel is allocated 3 units, identified by similar patterns in the structures shown in Figure 7. Hopping may also be used to randomize inter-cell interference. The details of the hopping pattern are TBD.

The logical HARQ channel index shall be configured by the MAC layer. The mapping between logical HARQ channel index and physical HARQ channel index is TBD.

Sequences [1,] or [1,] shall be used to transmit an ACK or NACK respectively for each HARQ feedback channel. is a sector specific phase rotation that shall be configured to avoid inter cell interference. The value of  for the three FMTs may be different and the detailed design of  is TBD. A single allocation of 3 distributed FMTs to HARQ feedback can therefore support 6 HARQ feedback channels.

Since each HARQ channel transmission is on only 1 subcarrier in each OFDM symbol, the power of each subcarrier can be boosted by 7.7dB compared to the design in which 6 subcarriers are used in each OFDM symbol.

# UL Sounding Channel

The UL sounding channel is used by an aMS to acquire UL channel information at the aBS. The sounding sequence shall occupy specific UL sub-bands or the whole UL frequency partition.

# Multiplexing with control information and data symbols

The aBS can configure an aMS to transmit an UL sounding signal on specific UL sub-bands or across the whole UL frequency partition. The periodicity of the sounding signal for each aMS shall be configured by the aBS.

The subcarriers allocated for UL sounding shall be FDM and/or TDM with subcarriers used to transmit control information and data symbols.

# Multiplexing sounding sequences for multiple aMSs

The aBS can configure multiple aMSs to transmit UL sounding signal. The subcarriers allocated for UL sounding from the aMS can be CDM, FDM, or TDM with the UL sounding subcarriers used by other aMSs.

The aBS can configure the aMS to perform UL sounding from multiple antennas. The subcarriers allocated for UL sounding from one antenna of the aMS can be CDM, FDM, or TDM with the UL sounding subcarriers used by other antennas.

# UL Sounding PHY structure

Sounding sequence design and mapping to subcarriers is TBD.

# Ranging Channel

The UL ranging channel shall be used for UL synchronization. The UL ranging channel shall be further classified into ranging channel for non-synchronized mobile stations and synchronized mobiles stations. A random access procedure, which can be contention based or non-contention based shall be used for ranging. Contention-based random access shall be used for initial ranging, periodic ranging and handover. Non-contention based random access shall be used for periodic ranging and handover.

# Ranging Channel for Non-Synchronized Mobile Stations

The ranging channel for non-synchronized aMSs shall be used for initial access and handover.

# Multiplexing with other control channels and data channels

The UL ranging channel for non-synchronized aMSs shall start at a configurable location with the configuration defined in a DL broadcast control message.

The UL ranging channel for non-synchronized aMSs shall be FDM with other UL control channels and data channels.

# PHY structure

The physical ranging channel for non-synchronized mobile stations shall consist of three parts: 1) ranging cyclic prefix (RCP), 2) ranging preamble (RP) and 3) guard time (GT). The length of RCP shall not be shorter than the sum of the maximum channel delay spread and round trip delay (RTD) of supported cell size. The length of GT shall not be also shorter than the RTD of supported cell size. The length of ranging preamble shall be equal to or longer than RCP length of ranging channel. The details on the length of each part and its configurations are FFS. To support large cell sizes, the ranging channel for non-synchronized aMSs can span multiple concatenated subframes.

The physical resource of ranging channel for non-synchronized mobile stations shall be Nrsc consecutive ranging subcarriers (BWRCH-NS Hz corresponding to continuous Nrru CRUs) and Nrsym OFDMA symbols (TRCH-NS sec). As a default configuration, Nrsc and Nrsym are equal to [TBD] ranging subcarriers and Nsym OFDMA symbols, respectively, where Nsym depends on the subframe type as described in section 11.6.

Figure 8 shows the default ranging channel structure spanning one subframe. The ranging preamble shall be repeated as a single opportunity. A single preamble can be used by different non-synchronized aMSs for increasing ranging opportunities. When the preamble is repeated as a single opportunity, the second RCP can be omitted for coverage extension. The guard subcarriers shall be reserved at the edge of non-synchronized ranging channel(s) physical resource. CDM allows multiple aMSs to share the same ranging channel. The details of the ranging structure within the localized resource are FFS. In the TDD mode, the GT can be omitted for extending the length of RCP.

Figure 8: The default ranging structure for non-synchronized aMSs

Support for multi-antenna transmission is FFS. When multi-antenna transmission is supported by an aMS, it can be used to increase the ranging opportunity by spatial orthogonality.

# Ranging Channel for synchronized mobile stations

The ranging channel for synchronized aMSs shall be used for periodic ranging. The use of the ranging channel for synchronized aMSs for handover is FFS.

# Multiplexing with other control channels and data channels

The UL ranging channel for synchronized aMSs shall start at a configurable location with the configuration defined in a DL broadcast control message.

The UL ranging channel for synchronized aMSs shall be FDM with other UL control channels and data channels.

# PHY structure

The ranging sequence design and mapping to subcarriers are TBD.

# Bandwidth Request Channel (BRCH)

Contention based or non-contention based random access shall be used to transmit bandwidth request information on this control channel. Prioritized bandwidth requests shall be supported on the bandwidth request channel. The mechanism for such prioritization is TBD.

The random access based bandwidth request procedure is described in Figure 9. A 5-step regular procedure (step 1 to 5) or an optional 3-step quick access procedure (step 1,4 and 5) may be supported concurrently. Step 2 and 3 shall be used only in 5-step regular procedure. In step 1, the aMS shall send a bandwidth request indicator and a message for quick access that may indicate information such as aMS addressing and/or request size (FFS) and/or uplink transmit power report (FFS), and/or QoS identifiers (FFS), and the aBS may allocate uplink grant based on certain policy. The 5-step regular procedure shall be used independently or as a fallback mode for the 3-step bandwidth request quick access procedure. The aMS may piggyback additional BW REQ information along with user data during uplink transmission (step 5). In step 2 and step 4, the aBS may send message to acknowledge the reception status.

Figure 9: Bandwidth Request Procedure

# Multiplexing with other control channels and data channels

A bandwidth request channel in the Advanced Air Interface specifications shall start at a configurable location with the configuration defined in a DL broadcast control message. The bandwidth request channel is FDM with other UL control and data channels. Multiple BW REQ channels may be allocated per subframe using FDM.

# PHY structure

A BW REQ tile in the Advanced Air Interface specifications is defined as 6 contiguous subcarriers by 6 OFDM symbols. As illustrated in Figure 9, a BW REQ access sequence (BW REQ preamble) shall be transmitted in step-1 of the bandwidth request procedure. As shown in Figure 10, the BW REQ preamble shall be transmitted on a resource that spans 4 subcarriers by 6 OFDM symbols. Additionally, a quick access BW REQ message shall be carried in the data portion of the tile that spans 2 contiguous subcarriers by 6 OFDM symbols. Each BW REQ channel shall comprise of 3 distributed BW REQ tiles for frequency diversity. The procedure for allocation of resources for transmission of UL control information and the formation of DRUs for such transmission is TBD.

Figure 10: 6x6 BW REQ Tile Structure in the Advance Air Interface

Let  denote a total of 16 bits of information which can be carried in the quick access message. The first 4 bits,  shall be carried in the BW REQ preamble using the preamble index. The combined resource in the data portions of the three tiles that form the BRCH shall be used to transmit the remaining 12 bits of information, . Figure 11 illustrates the construction of the BW REQ preamble and quick access message for a 6x6 tile structure.

Figure 11: Bandwidth Request Channel structure with 6x6 tiles

The logical preamble sequence index, BWREQ_PREAMBLE_INDEX, which is configured by the upper layer and 4 bits of the bandwidth request message shall be used to select three sequences of length 24 from Table 5 in order to construct 72 preamble symbols. As shown in Figure 11 , the 72 preamble symbols shall be scrambled before multiplexing with data symbols in the BRCH.

Table 5: BRCH Preamble sequences

| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 |
| 2 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 |
| 3 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 |
| 4 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 |
| 5 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 |
| 6 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 |
| 7 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 |
| 8 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| 9 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 |
| 10 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 |
| 11 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 |
| 12 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 13 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 |
| 14 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 |
| 15 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 |
| 16 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 |
| 17 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 |
| 18 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 |
| 19 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 |
| 20 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 21 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 |
| 22 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 |
| 23 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 |

The preamble sequences transmitted in the three BW REQ tiles of a BRCH are defined as

where  is BRCH channel index,  is symbol index in the preamble portion of a BW REQ tile,  is BW REQ tile index,  is sequence index in BW REQ tile .

The mapping between the combination of the logical preamble sequence index, BWREQ_PREAMBLE_INDEX and the 4 bits of the bandwidth request message  to the physical preamble index,  is TBD.

The remaining 12 bits of information  in the quick access message transmitted in the BRCH shall be encoded into 36 bits b0,b1,b2,⋯b35using the linear block code (36, 12) described in section 15.x.2.1.2.2.1. The 36 coded bits shall then be BPSK modulated as described in Section TBD and scrambled to generate 36 data symbols, .  The combined data portions of the three distributed BW REQ tiles that form the BRCH shall be used to transmit these data symbols. The data symbols shall be scaled by  before multiplexing with preamble symbols and the scaling factor  is configured by the MAC layer. The mapping of the 36 data symbols to the data portions of three distributed 6×6 BW REQ tiles in a BRCH is given in Table 6. The final 108 modulated BRCH symbols shall be scaled by a factor of  to normalize the total transmitted energy.

Table 6: Mapping of data symbols to subcarriers in the data portion of the BW REQ Tile

| BW REQ tile index |  |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |

In order to support operation in the legacy mode, a BW REQ tile shall be defined as 4 contiguous subcarriers by 6 OFDM symbols. As shown in Figure 12, only the BW REQ access sequence or BW REQ preamble shall be transmitted in all 24 subcarriers that form the BW REQ tile. In this case, the BWREQ_PREAMBLE_INDEX shall be randomly selected from all available logical preamble sequences, as shown in Figure 12.

Figure 12: 4x6 BW REQ tile structure

Figure 13: 4x6 Bandwidth Request Channel structure with 4x6 tiles

# UL Inband Control Signaling

Uplink control information can be multiplexed with data on the UL data channels as MAC headers or MAC management messages. Inband control signaling can contain information such as uplink bandwidth requests or bandwidth assignment updates.

# Mapping of UL control information to UL control channels

| Information | Channel |
| --- | --- |
| Channel quality feedback | UL Fast Feedback Channel |
|  | UL Sounding Channel |
| MIMO feedback | UL Fast Feedback Channel |
|  | UL Sounding Channel |
| HARQ feedback | UL HARQ Feedback Channel |
| Synchronization | UL Ranging Channel |
| Bandwidth request | Bandwidth Request Channel |
|  | UL Inband Control Signaling |
|  | UL Fast Feedback Channel*(FFS) |
| E-MaBS feedback | FFS |

* Transmission of BW REQ indicators on the UL Fast Feedback Channel is FFS