| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Channel Coding and HARQ Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Chongli Liu, Rongdao Yu, Tao Wu, Ying Jin, Qitao Song, Jung Woon Lee Huawei | Voice: 86-755-28970182 E-mail: liuchongli@huawei.com |
| Re: | IEEE 802.16m-08/53r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document”. Target topic: “Channel Coding and HARQ” |  |
| Abstract | The contribution proposes the text of Channel Coding and HARQ section to be included in the 802.16m amendment. |  |
| Purpose | For discussion and approval by TGm. for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Channel Coding and HARQ
for the IEEE 802.16m Amendment

Chongli Liu, Rongdao Yu, Tao Wu, Ying Jin, Qitao Song, Jung Woon Lee

Huawei

# Introduction

The contribution proposes the text of Channel Coding and HARQ section to be included in the 802.16m amendment. It’s based on the sub-clause 11.13 in the IEEE 802.16m SDD [1]

# Reference

[1] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[2] IEEE C802.16m-08/1237, “Style guide for writing the IEEE 802.16m amendment”

[3] IEEE 802.16m-08/050 “IEEE 802.16m Amendment Working Document”

# Text Proposal for 802.16m amendment

------------------------------------Start text proposal------------------------------------

- Abbreviations and acronyms

[Insert the following at the end of section 4]

CRC     cyclic redundancy check

CTC     convolutional turbo code

FEC     forward error correction

HARQ   hybrid automatic repeat request

MAC    media access control

PDU     protocol data unit

PHY     physical

QAM    quadrature amplitude modulation

QPSK    quadrature phase-shift keying

[Insert a new sub-clause into section 15 ]

# Advanced Air Interface

# Physical Layer

# Channel coding and HARQ

# Channel coding

# Burst partition

# FEC block CRC encoding

# FEC encoding

# Convolutional turbo codes

The CTC encoder, including its constituent encoder, is depicted in Figure 1. It uses a double binary Circular Recursive Systematic Convolutional code. The bits of the data to be encoded are alternately fed to A and B, starting with the MSB of the first byte being fed to A. The encoder is fed by blocks of k bits or N couples (k=2×Nbits).For the entire frame sizes, k is a multiple of 8 and N is a multiple of 4.

The polynomials defining the connections are described in octal and symbol notations as follows:

- For the feedback branch :0xB, equivalently 1+D+D3(in symbolic notation)

- For the Y parity bit: 0xD,equivalently 1+D2+D3

- For the W parity bit: 0x9,equivalently 1+D3

Figure 1 - CTC encoder

First, the encoder (after initialization by the circulation stateSc1) is fed the sequence in the natural order (position 1) with the incremental address i=0,1,2,…,N−1.The first encoding is called C1 encoding. Then the encoder (after initialization by the circulation stateSc2) is fed by the interleaved sequence (switch in position 2) with incremental address j=0,1,2,…,N−1.This second encoding is called C2 encoding.

The order in which the encoded bits shall be fed into the subblock interleaving block is

A,B,Y1,Y2,W1,W2=A0,A1,⋯,AN−1,B0,B1,⋯,BN−1,Y1,0,Y1,1,⋯,Y1,N−1,Y2,0,Y2,1,⋯,Y2,N−1,W1,0,W1,1,⋯,W1,N−1,W2,0,W2,1,⋯,W2,N−1

The inner interleaver of CTC requires the parametersP0,P1,P2and P3shown in Table 1.

Table 1 - CTC interleaver parameters

| i | K | N | P0 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 48 | 24 | 5 | 0 | 0 | 0 |
| 2 | 72 | 36 | 11 | 18 | 0 | 18 |
| 3 | 96 | 48 | 13 | 24 | 0 | 24 |
| 4 | 120 | 60 | 7 | 30 | 28 | 30 |
| 5 | 144 | 72 | 11 | 6 | 0 | 6 |
| 6 | 192 | 96 | 7 | 48 | 24 | 72 |
| 7 | 216 | 108 | 11 | 54 | 56 | 2 |
| 8 | 240 | 120 | 13 | 60 | 0 | 60 |
| 9 | 264 | 132 | 5 | 6 | 24 | 6 |
| 10 | 288 | 144 | 17 | 74 | 72 | 2 |
| 11 | 312 | 156 | 5 | 2 | 44 | 54 |
| 12 | 360 | 180 | 11 | 48 | 48 | 4 |
| 13 | 384 | 192 | 5 | 4 | 20 | 44 |
| 14 | 408 | 204 | 11 | 12 | 44 | 8 |
| 15 | 432 | 216 | 7 | 54 | 32 | 26 |
| 16 | 456 | 228 | 13 | 20 | 44 | 8 |
| 17 | 480 | 240 | 19 | 34 | 24 | 94 |
| 18 | 528 | 264 | 5 | 2 | 36 | 82 |
| 19 | 576 | 288 | 11 | 34 | 4 | 78 |
| 20 | 624 | 312 | 11 | 46 | 52 | 6 |
| 21 | 720 | 360 | 13 | 2 | 12 | 58 |
| 22 | 768 | 384 | 11 | 84 | 84 | 80 |
| 23 | 816 | 408 | 5 | 6 | 88 | 62 |
| 24 | 864 | 432 | 19 | 84 | 56 | 72 |
| 25 | 912 | 456 | 37 | 8 | 24 | 16 |
| 26 | 960 | 480 | 53 | 62 | 12 | 2 |
| 27 | 1056 | 528 | 43 | 66 | 4 | 90 |
| 28 | 1152 | 576 | 29 | 60 | 16 | 24 |
| 29 | 1248 | 624 | 31 | 62 | 8 | 22 |
| 30 | 1440 | 720 | 37 | 86 | 32 | 42 |
| 31 | 1536 | 768 | 47 | 38 | 28 | 98 |
| 32 | 1632 | 816 | 43 | 86 | 12 | 58 |
| 33 | 1728 | 864 | 53 | 18 | 20 | 90 |
| 34 | 1824 | 912 | 53 | 4 | 24 | 60 |
| 35 | 1920 | 960 | 49 | 80 | 68 | 40 |
| 36 | 2112 | 1056 | 47 | 34 | 56 | 62 |
| 37 | 2208 | 1104 | 67 | 2 | 32 | 10 |
| 38 | 2304 | 1152 | 67 | 22 | 96 | 10 |
| 39 | 2400 | 1200 | 59 | 22 | 32 | 26 |
| 40 | 2496 | 1248 | 53 | 60 | 64 | 92 |
| 41 | 2592 | 1296 | 47 | 96 | 28 | 40 |
| 42 | 2784 | 1392 | 59 | 20 | 16 | 96 |
| 43 | 2880 | 1440 | 47 | 20 | 60 | 60 |
| 44 | 2976 | 1488 | 47 | 72 | 56 | 96 |
| 45 | 3072 | 1536 | 59 | 28 | 72 | 44 |
| 46 | 3168 | 1584 | 61 | 52 | 80 | 32 |
| 47 | 3264 | 1632 | 37 | 66 | 12 | 94 |
| 48 | 3456 | 1728 | 53 | 22 | 28 | 94 |
| 49 | 3552 | 1776 | 53 | 20 | 60 | 64 |
| 50 | 3648 | 1824 | 59 | 46 | 52 | 82 |
| 51 | 3744 | 1872 | 31 | 36 | 96 | 24 |
| 52 | 3840 | 1920 | 71 | 88 | 8 | 20 |
| 53 | 3936 | 1968 | 67 | 6 | 88 | 54 |
| 54 | 4128 | 2064 | 67 | 14 | 8 | 58 |
| 55 | 4224 | 2112 | 59 | 16 | 8 | 40 |
| 56 | 4320 | 2160 | 37 | 54 | 24 | 66 |
| 57 | 4416 | 2208 | 37 | 54 | 4 | 82 |
| 58 | 4512 | 2256 | 53 | 68 | 44 | 4 |
| 59 | 4608 | 2304 | 31 | 68 | 4 | 88 |
| 60 | 4800 | 2400 | 67 | 98 | 80 | 90 |

The two-step interleaver shall be performed as follows:

Step 1: Switch alternate couples

Let the sequence u0=[(A0,B0),(A1,B1),(A2,B2)(A3,B3),…,(AN−1,BN−1)]be the input to first encoding C1.

For i=0,1,2,…,N−1

if (i mod 2),let (Ai,Bi)→(Bi,Ai)(i.e., switch the couple )

This step gives a sequence u1=[(A0,B0),(B1,A1),(A2,B2),(B3,A3),…,(BN−1,AN−1)]

Step 2:P(j)

The function P(j) provides the address of the couple of the sequence u1that shall be mapped onto the address j of the interleaved sequence (i.e.u2(j)=u1(P(j))).

For j=0,1,2,…,N−1

switch (j mod 4)

case 0: P(j)=(P0⋅j+1)modN

case 1:  P(j)=(P0⋅j+1+N2+P1)modN

case 2: P(j)=(P0⋅j+1+P2)modN

case 3: P(j)=(P0⋅j+1+N2+P3)modN

This step gives a sequence u2=u1(P(0)),u1(P(1)),u1(P(2)),u1(P(3)),…,u1(P(N−1))

=[(BP(0),AP(0)),(AP(1),BP(1)),(BP(2),AP(2)),(AP(3),BP(3)),…,(AP(N−1),BP(N−1)).Sequence u2is the input to the second encoding C2.

The state of the encoder is denoted S (0≤S≤7) with S=4s1+2s2+s3.The circulation states Sc1and Sc2are determined by the following operations:

- Initialize the encoder with state 0.Encode the sequence in the natural order for the determination of Sc1or in the interleaved order for determination of Sc2.In both cases the final state of the encoder is S0N−1;

- According to the length N of the sequence, use Table 2 to find Sc1orSc2.

Table 2 - Circulation state lookup table

| Nmod7 | S0N−1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 0 | 6 | 4 | 2 | 7 | 1 | 3 | 5 |
| 2 | 0 | 3 | 7 | 4 | 5 | 6 | 2 | 1 |
| 3 | 0 | 5 | 3 | 6 | 2 | 7 | 1 | 4 |
| 4 | 0 | 4 | 1 | 5 | 6 | 2 | 7 | 3 |
| 5 | 0 | 2 | 5 | 7 | 1 | 3 | 4 | 6 |
| 6 | 0 | 7 | 6 | 1 | 3 | 4 | 5 | 2 |

# 15.3.X.1.3.1.2 Subblock interleaving

Block diagram of the interleaving scheme for CTC is shown in Figure 2. First, all of the encoded symbols shall be demultiplexed into six subblocks denoted A, B, Y1, Y2, W1 and W2. The encoder output symbols shall be sequentially distributed into six subblocks with the first N encoder output symbols going to the A subblock, the second N encoder output going to the B subblock, the third N to the Y1 subblock, the fourth N to the Y2 subblock, the fifth N to the W1 subblock, the sixth N to the W2 subblock.

Figure 2 - Block diagram of interleaving scheme

The six subblocks shall be interleaved separately. The interleaving is performed by the unit of symbol. The sequence of interleaver output symbols for each subblock shall be generated by the procedure described below. The entire subblock of symbols to be interleaved is written into an array at addresses from 0 to the number of the symbols minus one (N–1), and the interleaved symbols are read out in a permuted order with the i-th symbol being read from an address, ADi ( i = 0... N–1), as follows:

- Determine the subblock interleaver parameters, m and J. Table 3 gives these parameters.

- Initialize i and k to 0.

- Form a tentative output address Tk according to the following formula:

Tk =2m(k mod J) + BROm(k/J )

where BROm(y) indicates the bit-reversed m-bit value of y (i.e., BRO3(6) = 3).

- If Tk is less than N, ADi = Tk and increment i and k by 1. Otherwise, discard Tk and increment k only.

- Repeat steps c) and d) until all N interleaver output addresses are obtained.

[Editor’s Note: The parameters for the subblock interleavers are FFS. ]

Table 3 - Parameters for the subblock interleavers

| Block size (bits) NEP | N | Subblock interleaver parameters |  |
| --- | --- | --- | --- |
|  |  | m | J |

After interleaving, every C bits of the interleaved subblock are further permuted to prevent the contiguous bits from to be allocated into same level in constellation bits at 64QAM.  Each interleaved subblock is formed into R groups, where each group has C elements, where C is set to the modulation order (QPSK=2, 16QAM = 4, 64QAM = 6) and R= N/C .  Bits in each group of six interleaved subblocks are further permuted as followings;

Ai(j)=(j+(imodC))modC

Bi(j)=(j+((i+1)modC))modC

Y1,i(j)=(j+(imodC))modC

W1,i(j)=(j+((i+1)modC))modC

Y2,i(j)=(j+((i+2)modC))modC

W2,i(j)=(j+((i+3)modC))modC

Where Ai(j), Bi (j), Y1, i (j), W1, i (j), Y2, i (j), and W2, i (j), indicate the permutation formula for j -th element of i-th group of sub-block A, B, Y1, W1, Y2 and W2, respectively.  It’s noted that different blocks may have different permutation formulas to avoid the parity bits of each systematic bit being allocated into same constellation layer as the corresponding systematic bit.

The channel interleaver output sequence shall consist of the interleaved A and B subblock sequence, followed by a symbol-by-symbol multiplexed sequence of the interleaved Y1 and Y2 subblock sequences, followed by a symbol-by-symbol multiplexed sequence of the interleaved W1 and W2 subblock sequences. The symbol-by-symbol multiplexed sequence of interleaved Y1 and Y2 subblock sequences shall consist of the first output bit from the Y1 subblock interleaver, the first output bit from the Y2 subblock interleaver, the second output bit from the Y1 subblock interleaver, the second output bit from the Y2 subblock interleaver, etc. The symbol-by-symbol multiplexed sequence of interleaved W1 and W2 subblock sequences shall consist of the first output bit from the W1 subblock interleaver, the first output bit from the W2 subblock interleaver, the second output bit from the W1 subblock interleaver, the second output bit from the W2 subblock interleaver, etc. Figure 2 shows the interleaving scheme.

# Bit selection and repetition

# Encoded blocks concatenation

# Modulation

The data bits are entered serially to the constellation mapping. Gray-mapped QPSK、16-QAM and 64-QAM (as shown in Figure 3) shall be supported. The constellations (as shown in Figure 3) shall be normalized by multiplying the constellation point with the indicated factor c to achieve equal average power.

Each M interleaved bits (M = 2, 4, 6) shall be mapped to the constellation bits b(M – 1) – b0 in MSB-first order (i.e., the first bit shall be mapped to the higher index bit in the constellation). The constellation-mapped data shall be subsequently modulated onto the allocated data subcarriers.

Figure 3 - QPSK, 16-QAM, and 64-QAM constellations

------------------------------------End text proposal------------------------------------