| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Structured LDPC coding for IEEE 802.16m |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Jin Xu, Bo Sun ZTE Corporation | E-mail: xu.jin7@zte.com.cn, sun.bo1@zte.com.cn, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/053r1 –Call for Comments and Contributions on Project 802.16m Amendment Working Document; Channel Coding and HARQ |  |
| Abstract | This proposal gives a description about structured LDPC coding. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Structured LDPC coding for IEEE 802.16m

Jin Xu, Bo Sun

ZTE Corporation

# 1 Introduction

As defined in IEEE 802.16m SDD_r6 [1], LDPC is FFS.

This proposal is to suggest structured LDPC codes, which can not only provide flexible code rate and code size, but also support IR-HARQ. Our scheme includes matrix generation and encoding and rate matching (RM).

# 2 Discussion on LDPC Coding

### 2.1 Matrix definition and description of structured LDPC codes

The Parity check matrix of structured LDPC Codes is defined by a matrix H of size(mb×z)×(nb×z), which consists of blocks of circularly shifted identity matrices or zero matrices of sizez×z, of the form as following:

H=Ph00bPh01bPh02b⋯Ph0nbbPh10bPh11bPh12b⋯Ph1nbb⋯⋯⋯⋯⋯Phmb0bPhmb1bPhmb2b⋯Phmbnbb=PHb

If integerhijb=−1, definePhijb=0.

If integer, definePhijb=(P)hijband P is a z×zstandard permutation matrix of the form:

P=010⋯0001⋯0⋯⋯⋯⋯⋯000⋯1100⋯0

The size of H isM×N, hereN=nb×z,M=mb×z. And Hb of size mb×nb has the form:

For example, a matrix H is shown as following:

Actually a base matrix Hb and an expand factor z together can be used to represent H.

z = 3   and   Hb＝

Due to the structured characteristic of parity check matrix, the parity check matrix of LDPC codes can be fully described by small set parameters, which results in very low complexity implementations. Actually we only need base matrixHb, instead of parity check matrix H, to perform encoding and decoding. The encoding and decoding algorithms become the matrix calculation of sizemb×nb, the basic structure of encoder and decoder only depends on the positions of the non-negative-one elements in base matrix.

### Mother Code Set of structured LDPC codes to support single code rate and different code sizes

To generate a LDPC code set of a certain code rate and various code sizes, a uniform base matrix is defined. In order to obtain the base matrix of certain code size, the uniform base matrix has to be modified to generate a modified base matrix, which will really be used in the encoder/decoder of the LDPC code of certain code size. That is to say, for the LDPC codes of different code sizes and the same code rate, the positions of non-negative-one elements of their base matrices is the same, and the values of non-negative-one elements of their base matrices need to be changed.

Here is an example of rate 1/2 LDPC codes with different code sizes. A uniform base matrix constructed by us is used to describe the LDPC code set. The code rate of the designed LDPC codes is always 1/2, and the designed information block sizes increase from 192 to 10240, and the increasing step is 16, That is to say, K=192,192+16,192+2*16,192+3*16,…, 10240. The size of base matrix is 16×32，namely, mb=16and. So expand factors z =12:1:640. One uniform base matrix of rate 1/2 LDPC code set is defined as following:

To obtain the LDPC code of certain code size, the uniform base matrix above has to be modified and then can be regarded as the base matrix, which together with expand factor z can be used to define the parity check matrix H of the LDPC code.

Base Matrix Modification Rule is defined as following:

For each non-negative-one elements of the uniform base matrix above, the value hijbshould be modified. Let （hijb）modifiedrepresents the i-th row, j-th column element of modified base matrix, (hijb)uniform represents the i-th row, j-th column element of the uniform base matrixgiven by us. Then

zmaxis the largest expand factor, here equals to 640, and z is the currently used expand factor uniquely corresponding to the currently used code size.  denotes the operation that rounds the elements in it to the nearest integers towards minus infinity.

The characteristic of our matrix design includes:

1   For all code sizes, the best degree distribution has been selected.

2   For all code sizes, suitable for shortening and puncturing.

3   For all rows of parity check matrix, row weight is always 8, which decides the complexity of decoder.

Here are two examples of modified base matrix created by single uniform base matrix .

Information block size K= 288

Information block size K=5120

### Basematrix Generation for  LDPC Encode Process

The maximum number of soft channel bits available in the virtual buffer is, which is signaled by higher layers of each HARQ transmission. is the maximum possible codeword size for the encode of K information bits in the scenario of full Incremental Redundancy.

Here, and let .

Let. There are three steps to generate the needed base matrix. If ，Step 1 is needed; Otherwise, Step 1 is neglect, and and .

- Step 1: Compute, and then  rows and  columns shall be added to the last row and the last column of  to generate an extended base matrix of size, here.  is depicted by Figure 1 and is defined as follows:

Figure 1 Extended LDPC base matrix

for k = 1 to k =,

, , , ,

,, others in the k-th row are -1.

end for

Here, k indicates the row index for the added  rows, and denotes the i-th row and the j-th column element of the matrix part of the added  rows defined by Figure 1 and respectively. Where, the row and column index begins from 1.

-  Step 2: Compute the expand factor.

- Step 3: can be modified into by Scale-Floor rule in Section II.B.  is the base matrix of  LDPC code.

### LDPC HARQ subpacket generation

Each HARQ transmission transmits a HARQ subpacket, and then the generation of HARQ subpacket needs to be defined. Proposed scheme selects some specific bits from the codeword produced by encoder to generate a HARQ subpacket. Figure 2 shows a block diagram of subpacket generation. For the first transmission, the subpacket is generated to select the consecutive interleaved bit sequence that starts from the first bit of the systematic part of the codeword from LDPC encoder. The length of the subpacket is chosen according to the needed coding rate reflecting the channel condition. The first subpacket can also be used as a codeword with the needed coding rate for a packet where HARQ is not applied.

Figure 2 Block diagram of LDPC HARQ subpacket generation

Let the input information block (packet) size is K. Based on rate compatible codes, the generation procedure has been described as following:

Step 1: for any desired packet size K, choose values of kB and l such that kB*2l is greater than k by the smallest possible amount.

Step 2: pad  d= kB*2l -K known bits into K bits information block, the position for padded bits are chosen from the second information column of the base parity check matrix.

Step 3: encode kB*2l bits information block with padded zero, and N bits codeword is generated, update HARQ buffer to orderly store the generated codeword exceptpadded known bits.

When,     N=

Base matrix  and expand factor l can be applied for encoding.

When,     N=

Base matrix  and expand factor l can be applied for encoding.

Others,    N=

Base matrix  and expand factor l can be applied for encoding. nB is chosen to be the smallest integer such that nB*2l>=  + d, Base matrix  and expand factor l can be applied for encoding, the codeword of size nB*2l can be generated, the last nB*2l - of the codeword will be punctured.

Here, let,

is the mother code rate, puncturing mother codeword can attain the LDPC codes of any rate larger than , extending mother codeword can attain the LDPC codes of any rate less than .

be the lowest code rate in the scenario of full Incremental Redundancy; for MS,  can be defined by the capability of MS.

be the length of the k-th subpacket

Permute puncturing parity bits of LDPC HARQ mother codeword and keep the order of systematic bits and extending parity. Let the coded bits before the permutation be, let the coded bits after the permutation be. The permutation is defined by

Here,  indicates the i-th element of the puncturing pattern vector, . PV=[16,26,20,30,28,22,24,18,31,29,27,25,23,21,19,17];

Step 5: Select  bits from said HARQ buffer to form the k-th subpacket. The selection rule has been defined by the following formula, the right side of the formula denotes some position in N bits codeword, the codeword bit in this position is selected and then located in the i-th positon of the k-th HARQ subpacket. Then, the index of the i-th symbol for the k-th subpacket shall be:

Let

be zero.

k	be the subpacket index when HARQ is enabled k=0 for the first transmission and increases by one for the next subpacket. k=0 when HARQ is not used

be the length of the k-th subpacket

be the lowest code rate in the scenario of full Incremental Redundancy; for MS,  can be defined by the capability of MS

be the i-th index of the k-th subpacket, here the position index i starts from zero

Downlink/Uplink HARQ buffering capability indicates the maximal number of data bits the BS/MS can store for downlink/uplink HARQ. It needs to be defined by system.

### Vector Encoding of Structured LDPC codes

One encoding method has been shown; the encoding structure is very simple. For certain code rate, LDPC codes of different code sizes will use a uniform hardware structure. Here we have suggested another more effective encode method.

Encoding is the process of determining the parity sequence, given an information sequence. To encode, the information block s is divided into kb=nb−mb groups of z bits. Let this grouped s be denoted u,

Where each element of u is a column vector as follows

u(i) = [siz,siz+1,siz+2,⋯,s(i+1)z−1]T

Using the modified base matrix, the parity sequence p is determined in groups of z. Let the grouped parity sequence p be denoted v,

Where each element of v is a column vector as follows

v(i) = [piz,piz+1,piz+2,⋯,p(i+1)z−1]T

Define the temporary block is divided into mbgroups of z bits. Let this grouped  be denoted

where each element of  is a column vector whose size equals to z.

Encoding proceeds in four steps:

(1) Compute temporary block.

(2) Determines v(0) .

(3) Determines v(i+1) from v(i) ,here , and  indicates the row number of the second element of three non-negative-one elements in the kb-th column of base matrix Hb.

(4) Determines v(mb-1) from v(0), and determines v(i) from v(i-1) , here .

First step, compute, each element of  can be calculated by the following formula,

needs to be saved in the LDPC encoder, and the needed memory for saving  is only the maximum code size.

Second step, compute v(0) ,

.

So

Third step, compute v(i+1), here .

Four step, compute v(i),  here .

means the operation that circularly shifts the values in the vector u(j) by h(i,j)b elements. Thus only the operations of add and shift are needed to perform the encode process of structured LDPC codes.

In the second step, parallel processing can be adopted. The third step and the fourth step can be performed at the same time to improve the throughput of the encoder.

# 3 Feature

- Flexible enough to support any code rate from 1/3 to 5/6 and any code size from 192 to 10240.

- Support Full IR -HARQ with the lowest rate 1/3.

- Support Bit Re-arrangement for high order modulation.

# 4 Simulation

This part evaluates the error rate performance for proposed structured LDPC coding.

Simulation conditions:

- Gaussian Channel

- QPSK modulation

- code rates: 1/3, 1/2, 2/3, 3/4 , 5/6

- information block lengths: k=384 bits

- Simulation is stopped when 100 information blocks are wrong.

- BP decoding, 50 iterations for structured LDPC

- Max-Log-MAP algorithm with maximum 8 iterations with early stopping for CTC.

Figure 3 gives the error rate performance for proposed LDPC comparing with 16e’s CTC coding. The performance of these two coding scheme is very close.

Figure 3 the error rate performance for proposed LDPC comparing with 16e’s CTC coding

# 5 Conclusion

From the analysis and simulation above, we find that the proposed structured LDPC has a closed performance comparing to 16e’s CTC [2]. However, considering of the feature for LDPC, such as more adequate for longer codeword, easy to processes parallel decoding, and achieve high throughput, and unequal protection for encoding bits, we suggest this proposed structured LDPC be considered by 16m FEC.

# 6 Reference

[1] IEEE 802.16-08/003r6, “The Draft IEEE 802.16m System Description Document,” December 12, 2008.

[2] P80216Rev2_D7, "Part 16: Air interface for fixed broadband wireless access systems," October, 2008.

# Text Proposal for IEEE 802.16m Amendment Working Document

============================== Start of Proposed Text ================================

15.3.x   Channel Encoding and HARQ

15.3.x.x  Channel coding

15.3.x.x.x  LDPC coding

15.3.x.x.x.1	Code description of Structured LDPC coding

The Parity check matrix of structured LDPC Codes is defined by a matrix H of size(mb×z)×(nb×z), which consists of blocks of circularly shifted identity matrices or zero matrices of sizez×z, of the form:

| H=Ph00bPh01bPh02b⋯Ph0nbbPh10bPh11bPh12b⋯Ph1nbb⋯⋯⋯⋯⋯Phmb0bPhmb1bPhmb2b⋯Phmbnbb=PHb | (15.3.x. x .x .1-1) |
| --- | --- |

If integerhijb=−1, definePhijb=0 ; otherwise, definePhijb=(P)hijb.

P of size z×z and base matrixHb of size mb×nb has the form:

| P=010⋯0001⋯0⋯⋯⋯⋯⋯000⋯1100⋯0 | (15.3.x.x.x.1-2) |
| --- | --- |

consists of two sections, where  corresponds to the systematic bits and  corresponds to the parity-check bits, such that

15.3.x.x.x.2	Mother Code Set of Structured LDPC codes

The  information bits are encoded into  bits by using the LDPC code. A Mother Code Set shall be defined to consist ofLDPC codes of the same code rate and different code sizes. Here z is the expand factor of structured LDPC code,  , , and .

A uniform base matrix  is defined as the following:

-1 605 173 110 -1 274 -1 8 -1 -1 -1 -1 -1 108 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 194 77 265 571 -1 -1 -1 242 -1 -1 -1 246 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 63 0 538 -1 -1 194 -1 -1 358 -1 -1 -1 411 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 348 23 271 260 166 -1 -1 -1 -1 -1 -1 566 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 238 241 574 -1 466 -1 -1 -1 -1 111 -1 -1 -1 -1 575 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 601 442 474 218 -1 -1 321 -1 -1 -1 446 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 56 148 618 -1 -1 363 -1 485 -1 454 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 495 178 459 270 -1 -1 -1 -1 112 -1 -1 -1 -1 193 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 346 611 380 170 -1 -1 -1 -1 241 -1 -1 -1 -1 -1 -1 -1 135 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 -1 -1 343 255 353 405 317 -1 -1 -1 436 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 552 630 42 282 -1 -1 -1 436 -1 -1 -1 -1 96 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 -1 194 479 621 170 -1 567 -1 -1 -1 -1 -1 -1 12 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 -1 352 158 73 -1 -1 -1 471 187 -1 -1 -1 -1 -1 218 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 -1 524 595 329 185 -1 532 -1 -1 -1 -1 -1 -1 -1 -1 501 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 -1 573 48 108 -1 452 -1 -1 -1 -1 -1 195 -1 -1 536 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0 0 -1 293 5 409 9 -1 -1 -1 -1 -1 285 -1 552 -1 -1 -1 0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 0

Here z is the expand factor of structured LDPC code, ,

shall be modified to generate the base matrixof LDPC codes .Let denotes the i-th row, j-th column element of ,  denotes the i-th row, j-th column element of  . is given by Scale-Floor rule:

|  | (15.3.x.x.x.2-1) |
| --- | --- |

Here,   rounds the elements in it to the nearest integers towards negative infinity.

15.3.x.x.x.3	Base Matrix Generation for LDPC encode process

The maximum number of soft channel bits available in the virtual buffer is which is signaled from higher layers from each HARQ transmission. is the maximum possible codeword size for the encode of k information bits in the scenario of full Incremental Redundancy.

Here, and let .

Let. There are three steps to generate the needed base matrix. If ，Step 1 is needed; Otherwise, Step 1 is neglect, and and .

- Step 1: Compute, and then  rows and  columns shall be added to the last row and the last column of  to generate an extended base matrix of size, here.  is depicted by Figure 15.3.x.x.x.3-1 and is defined as follows:

for k = 1 to k =,

, , , ,

,, others in the k-th row are -1.

end for

Here, k indicates the row index for the added  rows, and denotes the i-th row and the j-th column element of the matrix part of the added  rows defined by Figure 15.3.x.x.x.3-1 and respectively. Where, the row and column index begins from 1.

- Step 2: Compute the expand factor.

- Step 3: can be modified into by Scale-Floor rule in Section 15.3.x.x.2  is the base matrix of  LDPC code.

Figure 15.3.x.x.3-1. Extended LDPC base matrix

15.3.x.x.x.4	LDPC encode process

The LDPC HARQ mother codeword shall be generated and then sent to HARQ buffer. There are two steps for encode process:

Step 1 Pad  zero bits before K bits information block and perform the encode process of LDPC code.  parity bits shall be generated.

Step 2 Delete the padded zero bits in Step 1. If the number of codeword bits after the deletion doesn’t equal to,  parity bits in the tail shall be deleted.

15.3.x.x.x.5	HARQ mother codeword permutations

Permute puncturing parity bits of LDPC HARQ mother codeword and keep the order of systematic bits and extending parity. Let the coded bits before the permutation be, let the coded bits after the permutation be. The permutation is defined by

|  | (15.3.x.x.x.5-1) |
| --- | --- |

Here,  indicates the i-th element of the puncturing pattern vector, . PV=[16,26,20,30,28,22,24,18,31,29,27,25,23,21,19,17];

15.3.x.x.x.6	HARQ codeword bits selection

Select  bits from said HARQ buffer to generate the k-th subpacket, which is defined by:

|  | (15.3.x.x.x.6-1) |
| --- | --- |

Let k be the subpacket index when HARQ is enabled k=0 for the first transmission and increases by one for the next subpacket. k=0 when HARQ is not used. Let  be the length of the k-th subpacket, be andbe 0. Let the coded bits of the k-th subpacket be. Let the coded bits of HARQ mother codeword be.

============================== end of Proposed Text ================================