| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for multi-hop relay simulation test scenario in Relay Evaluation Methodology |  |
| Date Submitted | 2009-01-15 |  |
| Source(s) | Young-il Kim, Perumal Ramesh Babu, Jisoo Park, Khishigjargal ETRI 161 Gajeong-dong Yuseong-gu, Taejeon 305-700, Korea Kyu Ha Lee Samsung Thales San-14, Nongseo-Dong, Giheung-Gu, Yongin , Gyeonggi-Do, Korea 446-712 Suk Chan Kim, Dong Heon Lee Pusan National University Jangjeon-dong, Geumjeong-gu, Busan, 609-735, Korea | Voice: +82 42 860 5399 (Young-il Kim) +82 42 860 1755 (Perumal Ramesh Babu) +82 42 860 5748 (Jisoo Park) +82 42 860 1779 (Khishigjargal) E-mail: yikim@etri.re.kr (Young-il Kim) ramesh.babu@etri.re.kr(Perumal Ramesh Babu) jsp@etri.re.kr (Jisoo Park) khishig@etri.re.kr (Khishigjargal) Voice: +82-31-280-9917 Fax : +82-31-280-1620 E-mail: kyuha.lee@samsung.com Voice: +82 51 510 2380 (Suk Chan Kim) +82 51 515 5190 E-mail: sckim@pusan.ac.kr(Suk Chan Kim) leedheon@pusan.ac.kr(Dong Heon Lee) |
| Re: | Change Request for 16m EMD |  |
| Abstract | Relay evaluation methodology was accepted in #57 meeting. In this evaluation methodology, throughput enhancement scenario was included without the test scenario for cell coverage extension evaluation. In this document we define the scenario for cell coverage extension evaluation. |  |
| Purpose | For consideration and adoption into the 16m EMD document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for multi-hop relay simulation test scenario in Relay Evaluation Methodology

Young-il Kim, Perumal Ramesh Babu, Jisoo Park, Khishigjargal

ETRI

Kyu Ha Lee

Samsung Thales

Suk Chan Kim, Dong Heon Lee

Pusan National University

# Introduction

# Relay evaluation methodology was accepted within 16m EMD in #57 meeting. In this evaluation methodology, test scenario was included for ART(Above Rooftop) RS. The test scenario mainly was focused on the evaluation of throughput enhancement. The goal of Relay is for cell throughput enhancement and cell coverage extension.  Therefore the test scenario for cell coverage extension has to be included. The test scenarios had been evaluated in [2]. In this document we define the scenario for cell coverage extension evaluation.

# Text Proposal

[Insert the following text as section 14 of the 16m EMD document:]

# Test Scenarios

Two basic scenarios with Relay Stations (RS) are defined for the purpose of system level simulations:

- Above Rooftop (ART) RS scenario

  - Two relays per sector

- Below Rooftop (BRT) RS scenario

  - Six relays per sector (1.5 km BS site-to-site distance)

  - TBD relays per sector (3.0 km BS site-to-site distance)

- Manhattan deployment scenario (Optional)

- Scenarios according to the purpose of RS deployments (Optional)

  - Throughput enhancement scenario

    - Two relays per sector

  - Coverage extension scenario

    - Cell edge and outside scenario

    - Artificial Shadow Region (ASR) scenario

# Above Rooftop RS Scenario

# Two Relays per Sector

In this scenario two RSs are deployed in each sector. The positions of the relays are determined by the BS-RS distance r and the angle  between the boresight direction of the BS sector antenna and the LOS to the RS (refer to Figure 28). By default the distance r is equal to 3/8 of the site-to-site distance and the angle  is 26º. A value of 30º may optionally be used. Note that specified recommended values of r and  are currently aligned for DL spatial multiplexing of relay links assuming that the BS is equipped with 4 antenna elements and antenna spacing equals 4 wavelengths. The values of r and  for the BS equipped with 2 antenna elements are TBD. The optional parameters are aligned with the case when no beamforming and spatial multiplexing techniques are applied. The defined values for r and  are not obligatory and may be changed for other simulation scenarios, but in this case their values must be specified by the proponents. The particular choice may be justified by specific BS antenna system parameters (i.e., antenna spacing and number of antenna elements) and used signal processing techniques. For instance, the angles may be selected to reduce the amount of interference from neighboring cells or to increase the performance of spatially multiplexed relay links for a given BS antenna configuration.

# Below Rooftop RS Scenario

# Manhattan deployment scenario

# Scenarios according to the purpose of RS deployment (Optional)

Relay Stations (RSs) may be deployed to provide improved coverage and/or capacity. It is necessary to evaluate the system performance based on the purpose of RS deployment. The 802.16j provided harmonized contribution on multi-hop relay usage models [4] and the purpose of usage models is to define different deployment scenarios. Some examples of RS deployment are illustrated in Figure 36. This section provides three scenarios from the purpose of RS deployment based on usage models and channel models by using the previous ART or BRT scenario.

Figure 36: RS Usage Model

# Throughput Enhancement Scenario

Fixed RSs are deployed to enhance throughputs and to improve capacity. In this scenario, RS allocation and the assumptions are same with ART RS and BRT RS scenario as described in the Section 14.1.1.

# Coverage Extension Scenario

# Cell Edge and Outside Scenario

For the evaluation of cell coverage extension, two or three RSs in each sector to configure multi hop link are deployed. Distances between BSs are widened and RSs are positioned at the edge and outside of the BS coverage as shown in figure 36. Figure 37 illustrates the deployment scenario with ART RSs placement more specifically. First and second sector has three RSs but third sector has two RSs. In this topology, rcell denotes the cell radius covered by the BS and R is the site-to-site distance. The positions of bth relay of ath sector are determined by the distance rab and the angle ab between the boresight direction of the BS sector antenna and the LOS to the RS. By default, distances r11, r21, and r31 is 3/8 of R and r12, r22, and r32 is 4/8 of site-to-site distance, and r13, and r23 is  of site-to-site distance.  Angles 11, 12, 21, 22, 31, and 32 are 0 º, 13 is 30 º, and 23 is -30º. (refer to Figure 37). We find that this topology can cover whole outside of BS coverage as shown in the centre cell. The defined values for rab and ab are not obligatory and may be changed for other simulation scenarios, but in this case their values must be specified by the proponents and be able to cover whole outside of BS coverage.

Figure 37: Scenario with RS deployments at the edge and the outside of the cell (19 cell)

Figure 38: Three RSs in the first and second sector and two RSs in the third sector

# Artificial Shadow Region (ASR) Scenario

This scenario is also for the evaluation of coverage extension. The RS can provide service for users in coverage holes that exist due to deep-shadow fading and in valleys between buildings. Shadow fading values for MSs, located in the ASR, are artificially raised above the threshold, which depends on the shadow fading model. Positions of ASR are determined by the same as the way described in Section 14.1.1.1 and RSs are located in that region. Figure 39 and Figure 40 illustrates the deployment scenario with two ASRs and two ART RSs per sector. The radius rASR of ASR may be calculated by

where, A is the cell area(km2), R is the constant that can be determined from the threshold value of  the magnitude of shadow fading, and N denotes the number of ASR per cell.

Figure 39: Two ASR and ART deployment scenario (19 cell)

Figure 40: Two ASR and ART deployment scenario

Figure 41 (a) shows the example of shadow fading generated by spatial correlation model [3], and (b) is the CDF of shadow fading. R can be obtained by determining the threshold in the CDF. If the magnitude of shadow fading of ASR is higher than the threshold value of 15dB, then R is 3 %.

(a)							       (b)

Figure 41:  shadow fading (a) magnitude(dB) (b) CDF (standard deviation : 8dB)

# Basic Parameters

| Scenario/ parameters | ART RS scenario | BRT RS scenario | Manhattan deployment scenario |
| --- | --- | --- | --- |
| Carrier Frequency | Refer to Baseline configuration (Table 3) |  |  |
| Operating Bandwidth | Refer to Baseline configuration (Table 3) |  |  |
| Frequency Reuse | 1x3x1 (required)* |  |  |
| Number of RS per sector | 2** | 6 (1.5 km site-to-site distance) 6-12 (3.0 km site-to-site distance) | 1,2,3 |
| BS Site-to-site distance | 1.5km (mandatory) 3.0km (optional) | 1.5 km (mandatory) 3.0 km (optional) | TBD |
| RS placement distance (r) | 2 RSs per sector – 3/8 of site-to-site distance | 6 RSs per sector – symmetrical positioning (hexagonal) | TBD |
| RS placement angle ( ) | 2 RSs per sector - 26° (Default); 30° (Optional) | 6 RSs per sector – symmetrical positioning (hexagonal) | TBD |
| MS mobility | Refer to Baseline configuration (Table 3) |  |  |
| Scenario/ parameters | Cell Edge and Outside Scenario (optional) | ASR Scenario (optional) |  |
| Carrier Frequency | Refer to Baseline configuration (Table 3) |  |  |
| Operating Bandwidth | Refer to Baseline configuration (Table 3) |  |  |
| Frequency Reuse | 1x3x1 (required)* |  |  |
| Number of RS per sector | 3 (2.0 km site-to-site distance), 3 hops 5 (3.0 km site-to-site distance), 4 hops | 2** |  |
| BS Site-to-site distance | 2.0km (mandatory) 3.0km (optional) | 1.5km (mandatory) 3.0km (optional) |  |
| RS placement distance (r) | 3/8, 4/8, of site-to-site distance(2.0 km site-to-site distance) 2/8, 3/8, 4/8, , of site-to-site distance(3.0 km site-to-site distance) | 2 RSs per sector – 3/8 of site-to-site distance |  |
| RS placement angle ( ) | 0°, 30°, -30° | 2 RSs per sector - 26° (Default); 30° (Optional) |  |
| MS mobility | Refer to Baseline configuration (Table 3) |  |  |

Table 46: Test Scenarios

* In a frequency reuse pattern of NxSxK, the network is divided into clusters of N cells (each cell in the cluster has a different frequency allocations), S sectors per cell, and K different frequency allocations per cell.

**Two RSs per sector are recommend here because the other parameters(e.g. RS placement distance, RS placement angle) are dependant on the number of RS.

| Parameter | Value |  |
| --- | --- | --- |
|  | ART RS and BRT RS scenario | Manhattan deployment scenario (optional) |
| BS Tx Power per sector | Refer to Table 4 | 46 dBm (Refer to Table 4) |
|  |  | 36 dBm (optional) |
| Base station antenna height | Refer to Table 4 | 12.5m |
| Number of transmit antennas per sector | 2 (Mandatory) 4 (Optional) |  |
| Number of receive antennas per sector | 2 (Mandatory) 4 (Optional) |  |
| Number of sectors | Refer to Table 4 | 4 (oriented along the streets) |
| Antenna gain (boresight) | Refer to Table 4 | TBD |
| Antenna 3-dB beamwidth | Refer to Table 4 | TBD |
| Antenna front-to-back power ratio | 30 dB (Mandatory) 20 dB (Optional) (Refer to Table 4 ) | TBD |
| Antenna spacing | 4λ (Mandatory) (Refer to Table 4) 0.5λ (Optional) |  |
| Noise figure | Refer to Table 4 |  |
| Cable loss | Refer to Table 4 |  |
| Parameter | Value |  |
|  | Cell Edge and Outside, ASR Scenario (optional) |  |
| BS Tx Power per sector | Refer to Table 4 |  |
| Base station antenna height | Refer to Table 4 |  |
| Number of transmit antennas per sector | 2 (Mandatory) 4 (Optional) |  |
| Number of receive antennas per sector | 2 (Mandatory) 4 (Optional) |  |
| Number of sectors | Refer to Table 4 |  |
| Antenna gain (boresight) | Refer to Table 4 TBD |  |
| Antenna 3-dB beamwidth | Refer to Table 4 TBD |  |
| Antenna front-to-back power ratio | 30 dB (Mandatory) 20 dB (Optional) (Refer to Table 4 ) |  |
| Antenna spacing | 4λ (Mandatory) (Refer to Table 4) 0.5λ (Optional) |  |
| Noise figure | Refer to Table 4 |  |
| Cable loss | Refer to Table 4 |  |

Table 47: BS Equipment Model

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | ART RS scenario | BRT RS scenario |  | Manhattan deployment scenario (optional) |
|  | Relay Link |  |  |  |
| RS Tx Power | 36 dBm per antenna | 27 dBm per antenna |  | 36 dBm per antenna (Mandatory) 27 dBm per antenna (Optional) |
| Relay station antenna height | 32 m | 10m |  | 10 m |
| Number of transmit antennas | 1 | 2 |  | 1 |
| Number of receive antennas | 1 | 2 |  | 1 |
| Antenna type | Directional | Omni in horizontal plane |  | Directional |
| Antenna gain (boresight) | 20 dBi | 7 dBi |  | 20 dBi |
| Antenna 3-dB beamwidth | 200 | N/A |  | 200 |
| Antenna front-to-back power ratio | 23 dB | N/A |  | 23 dB |
| Antenna spacing | N/A | 2λ |  | N/A |
| Antenna orientation | Antenna array broadside pointed to BS direction | Antenna array broadside pointed to BS direction |  | Antenna array broadside pointed to the link’s source direction (BS or another RS) |
| Noise figure | 5 dB |  |  |  |
| Cable loss | 2 dB |  |  |  |
|  | Access Link |  |  |  |
| RS Tx Power | 36 dBm per antenna | 27 dBm per antenna |  | 36 dBm per antenna (Mandatory) 27 dBm per antenna (Optional) |
| Relay station antenna height | 32m | 10m |  | 10 m |
| Number of transmit antennas | 2 baseline/ 4 optional |  |  | TBD |
| Number of receive antennas | 2 baseline/ 4 optional |  |  | TBD |
| Number of sectors | 1 |  |  | 4 oriented along the streets (or less) |
| Antenna type | Omni in horizontal plane |  |  | Directional |
| Antenna gain (boresight) | 7 dBi |  |  | TBD |
| Antenna 3-dB beamwidth | N/A | N/A |  | TBD |
| Antenna front-to-back power ratio | N/A | N/A |  | TBD |
| Antenna spacing | 4λ | 2λ |  | TBD |
| Antenna orientation | Antenna array broadside pointed to BS direction | Antenna array broadside pointed to BS direction |  | TBD |
| Noise figure | 5 dB |  |  |  |
| Cable loss | 2 dB |  |  |  |
| Parameter | Value |  |  |  |
|  | Cell Edge and Outside Scenario | ASR Scenario |  |  |
|  | Relay Link |  |  |  |
| RS Tx Power | 36 dBm per antenna |  |  |  |
| Relay station antenna height | 32 m |  |  |  |
| Number of transmit antennas | 1 | 2 |  |  |
| Number of receive antennas | 1 | 2 |  |  |
| Antenna type | Directional |  |  |  |
| Antenna gain (boresight) | 20 dBi |  |  |  |
| Antenna 3-dB beamwidth | 200 |  |  |  |
| Antenna front-to-back power ratio | 23 dB |  |  |  |
| Antenna spacing | N/A |  |  |  |
| Antenna orientation | Antenna array broadside pointed to BS direction |  |  |  |
| Noise figure | 5 dB |  |  |  |
| Cable loss | 2 dB |  |  |  |
|  | Access Link |  |  |  |
| RS Tx Power | 36 dBm per antenna |  |  |  |
| Relay station antenna height | 32m |  |  |  |
| Number of transmit antennas | 2 baseline/ 4 optional |  |  |  |
| Number of receive antennas | 2 baseline/ 4 optional |  |  |  |
| Number of sectors | 1 |  |  |  |
| Antenna type | Omni in horizontal plane |  |  |  |
| Antenna gain (boresight) | 7 dBi |  |  |  |
| Antenna 3-dB beamwidth | N/A |  |  |  |
| Antenna front-to-back power ratio | N/A |  |  |  |
| Antenna spacing | 4λ |  | 2λ |  |
| Antenna orientation | Antenna array broadside pointed to BS direction |  |  |  |
| Noise figure | 5 dB |  |  |  |
| Cable loss | 2 dB |  |  |  |

Table 48: RS Equipment Model

*Parameters for mobile vehicle usage scenario are MRS’s.

# Channel Models

This section describes the channel models used to model propagation conditions between BS, RS, and MS for two the different RS deployment scenarios – ART RS and BRT RS. Note that whole of the channel models for optional Throughput Enhancement Scenario are same with those of ART RS scenario.

# Pathloss Models

# ART RS Scenario

# BRT RS Scenario

# Manhattan deployment scenario (optional)

# Scenarios according to the purpose of RS deployment (optional)

# Throughput enhancement and Cell Edge and Outside scenario

The pathloss models for the Throughput Enhancement and Cell Edge and Outside scenario are same with those of ART RS scenario.

# ASR scenario

Pathloss models for the ASR scenario are defined in Error: Reference source not foundX1.

| Link | Pathloss model |  |
| --- | --- | --- |
| BS-MS | Baseline test scenario (Mandatory) (Refer to Section Error: Reference source not found) Urban Macrocell test scenario (Optional) (Refer to Section Error: Reference source not found). |  |
| RS-MS | MSs which are not in the ASR | Baseline test scenario (Mandatory) (Refer to Section Error: Reference source not found) Urban Macrocell test scenario (Optional) (Refer to Section Error: Reference source not found). |
|  | MSs in the ASR | Urban Microcell propagation : Urban Microcell COST-Walfish-Ikegami pathloss model (Mandatory) Error: Reference source not foundError: Reference source not found Urban Microcell test scenario pathloss model (Optional) (Refer to Section Error: Reference source not found) |
|  |  | Outdoor to Indoor propagation : Outdoor to Indoor test scenario pathloss model (Optional) (Refer to Section Error: Reference source not found) |
| BS-RS and RS-RS | 802.16j EVM Type H (Mandatory) [81] |  |

Table X1: Path loss models for ASR Scenario

# BS-MS and RS-MS links

The BS-MS link for the ASR scenario is typical ART to BRT links of the ART RS scenario. The RS-MS links for the MSs which are not in the ASR is also typical ART to BRT link. In this scenario, we assume that RS provides service for MSs in coverage holes, shadow of buildings, and valleys between buildings or for MSs in buildings. For MSs in the ASR, two propagation sub-scenarios for RS-MS links are considered as those of the BRT RS scenario. The Urban Microcell propagation scenario is used where MSs are located outdoors, and the Outdoor to Indoor propagation scenario is used where MSs are located inside the buildings.

# BS-RS and RS-RS links

BS-RS and RS-RS links are assumed to be Urban LOS ART to ART links. RS is assumed to be located above rooftop and at the region suffered from deep shadow fading in the urban. Since there are no suitable channel model scenarios defined in Section Error: Reference source not found, the pathloss models described in the 802.16j EVM Error: Reference source not found is used. IEEE 802.16j EVM Type H model is detremind by the COST 231 Walfish-Ikegami pathloss model Error: Reference source not found, Error: Reference source not found, but excluding the rooftop-to-MS diffraction loss term and modifying the multiscreen diffraction loss term [81].

# Spatial channel models

# ART RS scenario

# BRT RS scenario

# Manhattan deployment scenario (optional)

# Scenarios according to the purpose of RS deployment (optional)

# Throughput enhancement and Cell Edge and Outside scenario

The spatial channel models for the Throughput Enhancement and Cell Edge and Outside scenario are same with that of scenario.

# ASR scenario

| Link | Spatial Channel Models |  |
| --- | --- | --- |
| BS-MS | MSs which are not in the ASR | Baseline test scenario model (Mandatory) (Refer to Section Error: Reference source not found) Urban Macrocell test scenario model (Optional) (Refer to Section Error: Reference source not found). |
|  | MSs in the ASR | Bad Urban Macrocell (Mandatory) (Refer to Section 3.2.5.1) |
| BS-RS and RS-RS | WINNER B5a model Error: Reference source not foundError: Reference source not found |  |
| RS-MS | MSs which are not in the ASR | Baseline test scenario model (Mandatory) (Refer to Section Error: Reference source not found) Urban Macrocell test scenario model (Optional) (Refer to Section Error: Reference source not found). |
|  | MSs in the ASR | Urban Microcell propagation : Bad Urban Microcell (Mandatory) (Refer to Section 3.2.5.3) |
|  |  | Outdoor to Indoor propagation : Outdoor to Indoor test scenario model (Optional) (Refer to Section Error: Reference source not found) |

Table X3: Spatial channel models for ASR Scenario

# BS-MS link

The mandatory baseline and optional Urban Macrocell and Suburban Macrocell test scenario spatial channel models described in Section Error: Reference source not found are used for BS-MS link simulations in which MSs are not in the ASR. For the MSs in the ASR, the Bad Urban Macrocell CDL model is used for BS-MS links, and the CDL parameters are given in Table 11.

# BS-RS and RS-RS links

The WINNER B5a CDL model is used as the spatial channel model for BS-RS and RS-RS links. Error: Reference source not found and Error: Reference source not found provide a short summary of the parameters associated with this channel model.

# RS-MS links

The mandatory baseline and optional Urban Macrocell and Suburban Macrocell test scenario spatial channel models described in Section Error: Reference source not found are used for BS-MS link simulations in which MSs are not in the ASR. For the MSs in the ASR, the mandatory Bad Urban Microcell CDL model and the optional Outdoor to Indoor spatial channel models described in Section Error: Reference source not found can be used for RS-MS link simulations without any modifications. The CDL parameters are given in Table 15.

# Shadowing models

The shadowing factor (SF) has a log-normal distribution with a standard deviation that is different for different scenarios as shown in Table. The values specified in Error: Reference source not found have been derived based on the baseline model in this document, 802.16j EVM Error: Reference source not found, SCM, and WINNER models.

In the ART RS scenario for BS-MS and RS-MS links, the shadowing standard deviation is 8 dB according to Section Error: Reference source not found. For BS-RS and RS-RS links, the shadowing standard deviation is 3.4 dB according to the 802.16j EVM Type D Error: Reference source not found and WINNER B5a channel models Error: Reference source not found.

In the BRT RS scenario for the BS-MS link the shadowing standard deviation is 8 dB according to Section Error: Reference source not found. For the BS-RS link “pre-planned” RS placement by operators is assumed so the shadowing standard deviation is set to 6 dB and the mean value of the shadowing factor is set to 2 dB providing a positive shift in log-normal curve. This ensures better propagation conditions than for typical BS-MS links in which MSs are assumed to be randomly located. For the RS-MS (in the Urban Microcell scenario) and RS-RS links, the shadowing standard deviation is 4 dB for NLOS and 3 dB for LOS propagation according to Section Error: Reference source not found and the WINNER Urban Microcell channel models Error: Reference source not found. For the Outdoor-to-Indoor test scenario RS-MS link shadowing standard deviation is 7 dB according to Section Error: Reference source not found.

In the Manhattan deployment scenario for the BS-MS, RS-MS links with Urban Microcell propagation conditions and BS-RS, RS-RS links, the shadowing standard deviation is 4 dB for NLOS and 3 dB for LOS propagation according to Section Error: Reference source not found.and the WINNER models Error: Reference source not found. For the BS-MS and RS-MS links with Outdoor-to-Indoor propagation conditions shadowing standard deviation is 7 dB according to Section Error: Reference source not found.

In the Cell Edge and Outside scenario and ASR scenario for all of links, shadowing standard deviations are same with those of ART RS scenario.

|  | BS-RS |  | BS-MS |  | RS-RS | RS-MS |
| --- | --- | --- | --- | --- | --- | --- |
| ART RS | 3.4 dB |  | 8 dB |  | 3.4 dB | 8 dB |
| BRT RS | 6 dB and 2 dB mean value positive shift |  | 8 dB |  | NLOS: 4 dB LOS: 3 dB | Urban Microcell propagation: NLOS: 4 dB LOS: 3 dB |
|  |  |  |  |  |  | Outdoor to Indoor propagation (Optional): 7 dB |
| Manhattan deployment scenario (optional) | NLOS: 4 dB LOS: 3 dB |  | Urban Microcell propagation: NLOS: 4 dB LOS: 3 dB |  | NLOS: 4 dB LOS: 3 dB | Urban Microcell propagation: NLOS: 4 dB LOS: 3 dB |
|  |  |  | Outdoor to Indoor propagation (Optional): 7 dB |  |  | Outdoor to Indoor propagation (Optional): 7 dB |
|  | BS-RS |  | BS-MS |  | RS-RS | RS-MS |
| Cell Edge and Outside (optional) | 3.4 dB |  | 8 dB |  | 3.4 dB | 8 dB |
| ASR (optional) | 3.4 dB |  | 8 dB |  | 3.4 dB | 8 dB |

Table 57: Shadowing standard deviation

The correlation model for shadow fading is the same as the one described in this document, but the correlation distance for shadowing is corrected according to Table. The parameters in Table have been derived based on the baseline model in this document, 802.16j EVM Error: Reference source not found, SCM, and WINNER models.

In the ART RS scenario for BS-MS and RS-MS links, the shadowing correlation distance is chosen to be 50 m according to Section Error: Reference source not found. For the BS-RS and RS-RS links, the shadowing correlation distance is chosen to be 100 m according to typical values of the LOS channel correlation distance.

In the BRT RS scenario for the BS-MS and BS-RS links, the shadowing correlation distance is chosen to be 50 m according to Section 3.2.4. For the RS-RS and RS-MS links, the shadowing correlation distance is chosen to be 12 m for NLOS and 14 m for LOS conditions according to typical values of correlation distance given in WINNER Error: Reference source not found for Urban Microcell scenarios. For the RS-MS links in Outdoor to Indoor propagation conditions the shadowing correlation distance is chosen to be 7 m according to WINNER Error: Reference source not found.

In the Manhattan deployment scenario, for the BS-RS, RS-RS links and BS-MS, RS-MS links with Urban Microcell propagation conditions, the shadowing correlation distance is chosen to be 12 m for NLOS and 14 m for LOS conditions according to typical values of correlation distance given in WINNER Error: Reference source not found. For the BS-MS, RS-MS links with Outdoor to Indoor propagation conditions the shadowing correlation distance is chosen to be 7 m according to WINNER Error: Reference source not found.

In the Cell Edge and Outside scenario and ASR scenario for all of links, shadowing standard deviations are same with those of ART RS scenario.

|  | BS-RS |  | BS-MS |  | RS-RS | RS-MS |
| --- | --- | --- | --- | --- | --- | --- |
| ART RS | 100 m |  | 50 m |  | 40 m | 50 m |
| BRT RS | 50 m |  | 50 m |  | NLOS: 12 m LOS: 14 m | Urban Microcell propagation NLOS: 12 m LOS: 14 m |
|  |  |  |  |  |  | Outdoor to Indoor propagation (Optional): 7 m |
| Manhattan deployment scenario (optional) | NLOS: 12 m LOS: 14 m |  | Urban Microcell propagation: NLOS: 12 m LOS: 14 m |  | - NLOS: 12 m LOS: 14 m | Urban Microcell propagation: - NLOS: 12 m LOS: 14 m |
|  |  |  | Outdoor to Indoor propagation (Optional): 7 m |  |  | Outdoor to Indoor propagation (Optional): 7 m |
|  | BS-RS |  | BS-MS |  | RS-RS | RS-MS |
| Cell Edge and Outside (optional) | 100 m |  | 50 m |  | 40 m | 50 m |
| ASR (optional) | 100 m |  | 50 m |  | 40 m | 50 m |

Table 58: Correlation distance for shadowing

# Summary

| BS-RS link | ART RS scenario | BRT RS scenario | Manhattan deployment scenario |
| --- | --- | --- | --- |
| Penetration Loss | 0dB | 0 dB | 0 dB |
| Pathloss Model | IEEE 802.16j EVM Type D pathloss model (Mandatory) | Modified Baseline Model (Mandatory) Modified Urban and Suburban Macrocell (Optional) | Urban Microcell pathloss model (Mandatory) (To be further modified) WINNER B1 Urban Microcell pathloss model (Optional) (To be further modified) |
| Lognormal Shadowing Standard Deviation | 3.4dB | 6 dB and 2 dB mean value positive shift | NLOS: 4dB LOS: 3dB |
| Correlation Distance for Shadowing | 100m | 50m | NLOS: 12m LOS: 14m |
| Channel Mix | Single static channel | Single static channel | Single static channel |
| Spatial Channel Model | WINNER B5a | Baseline Model (Mandatory) Urban and Suburban Macrocell (Optional) | Modified Urban Microcell |
| RS-RS link |  |  |  |
| Penetration Loss | 0dB | 0 dB | 0 dB |
| Pathloss Model | IEEE 802.16j EVM Type D pathloss model (Mandatory) | Walfish-Ikegami LOS and NLOS pathloss models (Mandatory) (To be further modified) Urban Microcell (Optional) (To be further modified) | Urban Microcell pathloss model (Mandatory) (To be further modified) WINNER B1 Urban Microcell pathloss model (Optional) (To be further modified) |
| Lognormal Shadowing Standard Deviation | 3.4dB | NLOS: 4dB LOS: 3dB | NLOS: 4dB LOS: 3dB |
| De-correlation Distance for Shadowing | 40m | NLOS: 12m LOS: 14m | NLOS: 12m LOS: 14m |
| Channel Mix | Single static channel | Single static channel | Single static channel |
| Spatial Channel Model | WINNER B5a | Modified Urban Microcell | Modified Urban Microcell |
| RS-MS link |  |  |  |
| Penetration Loss | 10dB | Urban Microcell propagation: LOS: 0 dB NLOS: 10 dB Outdoor to Indoor propagation: 0 dB | Urban Microcell propagation: LOS: 0 dB NLOS: 10 dB Outdoor to Indoor propagation: 0 dB |
| Pathloss Model | Baseline Model (Mandatory) Urban and Suburban Macrocell (Optional) | COST Walfish-Ikegami LOS and NLOS pathloss models (Mandatory) Urban Microcell (Optional) Outdoor to Indoor (Optional) | Urban Microcell pathloss model (Mandatory) WINNER B1 Urban Microcell pathloss model (Optional) Outdoor to Indoor pathloss model (Optional) |
| Lognormal Shadowing Standard Deviation | 8dB | Urban Microcell propagation: NLOS: 4dB LOS: 3dB Outdoor to Indoor propagation: 7 dB | Urban Microcell propagation: NLOS: 4dB LOS: 3dB Outdoor to Indoor propagation: 7 dB |
| Correlation Distance for Shadowing | 50m 50% BSs, RSs correlation | Urban Microcell propagation: NLOS: 12m LOS: 14m Outdoor to Indoor propagation: 7 m | Urban Microcell propagation: NLOS: 12m LOS: 14m Outdoor to Indoor propagation: 7 m |
| Channel Mix | ITU Pedestrian B and Vehicular A channel models ITU PB 3kmph - 60% ITU VA 30kmph - 30% ITU VA 120kmph – 10% | Urban Microcell propagation: 3kmph – 60% 60kmph – 30% 120kmph – 10% Outdoor to Indoor propagation: TBD | Urban Microcell propagation: 3kmph – 60% 60kmph – 30% 120kmph – 10% Outdoor to Indoor propagation: TBD |
| Spatial Channel Model | Baseline model (Mandatory) 802.16m EMD Urban and Suburban Macrocell (Optional) | Urban Microcell (Mandatory) Outdoor to Indoor (Optional) | Urban Microcell (Mandatory) Outdoor to Indoor (Optional) |
| BS-MS link |  |  |  |
| Penetration Loss | 10dB | 10dB | Urban Microcell propagation: LOS: 0 dB NLOS: 10 dB Outdoor to Indoor propagation: 0 dB |
| Pathloss Model | Baseline model (Mandatory) Urban and Suburban Macrocell (Optional) | Baseline model (Mandatory) Urban and Suburban Macrocell (Optional) | Urban Microcell pathloss model (Mandatory) WINNER B1 Urban Microcell pathloss model (Optional) Outdoor to Indoor pathloss model (Optional) |
| Lognormal Shadowing Standard Deviation | 8dB | 8dB | Urban Microcell propagation: NLOS: 4dB LOS: 3dB Outdoor to Indoor propagation: 7 dB |
| Correlation Distance for Shadowing | 50m 50% BSs correlation | 50m 50% BSs correlation | Urban Microcell propagation: NLOS: 12m LOS: 14m Outdoor to Indoor propagation: 7 m |
| Channel Mix | 802.16m ITU Pedestrian B and Vehicular A channel models ITU PB 3kmph - 60% ITU VA 30kmph - 30% ITU VA 120kmph – 10% | 802.16m ITU Pedestrian B and Vehicular A channel models ITU PB 3kmph - 60% ITU VA 30kmph - 30% ITU VA 120kmph – 10% | Urban Microcell propagation: 3kmph – 60% 60kmph – 30% 120kmph – 10% Outdoor to Indoor propagation: TBD |
| Spatial Channel Model | Baseline model (Mandatory) Urban and Suburban Macrocell (Optional) | Baseline model (Mandatory) Urban and Suburban Macrocell (Optional) | Urban Microcell (Mandatory) Outdoor to Indoor (Optional) |
| Error Vector Magnitude (EVM) | Ideal | Ideal | Ideal |
| BS-RS link | Cell Edge and Outside scenario | ASR scenario |  |
| Penetration Loss | 0dB | 0 dB |  |
| Pathloss Model | IEEE 802.16j EVM Type D pathloss model (Mandatory) | 802.16j EVM Type H (Mandatory) |  |
| Lognormal Shadowing Standard Deviation | 3.4dB | 3.4dB |  |
| Correlation Distance for Shadowing | 100m | 100m |  |
| Channel Mix | Single static channel | Single static channel |  |
| Spatial Channel Model | WINNER B5a | WINNER B5a |  |
| RS-RS link |  |  |  |
| Penetration Loss | 0dB | 0 dB |  |
| Pathloss Model | IEEE 802.16j EVM Type D pathloss model (Mandatory) | 802.16j EVM Type H pathloss model (Mandatory) |  |
| Lognormal Shadowing Standard Deviation | 3.4dB | 3.4dB |  |
| De-correlation Distance for Shadowing | 40m | 40m |  |
| Channel Mix | Single static channel | Single static channel |  |
| Spatial Channel Model | WINNER B5a | WINNER B5a |  |
| RS-MS link |  |  |  |
| Penetration Loss | 10dB | 10dB |  |
| Pathloss Model | Baseline Model (Mandatory) Urban and Suburban Macrocell (Optional) | MSs which are not in the ASR : Baseline Model (Mandatory) Urban and Suburban Macrocell (Optional) |  |
|  |  | MSs in the ASR Urban Microcell propagation : Urban Microcell COST-Walfish-Ikegami pathloss model (Mandatory) Urban Microcell test scenario pathloss model (Optional) |  |
|  |  | MSs in the ASR Outdoor to Indoor propagation : Outdoor to Indoor test scenario pathloss model (Optional) |  |
| Lognormal Shadowing Standard Deviation | 8dB | 8dB |  |
| Correlation Distance for Shadowing | 50m 50% BSs, RSs correlation | 50m 50% BSs, RSs correlation |  |
| Channel Mix | ITU Pedestrian B and Vehicular A channel models ITU PB 3kmph - 60% ITU VA 30kmph - 30% ITU VA 120kmph – 10% | ITU Pedestrian B and Vehicular A channel models ITU PB 3kmph - 60% ITU VA 30kmph - 30% ITU VA 120kmph – 10% |  |
| Spatial Channel Model | Baseline model (Mandatory) 802.16m EMD Urban and Suburban Macrocell (Optional) | MSs which are not in the ASR : Baseline test scenario model (Mandatory) Urban Macrocell test scenario model (Optional) |  |
|  |  | MSs in the ASR : Urban Microcell propagation : Bad Urban Microcell model (Mandatory) Outdoor to Indoor propagation : Outdoor to Indoor test scenario model (Optional) |  |
| BS-MS link |  |  |  |
| Penetration Loss | 10dB | 10dB |  |
| Pathloss Model | Baseline model (Mandatory) Urban and Suburban Macrocell (Optional) | MSs which are not in the ASR : Baseline model (Mandatory) Urban and Suburban Macrocell (Optional) MSs in the ASR : Bad Urban Macrocell model (Mandatory) |  |
| Lognormal Shadowing Standard Deviation | 8dB | 8dB |  |
| Correlation Distance for Shadowing | 50m 50% BSs correlation | 50m 50% BSs correlation |  |
| Channel Mix | 802.16m ITU Pedestrian B and Vehicular A channel models ITU PB 3kmph - 60% ITU VA 30kmph - 30% ITU VA 120kmph – 10% | 802.16m ITU Pedestrian B and Vehicular A channel models ITU PB 3kmph - 60% ITU VA 30kmph - 30% ITU VA 120kmph – 10% |  |
| Spatial Channel Model | Baseline model (Mandatory) Urban and Suburban Macrocell (Optional) | Baseline model (Mandatory) Urban and Suburban Macrocell (Optional) |  |
| Error Vector Magnitude (EVM) | Ideal | Ideal |  |

Table 62: Summary of pathloss and channel models

------------------------------------------------------------End of the text----------------------------------------------------------

References

[1] R. Srinivasan, J. Zhuang, L. Jalloul, R. Novak, J. Park, “Project 802.16m Evaluation Methodology Document (EMD),” IEEE C80216m-08-004/r3, Oct. 2, 2008.

[2]  D.H. Lee, S.C. Kim, D.C. Park, S.S. Hwang, and Y.-I. Kim "Performance Evaluation of Multi-hop Relay System with Deployment Scenarios," in proc. MIL. Commun. Conf. , San Diego, USA, Nov. 2008.

[3]  IEEE 802.16j-06/013r3, “Multi-hop Relay System Evaluation Methodology [Channel Model and Performance Metric],” IEEE 802.16's Relay Task Group, Feb. 2007.

[4]  IEEE 802.16j-06/015, “Harmonized Contribution on 802.16j (Mobile Multihop Relay) Usage Models,” IEEE 802.16's Relay Task Group, Sep. 2006.