| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Rate Compatible LDPC coding for IEEE 802.16m Amendment |  |
| Date | 2008-09-07 |  |
| Source(s) | Wataru Matsumoto, Toshiyuki Kuze Mitsubishi Electric Corporation 5-1-1 Ofuna Kamakura, Kanagawa 2478501, JAPAN Philip V. Orlik, Andreas F. Molisch, Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139, USA | Matsumoto.Wataru@aj.MitsubishiElectric.co.jp kuze.Toshiyuki@ah.MitsubishiElectric.co.jp {porlik, molisch, tao, jzhang}@merl.com |
| Re: | “802.16m amendment text”: IEEE 802.16m-08/042, “Call for Contributions on Project 802.16m Draft Amendment Content”. Target topic: “11.13 Channel Coding and HARQ”. |  |
| Abstract | Proposed Text of LDPC coding with rate matching for the IEEE 802.16m Amendment |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Rate Compatible LDPC coding  for IEEE802.16m Amendment

Wataru Matsumoto ,Toshiyuuki Kuze

Mitsubishi Electric Corp

Philip V. Orlik, Andreas F. Molisch, Zhifeng (Jeff) Tao, Jinyun Zhang

MERL

# 1. Introduction

One of the most important advantages of LDPC is its higher decoding efficiency compared to Turbo codes. Therefore, a high throughput link such as BS-MS could be achieved with lower hardware cost.  Unfortunately, current 802.16e LDPC is not rate compatible and does not support 1/3 code rate.  In addition, the current LDPC only support HARQ with chase combining but not incremental redundancy (IR).

1/3 code rate and IR HARQ are known to improve reliability or robustness in links with hostile channel conditions.

We proposed an enhanced version of 802.16 LDPC using the current 802.16e LDPC as the baseline.  We called rate compatible RC-LDPC will support 1/3 rate code and also provide the means to support HARQ with incremental redundancy to provide the BS-MS  links operating in hostile channel condition, with high throughput, improved robustness and increased decoding efficiency.

# 2. Summary of Proposal

We proposed the construction of an extended parity-check matrix of the RC (Rate-Compatible)-LDPC codes to achieve good performance for wide range of code word length and code rate. The parity-check matrix construction method can support code rates ranging from 1/3 to 4/5 and codeword lengths ranging from 288 bits to  14400bits. The RC-LDPC codes are constructed using a code-rate 1/2 parity-check matrix for code-rate greater than or equal to 1/2. For code rate smaller than ½ an additional parity check matrix is being extended to the ½ parity-check matrix.

The proposed RC-LDPC uses the current 802.16 LDPC as the baseline matrix.  It also uses the same four sub-packets protocol for its HARQ operation.  The proposal provides some small additions and requires no change to the current 802.16e LDPC.

# 3. Performance Results

We show the complexity comparison between LDPC and CTC decoder in table 1 and the performance of the proposed RC-LDPC codes as shown in figures 1 and 2. As for the complexity comparison, under the condition of same constraint length as the 3GPP turbo code in [1] [2], the total cost of CTC is twice than that of 3GPP turbo codes. So, from the reference paper [2], we calculate the cost value of LDPC vs CTC as shown in table 1.

Table 1:Operations count comparison of sub-optimal decoders LDPC and CTC decoders

|  | LDPC | CTC | Complexity of LDPC / Complexity of CTC |
| --- | --- | --- | --- |
| Algorithm | LBP Min-Sum+Offset | Max Log Map +extrinsic scaling |  |
| Number of Iterations | 20 | 8 |  |
| Total cost (R=1/2) | 28.8K x 20 = 576K | 171K x 8 x 2 = 2736K | 21% |
| Total cost (R=3/4) | 20.6K x 20 = 412K | 171K x 8 x2 = 2736K | 15% |

Table 2 gives the parameters for simulation.

Table 2 Parameters

| Parameter | Value |
| --- | --- |
| Coding rate | 1/3,1/2,2/3,3/4,5/6 |
| Modulation scheme | QPSK |
| Channel model | AWGN |
| Decoding scheme | LDPC: Log-BP MaxIterner=30 |

Figure 1: Performance for RC LDPC codes based on the 16e LDPC codes

Figure 2: Performance of RC LDPC codes based on the 16e LDPC

The codeword with less than code-rate 1/3 can be constructed by cyclically repeated from the rate 1/3 codeword.

Figure 2 show the performance comparison between a IR code (cyclically repeated code) and a CC code (simple repeat code).

The IR code with rate 1/4 (cyclically repeat from rate 1/3 code) out performs the CC code with rate 1/4(simple repeat of rate 1/2 code) by 0.5dB. So we should make low-rate code by RC-LDPC codes based on the 1/3 code.

# 4. Conclusions

LDPC support high throughput with less hardware complexity and lower cost compared to Turbo Codes.  Our RC-LDPC is an enhanced version of the current 802.16e LDPC.  It uses the 802.16e LDPC as a baseline.  RC-LDPC is rate compatible and can provide 1/3 code rate and Hybrid ARQ with incremental redundancy. For operation in very hostile channel conditions as shown in Figure 3, such as those encountered by mobile MS or non-LOS MS and nomadic MS, RC-LDPC would be able to provide improved robustness to the BS-MS links.

Figure 6: RC-LDPC is best for hostile channel conditions

[References]

[1] TS 25.222 V3.1.1 multiplexing and channel coding (FDD), 3GPP TSG RAN WG1, Dec. 1999

[2] R1-060874, Intel ,ITRI, LGE, Mitsubishi, Motorola, Samsung, ZTE,”Complexity Comparison of LDPC Codes and Turbo Codes”, 3GPP TSG RAN WG1 #44bis, Athens, Greece, Mar 27-31, 2006.

# 5. Text proposal for inclusion in the 802.16m amendment

===================Start of Proposed Text====================
15.13.X Channel Coding and HARQ

15.13.X.X LDPC Code structure and code description

15.13.X.X.1 Basic description of structured LDPC codes

##### Based on 8.4.9.2.5.1 Code description on P802.16Rev2/D2(December 2007)

Insert new text at end of the subsection.

We propose the parity check matrix for rate-compatible LDPC(RC-LDPC) codes using the parity-check matrix for rate-1/2 specified in the 802.16e as the following Figure 4.

Extend for Code Rate=1/3

Figure 4 Parity check matrix for rate compatible LDPC codes

Let u and r be information bits as u=u1u2⋯uK and parity bits as r=r1r2⋯rM, respectively.  And Let v be a systematic codeword such that:

v:=u1u2⋯uKr1r2⋯rM. satisfying HM⋅vT=0.

The RC-LDPC encoder consists of a common LDPC encoder and a puncturing device that punctures the parity bits. The systematic bits are not punctured. The decoder for RC-LDPC codes is the same as an ordinary LDPC decoding algorithm with received LLR=0 for puncturing bits.

A set of code rates and un-puncturing parity bits set r for RC-LDPC codes can be represented by:

For the number of un-puncturing bits i(≤M2), r=rl,

For p=0 to 11 { c(p)=0 }

For p=0 to 11 {

For  q=0 to z−1 {

wj,c(j)=rk+1  where j=(p⋅z+q)mod8, k=p⋅z+((p+q)mod8),

c(j)=c(j)+1

}

}

l=0,4,2,6,1,5,3,7,k=1

For p=0 to 7 {

For q=0 to c(p)−1 {

rk=wl(p),q,

If k=i then exit

k=k+1

}

}

For the number of un-puncturing bits i(>M2),

r=rl,  for M2≤l≤i,

where, l is a vector such that l=l(0),l(1),...,l(i),...,l(7),0≤i≤7.

15.13.X.X.2 Code block size adjustment

##### Based on 8.4.9.2.5.2 Code block size adjustment on P802.16Rev2/D2(December 2007)

The LDPC code flexibly supports different block sizes for each code rate through the use of an expansion factor.

Each base model matrix has nb=24 columns, and the expansion factor (z factor) is equal to n/24 for code length n. In each case, the number of information bits is equal to the code rate times the coded length n.

Insert into  RC-LDPC block Sizes and the number of subchannels.

The code length N shall cover from 576 bits to 14400 bits for the RC-LDPC code. The supported code-rate shall be equal to and more than 1/3.

The number of slots shall be calculated as follows,

If N (bits) is a multiple of 96,

QPSK can be used as Nsubchannels = N/96,

else if N (bits) is a multiple of 96x2,

16QAM can be used as Nsubchannels = N/(96x2),

else if N (bits) is a multiple of 96x3,

64QAM can be used as Nsubchannels = N/(96x3),

end.

====================End of Proposed Text====================