| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink OL SU-MIMO Amendment Text |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Kiran Kuchi, J Klutto Milleth, Vinod R, Dileep M K, Divagar , Padmanabhan M S, Bhaskar R, Giridhar K ¦ CEWiT, India | E-mail: kkuchi@cewit.org.in, klutto@cewit.org.in |
| Re: | In response to the TGm Call for Contributions and Comments IEEE 802.16m-08/053r1. 802.16m amendment working document Section 11.8 DLMIMO |  |
| Abstract | This contribution proposes the amendment text for DL OL SU-MIMO |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment during Session #59 meeting in San Diego. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Downlink OL SU-MIMO Amendment Text

Kiran Kuchi, J Klutto Milleth, Vinod R, Dileep M K,

Divagar , Padmanabhan M S, Bhaskar R, Giridhar K

CEWiT, India

# Introduction

# This contribution proposes amendment text for the following downlink OL SU-MIMO schemes: rank-1 precoding for 2,4,8 Tx antennas, SFBC+rank-1 precoding for 4,8 Tx antennas, SM+rank-1 precoding for 4,8 Tx antennas. The system model is described in more detail in the Appendix. Simulation results comparing different code books are given in C80216m-09_0219.ppt.

# Proposed Amendment Text

----------------------------------------------------------- Text Start---------------------------------------------------------

11.8.2.1.1

System model

y = W(p,q) . z

y = The output of the precoder

W=Precoder  (NT × NS)

z  = Data matrix as defined in SDD

NT = Number of Tx antennas

NS = Number of columns of the precoder

The notation (p,q) denotes the index of the physical resource unit (PRU) in two dimensional frequency-time plane, where ‘p’ denotes PRU index along frequency, and ‘q’ denotes PRU index along time dimension.  Each PRU is composed of P subcarriers and Q OFDM symbols.

Precoder definition

W(p,q)=W(p).W(q)

where W(p).W(q) is the element wise multiplication of W(p) and W(q).

Precoder for 2-Tx rank-1 precoding

W(p)=C1(m), where m=mod(p,M1)+1 where M1 is the size of code book C1

W(q)=C2(n), where n=mod(q,M2)+1 where M2 is the size of code book C2

The entries of the code book C1 for M1=4 are given by:

C1(1)=1211, C1(2)= 121−1, C1(3)= 121j, C1(4)= 121−j

The entries of the code book C2 for M2=2 are given by:

C2(1)=11, C2(2)= 1−1

Precoder for 4-Tx rank-1 precoding

W(p)=C1(m), where m=mod(p,M1)+1 where M1 is the size of code book C1

W(q)=C2(n), where n=mod(q,M2)+1 where M2 is the size of code book C2

The entries of the code book C1 for M1=4 are given by:

C1(1)=141111, C1(2)= 141−11−1, C1(3)= 1411−1−1, C1(4)= 141−1−11

The entries of the code book C2 for M2=4 are given by:

C2(1)=141111, C2(2)= 1411−1−1, C2(3)= 141−11−1, C2(4)= 141−1−11

Precoder for 8-Tx rank-1 precoding

W(p)=C1(m), where m=mod(p,M1)+1 where M1 is the size of code book C1

W(q)=C2(n), where n=mod(q,M2)+1 where M2 is the size of code book C2

The entries of the code book C1 for M1=8 are given by:

C1(1)= 1811111111, C1(2)= 181−11−11−11−1, C1(3)= 1811−1−111−1−1, C1(4)= 181−1−111−1−11, C1(5)= 181111−1−1−1−1, C1(6)= 181−11−1−11−11, C1(7)= 1811−1−1−1−111, C1(8)= 181−1−11−111−1

The entries of the code book C2 for M2=8 are given by:

C2(1)= 1811111111, C2(2)= 181111−1−1−1−1, C2(3)= 1811−1−111−1−1, C2(4)= 1811−1−1−1−111, C2(5)= 181−11−11−11−1, C2(6)= 181−11−1−11−11, C2(7)= 181−1−111−1−11, C2(8)= 181−1−11−111−1

Precoder for 4-Tx SFBC+rank-1/SM+rank-1 precoding

W(p)=C1(m), where m=mod(p,M1)+1 where M1 is the size of code book C1

W(q)=C2(n), where n=mod(q,M2)+1 where M2 is the size of code book C2

The entries of the code book C1 for M1=4 are given by:

C1(1)= 1410011001, C1(2)= 141001−100−1, C1(3)= 141001j00j, C1(4)= 141001−j00−j

The entries of the code book C2 for M2=2 are given by:

C2(1)=11111111, C2(2)= 1111−1−1−1−1

Precoder for 8-Tx SFBC+rank-1/SM+rank-1 precoding

W(p)=C1(m), where m=mod(p,M1)+1 where M1 is the size of code book C1

W(q)=C2(n), where n=mod(q,M2)+1 where M2 is the size of code book C2

The entries of the code book C1 for M1=4 are given by:

C1(1)= 181001100110011001, C1(2)= 181001−100−11001−100−1, C1(3)= 1810011001−100−1−100−1, C1(4)= 181001−100−1−100−11001

The entries of the code book C2 for M2=4 are given by:

C2(1)= 1111111111111111, C2(2)= 11111111−1−1−1−1−1−1−1−1, C2(3)= 1111−1−1−1−11111−1−1−1−1, C2(4)= 1111−1−1−1−1−1−1−1−11111

------------------------------------------------------------Text End---------------------------------------------------------

# Appendix

The system model

y = W(p,q) . z

y = The output of the precoder

W=Precoder  (NT × NS)

z  = 'data matrix as defined in SDD '

NT = number of Tx antennas

NS = number of columns of the precoder

The notation (p,q) denotes the index of the physical resource unit (PRU) in two dimensional frequency-time plane, where ‘p’ denotes PRU index along frequency, and ‘q’ denotes index in time dimension.  Each PRU is composed of P subcarriers and Q OFDM symbols.

Rank-1 Precoding for 2Tx antennas (NT =2)

NS = 1;

W(p,q) = [1  ejθ(p,q)]T

θ(p,q) = the phase which is constant in a RB. It can be varied over time/frequency. The choice of the values of θ(p,q) is chosen from the set {0,π/2,π,3π/2} in a specific order to maximize the diversity benefit for both 1D and 2D allocations.

Rank-1 Precoding for 4Tx antennas (NT =4)

Ns = 1.

W(p,q) = [w1 w2 w3 w4]T .  The elements are chosen from hadamard sequence.

| w1 | w2 | w3 | w4 |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 1 |
| 1 | -1 | 1 | -1 |
| 1 | 1 | -1 | -1 |
| 1 | -1 | -1 | 1 |

The assignment of different codewords to RBs can be done either in 1D (freequency only) or in 2D (frequency and time).The precoder W(p,q) is chosen to maximize the diversity benefit for both 1D and 2D allocations.

Rank-1 Precoding for 8Tx antennas (NT =8)

Ns = 1.

W(p,q) = [w1 w2 w3 w4 w5 w6 w7 w8]T .  The elements are chosen from 8 X 8 hadamard matrix. The precoder W(p,q) is chosen to maximize the diversity benefit for both 1D and 2D allocations.

4-Tx SFBC/SM+ Rank-1 Precoding

Ns = 2;      The 4×2   W(p,q) matrix is as shown:

| 1 | 0 |
| --- | --- |
| 0 | 1 |
| ejθ(p,q) | 0 |
| 0 | ejθ(p,q) |

z = [s1 s2*] T  or [s2 -s1*] T   for SFBC and

z = [s1 s2] T   for SM.

θ(p,q) = the phase which is constant in a RB. It can be varied over time/frequency. The choice of the values of θ(p,q) is made from the set {0,π/2,π,3π/2} in a specific order to maximize the diversity benefit for both 1D and 2D allocations.

8-Tx SFBC/SM+ Rank-1 Precoding

Ns = 2;      The 8×2   W(p,q) matrix is as shown in Table below:

| w1 | 0 |
| --- | --- |
| 0 | w1 |
| w2 | 0 |
| 0 | w2 |
| w3 | 0 |
| 0 | w3 |
| w4 | 0 |
| 0 | w4 |

z = [s1 s2*] T  or [s2 -s1*] T   for SFBC and

z = [s1 s2] T   for SM.

The elements [w1 w2 w3 w4] are chosen from the columns of the  4×4 Hadamard matrix. The precoder W(p,q) is chosen to maximize the diversity benefit for both 1D and 2D allocations.