04

| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to Synchronization Channel Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Zheng Yan-Xiu, Tsung-Pao Chen, Yu-Tao Hsieh, Pang-An Ting, Richard Li ITRI | Voice : E-mail: zhengyanxiu@itri.org.tw |
| Re: | IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document” |  |
| Abstract | This contribution proposes the text of synchronization channel section for the 802.16m amendment. |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to Synchronization Channel Section for the IEEE 802.16m Amendment

Zheng Yan-Xiu, Tsung-Pao Chen, Yu-Tao Hsieh, Pang-An Ting, Richard Li

ITRI

# Introduction

The proposed text for the synchronization channel (SCH) sections in the IEEE 802.16m Amendment is included in this contribution.

# Proposed Descriptions

### 2.1. Index mapping of P-SCH

The series (Wk) used for the P-SCH modulation series is defined in Table X-1.

According to the SDD, Primary synchronization channel (P-SCH) should contain partial cell ID information and limited signaling, and a complete cell ID should be transmitted in Secondary synchronization channel (S-SCH).

The BS type and sector information are discussed as a part of cell ID information, and system bandwidth and carrier information are considered as limited signaling. Therefore, the index equation, which maps sector information, BS type, system bandwidth, and carrier information into P-SCH index,, is proposed as follows:

,		(Eq. 1)

where

- denotes the sector index with 0, 1, and 2.

- NBSType denotes the type of BSs with 0, and 1. The indices 0, and 1 are mapped to femto, and Macro BSs, respectively.

- denotes the bandwidth indication with 0, 1, and 2, and thus, the indices 0, 1, and 2 represent 512-FFT, 1024-FFT, and 2048-FFT, respectively.

- represents the carrier type whether this carrier is a partially-configured carrier, i.e., NCarrier=0, or a fully-configured carrier, i.e., NCarrier=1.

Table X-1 P-SCH modulation series [TBD]

| IDP−SCH | Segment | BS type | Bandwidth | Carrier type | Series to modulate (Wk) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | Macro | 512-FFT | Fully-configured | … |
| 1 | 1 | Macro | 512-FFT | Fully-configured | … |
| 2 | 2 | Macro | 512-FFT | Fully-configured | … |
| 3 | 0 | Macro | 1024-FFT | Fully-configured | … |
| 4 | 1 | Macro | 1024-FFT | Fully-configured | … |
| 5 | 2 | Macro | 1024-FFT | Fully-configured | … |
| 6 | 0 | Macro | 2048-FFT | Fully-configured | … |
| 7 | 1 | Macro | 2048-FFT | Fully-configured | … |
| 8 | 2 | Macro | 2048-FFT | Fully-configured | … |
| 9 | 0 | Femto | 512-FFT | Fully-configured | … |
| 10 | 0 | Femto | 1024-FFT | Fully-configured | … |
| 11 | 0 | Femto | 2048-FFT | Fully-configured | … |
| 12 | The same as Fully-configured |  |  | Partially-configured | … |

### 2.2. Proposed Synchronization Channel Architecture

The proposed SCH architecture includes P-SCH and S-SCH. The superframe timing synchronization, radio frame timing synchronization, and some system information are obtained in P-SCH. S-SCH is responsible for radio frame timing synchronization and a complete cell ID information. Figure 1 shows the proposed SCH architecture.

Figure 1 SCH architecture for 16m-only mode

# Text proposal for the 802.16m amendment

============================== Start of Proposed Text =================================

[Adopt the following modifications:]

# 15.3.7 DL Control Structure

# 15.3.7.1 DL Control Information Classification

# 15.3.7.2 Transmission of DL Control Information

## 15.3.7.2.1 Synchronization Channel (SCH)

The synchronization channel (SCH) is a DL physical channel which provides a reference signal for time, frequency, and frame synchronization, RSSI estimation, channel estimation, and BS identification.

Two levels of synchronization hierarchy exist. These are called the primary synchronization channel (P-SCH) and secondary synchronization channel (S-SCH).

The P-SCH transmits one of [13] unique identifications to support the acquisition of physical cell/sector identifications transmitted in S-SCH.

The S-SCH transmits one of [512] complete physical cell/sector identifications.

Figure X-1 Transmission of SCH symbols

### 15.3.7.2.1.1 Primary Synchronization Channel (P-SCH)

The P-SCH is used for symbol synchronization, frame synchronization, bandwidth identification, sector identification, partially configurable carrier identification, cell type identification.

#### 15.3.7.2.1.1.1 P-SCH Modulation Series

The P-SCH depends on the sector information, BS type, system bandwidth, and carrier information. [Note: The section 15.3.X.Y should include modulation and power boosting of P-SCH.]

The series (Wk) used for the P-SCH modulation series is defined in Table X-1.

The equation (X-1) defines the mapping rule of the sector information, system bandwidth, and carrier information into P-SCH index, IDP−SCH as follows:

,		(X-1)

where

- denotes the sector index with 0, 1, and 2.

- NBSType denotes the type of BSs with 0, and 1. The indices 0, and 1 are mapped to femto, and Macro BSs, respectively.

- denotes the bandwidth indication with 0, 1, and 2, and thus, the indices 0, 1, and 2 represent 512-FFT, 1024-FFT, and 2048-FFT, respectively.

- represents the carrier type whether this carrier is a partially-configured carrier, i.e., NCarrier=0, or a fully-configured carrier, i.e., NCarrier=1.

[Note: The expression of P-SCH series can be either equations or tables.]

Table X-1 P-SCH modulation series [TBD]

| IDP−SCH | Segment | BS type | Bandwidth | Carrier type | Series to modulate (Wk) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | Macro | 512-FFT | Fully-configured | … |
| 1 | 1 | Macro | 512-FFT | Fully-configured | … |
| 2 | 2 | Macro | 512-FFT | Fully-configured | … |
| 3 | 0 | Macro | 1024-FFT | Fully-configured | … |
| 4 | 1 | Macro | 1024-FFT | Fully-configured | … |
| 5 | 2 | Macro | 1024-FFT | Fully-configured | … |
| 6 | 0 | Macro | 2048-FFT | Fully-configured | … |
| 7 | 1 | Macro | 2048-FFT | Fully-configured | … |
| 8 | 2 | Macro | 2048-FFT | Fully-configured | … |
| 9 | 0 | Femto | 512-FFT | Fully-configured | … |
| 10 | 0 | Femto | 1024-FFT | Fully-configured | … |
| 11 | 0 | Femto | 2048-FFT | Fully-configured | … |
| 12 | The same as Fully-configured |  |  | Partially-configured | … |

#### 15.3.7.2.1.1.2 Transmission (Mapping) of P-SCH series

The PCH sequence is located at the center frequency of a carrier and the bandwidth is 5 MHz. In time domain, the P-SCH symbol has two repeated waveforms. In frequency domain, the P-SCH sequence is placed on [odd|even] subcarriers. The subcarriers on the guardband carry null symbols.

============================== End of Proposed Text =================================