| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections to some formulas in “Downlink Physical Structure” Section of IEEE 802.16m Amendment Document |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Sudhir Ramakrishna | sudhir.r@samsung.com |
| Re: | “802.16m amendment working document”: IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document”. |  |
| Abstract | This contribution proposes corrections to some formulas in Section 15.3.5 (“Downlink Physical Structure”) in Contribution IEEE 802.16m-08/50, “IEEE 802.16m Amendment Working Document”. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Corrections to some formulas in Section 15.3.5 (“Downlink Physical Structure”) of the IEEE 802.16m Amendment Working Document

Sudhir Ramakrishna

Samsung Telecommunications America

- Introduction

In contribution [1], IEEE 802.16m-08/50, “IEEE 802.16m Amendment Working Document”, in addition to other sections, text is proposed for Section 15.3.5, “Downlink Physical Structure”. This contribution proposes corrections to Formula 178 in Section 15.3.5.2.3, and the formulas on Lines 25 & 26 on Page 29 of Section 15.3.5.3.1, which are incomplete/erroneous.

- Corrections to Formula 178 in Section 15.3.5.2.3, and to the Formulas on Lines 25/26 on Page 29 in Section 15.3.5.3.1

- Formula 178 in IEEE 802.16m-08/50 should be replaced by the formula below.

(178)

where

and

- The formulas on Lines 25/26 on Page 29 of Section 15.5.3.5.1 should be replaced by the following:

- References

[1] IEEE 802.16-08/050, “IEEE 802.16m Amendment Working Document”, December 11, 2008.