| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on SCH for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Robert Novak, Mo-Han Fong, Sophie Vrzic, Dongsheng Yu, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan Nortel Networks | rnovak@nortel.com mhfong@nortel.com |
| Re: | 802.16m amendment working document In response to IEEE 802.16m-08/053r1 “Call for Contributions and Comments on Project 802.16m Amendment Working Document” for Session 59 |  |
| Abstract | Contribution with proposed text on the SCH for the IEEE 802.16m Amendment (IEEE 802.16m-08/050) |  |
| Purpose | Propose to be discussed and adopted by TGm for the use in Project 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on SCH for the IEEE 802.16m Amendment

Robert Novak , Mo-Han Fong, Sophie Vrzic, Dongsheng Yu, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan

Nortel Networks

- Introduction

This contribution presents text on the synchronization channel (SCH) for IEEE 802.16m. The text is based on the section text of the IEEE 802.16m SDD (IEEE 802.16m-08/003r6).

- Text Proposal

--------------------------------------------------------Start of the Text-----------------------------------------------------------

[ Add the text below at line 22, page 30 of IEEE 802.16m-08/050]

### DL Control Structure

#### Synchronization Channel (SCH)

The synchronization channel (SCH) is a DL physical channel which provides a reference signal for time, frequency, and frame synchronization, RSSI estimation, channel estimation, and ABS identification.

##### Hierarchy

The SCH shall consist of the primary synchronization channel (P-SCH) and secondary synchronization channel (S-SCH). The P-SCH shall used for initial acquisition, superframe synchronization and sending additional information. The S-SCH shall be for fine synchronization, and cell/sector identification (ID).

##### Location and multiplexing of synchronization symbols

P-SCH and S-SCH shall be transmitted in different symbols of the same subframe..The SCH shall be located in the first subframe of the superframe, and shall be transmitted in each superframe

The timing of the SCH is indicated in Figure 1.

Figure 1 Timing of IEEE 802.16m SCH symbols in16m-only mode

##### P-SCH & S-SCH

The AMS shall the Cell ID by concatenation of the P-SCH_ID ( 5 bits) and S-SCH_ID (variable). The S-SCH is padded with zeros to the maximum length of the field.

Table 1 – P-SCH ID logical sequence index

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Cell Class | 2 | 0b00=femto cell 0b01=micro cell 0b10=macro cell 0b11=reserved |
| Cell Group ID | 3 |  |

Cell Class – This field indicates the class of the cell.

Cell Group ID – This field indicates the ID for local group of cells of the same class.

The P-SCH sequence shall be mapped to subcarriers n, n+2, n+4 …N of the symbol for 384 subcarriers. Frequency reuse of 1 is applied to P-SCH.

Table 2 – S-SCH ID logical sequence index

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| S-SCH Mapping | 2 | 0b00= Subcarrier mapping from n 0b01= Subcarrier mapping from n+1 0b10= Subcarrier mapping from n+2 0b11= not used |
| Local_Cell ID | variable | 7 bits for 5 MHz 8 bits for 10 MHz 10 bits for 20 MHz |

S-SCH Mapping  - The S-SCH shall mapped to subcarriers n, n+3, n+6 …, or n+1, n+4, or n+2, n+5 … for all subcarriers of the symbol according to the S-SCH_Mapping field. Frequency reuse of 3 is applied to S-SCH.

Local_Cell ID -  This field shall indicate the ID for the cell within the local group of cells.

The power of synchronization channels may be boosted.

-----------------------------------------------------------End of the Text---------------------------------------------------------