| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Subblock interleaver for CTC and Symbol interleaver |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Isamu Yoshii Kenichi Kuri | Voice: +81-50-3687-6548 E-mail: yoshii.isamu@jp.panasonic.com |
| Re: | 802.16m amendment working document |  |
| Abstract | Amendment text on symbol interleaver |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Sub-block interleaver for CTC and Symbol interleaver

Isamu Yoshii, Kenichi Kuri

Panasonic Corporation

- Introduction

In SDD, the sub-block interleaver details are FFS as following.

------------------------------------------------------SDD Text ----------------------------------------------------------------

The encoder block depicted in Figure 53 includes the sub-block interleavers. The interleaving details are FFS.

-------------------------------------------------------------------------------------------------------------------------------------

Here, we point out the problem of  mapping 16 QAM/64QAM based on 16e sub-block interleaver and propose to delete Sub-block interleaver and introduce symbol interleaver.

- Analysis on the mapping based on subblock interleaver for CTC

Figure 1 Subblock interleaver for CTC

Figure 1 shows the mapping of the parity bit sequence in case of 16 QAM transmission in case of  Nep=48, Sub-block parameter, m=3, J=3. The dark blue bit is assigned at the higher reliable position in 16QAM, the light blue bits is the lower reliable bits. For analyzing the distribution of the reliable bits for original parity sequence, the de-interleaved bits sequence is shown. In de-interleaved bits, 4 or 8 consecutive higher reliable bits and 4 or 8 lower reliable bits are allocated alternately. This allocation causes the performance degradation for Turbo decoding. We observe same problem in other parameter case like 16QAM case,(Nep=480,  m=7, J=2), 64QAM cae (Nep=384, m=6, J=3) etc. So, it is necessary to modify the sub-block interleaver for higher order modulation case.

- Sub-block interleaver and symbol interleaver

In order to avoid the continuous mapping of higher/lower reliable bits, we propose to delete the sub-block interleaver as Figure 2.

Figure 2 Subblock interleaver

- LLS comparison

Table 1 shows the LLS parameter and in Figure 3-4, the comparison between 16e interleaver case and proposal(non-subblock IL) are shown. The proposal outperforms 16e interleaver case in AWGN and fading case. Also, in fading case, we can observe the diversity gain of symbol interleaver.

Table 1 LLS parameter

Figure 3 AWGN (64QAM)

Figure 4 TU 6path (64QAM)

- Coding Chain

Figure 5 shows the detail of coding chain including symbol interleave part and deleting 16e sub-block interleaver.

Figure 5 Coding Chain for SCW(2 streams)

- Conclusion

We propose following text for amendment text on symbol interleaver.

---------------------------- Begin Proposed Text -------------------------------------------------------------------------

11.13.x.y  Symbol Interleaver

The modulated symbol per stream shall be interleaved separately. The interleaving is performed by the unit of symbols. The sequence of interleaver output symbols for each stream is shown in the table yyy.

Table yyy

| Symbol Interleaver size | Interleave Pattern |
| --- | --- |
| TBD | TBD |

---------------------------End of Text Proposal ---------------------------------------------------------------------------

Reference

[1] C802.16m-08/1342r2　 A new constellation permutation scheme for channel coding

[2] IEEE 802.16m System Description Document [draft], IEEE 802.16m-08/003r6

[3] IEEE 802.16Rev2-D7