| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications on section 15.3.5.3.1 and 15.3.5.3.2 of 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Xin Qi, Shaohua Li, Chao Wei Nokia Siemens Networks | E-mail: xin.qi@nsn.com shaohua.li@nsn.com |
| Re: | 802.16m amendment working document Target topic: “Comments on Downlink Physical Structure”. |  |
| Abstract | The contribution proposes modifications on IEEE 802.16m amendment working document IEEE 80216m-08_050. |  |
| Purpose | To be incorporated into the IEEE 802.16 amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications on section 15.3.5.3.1 and 15.3.5.3.2 of 802.16m Amendment

- 15.3.5.3.1. CRU/DRU allocation

The partition between CRUs and DRUs is done on a sector specific basis. DRU allocation is signaled in two step process.

There are four possible pre-configured allocations signaled in two bits:

00 --  All PRUFPis are allocated to DRUs
01 --  All PRUFPis are allocated to CRUs
10 --  All subband PRUs are allocated to CRUs and all miniband PRUs are allocated to DRUs
11 --  The mapping is signaled explicitly

When explicit mapping is indicated, additional 4-bit (TBD) CRU allocation size (CAS) field is sent in the BCH for each allocated frequency partition. CASi indicates the number of allocated CRUs for partition FPi in the unit of subband size.

The number of CRUs in each frequency partition is denoted by LCRU,FPi., where

- LCRU,FPi = CASi*N1 for 0 ≤ i < FPCT

- The number of DRUs in each frequency partition is denoted by LDRU,FPi, where

LDRU,FPi = FPSi*N2 - CASi*N1 for 0 ≤ i < FPCT

If , the mapping of PRUFPi to CRUFPi is based on following equation

If , the mapping of PRUFPi to CRUFPi is based on following equation

where .  is the CRU/DRU-allocation sequence (TBD), and . The remainders of the PRUs in the frequency partition are allocated for DRUFPi and renumbered from 0 to LDRU,FPi-1.

Figure xxxx illustrates the CRU/DRU-allocation and subcarrier permutation, CASi=2, FPSi=6.

- 15.3.5.3.2. Subcarrier permutation

Figure xxxx --- CRU/DRU-allocation and subcarrier permutation, CASi=2, FPSi=6