| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of UL Pilot Patterns Sub-Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Jong-Kae (JK) Fwu, Minh-Anh Vuong, Yuval Lomnitz, Huaning Niu, Rongzhen Yang, Wei Guan, Furuzan Atay, Sassan Ahmadi, Hujun Yin Intel Corporation Jerry Pi, Taeyoung Kim, Jeongho Park, David Mazzarese, Heewon Kang, Kaushik Josiam, Sudhir Ramakrishna, Hai Wang Samsung Electronics | E-mail: Jong-kae.fwu@intel.com |
| Re: | IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document” Target topic: “11.6 Uplink Physical Structure”. Target topic: Call for Comment on Amendment Working Document Text (IEEE 802.16m-08/050). |  |
| Abstract | The contribution proposes the text of uplink pilot patterns sub-section (11.6.3) to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment Working Document.. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Uplink Pilot Patterns Sub-Section

for the IEEE 802.16m Amendment

Jong-Kae (JK) Fwu, Minh-Anh Vuong, Furuzan Atay, Yuval Lomnitz, Huaning Niu, Sassan Ahmadi, Hujun Yin

Intel Corporation

Jerry Pi, Taeyoung Kim, Jeongho Park, David Mazzarese, Heewon Kang, Kaushik Josiam, Sudhir Ramakrishna, Hai Wang

Samsung Electronics

# Introduction

# This contribution proposes the text of uplink pilot patterns sub-section to be included in the 802.16m amendment. The proposed text is developed so that it is compliant to the 802.16m SRD [2] and the 802.16m SDD [1] and follows the style and format guidelines in [3].

# Modifications to the SDD [1] text

The text proposed in this contribution is based on subclause 11.6 in the IEEE 802.16m SDD ‎[3]. The modifications to the SDD text are summarized below:

- Replace the figure that illustrates the UL pilot structure intended for contiguous LRUs in cases of 1, 2, 3 or 4 UL streams.

- Clean up the text of Section 15.3.6.4.

# References

- IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”.

- IEEE 802.16m-07/002r6, “802.16m System Requirements”.

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”.

# Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------- Text Start---------------------------------------------------------

## Advanced Air Interface

### Physical Layer

#### Uplink Physical Structure

##### Pilot Structure

Uplink pilot is dedicated to each user and can be precoded or beamformed in the same way as the data subcarriers of the resource allocation. The pilot structure is defined for up to 4 transmission streams.

The pilot pattern may support variable pilot boosting. When pilots are boosted, each data subcarrier should have the same Tx power across all OFDM symbols in a resource block. The boosting values are TBD.

Pilot structures in 18-by-6 allocations used for contiguous LRUs and in 6-by-6 tiles for distributed LRUs are shown in Figures 1 and 2 respectively. Figure 1 shows the pilot structure for contiguous LRUs where the number of streams is one, two, three or four. Note that the pilot patterns for UL contiguous LRUs are same as in the downlink case. Figure 2 shows the pilot structure for distributed LRUs where the number of streams is one or two.

Figure 1 – Pilot patterns for contiguous LRUs in cases of 1, 2, 3 or 4 TX streams

Figure 2 – Pilot patterns for distributedLRUs in cases of 1 or 2 TX streams

------------------------------------------------------------Text End---------------------------------------------------------