| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes of UL Physical Layer Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2008-11-03 |  |
| Source(s) | Jong-Kae (JK) Fwu, Minh-Anh Vuong, Yuval Lomnitz, Huaning Niu, Rongzhen Yang, Wei Guan, Furuzan Atay, Sassan Ahmadi, Hujun Yin Intel Corporation | E-mail: Jong-kae.fwu@intel.com |
| Re: | IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document” Target topic: “11.6 Uplink Physical Structure”. Target topic: Call for Comment on Amendment Working Document Text (IEEE 802.16m-08/050). |  |
| Abstract | The contribution proposes the text of uplink physical structure section (11.6) to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes of Uplink Physical Layer Section

for the IEEE 802.16m Amendment

Jong-Kae (JK) Fwu, Minh-Anh Vuong, Furuzan Atay, Yuval Lomnitz, Huaning Niu, Sassan Ahmadi, Hujun Yin

Intel Corporation

# Introduction

# This contribution proposes the text changes of UL permutation to be included in the uplink physical structure section of 802.16m amendment.

----------------------------------------------------------- Text Start---------------------------------------------------------

Text change 1:

## Advanced Air Interface

### Physical Layer

#### Uplink Physical Structure

###### 15.3.x.x Tile Permutation

Each of the DRUs of an UL frequency partition is divided into 3 tiles of 6 adjacent subcarriers over Nsym symbols. The tiles within a frequency partition are collectively inner-permuted to frequency-diversify across the allocated resources.

The inner permutation that allocates physical tiles of DRUs to logical tiles of subchannels is performed in the following manner:

Tile(s,n,t)=LDRU,FPi⋅f(n,s)+g(PermSeq(),s,n,t,ULPermBase)	Eqn. XX1

where

- Tiles(n,s,t) is the tile index of the n-th tile in the s-th distributed LRU of the t-th subframe.

- n is the tile index, 0 to 2, in a distributed LRU.

- t is the subframe index with respect to the frame s.

- s is the distributed LRU index, 0 to LDRU, FP,i-1.

- PermSeq() is the permutation sequence generated by a function or by a lookup table

- UL_PermBase is an integer value which is assigned by a management entity.

- g(PermSeq(),s,n,t,ULPermBase) is a function of n, s, t, UL_PermBase and PermSeq().

- f(n,s) is a function of n and/or s

Text change 2:

[Editor’s note: following options are proposed in 1444r2]

- f(n,s) =n     Eqn. XX2

- g(PermSeq(), s,n,t, UL_PermBase)=PermSeqLDRU,FPi;(n+s+t)modLDRU,FPi+ULPermBasemodLDRU,FPi	Eqn. XX3

Where PermSeq(L;i) is the i-th entry of the permutation sequence of length L as defined by the following equation.

PermSeq(L;i)=PermSeqTable−15.3.5.1(L;i)forL≤48PermSeqTable−15.3.5.1L2−(Lmod2)⋅(imod2);i2+L2⋅(imod2)for48<L≤96 Eqn. XX4

------------------------------------------------------------Text End---------------------------------------------------------