| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Input for Sub-carrier Alignment based on P802.16m SDD |  |
| Date Submitted | 2009-01-12 |  |
| Source(s) | I-Kang Fu, Pei-Kai Liao, Yih-Shen Chen, Kelvin Chou, Chih-Yuan Lin, Paul Cheng MediaTek Inc. Jia Lin, Junxian Mo, Hongjie Si Huawei Technologies Richard Li, Zheng Yan-Xiu, Chie-Ming Chou ITRI | IK.Fu@mediatek.com linjia@huawei.com richard929@itri.org.tw |
| Re: | IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document.” - Comments on Amendment Working Document Text |  |
| Abstract | Based on the consensus agreed by members during Session#55, sub-carrier alignment for the OFDMA signals transmitted over adjacent RF carriers will be required by IEEE 802.16m. The text proposal of this contribution is based on the current text in SDD, and the motivation is to ensure the text in Amendment Working Document will be aligned with SDD. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Input for Sub-carrier Alignment based on P802.16m SDD

I-Kang Fu, Pei-Kai Liao, Yih-Shen Chen, Kelvin Chou, Chih-Yuan Lin, Paul Cheng

MediaTek Inc.

Jia Lin, Junxian Mo, Hongjie Si

Huawei Technologies

Richard Li, Zheng Yan-Xiu, Chie-Ming Chou

ITRI

- Current Text in P802.16m SDD relative to Sub-carrier Alignment

[start from line#19, page#24 of IEEE 802.16m-08/003r6]

Multi-carrier (MC) block enables a common MAC entity to control a PHY spanning over multiple frequency channels. The channels may be of different bandwidths (e.g. 5, 10 and 20 MHz), be non-contiguous or belong to different frequency bands. The channels may be of the same or different duplexing modes, e.g. FDD, TDD, or a mix of bidirectional and broadcast only carriers. For contiguous frequency channels, the overlapped guard sub-carriers shall be aligned in frequency domain in order to be used for data transmission.

[start from line#9, page#138 of IEEE 802.16m-08/003r6]

19.2 Subcarrier Alignment for Utilization of Guard Subcarriers of Adjacent Frequency Channels

When multiple contiguous frequency channels are available, the guard sub-carriers between contiguous frequency channels can be utilized for data transmission only if the sub-carriers from adjacent frequency channels are well aligned. In order to align those sub-carriers from adjacent frequency channel, a frequency offset (Δf’) can be applied to its FA. The basic idea is shown by the example in Figure 59.

Figure 59 Sub-carrier alignment by applying a fraction of sub-carrier spacing to the FA of adjacent frequency channel

In order to utilize the guard sub-carrier for data transmission, the information of the available guard sub-carriers eligible for data transmission shall be sent to AMS. This information shall include the numbers of available sub-carriers in upper side and in lower side with respect to the DC sub-carrier of carrier.

- Proposed Text Input

--------------------------------------------------------Start of the Text------------------------------------------------------------

[add the following section title and text into the IEEE 802.16m-08/050]

15.3.3.5.2 Sub-carrier alignment for the frame structure support multi-carrier operation

When contiguous carriers are involved in multicarrier operation, the overlapped guard sub-carriers shall be aligned in frequency domain. In order to align the overlapped sub-carriers of the OFDMA signals transmitted over adjacent carriers, a permanent frequency offset (Δf’) will be applied over the original center frequency. The basic principle is shown by the example in Figure xxx.

Figure xxx Example on sub-carrier alignment by applying a fraction of sub-carrier spacing over the center frequency of the adjacent carriers

During the network entry procedure defined in [TBD], the ABS will notify the frequency offset to be applied over each carrier for sub-carrier to the AMS.

15.3.3.5.3 Data Transmission over guard sub-carriers for the frame structure support multi-carrier operation

When contiguous carriers are involved in multicarrier operation, the guard sub-carriers between contiguous frequency channels may be utilized for data transmission. During the network entry procedure defined in [TBD], the ABS will notify the information on available guard sub-carriers eligible for data transmission to the AMS.

---------------------------------------------------------End of the Text------------------------------------------------------------