| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Input for Pilot Patterns based on P802.16m SDD |  |
| Date Submitted | 2009-01-12 |  |
| Source(s) | I-Kang Fu, Chih-Yuan Lin, Pei-Kai Liao, Yih-Shen Chen, Kelvin Chou, Yu-Hao Chang, Paul Cheng MediaTek Inc. Chung-Lien Ho, Zheng Yan-Xiu, Richard Li ITRI | IK.Fu@mediatek.com richard929@itri.org.tw |
| Re: | IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document.” - Comments on Amendment Working Document Text |  |
| Abstract | Based on the consensus agreed by members during Session#56 and #57, the basic pilot patterns to be applied in IEEE 802.16m DL and UL transmission had been agreed and adopted into P802.16m SDD. This contribution proposes to forward those agreed pilot structure into the Amendment Working Document to facilitate other stage 3 proposals preparation. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Input for Pilot Patterns based on P802.16m SDD

I-Kang Fu,Chih-Yuan Lin, Pei-Kai Liao, Yih-Shen Chen, Kelvin Chou Yu-Hao Chang, Paul Cheng

MediaTek Inc.

Chung-Lien Ho, Zheng Yan-Xiu, Richard Li

ITRI

- Current Pilot Pattern Text in P802.16m SDD

11.5.3 Pilot Structure

The transmission of pilot subcarriers in the downlink is necessary for enabling channel estimation, measurements of channel quality indicators such as the SINR, frequency offset estimation, etc. To optimize the system performance in different propagation environments and applications, IEEE 802.16m supports both common and dedicated pilot structures. The categorization in common and dedicated pilots is done with respect to their usage. The common pilots can be used by all AMSs. Dedicated pilots can be used with both localized and distributed allocations.  Pilot subcarrriers that can be used only by a group of AMSs is a special case of common pilots and are termed shared pilots. The dedicated pilots are associated with a specific resource allocation, can be only used by the AMSs allocated to said specific resource allocation, and therefore can be precoded or beamformed in the same way as the data subcarriers of the resource allocation. The pilot structure is defined for up to four transmission (Tx) streams and there is a unified pilot pattern design for common and dedicated pilots. There is equal pilot density per Tx stream, while there is not necessarily equal pilot density per OFDMA symbol of the downlink subframe. Further, within the same subframe there is equal number of pilots for each PRU of a data burst assigned to one AMS.

Pattern A is used for 1 and 2 DL data streams dedicated and common pilot scenarios.

Figure 1 Pattern A for 1 or 2 pilot streams.

For the subframe consisting of 5 symbols, the last OFDM symbols in the figure is deleted.  For the subframe consisting of 7 symbols,  the first OFDM symbols in the figure is added as 7-th symbol.
The interlaced pilot patterns can be generated by cyclic shifting the base pilot pattern. The interlaced pilot patterns are shown in Figure 2 and can be optionally used by different ABSs. The use of interlaced pilot pattern is FFS. Pattern B is used for 4 data streams DL dedicated and common pilot pattern.

Figure 2 Interlaced Pattern A for 1 or 2 pilot streams.

Figure 3 Pilot Pattern B for 4 stream pilots, Pk denotes pilot for stream k.

The pilot pattern of the subframe consisting of 5 OFDM symbols is obtained by deleting the third OFDM symbol of the type-1 subframe. The pilot pattern of the type-2 sybframe is obtained by adding the third OFDM symbol of the type-1 subframe to the end of the type-1 subframe.

11.6.3 Pilot Structure

The transmission of pilot subcarriers in the uplink is necessary for enabling channel estimation, measurement of channel quality indicators such as SINR, frequency offset and timing offset estimation, etc. The uplink pilots are dedicated to localized and distributed resource units and are precoded using the same precoding as the data subcarriers of the resource allocation. The pilot structure is defined for up to 4 Tx streams with orthogonal patterns.

The pilot pattern may support variable pilot boosting. When pilots are boosted, each data subcarrier should have the same Tx power across all OFDM symbols in a resource block. The boosting values are TBD.

The DL 18x6 pilot patterns defined in Section Error: Reference source not found are used for UL 18x6 pilots, which include pilots up to 4 TX streams.

For 6-by-6 UL tile, the UL pilot pattern for DRU is shown in Figure 4

Figure 4 UL DRU tile structure.

- Proposed Text Input to P802.16m Amendment Working Document

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[add the following section titles and text into the IEEE 802.16m-08/050]

15.3.5.4 Pilot Structure

The transmission of pilot subcarriers is for enabling channel estimation, measurements of channel quality indicators. In IEEE 802.16m, both common pilot structure and dedicated pilot structure are supported. The common pilots can be used by all AMSs. Dedicated pilots can be used with both localized and distributed allocations.

The dedicated pilots are associated with a specific resource allocation, can be only used by the AMSs allocated to said specific resource allocation, and therefore can be precoded or beamformed in the same way as the data subcarriers of the resource allocation. The pilot structure is defined for up to four transmission (Tx) streams and there is a unified pilot pattern for common and dedicated pilots.

15.3.5.4.1 Pilot pattern for 1 or 2 streams

For 1 and 2 downlink data streams, the dedicated and common pilot patterns are the same as Figure x. For the subframe consisting of 5 symbols, the last OFDM symbol in the Figure x will be deleted.  For the subframe consisting of 7 symbols, the first OFDM symbols in the Figure x is added as 7-th symbol.

Figure x Pilot pattern for 1 or 2 streams, Pk denotes the pilot of stream k.

15.3.5.4.2 Interlaced pilot patterns for 1 or 2 streams

The interlaced pilot patterns are generated by cyclic shifting the pilot pattern defined in 15.3.5.4.1. The interlaced pilot patterns are shown in Figure x+1and may be optionally used by different ABSs.

Figure x+1 Interlaced pilot pattern for 1 or 2 streams, Pk denotes the pilot of stream k.

15.3.5.4.2 Pilot patterns for 4 streams

For 4 downlink data streams, the dedicated and common pilot patterns are the same as Figure x+2. For the subframe consisting of 5 symbols, the 3rd OFDM symbol in the Figure x+2 will be deleted. For the subframe consisting of 7 symbols, the 3rd OFDM symbols in the Figure x+2 is added as 7-th symbol.

Figure x+2 Pilot pattern for 4 streams, Pk denotes the pilot of stream k.

15.3.6 Uplink physical structure

15.3.6.1 Physical and logical resource unit

15.3.6.2 Multi-cell resource mapping

15.3.6.3 Cell specific resource mapping

15.3.6.4 Pilot Structure

15.3.6.4.1 Pilot pattern for 1 or 2 streams

For 1 and 2 uplink data streams, the dedicated and common pilot patterns are the same as Figure y. For the subframe consisting of 5 symbols, the last OFDM symbol in the Figure y will be deleted. For the subframe consisting of 7 symbols, the first OFDM symbols in the Figure y is added as 7-th symbol.

Figure y Pilot pattern for 1 or 2 streams, Pk denotes the pilot of stream k.

15.3.6.4.2 Interlaced pilot patterns for 1 or 2 streams

The interlaced pilot patterns are generated by cyclic shifting the pilot pattern defined in 15.3.6.4.1. The interlaced pilot patterns are shown in Figure y+1and may be optionally used by different ABSs.

Figure y+1 Interlaced pilot pattern for 1 or 2 streams, Pk denotes the pilot of stream k.

15.3.6.4.3 Pilot patterns for 1 or 2 streams with 6×6 tile size

For 1 and 2 uplink data streams with 6×6 tile size, the dedicated and common pilot patterns are the same as Figure y+2. For the subframe consisting of 5 symbols, the last OFDM symbol in the Figure y+2 will be deleted. For the subframe consisting of 7 symbols, the first OFDM symbol in the Figure y+2 is added as 7-th symbol.

Figure y+2 Pilot pattern for 1 or 2 streams with 6×6 tile size, Pk denotes the pilot of stream k.

15.3.6.4.4 Pilot patterns for 4 streams

For 4 uplink data streams, the dedicated and common pilot patterns are the same as Figure y+3. For the subframe consisting of 5 symbols, the 3rd OFDM symbol in the Figure y+3 will be deleted. For the subframe consisting of 7 symbols, the 3rd OFDM symbol in the Figure y+3 is added as 7-th symbol.

Figure y+3 Pilot pattern for 4 streams, Pk denotes the pilot of stream k.

----------------------------------------------------------End of the Text-----------------------------------------------------------