| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Synchronization Channel for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Pei-Kai Liao, Yu-Hao Chang, Chih-Yuan Lin, Ciou-Ping Wu, Paul Cheng MediaTek Inc. | pk.liao@mediatek.com yuhao.chang@mediatek.com paul.cheng@mediatek.com |
| Re: | IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document”. - DL PHY control structure |  |
| Abstract | This contribution proposes SCH text proposal for P802.16m SDD. |  |
| Purpose | Propose to be discussed and adopted by TGm for the use in Project 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Synchronization Channel for IEEE 802.16m Amendment

Pei-Kai Liao, Yu-Hao Chang, Chih-Yuan Lin, Ciou-Ping Wu, Paul Cheng

MediaTek Inc.

- Introduction

This contribution is to propose text proposal to synchronization channel section for IEEE 802.16m Amendment.

- Proposed Synchronization Channel Architecture

There are two synchronization channel (SCH) types in the proposed SCH architecture — Primary synchronization channel (P-SCH) and Secondary synchronization channel (S-SCH). P-SCH is responsible for superframe timing / radio frame timing / OFDM symbol boundary synchronization and signaling some system information such as channel bandwidth and sector information. Following two figures illustrates the proposed SCH architecture for 16m-only mode and legacy support mode.

Figure 1 SCH architecture for 16m-only mode

Figure 2 SCH architecture for legacy support mode

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add the following into the TGm Amendment Document]

15.3.7.2.1 Synchronization Channel (SCH)

The synchronization channel (SCH) is a DL physical channel which provides a reference signal for time, frequency, and frame synchronization, RSSI estimation, channel estimation, and BS identification.

Two levels of synchronization hierarchy exist. These are called the primary synchronization channel (P-SCH) and secondary synchronization channel (S-SCH).

The P-SCH transmits one of [10] unique identifications to support the acquisition of physical cell/sector identifications transmitted in S-SCH.

The S-SCH transmits one of [520] complete physical cell/sector identifications.

There are 4 OFDM symbols located every 5 ms for P-/S-SCH in a single superframe and the P-SCH and S-SCH shall share one OFDM symbol in time length as shown in Figure X-1. P-SCH and S-SCH are multiplexed by TDM inside the OFDM symbol. Each pair of P-SCH and S-SCH is located in the first DL subframe of the frame and occupies the 1st symbol position within a subframe.

Figure X-1 Advanced Air Interface SCH Architecture

### 15.3.7.2.1.1 Primary Synchronization Channel (P-SCH)

In P-SCH, frequency reuse 1 shall be applied. The time length of P-SCH is  and the occupied channel bandwidth is 5 MHz. The P-SCH is used for initial acquisition, superframe synchronization, channel estimation, and sending additional information.

15.3.7.2.1.1.1 P-SCH Modulation Series

The length of each P-SCH modulation series is [97]. The modulation and power boosting of P-SCH is FFS. The P-SCH series depends on the sector information, system bandwidth, and carrier information.

The series (Wk) used for the P-SCH modulation is defined in Table X-1. Table X-1 includes a set of series in a hexadecimal format. The value of the P-SCH modulation series is obtained by converting the series (Wk) to a binary sequence and mapping the converted sequence starting from the MSB of each symbol to the LSB. (0 mapped to +1 and 1 mapped to –1. For example, Wk = 110000010010..., and the mapping shall follow: –1 –1 +1 +1 +1 +1 +1 –1 +1 +1 –1 +1 ...)

The equation (X-1) defines the mapping rule of the sector information, system bandwidth, and carrier information into P-SCH index, IDP−SCH as follows:

,						(X-1)

where

- NSector denotes the sector index with 0, 1, and 2.

- NBW denotes the bandwidth indication with 0, 1, and 2, and thus, the indices 0, 1, and 2 represent 512-FFT, 1024-FFT, and 2048-FFT, respectively.

- NCarrier represents the carrier type whether this carrier is a fully-configured carrier, i.e., NCarrier=0, or a partially-configure carrier, i.e., NCarrier=1.

Table X-1 P-SCH modulation series [TBD]

| IDP−SCH | Series to modulate (Wk) |
| --- | --- |
| 0 | … |
| 1 | … |
| 2 | … |
| 3 | … |
| 4 | … |
| 5 | … |
| 6 | … |
| 7 | … |
| 8 | … |
| 9 | … |

15.3.7.2.1.1.2 Transmission of P-SCH Series

The subcarrier spacing of P-SCH is four times larger than that of a regular data OFDM symbol. Each subcarrier is modulated using a boosted BPSK modulation with a specific series defined in 15.3.7.2.1.1.1. After inverse FFT, the time domain samples are duplicated into two copies to form one P-SCH. In other words, the P-SCH in the time domain has two repeated waveform.

In regular frame header, the subcarrier modulation for P-SCH is provided by the following equation (X-2)

(X-2)

where Subcarrier(x)P-SCH represents the subcarrier with running index x and Wk(l) represents the lth digit of the modulated P-SCH series. x ranges from 0 to 127 and offset is equal to 15 for the channel bandwidth of 5 MHz; x ranges from 0 to 255 and offset is equal to 79 for the channel bandwidth of 7, 8.75 and 10 MHz; x ranges from 0 to 511 and offset is equal to 207 for the channel bandwidth of 20 MHz. Figure X-2 illustrates an example of P-SCH frequency domain structure in regular frame header for channel bandwidths of 5, 10 and 20 MHz.

Figure X-2 Example of P-SCH Frequency Domain Structure in Regular Frame Header

In superframe header, the subcarrier modulation for P-SCH is provided by the following equation (X-3)

(X-3)

where Subcarrier(x)P-SCH represents the subcarrier with running index x and Wk(l) represents the lth digit of the modulated P-SCH series. x ranges from 0 to 127 and offset is equal to 15 for the channel bandwidth of 5 MHz; x ranges from 0 to 255 and offset is equal to 79 for the channel bandwidth of 7, 8.75 and 10 MHz; x ranges from 0 to 511 and offset is equal to 207 for the channel bandwidth of 20 MHz. Figure X-3 illustrates an example of P-SCH frequency domain structure in superframe header for channel bandwidths of 5, 10 and 20 MHz.

Figure X-3 Example of P-SCH Frequency Domain Structure in Superframe Header

### 15.3.7.2.1.2 Secondary Synchronization Channel (S-SCH)

In S-SCH, frequency reuse 3 shall be applied. The time length of S-SCH is  and S-SCH occupies full bandwidth. The S-SCH is used for fine synchronization, RSSI measurement, and cell/sector identification (ID).

15.3.7.2.1.2.1 S-SCH Modulation Series

The length of each S-SCH modulation series is [67] for any channel bandwidth. Full cell ID information shall be carried inside the minimal supported channel bandwidth – 5 MHz. The modulation and power boosting of S-SCH is FFS.

The S-SCH series modulating the subcarriers is generated from Zadoff-Chu sequences with parameters u and S defined in Table X-2.1~2.4, where Table X-2.4 is for femtocells only.

Table X-2.1 S-SCH modulation series [FFS]

| Cell ID | Segment | u | S | Cell ID | Segment | u | S | Cell ID | Segment | u | S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 60 | 0 | 9 | 0 | 120 | 0 | 17 | 0 |
| 1 | 1 | 1 | 5 | 61 | 1 | 9 | 5 | 121 | 1 | 17 | 5 |
| 2 | 2 | 1 | 10 | 62 | 2 | 9 | 10 | 122 | 2 | 17 | 10 |
| 3 | 0 | 1 | 15 | 63 | 0 | 9 | 15 | 123 | 0 | 17 | 15 |
| 4 | 1 | 1 | 20 | 64 | 1 | 9 | 20 | 124 | 1 | 17 | 20 |
| 5 | 2 | 1 | 25 | 65 | 2 | 9 | 25 | 125 | 2 | 17 | 25 |
| 6 | 0 | 1 | 30 | 66 | 0 | 9 | 30 | 126 | 0 | 17 | 30 |
| 7 | 1 | 1 | 35 | 67 | 1 | 9 | 35 | 127 | 1 | 17 | 35 |
| 8 | 2 | 1 | 40 | 68 | 2 | 9 | 40 | 128 | 2 | 17 | 40 |
| 9 | 0 | 1 | 45 | 69 | 0 | 9 | 45 | 129 | 0 | 17 | 45 |
| 10 | 1 | 1 | 50 | 70 | 1 | 9 | 50 | 130 | 1 | 17 | 50 |
| 11 | 2 | 1 | 55 | 71 | 2 | 9 | 55 | 131 | 2 | 17 | 55 |
| 12 | 0 | 2 | 0 | 72 | 0 | 10 | 0 | 132 | 0 | 18 | 0 |
| 13 | 1 | 2 | 5 | 73 | 1 | 10 | 5 | 133 | 1 | 18 | 5 |
| 14 | 2 | 2 | 10 | 74 | 2 | 10 | 10 | 134 | 2 | 18 | 10 |
| 15 | 0 | 2 | 15 | 75 | 0 | 10 | 15 | 135 | 0 | 18 | 15 |
| 16 | 1 | 2 | 20 | 76 | 1 | 10 | 20 | 136 | 1 | 18 | 20 |
| 17 | 2 | 2 | 25 | 77 | 2 | 10 | 25 | 137 | 2 | 18 | 25 |
| 18 | 0 | 2 | 30 | 78 | 0 | 10 | 30 | 138 | 0 | 18 | 30 |
| 19 | 1 | 2 | 35 | 79 | 1 | 10 | 35 | 139 | 1 | 18 | 35 |
| 20 | 2 | 2 | 40 | 80 | 2 | 10 | 40 | 140 | 2 | 18 | 40 |
| 21 | 0 | 2 | 45 | 81 | 0 | 10 | 45 | 141 | 0 | 18 | 45 |
| 22 | 1 | 2 | 50 | 82 | 1 | 10 | 50 | 142 | 1 | 18 | 50 |
| 23 | 2 | 2 | 55 | 83 | 2 | 10 | 55 | 143 | 2 | 18 | 55 |
| 24 | 0 | 3 | 0 | 84 | 0 | 12 | 0 | 144 | 0 | 21 | 0 |
| 25 | 1 | 3 | 5 | 85 | 1 | 12 | 5 | 145 | 1 | 21 | 5 |
| 26 | 2 | 3 | 10 | 86 | 2 | 12 | 10 | 146 | 2 | 21 | 10 |
| 27 | 0 | 3 | 15 | 87 | 0 | 12 | 15 | 147 | 0 | 21 | 15 |
| 28 | 1 | 3 | 20 | 88 | 1 | 12 | 20 | 148 | 1 | 21 | 20 |
| 29 | 2 | 3 | 25 | 89 | 2 | 12 | 25 | 149 | 2 | 21 | 25 |
| 30 | 0 | 3 | 30 | 90 | 0 | 12 | 30 | 150 | 0 | 21 | 30 |
| 31 | 1 | 3 | 35 | 91 | 1 | 12 | 35 | 151 | 1 | 21 | 35 |
| 32 | 2 | 3 | 40 | 92 | 2 | 12 | 40 | 152 | 2 | 21 | 40 |
| 33 | 0 | 3 | 45 | 93 | 0 | 12 | 45 | 153 | 0 | 21 | 45 |
| 34 | 1 | 3 | 50 | 94 | 1 | 12 | 50 | 154 | 1 | 21 | 50 |
| 35 | 2 | 3 | 55 | 95 | 2 | 12 | 55 | 155 | 2 | 21 | 55 |
| 36 | 0 | 4 | 0 | 96 | 0 | 14 | 0 | 156 | 0 | 22 | 0 |
| 37 | 1 | 4 | 5 | 97 | 1 | 14 | 5 | 157 | 1 | 22 | 5 |
| 38 | 2 | 4 | 10 | 98 | 2 | 14 | 10 | 158 | 2 | 22 | 10 |
| 39 | 0 | 4 | 15 | 99 | 0 | 14 | 15 | 159 | 0 | 22 | 15 |
| 40 | 1 | 4 | 20 | 100 | 1 | 14 | 20 | 160 | 1 | 22 | 20 |
| 41 | 2 | 4 | 25 | 101 | 2 | 14 | 25 | 161 | 2 | 22 | 25 |
| 42 | 0 | 4 | 30 | 102 | 0 | 14 | 30 | 162 | 0 | 22 | 30 |
| 43 | 1 | 4 | 35 | 103 | 1 | 14 | 35 | 163 | 1 | 22 | 35 |
| 44 | 2 | 4 | 40 | 104 | 2 | 14 | 40 | 164 | 2 | 22 | 40 |
| 45 | 0 | 4 | 45 | 105 | 0 | 14 | 45 | 165 | 0 | 22 | 45 |
| 46 | 1 | 4 | 50 | 106 | 1 | 14 | 50 | 166 | 1 | 22 | 50 |
| 47 | 2 | 4 | 55 | 107 | 2 | 14 | 55 | 167 | 2 | 22 | 55 |
| 48 | 0 | 7 | 0 | 108 | 0 | 16 | 0 | 168 | 0 | 24 | 0 |
| 49 | 1 | 7 | 5 | 109 | 1 | 16 | 5 | 169 | 1 | 24 | 5 |
| 50 | 2 | 7 | 10 | 110 | 2 | 16 | 10 | 170 | 2 | 24 | 10 |
| 51 | 0 | 7 | 15 | 111 | 0 | 16 | 15 | 171 | 0 | 24 | 15 |
| 52 | 1 | 7 | 20 | 112 | 1 | 16 | 20 | 172 | 1 | 24 | 20 |
| 53 | 2 | 7 | 25 | 113 | 2 | 16 | 25 | 173 | 2 | 24 | 25 |
| 54 | 0 | 7 | 30 | 114 | 0 | 16 | 30 | 174 | 0 | 24 | 30 |
| 55 | 1 | 7 | 35 | 115 | 1 | 16 | 35 | 175 | 1 | 24 | 35 |
| 56 | 2 | 7 | 40 | 116 | 2 | 16 | 40 | 176 | 2 | 24 | 40 |
| 57 | 0 | 7 | 45 | 117 | 0 | 16 | 45 | 177 | 0 | 24 | 45 |
| 58 | 1 | 7 | 50 | 118 | 1 | 16 | 50 | 178 | 1 | 24 | 50 |
| 59 | 2 | 7 | 55 | 119 | 2 | 16 | 55 | 179 | 2 | 24 | 55 |

Table X-2.2 S-SCH modulation series [FFS]

| Cell ID | Segment | u | S | Cell ID | Segment | u | S | Cell ID | Segment | u | S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 180 | 0 | 26 | 0 | 240 | 0 | 34 | 0 | 300 | 0 | 43 | 0 |
| 181 | 1 | 26 | 5 | 241 | 1 | 34 | 5 | 301 | 1 | 43 | 5 |
| 182 | 2 | 26 | 10 | 242 | 2 | 34 | 10 | 302 | 2 | 43 | 10 |
| 183 | 0 | 26 | 15 | 243 | 0 | 34 | 15 | 303 | 0 | 43 | 15 |
| 184 | 1 | 26 | 20 | 244 | 1 | 34 | 20 | 304 | 1 | 43 | 20 |
| 185 | 2 | 26 | 25 | 245 | 2 | 34 | 25 | 305 | 2 | 43 | 25 |
| 186 | 0 | 26 | 30 | 246 | 0 | 34 | 30 | 306 | 0 | 43 | 30 |
| 187 | 1 | 26 | 35 | 247 | 1 | 34 | 35 | 307 | 1 | 43 | 35 |
| 188 | 2 | 26 | 40 | 248 | 2 | 34 | 40 | 308 | 2 | 43 | 40 |
| 189 | 0 | 26 | 45 | 249 | 0 | 34 | 45 | 309 | 0 | 43 | 45 |
| 190 | 1 | 26 | 50 | 250 | 1 | 34 | 50 | 310 | 1 | 43 | 50 |
| 191 | 2 | 26 | 55 | 251 | 2 | 34 | 55 | 311 | 2 | 43 | 55 |
| 192 | 0 | 28 | 0 | 252 | 0 | 35 | 0 | 312 | 0 | 45 | 0 |
| 193 | 1 | 28 | 5 | 253 | 1 | 35 | 5 | 313 | 1 | 45 | 5 |
| 194 | 2 | 28 | 10 | 254 | 2 | 35 | 10 | 314 | 2 | 45 | 10 |
| 195 | 0 | 28 | 15 | 255 | 0 | 35 | 15 | 315 | 0 | 45 | 15 |
| 196 | 1 | 28 | 20 | 256 | 1 | 35 | 20 | 316 | 1 | 45 | 20 |
| 197 | 2 | 28 | 25 | 257 | 2 | 35 | 25 | 317 | 2 | 45 | 25 |
| 198 | 0 | 28 | 30 | 258 | 0 | 35 | 30 | 318 | 0 | 45 | 30 |
| 199 | 1 | 28 | 35 | 259 | 1 | 35 | 35 | 319 | 1 | 45 | 35 |
| 200 | 2 | 28 | 40 | 260 | 2 | 35 | 40 | 320 | 2 | 45 | 40 |
| 201 | 0 | 28 | 45 | 261 | 0 | 35 | 45 | 321 | 0 | 45 | 45 |
| 202 | 1 | 28 | 50 | 262 | 1 | 35 | 50 | 322 | 1 | 45 | 50 |
| 203 | 2 | 28 | 55 | 263 | 2 | 35 | 55 | 323 | 2 | 45 | 55 |
| 204 | 0 | 30 | 0 | 264 | 0 | 37 | 0 | 324 | 0 | 46 | 0 |
| 205 | 1 | 30 | 5 | 265 | 1 | 37 | 5 | 325 | 1 | 46 | 5 |
| 206 | 2 | 30 | 10 | 266 | 2 | 37 | 10 | 326 | 2 | 46 | 10 |
| 207 | 0 | 30 | 15 | 267 | 0 | 37 | 15 | 327 | 0 | 46 | 15 |
| 208 | 1 | 30 | 20 | 268 | 1 | 37 | 20 | 328 | 1 | 46 | 20 |
| 209 | 2 | 30 | 25 | 269 | 2 | 37 | 25 | 329 | 2 | 46 | 25 |
| 210 | 0 | 30 | 30 | 270 | 0 | 37 | 30 | 330 | 0 | 46 | 30 |
| 211 | 1 | 30 | 35 | 271 | 1 | 37 | 35 | 331 | 1 | 46 | 35 |
| 212 | 2 | 30 | 40 | 272 | 2 | 37 | 40 | 332 | 2 | 46 | 40 |
| 213 | 0 | 30 | 45 | 273 | 0 | 37 | 45 | 333 | 0 | 46 | 45 |
| 214 | 1 | 30 | 50 | 274 | 1 | 37 | 50 | 334 | 1 | 46 | 50 |
| 215 | 2 | 30 | 55 | 275 | 2 | 37 | 55 | 335 | 2 | 46 | 55 |
| 216 | 0 | 32 | 0 | 276 | 0 | 39 | 0 | 336 | 0 | 49 | 0 |
| 217 | 1 | 32 | 5 | 277 | 1 | 39 | 5 | 337 | 1 | 49 | 5 |
| 218 | 2 | 32 | 10 | 278 | 2 | 39 | 10 | 338 | 2 | 49 | 10 |
| 219 | 0 | 32 | 15 | 279 | 0 | 39 | 15 | 339 | 0 | 49 | 15 |
| 220 | 1 | 32 | 20 | 280 | 1 | 39 | 20 | 340 | 1 | 49 | 20 |
| 221 | 2 | 32 | 25 | 281 | 2 | 39 | 25 | 341 | 2 | 49 | 25 |
| 222 | 0 | 32 | 30 | 282 | 0 | 39 | 30 | 342 | 0 | 49 | 30 |
| 223 | 1 | 32 | 35 | 283 | 1 | 39 | 35 | 343 | 1 | 49 | 35 |
| 224 | 2 | 32 | 40 | 284 | 2 | 39 | 40 | 344 | 2 | 49 | 40 |
| 225 | 0 | 32 | 45 | 285 | 0 | 39 | 45 | 345 | 0 | 49 | 45 |
| 226 | 1 | 32 | 50 | 286 | 1 | 39 | 50 | 346 | 1 | 49 | 50 |
| 227 | 2 | 32 | 55 | 287 | 2 | 39 | 55 | 347 | 2 | 49 | 55 |
| 228 | 0 | 33 | 0 | 288 | 0 | 41 | 0 | 348 | 0 | 50 | 0 |
| 229 | 1 | 33 | 5 | 289 | 1 | 41 | 5 | 349 | 1 | 50 | 5 |
| 230 | 2 | 33 | 10 | 290 | 2 | 41 | 10 | 350 | 2 | 50 | 10 |
| 231 | 0 | 33 | 15 | 291 | 0 | 41 | 15 | 351 | 0 | 50 | 15 |
| 232 | 1 | 33 | 20 | 292 | 1 | 41 | 20 | 352 | 1 | 50 | 20 |
| 233 | 2 | 33 | 25 | 293 | 2 | 41 | 25 | 353 | 2 | 50 | 25 |
| 234 | 0 | 33 | 30 | 294 | 0 | 41 | 30 | 354 | 0 | 50 | 30 |
| 235 | 1 | 33 | 35 | 295 | 1 | 41 | 35 | 355 | 1 | 50 | 35 |
| 236 | 2 | 33 | 40 | 296 | 2 | 41 | 40 | 356 | 2 | 50 | 40 |
| 237 | 0 | 33 | 45 | 297 | 0 | 41 | 45 | 357 | 0 | 50 | 45 |
| 238 | 1 | 33 | 50 | 298 | 1 | 41 | 50 | 358 | 1 | 50 | 50 |
| 239 | 2 | 33 | 55 | 299 | 2 | 41 | 55 | 359 | 2 | 50 | 55 |

Table X-2.3 S-SCH modulation series [FFS]

| Cell ID | Segment | u | S | Cell ID | Segment | u | S | Cell ID | Segment | u | S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 360 | 0 | 51 | 0 | 380 | 2 | 53 | 40 | 400 | 1 | 57 | 20 |
| 361 | 1 | 51 | 5 | 381 | 0 | 53 | 45 | 401 | 2 | 57 | 25 |
| 362 | 2 | 51 | 10 | 382 | 1 | 53 | 50 | 402 | 0 | 57 | 30 |
| 363 | 0 | 51 | 15 | 383 | 2 | 53 | 55 | 403 | 1 | 57 | 35 |
| 364 | 1 | 51 | 20 | 384 | 0 | 55 | 0 | 404 | 2 | 57 | 40 |
| 365 | 2 | 51 | 25 | 385 | 1 | 55 | 5 | 405 | 0 | 57 | 45 |
| 366 | 0 | 51 | 30 | 386 | 2 | 55 | 10 | 406 | 1 | 57 | 50 |
| 367 | 1 | 51 | 35 | 387 | 0 | 55 | 15 | 407 | 2 | 57 | 55 |
| 368 | 2 | 51 | 40 | 388 | 1 | 55 | 20 | 408 | 0 | 58 | 0 |
| 369 | 0 | 51 | 45 | 389 | 2 | 55 | 25 | 409 | 1 | 58 | 5 |
| 370 | 1 | 51 | 50 | 390 | 0 | 55 | 30 | 410 | 2 | 58 | 10 |
| 371 | 2 | 51 | 55 | 391 | 1 | 55 | 35 | 411 | 0 | 58 | 15 |
| 372 | 0 | 53 | 0 | 392 | 2 | 55 | 40 | 412 | 1 | 58 | 20 |
| 373 | 1 | 53 | 5 | 393 | 0 | 55 | 45 | 413 | 2 | 58 | 25 |
| 374 | 2 | 53 | 10 | 394 | 1 | 55 | 50 | 414 | 0 | 58 | 30 |
| 375 | 0 | 53 | 15 | 395 | 2 | 55 | 55 | 415 | 1 | 58 | 35 |
| 376 | 1 | 53 | 20 | 396 | 0 | 57 | 0 | 416 | 2 | 58 | 40 |
| 377 | 2 | 53 | 25 | 397 | 1 | 57 | 5 | 417 | 0 | 58 | 45 |
| 378 | 0 | 53 | 30 | 398 | 2 | 57 | 10 | 418 | 1 | 58 | 50 |
| 379 | 1 | 53 | 35 | 399 | 0 | 57 | 15 | 419 | 2 | 58 | 55 |

Table X-2.4 S-SCH modulation series for femtocells [FFS]

| Cell ID | Segment | u | S | Cell ID | Segment | u | S | Cell ID | Segment | u | S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 420 | 0 | 60 | 0 | 454 | 1 | 64 | 50 | 488 | 2 | 14 | 60 |
| 421 | 1 | 60 | 5 | 455 | 2 | 64 | 55 | 489 | 0 | 16 | 60 |
| 422 | 2 | 60 | 10 | 456 | 0 | 65 | 0 | 490 | 1 | 17 | 60 |
| 423 | 0 | 60 | 15 | 457 | 1 | 65 | 5 | 491 | 2 | 18 | 60 |
| 424 | 1 | 60 | 20 | 458 | 2 | 65 | 10 | 492 | 0 | 21 | 60 |
| 425 | 2 | 60 | 25 | 459 | 0 | 65 | 15 | 493 | 1 | 22 | 60 |
| 426 | 0 | 60 | 30 | 460 | 1 | 65 | 20 | 494 | 2 | 24 | 60 |
| 427 | 1 | 60 | 35 | 461 | 2 | 65 | 25 | 495 | 0 | 26 | 60 |
| 428 | 2 | 60 | 40 | 462 | 0 | 65 | 30 | 496 | 1 | 28 | 60 |
| 429 | 0 | 60 | 45 | 463 | 1 | 65 | 35 | 497 | 2 | 30 | 60 |
| 430 | 1 | 60 | 50 | 464 | 2 | 65 | 40 | 498 | 0 | 32 | 60 |
| 431 | 2 | 60 | 55 | 465 | 0 | 65 | 45 | 499 | 1 | 33 | 60 |
| 432 | 0 | 63 | 0 | 466 | 1 | 65 | 50 | 500 | 2 | 34 | 60 |
| 433 | 1 | 63 | 5 | 467 | 2 | 65 | 55 | 501 | 0 | 35 | 60 |
| 434 | 2 | 63 | 10 | 468 | 0 | 66 | 0 | 502 | 1 | 37 | 60 |
| 435 | 0 | 63 | 15 | 469 | 1 | 66 | 5 | 503 | 2 | 39 | 60 |
| 436 | 1 | 63 | 20 | 470 | 2 | 66 | 10 | 504 | 0 | 41 | 60 |
| 437 | 2 | 63 | 25 | 471 | 0 | 66 | 15 | 505 | 1 | 43 | 60 |
| 438 | 0 | 63 | 30 | 472 | 1 | 66 | 20 | 506 | 2 | 45 | 60 |
| 439 | 1 | 63 | 35 | 473 | 2 | 66 | 25 | 507 | 0 | 46 | 60 |
| 440 | 2 | 63 | 40 | 474 | 0 | 66 | 30 | 508 | 1 | 49 | 60 |
| 441 | 0 | 63 | 45 | 475 | 1 | 66 | 35 | 509 | 2 | 50 | 60 |
| 442 | 1 | 63 | 50 | 476 | 2 | 66 | 40 | 510 | 0 | 51 | 60 |
| 443 | 2 | 63 | 55 | 477 | 0 | 66 | 45 | 511 | 1 | 53 | 60 |
| 444 | 0 | 64 | 0 | 478 | 1 | 66 | 50 | 512 | 2 | 55 | 60 |
| 445 | 1 | 64 | 5 | 479 | 2 | 66 | 55 | 513 | 0 | 57 | 60 |
| 446 | 2 | 64 | 10 | 480 | 0 | 1 | 60 | 514 | 1 | 58 | 60 |
| 447 | 0 | 64 | 15 | 481 | 1 | 2 | 60 | 515 | 2 | 60 | 60 |
| 448 | 1 | 64 | 20 | 482 | 2 | 3 | 60 | 516 | 0 | 63 | 60 |
| 449 | 2 | 64 | 25 | 483 | 0 | 4 | 60 | 517 | 1 | 64 | 60 |
| 450 | 0 | 64 | 30 | 484 | 1 | 7 | 60 | 518 | 2 | 65 | 60 |
| 451 | 1 | 64 | 35 | 485 | 2 | 9 | 60 | 519 | 0 | 66 | 60 |
| 452 | 2 | 64 | 40 | 486 | 0 | 10 | 60 |  |  |  |  |
| 453 | 0 | 64 | 45 | 487 | 1 | 12 | 60 |  |  |  |  |

15.3.7.2.1.2.2 Transmission of S-SCH Series

The subcarrier spacing of S-SCH is two times larger than that of a regular data OFDM symbol. Every third subcarrier is modulated using a boosted value with a specific series defined in 15.3.7.2.1.2.1. The subcarrier modulation for S-SCH is provided by the following equations, which are FFS. DC tone shall be nulled.

Figure X-4 illustrates an example of S-SCH frequency domain structure for channel bandwidths of 5, 10 and 20 MHz.

Figure X-4 Example of S-SCH Frequency Domain Structure

For the deployment of femtocells, interferences between femtocells and other cells in S-SCH shall be mitigated. The interference mitigation scheme is FFS.

### 15.3.7.2.1.3 Support of WirelessMAN-OFDMA

Advanced Air Interface shall exist in both green field and mixed deployments. In mixed deployments, the WirelessMAN-OFDMA preamble shall be always present. The Advanced Air Interface SCH shall enable AMSs to synchronize in frequency and time without requiring WirelessMAN-OFDMA preamble.

Figure X-5 shows an example of Advanced Air Interface SCH architecture in mixed deployments.

Figure X-5 Advanced Air Interface SCH Architecture Supporting WirelessMAN-OFDMA

-----------------------------------------------------------End of the Text---------------------------------------------------------