| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Channel coding and HARQ Sections for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Joon Chun, Xin Qi Nokia Siemens Networks | joon.chun@nsn.com xin.qi@nsn.com |
| Re: | 802.16m amendment working document Target topic: “channel coding and HARQ”. |  |
| Abstract | The contribution proposes IEEE 802.16m amendment text on channel coding and HARQ. |  |
| Purpose | To be incorporated into the IEEE 802.16 amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Channel Coding and HARQ Sections for the IEEE 802.16m Amendment

Joon Chun, Xin Qi

NSN

# Introduction

The contribution proposes text for channel coding section.

# Proposed text

----------------------------------------------------------------------------------------------------------------------------------------

15.3.x  Channel coding and HARQ

15.3.x.1 Channel coding

Channel coding procedures include FEC encoding, bit selection and repetition, and modulation.

Figure xxx Channel coding block diagram

The coding method used as the mandatory scheme shall be the tail-biting CTC (Convolutional Turbo Code) as specified in 15.3.x.1.3.

15.3.x.1.1 Burst partition into FEC blocks

[TBD]

15.3.x.1.2 CRC encoder

The sizes of both burst CRC and FEC block CRC are 16 bits (TBD) and the polynomial is as defined in ITU-T Recommendation X.25:

g(D)=D16+D12+D5+1

After CRC operation, if the size of burst does not fit exactly the amount of data allocated, flexible coding rate (see 15.3.x.1.3.1.1) shall be applied to the FEC blocks within the burst for size adjustment.

15.3.x.1.3 FEC encoding

15.3.x.1.3.1 Tail Biting CTC encoding

The CTC encoder, including its constituent encoder, is depicted in Figure XXXA. It uses a double binary Circular recursive Convolutional code. The bits of the data to be encoded are alternatively fed to A and B, starting with the MSB of the first byte being fed to A. The encoder is fed by blocks of k bits orNdcouples (k = 2*Nd bits). For all the frame sizes, k is a multiple of 8 and Nd is a multiple of 4.

The polynomials defining the connections are described in octal and symbol notations as follows:

- For the feedback branch: 0xB, equivalently 1+D+D3

- For the Y parity bit: 0xD, equivalently 1+D2+D3

- For the W parity bit: 0x9, equivalently 1+D3

Figure XXXA CTC encoder

First, the encoder (after initialization by circulation state Sc1, see 15.3.x.1.3.3) is fed the sequence in the natural order (position 1) with the incremental address  i=0,...,Nd−1. This first encoding is called C1 encoding. Then the encoder (after initialization by the circulation state Sc2, see 15.3.x.1.3.3) is fed by the interleaved sequence (switch in position 2) with incremental address j=0,...,Nd−1. This second encoding is called C2 encoding.

The order in which the encoded bit shall be fed into subpacket generation block (15.3.x.1.3.4) is:

A,B,Y1,Y2,W1,W2 =

A0,A1,...,ANd−1,B0,B1,...,BNd−1,Y1,0,Y1,1,...,Y1,Nd−1,Y2,0,Y2,1,...,Y2,Np−1,Y1,0,Y1,1,...,Y1,Nd−1                                     W1,0,W1,1,...,W1,Nd−1,W2,0,W2,1,...,W2,Nd−1

Table xxxb (TBD) gives the block sizes, code rates, and code parameters for the different modulation and coding schemes.

Table XXXb CTC channel coding per modulation

15.3.x.1.3.1.1 Flexible channel coding rate

Let

be the number of bits in the MAC PDUs to be transmitted in a burst.

be modulation and coding rate.

NRU     be the number of LRUs to transmit the burst, indicated by BS.

be the minimum number of LRUs to contain  bits using the selected MCS.

be the number of information bits which could be transmitted in a burst with , and the MIMO scheme selected.

be the number of information bits in the burst after flexible coding rate adjustment

could be determined by looking up table xxxc (TBD) using ,  and the MIMO scheme as the inputs [Table xxxc gives the valid burst size.]. Let  be the number of information which could be transmitted in  LRUs. .

When ,  non-informative padding bits needs to be added after the MAC PDUs to form the information bits of the burst.

In this case, the channel coding rate for the FEC blocks in the burst could be redefined either to save resource bandwidth by increasing coding rate, or to improve the performance of channel decoding by reducing coding rate. For the case of increasing coding rate, the  must be greater than 1. Selection of increasing or reducing coding rate is determined by channel state and the target BLER. The detail selection algorithm is TBD.

Procedure of flexible channel coding rate adjustment is as follows:

- Compute , .

- If α=0, there is no need for flexible channel coding rate adjustment. . Set .

- If α≠0, BS proceed the following flexible coding adjustment.

  - If the flexible coding rate adjustment is not activated,  non-informative bits are padded to the burst. The number of information bits transmitted in the burst is . Set .

  - If the flexible coding rate adjustment is activated, no non-informative padding bits are added to the burst. The number of information bits transmitted in the burst is. Decide whether coding rate is to be increased or to be reduced.

    - If coding rate is to be increased,

      - Set the flexible channel coding rate indicator, rfcr=1.

      - .

    - If coding rate is to be reduced:

      - Set the flexible channel coding rate indicator, rfcr=2.

      - .

      - The flexible channel coding rate indicatorrfcr shall be transmitted in USCCH (TBD). If , the value of  should also be indicated in USCCH. Upon reception of rfcr and ,  together with NRU,  and MIMO scheme, AMS can infer .

15.3.x.1.3.2 CTC interleaving

Same as 8.4.9.2.3.2.

15.3.x.1.3.3 Determination of CTC circulation states

Same as 8.4.9.2.3.3.

15.3.x.1.3.4 Subpacket generation

Same as 8.4.9.2.3.4.

15.3.x.1.3.4.1 Symbol separation

Same as 8.4.9.2.3.4.1

15.3.x.1.3.4.2 Subblock interleaving

[TBD]

15.3.x.1.4 Bit selection and repetition

[TBD]

15.3.x.1.5 Modulation

[TBD]

----------------------------------------------------------------------------------------------------------------------------------------

End of proposed text