| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame Structures for 7MHz and 8.75MHz Channel Bandwidths |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Jaeweon Cho, Hokyu Choi, Heewon Kang, Jeongho Park, Hwasun Yoo, Jaehee Cho Samsung Electronics Co., Ltd. Sungho Moon, Jin Sam Kwak LG Electronics | +82-31-279-5796 jaeweon.cho@samsung.com msungho@lge.com, samji@lge.com |
| Re: | “802.16m AWD” IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” – Call for Comments on Amendment Working Document. |  |
| Abstract | This contribution proposes frame structures for 7MHz and 8.75MHz channel bandwidths. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Frame Structures for 7MHz and 8.75Mhz Channel Bandwidths

Jaeweon Cho, Hokyu Choi, Heewon Kang, Jeongho Park, Hwasun Yoo, Jaehee Cho

Samsung Electronics Co., Ltd.

Sungho Moon, Jin Sam Kwak

LG Electronics

- Introduction

In the current 802.16m SDD [1] and AWD [2], for 7MHz and 8.75MHz channel bandwidths OFDMA parameters only are defined, no descriptions of frame structure there. In this contribution we discuss and propose frame structures for 7MHz and 8.75MHz for inclusion into the 802.16m Amendment.

Before discussing the detailed frame structures, we need to identify the issue and requirement in design of frame structures for 7MHz and 8.75MHz.

- The issue is that the 10MHz frame structure cannot be applied to 7MHz and 8.75MHz without modification. It is because the number of OFDMA symbols per frame is not the same among different channel bandwidths; i.e. 48 symbols for 10MHz, but 43 and 34 symbols for 8.75MHz and 7MHz, respectively. The subframe configuration in the 10MHz frame structure, i.e. 8 subframes per 5ms frame, therefore cannot be kept in 7MHz and 8.75MHz by using only a combination of type-1 subframe (6 symbols) and type-3 subframe (5 symbols).

- There may be two design approaches for 7MHz and 8.75MHz frame structures: One is to reduce the number of subframes per frame while employing the current three subframe types only (type-1, type-2, type-3). The other is to reduce the number of OFDMA symbols per subframe while keeping the configuration of 8 subframes per frame. Both options have their own drawbacks. Especially, the second approach may produce another new subframe type; it implies that a new PRU should be defined accordingly. An impact of the irregular subframe is more serious in UL than DL, because a new title structure for UL control channel is needed. Therefore, the first approach is more desirable in design of 7MHz and 8.75MHz frame structures, so as to avoid another new subframe type.

- In design of 7MHz and 8.75MHz frame structures, the legacy support requirements defined in SRD [3] shall be considered. In particular, the DL to UL symbol ratio employed in the current practical WiMAX systems shall be supported; i.e. DL:UL = 27:15 for 8.75MHz, DL:UL = 21:12 for 7MHz [4].

After due consideration of the issue and requirements above, we have developed the 7MHz and the 8.75MHz frame structures. They are presented and discussed in the following sections.

- Frame Structures for 7MHz

The proposed frame structure for 7MHz is shown in Figure 1. In this structure, a 5ms frame consists of six subframes. For the ratio of DL:UL = 21:12, downlink is comprised of one type-1 subframe and three type-3 subframes, uplink of two type-1 subframes. No irregular subframe type is employed in UL.

Figure 1. Frame Structure in TDD mode for 7 MHz channel bandwidths (CP=1/8 Tb)

- Frame Structures for 8.75MHz

Design of the 8.75MHz frame structure is more complicated and tricky. For DL to UL symbol ratio of 27:15, the uplink of 15 symbols should be divided into multiple subframes, but then it is not easy to avoid the use of irregular subframe type.

Various options for UL configuration for DL:UL = 27:15 are listed-up and compared in Table 1. All options in the table have the same downlink configuration of DL = {6, 6, 5, 5, 5}, i.e. two type-1 subframes and three type-3 subframes. Each option has its own pros and cons, but the most critical is whether the option mandates a new UL control structure design or not. Another UL control structure only for 8.75 MHz is much burden to the specification. In this sense, we have chosen the options C, which reuses the UL control structure of type-1 subframe. The option A-2 also allows to reuse the UL control structure of type-1 subframe, but it may degrade the performance of UL control channel due to one symbol truncation.

Table 1. Various options for UL configuration for 8.75MHz frame structure with DL:UL = 27:15

| UL configuration options |  | Pros | Cons |
| --- | --- | --- | --- |
| A-1 | UL = {5, 5, 5} | - No impact on upper-level control procedure (scheduling, HARQ, … ) - Easy new PRU design (the same PRU as in DL) | - Need New UL control design - Need New PRU design for 4 symbols (considering one sounding symbol) |
|  | New UL Control structure for 5 symbol |  |  |
| A-2 | UL = {5, 5, 5} | - No impact on upper-level control procedure (scheduling, HARQ, … ) - Easy new PRU design (the same PRU as in DL) - No need of new UL Control design | - Performance degradation of UL Control channel due to the truncation - Need New PRU design for 4 symbols (considering one sounding symbol) |
|  | Re-use UL Control structure of 6 symbol (by truncation) |  |  |
| B-1 | UL = {6, 6, 3} | - No impact on HARQ operation - Less burden to UL control design (Only New ACK design) | - Need New PRU design for 3 symbols - Need New ACK channel design |
|  | UL ACK channels only in 3-symbol subframe |  |  |
| B-2 | UL = {6, 6, 3} | - No need of new UL control design | - Need New PRU design for 3 symbols - Impact on DL HARQ operation (Different ACK timing from 10MHz frame structure) |
|  | No UL Control structure in 3-symbol subframe |  |  |
| C | UL = {9, 6} | - No need of new UL control design | - Need New PRU design for 9 symbols (but easier than for 3 symbols) - 7 subframes / frame (Different DL & UL HARQ timings from 10MHz frame structure) |
|  | Re-use UL Control structure of 6 symbol in 9-symbol subframe |  |  |

Figure 2 shows the proposed frame structure for 8.75MHz based on the option C. In this structure, one frame consists of seven subframes. For the ratio of DL:UL = 27:15, downlink is comprised of two type-1 subframes and three type-3 subframes. Uplink is comprised of one type-1 subframe and one new type-4 subframe, i.e.  9-symbol subframe.

Figure 2. Option C for 8.75 MHz Frame Structure with DL:UL = 27:15 (CP=1/8 Tb)

- Harmonization with IEEE C802.16-09/0473

Even though the proposed 8.75MHz frame structure in Figure 2 would be the best option for legacy support, it may not be easy with this structure to support various DL to UL ratios in greed-field deployment. It is because of many irregular subframes in this structure. For this reason, we propose the following frame structure comprised of seven subframes for green-field deployment; in TDD all subframes are type-1 subframes, and in FDD six subframes are type-1 and one subframe is type-2 subframe. Figure 3 shows various configurations of the proposed frame structure.

In summary, as the 8.75MHz frame structure, we propose the structure in Figure 3 for the green-field deployment, and the structure in Figure 2 for legacy support. Although these two structures have different combinations of subframes, the numbers of subframes per frame in these two structures are the same as 7.

It is also noted that this revised contribution includes only the proposed frame structure for 8.75MHz bandwidth. The proposed frame structure for 7MHz is incorporated into another harmonized contribution, IEEE C802.16m-09/0474r1.

Figure 3. 8.75 MHz Frame Structure for Green-filed deployment (CP=1/8 Tb)

- Text proposal for inclusion in the 802.16m AWD

[Remedy 1: Add the following text in 42 on the page 14, in 15.3.3.1:]

4) the type-4 subframe which consists of nine OFDMA symbols. This type shall be applied only to UL subframe for the 8.75MHz channel bandwidth when supporting the WirelessMAN-OFDMA frames.

[Remedy 2: Insert the following text and illustration after Figure 390 on page 16, in 15.3.3.2.1:]

Figure xxx illustrates an example FDD frame structure, which is applicable to the nominal channel bandwidth of 8.75 MHz with G = 1/8. In Figure xxx the fourth subframe is a type-2 subframe and the other subframes are type-1 subframes.

Figure xxx Frame structure in FDD for the 8.75 MHz channel bandwidth (CP = 1/8 Tb)

[Remedy 3: Change the text from line 39 to 40 on the page 16, in 15.3.3.2.2, as follows:]

In a TDD frame with DL to UL ratio of D:U, the 1st contiguous D subframes and the remaining U subframes are assigned for DL and UL, respectively, where D + U = 8 for 5, 10 and 20 MHz channel bandwidths, D + U = 7 for 8.75 MHz channel bandwidth, and D + U = 6 for 7 MHz channel bandwidth.

[Remedy 4: Insert the following text and illustration after Figure 391 on page 17, in 15.3.3.2.2:]

Figure yyy illustrates an example TDD frame structure with D:U =5:2, which is applicable to the nominal channel bandwidths of 8.75 MHz with G = 1/8. In Figure yyy all seven subframes in a frame are type-1 subframes. TTG and RTG are 87.2 µs and 74.4µs, respectively.

Figure xxy – Frame structure in TDD for 8.75 MHz channel bandwidth (CP=1/8 Tb)

[Remedy 5: Insert the following text and illustration after Figure 392 on page 18, in 15.3.3.3:]

For a channel bandwidth of 8.75 MHz, a frame shall have four type-1 subframes and three type-2 subframes for FDD, and five type-1 subframes and two type-2 subframes for TDD.

In the TDD frame, the first and last subframes within each frame shall be type-2 subframes. In the FDD frame, the first, forth, and last subframe within each frame shall be type-2 subframes.

Figure xyy illustrates an example of TDD and FDD frame structure for the 8.75 MHz channel bandwidth with a CP of 1/16 Tb. Assuming OFDMA symbol duration of 108.8 μs and a CP length of 1/16 Tb, the length of type-1 and type-2 subframes are 0.6528 ms and 0.7616 ms, respectively. TTG and RTG are 138.4 µs and 74.4µs, respectively.

Figure xyy — TDD and FDD frame structure for 8.75 MHz channel bandwidth with a CP of 1/16 Tb (DL to UL ratio of 4:3)

[Remedy 6: Insert the following text after “when FDM mode is used.” in line 2 on page 19, in 15.3.3.4.1:]

In case of 5/10/20 MHz, all UL subframes are type-1 subframes. In case of 8.75 MHz with 15 UL OFDM symbols, the [first] UL subframe is type-4 subframe and the [second] UL subframe is type-1 subframe.

[Remedy 7: Insert the following text after “when TDM mode is used.” in line 46 on page 19, in 15.3.3.4.1:]

In case of 5/10/20 MHz and 8.75 MHz, all subframes in the Advanced Air Interface UL Zone are type-1 subframes.

- References

[1] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document.”

[2] IEEE 802.16m-09/0010, “IEEE 802.16m Amendment Working Document.”

[3] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements.”

[4] WiMAX Plugfest 5 guideline, Dec 2007.

[5] IEEE C802.16m-09/0473, “Frame Structure for a System Bandwidth of 8.75 MHz.”

[6] IEEE C802.16m-09/0474r1, “Frame Structure for a System Bandwidth of 7 MHz.”