| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on channel coding |  |
| Date Submitted | 2009-01-10 |  |
| Source(s) | Hua Zhou, Sengbao Liang, Yuanrong Lan; Jie Zhang, Yuantao Zhang, Jinyin Xue, Jian Wang, Jun Tian Fujitsu R&D Center Ltd. | { zhouhua; sengbaol; lanyr; jie.zhang; zyuantao; xuejy;jianwang; tianjun}@cn.fujitsu.com |
| Re: | C80216m-08_053: Call for Contributions on Project 802.16m Draft Amendment Content Target Topic: Channel coding |  |
| Abstract | Proposal of amendment text on Channel coding |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m amendment text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text on Channel coding

Hua Zhou, Sengbao Liang, Yuanrong Lan;

Jie Zhang, Yuantao Zhang, Jinyin Xue, Jian Wang, Jun Tian

Fujitsu R&D Center Ltd.

# 1 Introduction

In this contribution, the amendment text is proposed on Channel coding in accordance with the IEEE 802.16m System Requirement Document [1] and System Description Document [2]. Any proposed text here is developed such that it can be readily combined with IEEE P802.16Rev2/D7 [3] and other active IEEE 802.16 amendment projects, as appropriate. In addition, this contribution follows the tentative outlines, style and format guidelines contained in [4].

# 2 Motivation

The existing 16e MCS for data transmission is based on punctured CTC and QAM, which have the following performance as showing blow.

Figure 1 BLER performance of MCS sets in 16E standard

Here, we find that if the target BLER is 10%, the SINR threshold for different MCS is 1.5dB, 4.5dB, 7.0dB, 10.5dB, 11.5dB, 15dB, 16dB, and 17.5dB. The SINR distance between two adjacent MCS levels is 3dB, 2.5dB, 3.5dB, 1.0dB, 3.5dB, 1dB, and 1.5dB, which have big variance. In some cases, this set of MCS levels will decrease system throughput, for example, most users in one system can have limited SINR range, such as, from 0dB to 10dB, but in this range, the MCS satisfied for target BLER for user can be only selected from 3 MCS levels (QPSK ½, QPSK ¾, 16QAM 1/2,). On the other hand, if the SINR distribution range can be roughly estimated, appropriate MCS levels will be assigned to optimize the system throughput.

Figure 2 shows the PDF of instantaneous SINR for 16e baseline system. From this figure, we can see that most of SINR value are between -10dB and 15dB. It means that a higher system throughput can be explored if the users are optimized with SINR values between -10dB and 15dB. One straightforward solution is to assign more MCS levels in certain SINR area, i.e., we design more MCS level for SNR with high probability, and less MCS level for SNR with less probability. It can be found that the MCS levels in 16e system are not reasonably designed from the BLER performance in figure 1, i.e., too many MCS levels greater than 15dB, less MCS levels between -10dB and 15dB. We recommend that more MCS levels should be added from -10dB to 15dB.

Figure 2 Distribution of Instantaneous SINR for 16e baseline system

# 3 Key proposals in this contribution

We design the whole 16 MCS levels with detailed interleaver parameters as in following table.

| MCS Index | Target Code Rate | Mod Order | Inf Byte | SNR | Spectral Efficiency |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.1333(2/15) | 2 | 240 | -4.97dB | 0.2667 |
| 1 | 0.2174(5/23) | 2 | 240 | -2.85dB | 0.4348 |
| 2 | 0.3448(10/29) | 2 | 240 | -0.84dB | 0.6897 |
| 3 | 0.4762(10/21) | 2 | 240 | 0.91dB | 0.9524 |
| 4 | 0.6000(3/5) | 2 | 288 | 2.52dB | 1.2000 |
| 5 | 0.3157(6/19) | 4 | 288 | 3.98dB | 1.2632 |
| 6 | 0.4167(5/12) | 4 | 240 | 5.40dB | 1.6667 |
| 7 | 0.4848(16/33) | 4 | 256 | 6.56dB | 1.9394 |
| 8 | 0.5556(5/9) | 4 | 240 | 7.52dB | 2.2222 |
| 9 | 0.6250(5/8) | 4 | 240 | 8.61dB | 2.5000 |
| 10 | 0.7142(5/7) | 4 | 240 | 9.94dB | 2.8571 |
| 11 | 0.5000(1/2) | 6 | 360 | 11.56dB | 3.0000 |
| 12 | 0.6111(11/18) | 6 | 176 | 13.19dB | 3.6667 |
| 13 | 0.7037(19/27) | 6 | 304 | 14.94dB | 4.2222 |
| 14 | 0.8000(4/5) | 6 | 288 | 16.64dB | 4.8000 |
| 15 | 0.9048(19/21) | 6 | 456 | 18.94dB | 5.4286 |

Table 1 Proposed 16m MCS set

| Data Block size (bytes) | N | P0 | P1 | P2 | P3 | M | J |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 176 | 704 | 53 | 62 | 12 | 2 | 8 | 3 |
| 240 | 960 | 43 | 64 | 300 | 824 | 9 | 2 |
| 256 | 1024 | 43 | 64 | 300 | 824 | 9 | 2 |
| 288 | 1152 | 43 | 64 | 300 | 824 | 9 | 3 |
| 304 | 1216 | 43 | 64 | 300 | 824 | 9 | 3 |
| 360 | 1440 | 43 | 64 | 300 | 824 | 10 | 2 |
| 456 | 1824 | 31 | 8 | 24 | 16 | 10 | 2 |

Table 2 Codeblock and interleave parameters for different data block size

In link level simulation, we can get single user throughput as showing below. It’s shown from this figure that new MCS has better throughput, especially in the SNR value about 10dB.

Figure 3 Throughput CDF of Single user link.

The following two figures are the CDF of user throughput with two different MCS set in system level simulation, from which, we can get the sector throughput and user coverage in the table.

Figure 4 User throughput for SISO system

Figure 5 user throughput for MIMO (2x2) system

|  | Sector Throughput (bps/Hz) |  | User Coverage 5%(bps/Hz) |  |
| --- | --- | --- | --- | --- |
|  | SISO | MIMO (2x2) | SISO | MIMO (2x2) |
| 16e MCS table | 1.06 | 1.41 | 0.020 | 0.027 |
| New MCS table | 1.13 | 1.57 | 0.022 | 0.032 |
| Performance Gain | 6.6% | 11.3% | 10% | 18% |

Table 3 Throughput comparison with existing 16e MCS.

# 4 References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

# 5 Proposed Amendment Text on 11.13 Channel coding

-----------------------------------------------------------Start of the Text---------------------------------------------------------

## 15 Advanced Air Interface

## 15.3 Physical Layer

## Channel coding

…

### 15.3.x Modulation and coding

Replace Table 526 as the following table

| MCS Index | Target Code Rate | Mod Order | Data Block size (Bytes | N | P0 | P1 | P2 | P3 | M | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.1333(2/15) | 2 | 240 | 960 | 43 | 64 | 300 | 824 | 9 | 2 |
| 1 | 0.2174(5/23) | 2 | 240 | 960 | 43 | 64 | 300 | 824 | 9 | 2 |
| 2 | 0.3448(10/29) | 2 | 240 | 960 | 43 | 64 | 300 | 824 | 9 | 2 |
| 3 | 0.4762(10/21) | 2 | 240 | 960 | 43 | 64 | 300 | 824 | 9 | 2 |
| 4 | 0.6000(3/5) | 2 | 288 | 1152 | 43 | 64 | 300 | 824 | 9 | 3 |
| 5 | 0.3157(6/19) | 4 | 288 | 1152 | 43 | 64 | 300 | 824 | 9 | 3 |
| 6 | 0.4167(5/12) | 4 | 240 | 960 | 43 | 64 | 300 | 824 | 9 | 2 |
| 7 | 0.4848(16/33) | 4 | 256 | 1024 | 43 | 64 | 300 | 824 | 9 | 2 |
| 8 | 0.5556(5/9) | 4 | 240 | 960 | 43 | 64 | 300 | 824 | 9 | 2 |
| 9 | 0.6250(5/8) | 4 | 240 | 960 | 43 | 64 | 300 | 824 | 9 | 2 |
| 10 | 0.7142(5/7) | 4 | 240 | 960 | 43 | 64 | 300 | 824 | 9 | 2 |
| 11 | 0.5000(1/2) | 6 | 360 | 1440 | 43 | 64 | 300 | 824 | 10 | 2 |
| 12 | 0.6111(11/18) | 6 | 176 | 704 | 53 | 62 | 12 | 2 | 8 | 3 |
| 13 | 0.7037(19/27) | 6 | 304 | 1216 | 43 | 64 | 300 | 824 | 9 | 3 |
| 14 | 0.8000(4/5) | 6 | 288 | 1152 | 43 | 64 | 300 | 824 | 9 | 3 |
| 15 | 0.9048(19/21) | 6 | 456 | 1824 | 31 | 8 | 24 | 16 | 10 | 2 |

Table 15.3.x.1 CTC channel coding per modulation

------------------------------------------------------------End of the Text---------------------------------------------------------