| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Downlink Pilot Transmission for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-08 |  |
| Source(s) | Jie Zhang, Hua Zhou, Jun Tian, Jinyin Xue, Yuantao Zhang, Jian Wang Fujitsu | E-mail: jie.zhang@cn.fujitsu.com zhouhua@cn.fujitsu.com tianjun@cn.fujitsu.com xuejy@cn.fujitsu.com zyuanyao@cn.fujitsu.com jianwang@cn.fujitsu.com |
| Re: | 802.16m amendment working document |  |
| Abstract | The contribution proposes the text of frame structure section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of 8 streams transmission for the IEEE 802.16m Amendment

Jie Zhang, Hua Zhou, Jun Tian, Jinyin Xue, Yuantao Zhang, Jian Wang

Fujitsu

# Introduction

This contribution proposes the text of pilot pattern for different downlink antenna configurations with 2, 4 and 8 streams transmission to be included in the 802.16m amendment. The proposed text is organized so that it can be compliant to the latest 802.16m SDD. In addition, some simulation results in both system level and link level are provided to verify the rationality of the proposed text.

# Motivation to adopt DL 8-stream transmission

Theoretically, the capacity of single link communication is proportional to the minimum value of numbers of transmit and receive antennas. In this section, we shall examine the performance improvement induced by increasing number of transmit/receive antenna pair in sector throughput and user throughput via system level simulation according to the specification in 802.16m EVM document.

Table.1 Simulation parameter settings in SLS

| Antenna configuration | 2x2, 4x4, 8x8 (uncorrelated) |
| --- | --- |
| Number of cell | 19 cells with 3 sectors per cell |
| Channel model | ITU PB 3km/h |
| Frame structure | Specified in latest 16m SDD draft |
| BS transmit power | 46.0 dBm |
| Scheduler | Proportional fairness |
| MCS | 16e MCS set (QPSK 1/2 with rep 1,2,4,6, QPSK 3/4,16QAM 1/2, 3/4, 64QAM 1/2, 2/3, 3/4, 5/6) |
| Coding | Convolutional Turbo coding |
| Link to system mapping method | RBIR |
| Traffic model | Full buffer |
| CQI feedback delay | 5 sub-frames |
| H-ARQ | Chase combining |
| Maximum number of retransmission | 3 |

Fig. 1. CDF curves of average user throughput obtained from 3 antenna configurations.

Fig. 2. CDF curves of average sector throughput obtained from 3 antenna configurations.

Table.2 Simulation results comparison

| Antenna configuration | Average sector throughput | User Coverage(5%tile) |
| --- | --- | --- |
| 2x2 | 1.62567 bps/Hz | 0.02137 bps/Hz |
| 4x4 | 2.49345 bps/Hz | 0.03959 bps/Hz |
| 8x8 | 4.26049 bps/Hz | 0.07667 bps/Hz |

Fig. 1 and 2 compare the sector throughput and user throughput obtained from 2, 4 and 8 streams downlink transmission with 2x2, 4x4 and 8x8 antenna configurations respectively. As antenna correlation is not specified in current EVM document right now, we assume uncorrelated antenna used at both transmit and receive sides in three antenna configurations. From two figures above, we observe obvious advantages in average sector throughput and user throughput from increasing number Tx/Rx antennas.

According to such throughput improvement of 8-stream transmission over 2x2 and 4x4 configurations, we proposed pilot pattern for 8Tx/8Rx antenna pair case. In our contribution “C80216m-09_0215.ppt”, we simulated the link level performance in channel estimate error, BLER and single link throughput. For some details in LLS, please refer to  .

# References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

# Text proposal for inclusion in the 802.16m amendment

-------------------------------------------------  Text Start  -------------------------------------------------

      - Pilot Structure

To facilitate the channel estimation, frequency offset correction, channel quality indicator (CQI) measurement, etc, pilot subcarriers are inserted into time-frequency grid of physical resource unit. Moreover, IEEE 802.16m supports both common and dedicated pilots to maximize the system performance under different channel condition.

##### Pilot Pattern

Pilot patterns are specified within a PRU.

##### 15.3.5.4.1.1 2x2 case

Fig. X.1. Pilot pattern used for 2 DL data streams

Fig X.1 shows the pilot patterns for 2 DL data streams in dedicated and common pilot scenarios. Pi (i=1, 2) denotes the pilot subcarrier used for i-th stream or i-th antenna. For irregular sub-frame with 5 OFDMA symbols, the third symbol is deleted. For irregular sub-frame with 7 OFDMA symbols, the third symbol is duplicated as the 7th symbol.

##### 15.3.5.4.1.2 4x4 case

Fig. X.2. Pilot pattern for 4 pilot streams

Fig X.2 shows the pilot patterns for 4 DL data streams in dedicated and common pilot scenarios. Pi (i=1, 2, 3, 4) denotes the pilot subcarrier used for i-th stream or i-th antenna. For irregular sub-frame with 5 OFDMA symbols, the third symbol is deleted. For irregular sub-frame with 7 OFDMA symbols, the third symbol is duplicated as the 7th symbol.

##### 15.3.5.4.1.3 8x8 case

Fig. X.3. Pilot pattern for 8 pilot streams

Fig X.3 shows the pilot patterns for 8 DL data streams in dedicated and common pilot scenarios. Pi (i =1 ~ 8) denotes the pilot subcarrier used for i-th stream or i-th antenna. For irregular sub-frame with 5 OFDMA symbols, the third symbol is deleted. For irregular sub-frame with 7 OFDMA symbols, the third symbol is duplicated as the 7th symbol.

# ------------------------------------------------  Text End  ----------------------------------------------------