| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for the 802.16m amendment (802.16m-08/050) for section 15.3.8 on Channel Coding and HARQ |  |
| Date Submitted | 2009-01-11 |  |
| Source(s) | Changlong Xu, Tom Harel, Yuval Lomnitz， Yang-seok Choi Intel Corporation | {changlong.xu, tom.harel, yuval.lomnitz, yang-seok.choi}@intel.com |
| Re: | 802.16m amendment text”: IEEE 802.16m-08/053r1, “Call for Contributions on Project 802.16m Draft Amendment Content”. Target topic: “Channel Coding and HARQ”. |  |
| Abstract | The contribution proposes the text of Channel Coding and HARQ section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for the 802.16m amendment (802.16m-08/050) for section 15.3.8 on Channel Coding and HARQ

Changlong Xu, Tom Harel, Yuval Lomnitz，Yang-seok Choi
Intel Corporation

- Introduction

The contribution proposes the text on channel coding and HARQ schemes for 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D7 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- Modifications to the SDD text

The text proposed in this contribution is based on sub clause 11.13 in the IEEE 802.16m SDD [3]. The modifications to the SDD text are summarized below:

- Removed unnecessary informative SDD text [3]

- Removed text describing general features (such as HARQ MIMO, frequency diversity).

- Added introductory text

- Added specific text regarding partitioning into FEC blocks (concatenation rule), CRC, Bit selection, Bit-Re.

- Proposed amendment text structure

We propose to separate the description of channel-coding and other aspects of HARQ. The channel coding is a mechanism used by the HARQ, and also related to non-HARQ transmission, e.g. encoding of control signaling.

HARQ is a process or protocol that has many aspects not related to channel coding:

- Protocol description – usage of ACKCH, difference between UL and DL, HARQ channels, ACID and relation to ARQ/higher layers

- HARQ timing for synchronous ACK in DL-HARQ and a-synchronous HARQ in UL

- Information elements structure in maps for controlling the protocol

Therefore HARQ protocol is not described in the channel coding section, but only the related mechanism of HARQ sub-packet generation.

This separation is in alignment with SDD table of content that separate the HARQ functions (sec. 10.2) and channel coding (sec. 11.13).

This contribution proposes text only for the channel coding section. Contributions [5] and [6] propose related text for the HARQ section of the amendment.

- References

[1] IEEE P802.16 Rev2/D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r6, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0286, “Proposed Text for the IEEE 802.16m Amendment on HARQ Timing”

[6] IEEE 802.16m-09/0306, “HARQ buffer management for IEEE 802.16m”

- Text proposal for inclusion in the 802.16m amendment

------------------------------- Text Start ---------------------------------------------------

## Channel coding

The following channel coding mechanisms are supported:

- CTC (convolutional turbo code) – traffic encoding is based on legacy code described in 8.4.9.2.3, is used for both UL and DL, and includes HARQ-IR support.

- TB-CC (tail biting convolutional code) – used for control channel

### CTC channel coding

#### Overview of channel coding process

Burst encoding process includes

- Appending burst CRC at the end of the burst. The next stages refer to the whole burst including the CRC.

- Partitioning into FEC blocks, each is separately encoded including:

  - Addition of FEC block CRC when more than one FEC block is encoded

  - CTC rate 1/3 encoding, including subblock interleaving

  - Bit selection & repetition – produces the different HARQ sub packet versions; applies repetition coding as particular case; determines mother code rate.

- Bit rearrangement and Modulation

- Bit collection – aggregation of subpackets generated separately for each FEC block

The coding rate is determined by the number of allocated QAM symbols and the number of bits to transmit. The number of inputs bits n divides by 8 (Byte) and is minimum 32. The number of output QAM symbols m divides by 2.

Figure 1 CTC channel coding block diagram

#### Burst CRC

A burst Cyclic Redundancy Check (CRC) is appended to a burst in HARQ mode before the burst is further partitioned into FEC blocks. The burst CRC is calculated based on all the bits in the burst. The detailed CRC is defined in section 15.3.8.1.4.

#### Partition into FEC blocks

The maximum uuencoded FEC block size is 4800 bits. When the burst size exceeds the maximum FEC block size, the burst is partitioned into a number of smaller blocks, each of which is encoded separately. The partitioning is done as follows:

The number of FEC blocks is

K=1n≤4800ceiln4800−16n>4800

The number of bits in FEC block k = 0,1,.. K-1 (before CRC addition) is

nk = 8  floor [ ( n/8 + k) / K ]

Note that n divides by 8 (Byte), nk divides by 8 so that the size of FEC block is in bytes granularity. The number of coded bits (3nk) divides by 24 and consequently by 2Ncbps (Ncbps = number of coded bits per QAM symbol = 2/4/6).

#### CRC encoding

If a burst is partitioned into more than one FEC block, an FEC block CRC is appended to each FEC block before the FEC encoding. The FEC block CRC of an FEC block is calculated based on all the bits in that FEC block.  The size of the CRC is 16 bits. CRC16-CCITT, as defined in ITU-T Recommendation X.25, shall be included at the end of each FEC block.

#### FEC encoding

A double binary Circular Recursive Systematic Convolutional code of code rate of 1/3 defined in section 8.4.9.2.3.1 is used for channel coding. The CTC scheme is extended to support additional FEC block sizes.

If  then 2 zero bits are inserted between the second and the third information bits. In this case, regardless of the transmitted information,. The block size for encoding is

.

At the output of the encoder, if , the following bits are discarded:. The number of coded bits is.

##### CTC inner interleaver

The CTC inner interleaver is specified in 8.4.9.2.3.2 and parameters for additional FEC block sizes are TBD.

##### Subblock interleaver

The outputs of encoder block depicted in Figure 1 are inputs to sub-block interleavers. The interleaving details are TBD.

#### Bit selection and repetition

The bit selection process is controlled by the following parameters:

- The number of coded bits 3nk

- Mother code rate RMC

- Number of coded bits per QAM symbol (2/4/6) Ncbps

- Number of QAM symbols mk allocated to the FEC block defined in 15.3.8.1.10.

- SPID (subpacket ID) [alternatively: Subpacket starting index i0]

The output of the bit selection is the sequence of mkNcbps bits and the parameters i0, NRMC.

The mother code rate is determined as follows. The default mother code rate is 1/3. The mother code rate is fixed within the HARQ session and may be changed (per MS) by the BS by sending a TBD control message specified in 15.X.X.X. The new mother code rate then applies to new generated HARQ sessions starting following and including the second frame after the frame in which the TBD control message was received (i.e. next – next frame). The mother code rate for non HARQ transmissions is 1/3. The mother code rates are specified in table 1

Table 1: mother code rates

| Index | Mother code rate (RMC) |
| --- | --- |
| 0 | 1 / 3 |
| 1 | 1 / 2 |
| 2 | 3 / 4 |
| 3 | 5 / 6 |

Denote the bit index following CTC encoding and subblock interleaving by i = 0, … , 3nk-1. The first

NRMC = 24  ceil(nk / (24  RMC))

coded bits are used for bit selection. The subpacket start index i0 is determined by SPID (subpacket ID = 0, .., 7) as

i0 = 24  floor[ SPIDnk / (192  RMC) ]

The output bit j (=0, .., mkNcbps-1) equals input bit

i = (i0 + j) mod NRMC

#### Modulation

Modulation constellations of QPSK, 16 QAM, and 64 QAM defined in section 8.4.9.4 is used for the legacy system. The mapping of bits to the constellation point depends on the bit-rearrangement (BitRe) version transmitted with two possible values (0,1) used for HARQ re-transmission and affect the mapping of bits over the QAM constellation and over 2 symbols. The flipping over 2 QAM symbols is intended for MIMO operation but is applied in all modes.

The bit rearrangement scheme is applied separately to each 2 Ncbps bits before modulation according to table 2:

Table 2 – Bit Rearrangement

| Constellation | Ncbps | Version (v) | Bits |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | First symbol |  |  |  |  |  | Second symbol |  |  |  |  |  |
| QPSK | 2 | 0 | b0 | b1 | - | - | - | - | b2 | b3 | - | - | - | - |
| QPSK | 2 | 1 | b2 | b3 | - | - | - | - | b0 | b1 | - | - | - | - |
| 16 QAM | 4 | 0 | b0 | b1 | b2 | b3 | - | - | b4 | b5 | b6 | b7 | - | - |
| 16 QAM | 4 | 1 | b5 | b4 | b7 | b6 | - | - | b1 | b0 | b3 | b2 | - | - |
| 64 QAM | 6 | 0 | b0 | b1 | b2 | b3 | b4 | b5 | b6 | b7 | b8 | b9 | b10 | b11 |
| 64 QAM | 6 | 1 | b8 | b7 | b6 | b11 | b10 | b9 | b2 | b1 | b0 | b5 | b4 | b3 |

The version v is determined as follows: v=BitRe Version (from USCCH described in 15.xx.xx) for the first kvf bits (kvf/Ncbps symbols) and is then flipped for the next NRMC bits (NRMC/Ncbps symbols) and flipped again each NRMC bits, i.e. for bit j (=0, .., mkNcbps-1) the version is:

vj = (floor[(j + i0) / NRMC] + BitReVersion) mod 2

Note that v is fixed over any symbol pair. Following the rearrangement every successive Ncbps bits are QAM modulated according to constellations defined in section 8.4.9.4. The constellations shall be normalized by multiplying the constellation point with the indicated factor c to achieve unit average power.

#### Collection

The total number m of available QAM symbols (m even) is partitioned into units of mk symbols as follows:

mk = 2  floor [ ( m/2 + k) / K ], 	k = 0, … K-1

Where K is the number of FEC blocks defined in 15.3.8.1.3. After modulation, the mk bits formed from each FEC block are concatenated in the same order (of k) to produce the m QAM symbols.

### TB-CC channel coding

Each USCCH block is encoded by the binary tail-biting convolutional encoder with rate of 1/4, a constraint length equal to K = 7, and shall use the following generator polynomials codes to derive its four code bits:

G1=171OCT，G2=133OCT，G3=165OCT，G4=117OCT

------------------------------- Text End ---------------------------------------------------

The referred section includes some text that is not relevant. We actually need to refer only to the first paragraphs (and figure) up to (and not including) the paragraph “Note that the interleaver (8.4.9.3) shall not be used when using CTC”.