| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Data mapping for downlink and uplink SFBC (amendment) |  |
| Date Submitted | 2009-1-7 |  |
| Source(s) | Xiaolong ZHU, Dong LI, Hongwei YANG, Keying WU Alcatel-Lucent Shanghai Bell Co., Ltd. | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 Mail to: Hongwei.yang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-08/053r1 Call for Comments and Contributions on Project 802.16m Amendment Working Document: 11.8 DL MIMO & 11.12 UL MIMO |  |
| Abstract | This contribution proposes the data mapping for downlink and uplink space-frequency block coding (SFBC), which aligns well with the one specified for space-time block coding (STBC) in legacy IEEE802.16e systems. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Data Mapping for Downlink and Uplink SFBC

Xiaolong ZHU, Dong LI, Hongwei YANG, Keying WU

Alcatel-Lucent Shanghai Bell Co., Ltd.

# 1 Introduction

In the current 16m SDD [1], space-frequency block coding (SFBC) is accepted as one of transmit diversity schemes for both downlink and uplink. However, the data mapping for SFBC has not been addressed yet.

2 Considerations on data mapping for SFBC

The proposed data mapping shall agree with SDD decisions made on single-user MIMO (SU-MIMO) [1]. Specifically, for the downlink (refer to 11.8.2.1.1.1) and uplink (refer to 11.12.2.1.1.1) transmit diversity scheme of SFBC, the input to the MIMO encoder is x=[s1;s2], and the MIMO encoder generates the following matrix

where the row indicates the index of transmit antennas, the column indicates the index of subcarriers, and the superscript “*” denotes complex conjugate.

The proposed data mapping shall also agree with the physical structures of downlink (refer to 11.5) and uplink (refer to 11.6) including the definitions of pilot patterns and resource units.

Finally, the proposed data mapping for SFBC shall align well with the one specified for space-time block coding (STBC) in legacy 802.16e systems as shown in Figure 0 (cf. Figure 274 in [2]). That is, it is desired that the legacy STBC encoder (decoder) can be reused for 802.16m SFBC by only adjusting the data mapping (de-mapping) pattern. This requirement will simply the implementation and reduce the cost.

The performance comparison between STBC and SFBC under various channel conditions (in terms of mobility and delay spread) can be found in [3]. According to [3], SFBC performs satisfactorily in most scenarios.

Figure 0 ---- Data mapping example for STBC in legacy 802.16e systems [2].

3. Text proposal for inclusion in the 802.16m amendment

--------------------------------------------- Start of Text Proposal ---------------------------------------------------------------

15.3.8 DL MIMO

15.3.8.X.1 Allocation of pilot subcarriers

Pattern A is used for one and two downlink data streams dedicated and common pilot scenarios, as shown in Figure 1.

The pilot locations are expressed by

Pilot Location for Stream #0 = 18*k – 8*mod(m+2,3) + 16

Pilot Location for Stream #1 = 18*k – 8*mod(m+2,3) + 17

Where m = [symbol index], symbol index 0 is the first symbol in which the STC zone is applied, k is the LRU index numbering from 0. That is, symbol index shall be reset to 0 when a new STC zone is applied. The first subcarrier of the first LRU is numbered from 0.

If interlaced pilot pattern A is used as shown in Figure 2, then m = mod([symbol index] - [cyclic shift], 6), where cyclic shift = 0,1,2 for the three interlacing patterns, respectively.

Figure 1 ---- Pattern A for one or two pilot streams.

Figure 2 ---- Interlaced Pattern A for one or two pilot streams.

For the type-1 subframe containing one idle symbol, the last OFDM symbol in Figure 1 (or Figure 2) is replaced with an idle symbol. For the type-2 subframe, the first OFDM symbol in Figure 1 (or Figure 2) is added as the 7th symbol.

15.3.8.X.2 Allocation of data subchannels

15.3.8.X.2.1 Open-loop SU-MIMO

15.3.8.X.2.1.1 Transmit Diversity

For the transmit diversity scheme with M = 2, the input to the MIMO encoder is represented as a 2x1 vector x=[s1;s2]. The MIMO encoder generates the SFBC matrix

, Equation 1

where the row indicates the index of transmit antennas, the column indicates the index of subcarriers, and the superscript “*” denotes complex conjugate. Then the output of the MIMO encoder is multiplexed by NTx2 matrix W, where W is described in Section 15.3.8.X.2.1.

The modulated data symbols shall be sequentially mapped for two transmit antennas in LRU-first order. The mapping inside one LRU continues first in an ascending manner along the subcarriers of the first OFDM symbol and then proceeds to the next OFDM symbol in time. When the LRU boundary is reached, the mapping proceeds to the next LRU. An illustration of the mapping rule for two transmit antennas is shown in Figure 3, assuming pilot pattern A with up to seven OFDM symbols in one LRU. For the general type-1 subframe, the seventh OFDM symbol is removed in Figure 3. For the type-1 subframe containing one idle symbol, the seventh OFDM symbol is removed and the sixth OFDM symbol in Figure 3 is replaced with an idle symbol.

Figure 3 ---- Data mapping of SFBC for 2Tx antennas using pilot Pattern A with up to 7 OFDM symbols in one LRU.

15.3.12 UL MIMO

15.3.12.X.1 Allocation of pilot subcarriers

The downlink 18x6 pilot pattern shown in Figure 1 is also applicable to the uplink 18x6 pilots. For the type-1 subframe containing one idle symbol, the last OFDM symbol in Figure 1 is replaced with an idle symbol. For the type-2 subframe, the first OFDM symbol in Figure 1 is added as the 7th symbol.

Dedicated pilots are applied on the uplink.

For the 6x6 uplink tile, the pilot pattern for DRU is shown in Figure 4. The pilot locations are expressed by

Pilot Location for Stream #0 = 6*t + 4*mod(m,4) with m ≠ 2 and m ≠ 3

Pilot Location for Stream #1 = 6*t + 4*mod(m,4) + 1 with m ≠ 2 and m ≠ 3

Where m = [symbol index], symbol index 0 is the first symbol in which the STC zone is applied, t is the tile index numbering from 0. That is, symbol index shall be reset to 0 when a new STC zone is applied. The first subcarrier of the first tile is numbered from 0.

Figure 4 ---- Uplink distributed RU tile structure.

15.3.12.X.2 Allocation of data subchannels

15.3.12.X.2.1 Open-loop SU-MIMO

15.3.12.X.2.1.1 Transmit Diversity

For the transmit diversity scheme with M = 2, the input to the MIMO encoder is represented as a 2x1 vector x=[s1;s2]. The MIMO encoder generates the SFBC matrix

, Equation 2

where the row indicates the index of transmit antennas, the column indicates the index of subcarriers, and the superscript “*” denotes complex conjugate. Then the output of the MIMO encoder is multiplexed by NTx2 matrix W, where W is described in section 15.3.12.X.2.1.

The data mapping rule for downlink SFBC as illustrated in Figure 3 is also used for the 18x6 uplink subchannel structure.

For the 6x6 uplink tile and the uplink physical structure for legacy support, the data mapping for SFBC is defined according to the following rule. The modulated data symbols shall be sequentially mapped for two transmit antennas in LRU-first order. When the LRU boundary is reached, the mapping proceeds to the next LRU. The mapping inside one LRU is performed in tile-first order and inside one tile the mapping continues first in an ascending manner along the subcarriers of the first OFDM symbol and then proceeds to the next OFDM symbol in time. When the tile boundary is reached, the mapping proceeds to the next tile. An illustration of data mapping for SFBC for the 6x6 tile structure is shown in Figure 5.

Figure 5 ---- Data mapping of SFBC for 2Tx antennas with 6x6 uplink tile structure.

---------------------------------------------- End of Text Proposal ---------------------------------------------------------------

References

- IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document.”

- IEEE P802.16Rev2/D8,

- December 2008.

- IEEE C802.16m-08/838, “Updated link-level simulation results for STBC and SFBC.”