| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Precoder for downlink open-loop SU-MIMO |  |
| Date Submitted | 2009-1-7 |  |
| Source(s) | Xiaolong ZHU, Hongwei YANG, Dong LI, Keying WU Alcatel Shanghai Bell Co., Ltd. | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 Mail to: Hongwei.yang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-08/053r1 Call for Comments and Contributions on Project 802.16m Amendment Working Document: 11.8 DL MIMO |  |
| Abstract | This contribution proposes the precoder codebook and parameters for downlink open-loop SU-MIMO. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Precoder for Downlink Open-Loop SU-MIMO

Xiaolong ZHU, Hongwei YANG, Dong LI, Keying WU

Alcatel Shanghai Bell Co., Ltd.

# 1 Introduction

In the current 16m SDD [1], precoder is accepted as an element of downlink open-loop (OL) single-user (SU-) MIMO transmit diversity and spatial multiplexing schemes. However, the detailed unitary codebook, the parameters u and v are FFS. Additionally, it is also open whether the CL SU-MIMO and OL SU-MIMO may use different unitary codebooks.

# 2 Considerations on precoder for downlink OL SU-MIMO

To allow implementation-specific precoder in the open-loop case, the OL SU-MIMO should have the possibility to use a different unitary codebook from the CL SU-MIMO.

Precoder codebook should allow the implementation of such schemes as MIMO plus cyclic delay diversity (CDD) and MIMO plus beamforming, which are supported already in WiMAX Forum mobile system profiles Release 1.0 and Release 1.5.

In addition, precoder codebook should also allow the implementation of antenna selection or its variants like antenna hopping, time/frequency switching diversity, etc. Antenna selection has the advantage of deploying less RF modules than the number of physical antennas, thus saving implementation cost among others.

For dedicated pilots, the pilot subcarriers are precoded using the same precoder as the data subcarriers. To avoid the adverse impact of precoder on channel estimation at the receiver side, the parameter u ( or v) should be set as multiple of the number of subcarriers (or OFDM symbols) contained in one unit of resource block for channel estimation.

# 3. Text proposal for inclusion in the 802.16m amendment

--------------------------------------------- Start of Text Proposal ---------------------------------------------------------------

15.3.8 DL MIMO

15.3.8.X.Y Open-Loop SU-MIMO

15.3.8.X.Y.Z Precoder

On a given subcarrier k, the precoding matrix P can be defined using the following equation:

P(k) = W(k), Equation 1

W(k) is an NT x Ns matrix, where NT is the number of transmit antennas and Ns is the number f streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u subcarriers and/or v OFDM symbols. The matrix W(k) can be identity matrix for 2Tx rate-2 and 4Tx rate-4. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix.

The OL SU-MIMO and CL SU-MIMO may use different unitary codebooks.

For NT = 4 and Ns = 2, the unitary codebook consists of all or a subset of the following matrices

,,,

,,,,,

where D is the 4x4 identity matrix or an implementation-specific diagonal unitary matrix.

The parameters are set as u = L1 x Psc and v = L2 x Nsym, where L1 and L2 are positive integers (default value = 1), Psc and Nsym are the number of subcarriers and the number of OFDM symbols inside one LRU, respectively. Take CRU and L1 = L2 = 1 as an example, u = 18, v = 6 for type-1 subframe and v = 7 for type-2 subframe. ABS exchanges with AMS about the current setting of L1 and L2 via IEs and/or MAC messages if they are not the default values.

---------------------------------------------- End of Text Proposal ---------------------------------------------------------------

References

- IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document.”