| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on UL Control Structure for the IEEE 802.16m Amendment Working Document |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Dongsheng Yu, Sophie Vrzic, Mo-Han Fong, Robert Novak, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan Nortel Networks | dongshengyu@nortel.com mhfong@nortel.com |
| Re: | 802.16m amendment working document In response to IEEE 802.16m-08/053r1 “Call for Contributions and Comments on Project 802.16m Amendment Working Document” for Session 59 |  |
| Abstract | Contribution with proposed text on UL PHY control for the IEEE 802.16m Amendment Working Document (IEEE 802.16m-08/050) |  |
| Purpose | Discuss and adopt into IEEE 802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on UL Control Structure for the IEEE 802.16m Amendment Working Document

Dongsheng Yu, Sophie Vrzic, Mo-Han Fong, Robert Novak, Hosein Nikopourdeilami,

Kathiravetpillai Sivanesan

Nortel Networks

- Introduction

This contribution proposes the text for the UL control structure section to be included in the 802.16m amendment. The proposed text is compliant to the 802.16m SRD (IEEE 802.16m-08/002r7) and the 802.16m SDD (IEEE 802.16m-08/003r6), and it follows the style and format guidelines in IEEE 802.16m-08/043.

- Text Proposal

--------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add the text below to page 30, line 22 of IEEE 802.16m-08/050]

15.3 Physical Layer

#### 15.3.8 UL Control Structure

15.3.8.1 UL Fast Feedback Channel

The UL fast feedback channel carries channel quality feedback, MIMO feedback and BW REQ indicators.

There are two types of UL fast feedback control channels: primary and secondary fast feedback channels. The UL primary fast feedback control channel provides wideband feedback information including channel quality and MIMO feedback. It is used to support robust feedback reports. The UL secondary fast feedback control channel carries narrowband CQI and MIMO feedback information. The UL fast feedback channel is FDM with other UL control and data channels.

The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an AMS are configurable based on traffic, channel conditions etc. The number of bits carried in the fast feedback channel can be adaptive. The specific type of feedback information carried on each fast feedback opportunity is different. The number of bits carried in the fast feedback channel can be adaptive.

The UL fast feedback channel carries one or more types of fast feedback information. Multiplexing fast feedback channels from one or more users is FDM.

A UL feedback mini-tile (FMT) is defined as 2 contiguous subcarriers by 6 OFDM symbols for subchhanelization shared with UL PUSC. A UL FMT of 6 contiguous subcarriers by 2 OFDM symbols is for other channelization. The primary and secondary fast feedback channels are comprised of NFMT distributed FMTs. One LRU consists of 9 FMTs and can be shared by multiple fast feedback channels.

No pilot is allocated in FMT for primary fast feedback channel.

Table XXX shows 64 sequences for primary fast feedback channel. This set of sequence can carrier up to 6 information bits. The mapping of these sequences on both feedback mini-tile is shown in Figures XXX and YYY.

Table 1 sequence of for primary fast feedback channel

| index | sequence | index | sequence | index | Sequence |
| --- | --- | --- | --- | --- | --- |
| 1 | 111111111111 | 22 | 101001101111 | 43 | 110010010101 |
| 2 | 101010101010 | 23 | 110000001001 | 44 | 100111000000 |
| 3 | 110011001100 | 24 | 100101011100 | 45 | 111101010110 |
| 4 | 100110011001 | 25 | 111111000101 | 46 | 101000000011 |
| 5 | 111100001111 | 26 | 101010010000 | 47 | 110001100101 |
| 6 | 101001011010 | 27 | 110011110110 | 48 | 100100110000 |
| 7 | 110000111100 | 28 | 100110100011 | 49 | 111110011100 |
| 8 | 100101101001 | 29 | 111100110101 | 50 | 101011001001 |
| 9 | 111111110000 | 30 | 101001100000 | 51 | 110010101111 |
| 10 | 101010100101 | 31 | 110000000110 | 52 | 100111111010 |
| 11 | 110011000011 | 32 | 100101010011 | 53 | 111101101100 |
| 12 | 100110010110 | 33 | 111110101001 | 54 | 101000111001 |
| 13 | 111100000000 | 34 | 101011111100 | 55 | 110001011111 |
| 14 | 101001010101 | 35 | 110010011010 | 56 | 100100001010 |
| 15 | 110000110011 | 36 | 100111001111 | 57 | 111110010011 |
| 16 | 100101100110 | 37 | 111101011001 | 58 | 101011000110 |
| 17 | 111111001010 | 38 | 101000001100 | 59 | 110010100000 |
| 18 | 101010011111 | 39 | 110001101010 | 60 | 100111110101 |
| 19 | 110011111001 | 40 | 100100111111 | 61 | 111101100011 |
| 20 | 100110101100 | 41 | 111110100110 | 62 | 101000110110 |
| 21 | 111100111010 | 42 | 101011110011 | 63 | 110001010000 |
|  |  |  |  | 64 | 100100000101 |

15.3.8.1.1 UL Fast Feedback Channel Formats

15.3.8.2 UL Sounding Channel

The UL sounding channel is used by an AMS to send a sounding signal for MIMO feedback, channel quality feedback and acquiring UL channel information at the ABS. The ABS can configure an AMS to transmit an UL sounding signal on specific UL sub-bands. The sounding signal is transmitted over predefined subcarriers within the intended sub-bands. The periodicity of the sounding signal for each AMS is configurable. The UL sounding channel is FDM with other control and data channels.

The ABS can configure multiple AMSs to transmit UL sounding signals on the corresponding UL sounding channels.  The UL sounding channels from multiple users or multiple antennas per user is FDM.

Antenna switching among available physical antenna at the MS is applied to make UL sounding channel providing full MIMO channel information for DL transmission.

-----------------------------------------------------------End of the Text---------------------------------------------------------