| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Padding / De-padding method for 16m CTC |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Kenichi KURI*, Isamu YOSHII*, * Panasonic Corporation | Voice: +81-50-3687-6556 E-mail: kuri.kenichi@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | 802.16m Amendment Working Document - Target topic: “11.13 Channel Coding and HARQ” |  |
| Abstract | We present padding / de-padding method for CTC (Convolutional Turbo code). |  |
| Purpose | Discussion and adoption for 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Padding / De-padding method for 16m CTC

Kenichi KURI*,  Isamu YOSHI*

* Panasonic Corporation

# Introduction

In 802.16m System Description Document is described that FEC block sizes are FFS[1]. And the contribution(08/1295) has proposed that the number of FEC block size is increased in 16m system[2]. If the input PHY burst size to CTC encoder doesn't fit any FEC block size, the encoder adds padding bits to PHY burst in order to adjust them to inner interleaver size of CTC. We recommend removing padding bits and related parity bits at rate-matching part for the purpose of improving transmission efficiency. This contribution proposes the partial de-padding method.

# De-padding method

There are four alternatives of de-padding method corresponding to Fig.1,2,3 and 4 (Full de-padding method, Partial de-padding method-1(A, B, Y1, W1), Partial de-padding method-2(A, B), No de-padding method).

Figure 1 Full de-padding method

Figure 2 Partial de-padding method-1 (A, B, Y1, W1)

Figure 3 Partial de-padding method-2 (A, B)

Figure 4 No de-padding method

Figure 5 shows an example of trellis diagram based on CTC tail-biting. This example assumes 4 information bits and 4 padding bits inserted to the front of information sequence. The parity bits related to padding bits are unknown because of tail-biting. Therefore full de-padding method and partial de-padding method-1 may be degraded by de-padding parity bits related to padding bits compared to partial de-padding method-2 and no de-padding method. The performance comparison among four de-padding methods is indicated in the next section.

Figure 5 example of trellis diagram based on CTC tail-biting

# Simulation results

Error: Reference source not found provides a list of simulation parameters. Error: Reference source not found shows coded bits and the puncturing condition of each de-padding method. All de-padding methods read out 3840 bits from circular buffer, these selection bits are different according to each de-padding method.

Table 1 Simulation parameters

| Parameter | Value |
| --- | --- |
| Number of information bits for packet | 1920 [bit] |
| Number of padding bit | 64 [bit] |
| coding rate (# of transmission bits) | R=1/2 (3840bit) |
| modulation | QPSK |
| Channel model | AWGN |

Figure 6 puncturing condition of each de-padding method

Error: Reference source not found shows the block error rate (BLER) vs. SNR performance comparison among four de-padding methods. The performance of full de-padding method and partial de-padding method-1 are almost same and better than partial de-padding method-2 and no de-padding method. We found that removing parity bits related to padding bits doesn’t affect error rate performance from this simulation result. Considering the complexity of de-padding parity bits(Y2, W2), the partial de-padding method-1 is simpler than full de-padding method. Therefore we propose that the partial de-padding method-1 are applied to rate-matching block for 16m CTC.

Figure 7 Block error rate vs. Eb/N0 performance comparison among four de-padding methods

# Conclusion

In this contribution de-padding method for rate-matching block is presented. We confirmed that the partial de-padding method-1 is the best compared to other methods in terms of error rate performance and processing complexity.

---------------------------- Begin Proposed Text --------------------------------------------------------------------------

15.3.x Channel coding and HARQ

15.3.x.1 Channel coding

15.3.x.1.3 FEC encoding

IEEE 802.16m uses the CTC (convolutional turbo code) of code rate 1/3 defined in the IEEE 802.16e standard where the CTC inner interleaver parameters for additional FEC block sizes are FFS while maintaining IEEE 802.16e CTC interleaver. The code rate of the “FEC Encoder” block in Figure 53 is termed mother code rate (RMC). The use of other coding schemes like CC and LDPC are FFS.

The CTC scheme is extended to support additional FEC block sizes. FEC block sizes larger than the legacy ones are supported. The FEC block sizes are FFS and they are independent of the transmission format, including code rate, modulation order, and resource allocation. Further, the FEC block sizes are regularly increased with pre-determined block size resolutions. The FEC block sizes which are multiple of 7 shall be removed for the tail-biting encoding structure. If the input size to CTC encoder after partition doesn't fit any FEC block size, the encoder adds padding bits to input burst in order to adjust them to inner interleaver size of CTC.

The encoder block depicted in Figure 53 includes the sub-block interleavers. The interleaving details

17 are FFS.

15.3.x.1.4 Bit selection and repetition

Bit selection and repetition are used in 802.16m to achieve rate matching. Bit selection adapts the number of coded bits to the size of the resource allocation (in QAM symbols) which may vary depending on the LRU and subframe type. The total subcarriers in the allocated LRU are segmented to each FEC block. Mother Code Bits, the total number of information and parity bits generated by FEC encoder, are considered as a maximum size of circular buffer. In case that the size of the circular buffer Nbuffer is smaller than the number of Mother Code Bits, the first Nbuffer bits of Mother Code Bits are considered as selected bits. Repetition is performed when the number of transmitted bits is larger than the number of selected bits. The selection of coded bits is done cyclically over the buffer. If the FEC block includes some padding bits, the padding bits (A, B) and related parity bits (Y1, W1) are skipped from circular buffer in selection of coded bits.

---------------------------End of Text Proposal ---------------------------------------------------------------------------

# Reference

- 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

- C802.16m-08/1295, “IEEE 802.16m CTC data block sizes”