| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Channel Coding and HARQ for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-08 |  |
| Source(s) | Sung-Eun Park, Chiwoo Lim, Seunghoon Choi, Songnam Hong, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. | +82-31-279-5249 se.park@samsung.com |
| Re: | “802.16m amendment working document”: IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document”. Target topic: “Channel Coding and HARQ”. |  |
| Abstract | The contribution proposes the text of channel coding and HARQ section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Channel Coding and HARQ 
for the IEEE 802.16m Amendment

Sung-Eun Park, Chiwoo Lim, Seunghoon Choi, Songnam Hong, Jaeweon Cho, Jaehee Cho, 
Heewon Kang, Hokyu Choi

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of channel coding and HARQ section to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- Modifications to the SDD text

The text proposed in this contribution is based on subclauses 11.13 in the IEEE 802.16m SDD [3]. The modifications to the SDD text are summarized below:

- MCS table has 16 levels with equi-distance SNR and includes different information for initial transmission and retransmission respectively. (Refer to IEEE C802.16m-09/0302 and IEEE C802.16m-09/0303 or their latest versions)

- CRC encoding is defined for both burst CRC and FEC block CRC.

- Burst partition scheme is provided with 155 information size. (Refer to IEEE C802.16m-09/0304 or its latest version)

- New CTC bit grouping to enhance the performance for higher order modulation is provided. (Refer to IEEE C802.16m-09/0301 or its latest version)

- Efficient starting position for adaptive HARQ is defined.

- Etc.

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r6, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- Abbreviations and acronyms

Insert the following at section 4 in alphabetic order:

CoRe		constellation re-arrangement

HARQ		hybrid-ARQ

IR			incremental redundancy

SPID			subpacket ID

Insert the following subsection at a new section 15:

  - Channel coding and HARQ

    - Channel coding

Channel coding procedures are shown in Figure aaa.

Figure aaa ― Channel coding procedure

      - MCS table

To determine the modulation and code rate for current transmission, the AMS shall read the 4-bit ’MCS index’ and 1-bit ‘AI-SN’ in MAP.

The MCS index represents the different information according to AI-SN. For AI-SN=0 (initial transmission), it denotes the modulation and code rate as shown in Table aaa, and for AI-SN=1 (retransmission), it denotes modulation, CoRe version, and SPID as shown in Table bbb.

In Table bbb, the “+” and “-“ are utilized when modulation at retransmission is not same to that at initial transmission as follows.

- When modulation is QPSK at initial transmission, “-” and “+” represent 64QAM and 16QAM at retransmission, respectively.

- When modulation is 16QAM at initial transmission, “-” and “+” represent QPSK and 64QAM at retransmission, respectively.

- When modulation is 64QAM at initial transmission, “-” and “+” represent 16QAM and QPSK at retransmission, respectively.

Table aaa ― MCS for initial transmission (AI-SN=0)

| MCS index | Modulation | Code rate |
| --- | --- | --- |
| 0 | QPSK | 31/256 |
| 1 | QPSK | 47/256 |
| 2 | QPSK | 70/256 |
| 3 | QPSK | 98/256 |
| 4 | QPSK | 131/256 |
| 5 | QPSK | 166/256 |
| 6 | QPSK | 199/256 |
| 7 | 16QAM | 123/256 |
| 8 | 16QAM | 149/256 |
| 9 | 16QAM | 176/256 |
| 10 | 16QAM | 204/256 |
| 11 | 16QAM | 229/256 |
| 12 | 64QAM | 173/256 |
| 13 | 64QAM | 196/256 |
| 14 | 64QAM | 218/256 |
| 15 | 64QAM | 234/256 |

Table bbb ― MCS for retransmission (AI-SN=1)

| MCS index | Modulation | SPID | CoRe version |
| --- | --- | --- | --- |
| 0 | Modulation is same as initial transmission. | 0 | 0 |
| 1 |  | 0 | 1 |
| 2 |  | 1 | 0 |
| 3 |  | 1 | 1 |
| 4 |  | 2 | 0 |
| 5 |  | 2 | 1 |
| 6 |  | 3 | 0 |
| 7 |  | 3 | 1 |
| 8 | - | 0 | 0 |
| 9 | - | 1 | 0 |
| 10 | - | 2 | 0 |
| 11 | - | 3 | 0 |
| 12 | + | 0 | 0 |
| 13 | + | 1 | 0 |
| 14 | + | 2 | 0 |
| 15 | + | 3 | 0 |

      - Randomization

Data randomization is performed on all data transmitted on the downlink and uplink. If the amount of data to transmit does not fit exactly the amount of data allocated, padding of 0xFF (“1” only) shall be added to the end of the transmission block, up to the amount of data allocated. The amount of data allocated shall be calculated including the CRC bits.

The PRBS generator shall be 1+X14+X15, as shown in Figure bbb. Each data to be transmitted shall enter sequentially into the randomizer, MSB first. SCH are not randomized. The seed value shall be used to calculate the randomization bits, which are combined in an XOR operation with the serialized bit. The randomizer is initialized with the vector [LSB] 0 1 1 0 1 1 1 0 0 0 1 0 1 0 1 [MSB].

Figure bbb ― The PRBS generator

The randomizer sequence is applied only to information bits. The bit issued from the randomizer shall be applied to the encoder.

      - Burst CRC encoding

Cyclic Redundancy Code (CRC) bits are used to detect errors in the received packets. The 16 CRC bits are appended to the burst with the following cyclic generator polynomial:

gCRC(D)=D16+D12+D5+1

Denote the bits in a input packet by d1, d2 ,d3, ⋯, dnand the parity bits are p1, p2 ,p3, ⋯, p16 wheren is the size of a input packet. The encoding is performed in a systematic form, which means that in GF(2), the polynomial:

d1Dn+15+ d2Dn+14+ ⋯ +dnD16+p1D15+p2D14+⋯+p15D1+p16

yields a remainder equal to 0 when divided by gCRC(D).

      - Burst partition

When the burst size including 16 burst CRC bits exceeds maximum FEC block size, the burst is partitioned into the NUM_FEC_BLOCK blocks. The size of each partitioned burst is same as NEP, the FEC input block size. NUM_FEC_BLOCK is calculated by following equation.

NUMFECBLOCK=MOD⋅CodeRate⋅NRENEPMAX

where

MOD 	is a modulation order specified by MCS index, 
CodeRate 	is a code rate specified by MCS index,
NRE 		is the number of resource element allocated for burst transmission, 
NEP_MAX	is the maximum FEC block size equal to 4800 bits.

The FEC encoder input size NEP for each FEC block is calculated by following equation.

NEP=argmaxX∈NEPX≤(MOD⋅CodeRate⋅NRENUMFECBLOCK)

where

{NEP} 	is a set of NEP shown in Table ccc.

Table ccc ― The set of encoder input

| Index | NEP | Index | NEP | Index | NEP | Index | NEP | Index | NEP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 48 | 32 | 344 | 63 | 720 | 94 | 1440 | 125 | 2880 |
| 2 | 64 | 33 | 352 | 64 | 736 | 95 | 1472 | 126 | 2944 |
| 3 | 72 | 34 | 360 | 65 | 752 | 96 | 1504 | 127 | 3008 |
| 4 | 80 | 35 | 368 | 66 | 768 | 97 | 1536 | 128 | 3072 |
| 5 | 88 | 36 | 376 | 67 | 792 | 98 | 1576 | 129 | 3144 |
| 6 | 96 | 37 | 384 | 68 | 800 | 99 | 1600 | 130 | 3200 |
| 7 | 104 | 38 | 400 | 69 | 816 | 100 | 1632 | 131 | 3264 |
| 8 | 120 | 39 | 408 | 70 | 832 | 101 | 1664 | 132 | 3328 |
| 9 | 128 | 40 | 416 | 71 | 848 | 102 | 1696 | 133 | 3392 |
| 10 | 136 | 41 | 424 | 72 | 864 | 103 | 1728 | 134 | 3456 |
| 11 | 144 | 42 | 432 | 73 | 880 | 104 | 1760 | 135 | 3520 |
| 12 | 152 | 43 | 440 | 74 | 904 | 105 | 1800 | 136 | 3592 |
| 13 | 160 | 44 | 456 | 75 | 912 | 106 | 1824 | 137 | 3648 |
| 14 | 176 | 45 | 464 | 76 | 928 | 107 | 1856 | 138 | 3712 |
| 15 | 184 | 46 | 472 | 77 | 944 | 108 | 1888 | 139 | 3776 |
| 16 | 192 | 47 | 480 | 78 | 960 | 109 | 1920 | 140 | 3840 |
| 17 | 200 | 48 | 488 | 79 | 976 | 110 | 1952 | 141 | 3904 |
| 18 | 208 | 49 | 496 | 80 | 992 | 111 | 1984 | 142 | 3968 |
| 19 | 216 | 50 | 512 | 81 | 1024 | 112 | 2048 | 143 | 4040 |
| 20 | 232 | 51 | 528 | 82 | 1056 | 113 | 2112 | 144 | 4096 |
| 21 | 240 | 52 | 544 | 83 | 1088 | 114 | 2176 | 145 | 4160 |
| 22 | 248 | 53 | 568 | 84 | 1128 | 115 | 2248 | 146 | 4224 |
| 23 | 256 | 54 | 576 | 85 | 1152 | 116 | 2304 | 147 | 4288 |
| 24 | 264 | 55 | 592 | 86 | 1184 | 117 | 2368 | 148 | 4352 |
| 25 | 272 | 56 | 608 | 87 | 1216 | 118 | 2432 | 149 | 4416 |
| 26 | 288 | 57 | 624 | 88 | 1248 | 119 | 2496 | 150 | 4488 |
| 27 | 296 | 58 | 640 | 89 | 1280 | 120 | 2560 | 151 | 4544 |
| 28 | 304 | 59 | 656 | 90 | 1312 | 121 | 2624 | 152 | 4608 |
| 29 | 312 | 60 | 680 | 91 | 1352 | 122 | 2696 | 153 | 4672 |
| 30 | 320 | 61 | 688 | 92 | 1376 | 123 | 2752 | 154 | 4736 |
| 31 | 328 | 62 | 704 | 93 | 1408 | 124 | 2816 | 155 | 4800 |

The burst size NB including burst CRC and FEC block CRC is defined by following equation.

NB = NUM_FEC_BLOCK × NEP

Then pure payload size excluding burst CRC and FEC block CRC is defined by following equation

NPAYLOAD = NUM_FEC_BLOCK × (NEP – k×NCRC_FEC) – NCRC_BURST

where,

k		is 0 when NUM_FEC_BLOCK=1, 1 when NUM_FEC_BLOCK>1,
NCRC_FEC	is the size of FEC block CRC equal to 16 bits,
NCRC_BURST	is the size of burst CRC equal to 16 bits.

.

      - FEC block CRC encoding

The 16 CRC bits are appended to each FEC block only if the burst is partitioned into the multiple FEC blocks. The cyclic generator in 15.x.1.3 is utilized for FEC block CRC encoding.

      - FEC encoding

The coding method used as the mandatory schemes shall be convolutional turbo encoding for data transmission and the tail-biting convolutional encoding for control information transmission.

        - Convolutional turbo codes (CTCs)

          - CTC encoder

The CTC encoder, including its constituent encoder, is depicted in Figure c.

It uses a double binary CRSC (Circular Recursive Systematic Convolutional) code.

The bits of the data to be encoded are alternatively fed to A and B, starting with the MSB of the first byte being fed to A, followed by the next bit being fed to B. The encoder is fed by blocks of NEP bits (NEP=2Nbits).

The polynomials defining the connections are described in octal and symbol notations as follows:

- For the feedback branch: 0xB, equivalently 1+D+D3 (in symbolic notation)
- For the Y parity bit: 0xD, equivalently 1+D2+D3
- For the W parity bit: 0x9, equivalently 1+D3

Figure ccc ― CTC encoder

First, the encoder (after initialization by the circulation state SC2, see 15.13.1.6.1.3Error: Reference source not found) is fed the sequence in the natural order (switch 1 in Figure ) with incremental address i=1,2,⋯,N. This first encoding is called C1 encoding.

Then the encoder (after initialization by the circulation state SC2, see 15.13.1.6.1.3) is fed by the interleaved sequence (switch 2 in Figure ) with incremental address i=1,2,⋯,N. This second encoding is called C2 encoding.

The order in which the encoded bits shall be fed into the bit separation block (15.x.1.6.1.4) is:

          - CTC interleaver

The CTC interleaver requires the parameters P0, P1, P2, and P3 shown in Table ddd of which NEP set corresponds to that in Table ccc.

The detailed interleaver structures except table for interleaver parameters correspond to 8.4.9.2.3.2.

Table ddd ― CTC Interleaver parameters (TBD)

          - Determination of CTC circulation states

Correspond to 8.4.9.2.3.3.

          - Bit separation

Correspond to 8.4.9.2.3.4.1.

          - Subblock interleaving

The subblock interleaver requires the parameters m and J shown in Table eee of which NEP set corresponds to that in Table ccc.

The detailed subblock interleaver structures except table for subblock interleaver parameters correspond to 8.4.9.2.3.4.2.

Table eee ― The parameters for the subblock interleavers (TBD)

          - Bit grouping

The channel interleaver output sequence shall consist a bit-by-bit multiplexed sequence of the interleaved A and B subblock sequence, followed by a bit-by-bit multiplexed sequence of the interleaved Y1 and Y2 subblock sequences, followed by a bit-by-bit multiplexed sequence of the interleaved W2 and W1 subblock sequences. The bit-by-bit multiplexed sequence of interleaved A and B subblock sequences shall consist of the first output bit from the A subblock interleaver, the first output bit from the B subblock interleaver, the second output bit from the A subblock interleaver, the second output bit from the B subblock interleaver, etc. The bit-by-bit multiplexed sequence of interleaved Y1 and Y2 subblock sequences shall consist of the first output bit from the Y1 subblock interleaver, the first output bit from the Y2 subblock interleaver, the second output bit from the Y1 subblock interleaver, the second output bit from the Y2 subblock interleaver, etc. The bit-by-bit multiplexed sequence of interleaved W2 and W1 subblock sequences shall consist of the first output bit from the W2 subblock interleaver, the first output bit from the W1 subblock interleaver, the second output bit from the W2 subblock interleaver, the second output bit from the W1 subblock interleaver, etc.

Figure ddd shows the interleaving scheme.

Figure ddd ― Block diagram of the interleaving scheme

        - Convolutional coding (CC)

Correspond to 8.4.9.2.1.

        - Low density parity check (LDPC) code

[TBD]

      - Bit selection and repetition

Bit selection and repetition are performed to generate the subpacket. If NUM_FEC_BLOCK is greater than 1, NREi resource elements for the i-th HARQ-subpacket are segmented into NUM_FEC_BLOCK blocks and then the number of resource elements for each block is defined by following equation.

NREPERFECBLOCKi,k=NREi−(NUMFECBLOCK−1)⋅NREiNUMFECBLOCK , k=0,NREiNUMFECBLOCKk=1,⋯,NUM_FEC_BLOCK-1

where

i	is a HARQ-subpacket index,
k	is an index from 0 to NUM_FEC_BLOCK-1 for k-th FEC block.

Let Ni,k be the number of codeword bits that shall be transmitted from the k-th FEC encoder output for the i-th HARQ-subpacket. Then Ni,k is calculated by following equation

Ni,k=NREPERFECBLOCKi,k⋅MODi

where MODi is a modulation order for the i-th HARQ-subpacket.

Also, let the selected bit be numbered from zero with the 0-th bit being the first bit in the sequence. Then, the index of the j-th bit from the k-th FEC encoder output for the i-th HARQ-subpacket shall be:

Si,j=(Pi+j)mod (3⋅NEP)                                                                                                                      where j=0,...,Ni,k−1, and Pi is the starting position for the i-th HARQ-subpacket in 15.x.2.1.

      - Bit collection

The selected bits from each FEC encoder output are collected in the order of FEC encoder for the HARQ transmission.

      - Modulation

Correspond to 8.4.9.4.2.

    - HARQ

      - IR HARQ

HARQ IR is used in 802.16m, by changing the starting position of the bit selection for HARQ retransmissions. For uplink, it is determined as the end of the last transmission. For downlink, it is determined as a function of SPID for the following Table fff.

Table fff ― Starting Position

| SPID | Starting position (Pi) |
| --- | --- |
| 0 | 0 |
| 1 | 3NEP2 |
| 2 | −Ni,k mod 3NEP |
| 3 | 3NEP2−Ni,k mod 3NEP |

      - Constellation re-arrangement (CoRe)

CoRe is supported in 16QAM and 64 QAM with 2 versions. All the QAM symbols in the same HARQ retransmission use same CoRe version. In addition, CoRe is utilized only if the modulation of retransmission is remained same as that of initial transmission. It is expressed by a bit-level interleaver within a QAM symbol for the following Table ggg.

Table ggg ― Bit interleaver within a QAM symbol

| CoRe version | 16 QAM | 64 QAM |
| --- | --- | --- |
| 0 | (b1,b2,b3,b4) | (b1,b2,b3,b4,b5,b6) |
| 1 | (b2,b1,b4,b3) | (b3,b2,b1,b6,b5,b4) |

      - Adaptive HARQ

[TBD]

      - MIMO HARQ

[TBD]

-------------------------------  Text End  ---------------------------------------------------