| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of DL MIMO Transmission Scheme Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Shaohua Li, Peter Wang, Adrian Boariu, Xin Qi Nokia Siemens Networks Zexian Li Nokia | Shaohua.li@nsn.com |
| Re: | “802.16m amendment text”: IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment working document” Target topic: “DL MIMO”. |  |
| Abstract | The contribution proposes the text of DL MIMO Transmission Scheme section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of DL MIMO Section for the IEEE 802.16m Amendment

Shaohua Li, et al.

Nokia Siemens Networks

- Introduction

The contribution proposes the text of DL MIMO section to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D7 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- References

[1] IEEE P802.16 Rev2/D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r6, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

Insert a new section 15:

### DL MIMO Transmission Scheme

      - DL MIMO Architecture and Data Processing

The architecture of downlink MIMO on the transmitter side is shown in the Figure 11

In SU-MIMO, only one user is scheduled in one Resource Unit (RU). In MU-MIMO, multiple users can be scheduled in one RU.

If vertical encoding is utilized, there is only one encoder block (one “layer”). If horizontal encoding is utilized, there are multiple encoders (multiple “layers”). A “layer” is defined as a coding / modulation path fed to the MIMO encoder as an input, and a “stream” is defined as each output of the MIMO encoder that is passed to the beamformer / precoder.

Figure 1 – DL MIMO Architecture

The encoder block contains the channel encoder, interleaver, rate-matcher, and modulator for each layer.

The Resource Mapping block maps the modulated symbols to the corresponding time-frequency resources in the allocated resource units (RUs).

The MIMO encoder block maps L (≥1) layers onto NS (≥L) streams, which are fed to the Beamformer/Precoder block.

The Beamformer/Precoder block maps streams to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

##### Layer to Stream Mapping

The complex-valued modulation symbols for each of the layers to be transmitted are mapped onto one or several streams. Complex-valued modulation symbols  d(q)(0),...,d(q)(Msymb(q)−1) for layer q shall be mapped onto the stream zi in the following manner:

zi=Sxi          i=0,1,...,Msymbstream−1

where

xi=s0i,...sM−1iT, i=0,1,...,Msymbstream−1

For SU-MIMO

smi=d0i∗M+m  i=0,1,...,Msymbstream−1, m=0,⋯,M−1

For MU-MIMO:

smi=dmi  i=0,1,...,Msymbstream−1, m=0,⋯,M−1

where M is the number of streams, Msymbq is the number of modulation symbols in layer q, and Msymbstream is the number of modulation symbols per stream. Sx is an STC matrix. It is defined in 15.3.9.2.1.1 for OL SU-MIMO, 15.3.9.2.1.2 for CL SU-MIMO, and  for CL MU-MIMO.

##### Stream to Antenna Mapping

The stream to antenna mapping is defined using the following equation

yi=Wkzi=y1,1iy1,2i⋯y1,NFiy2,1iy2,2i⋯y2,NFi⋮⋮⋱⋮yNT,1iyNT,2i⋯yNT,NFi i=0,1,...,Msymbstream−1

where yi is the output of the precoder/beamformer, yj,ki is the output symbol to be transmitted via the j-th antenna on the k-th subcarrier. Note NF is the number of subcarriers used to transmit the MIMO signals. For open-loop SU-MIMO, the rate of a mode is defined as R=MNF. Wk is an Nt  Ns precoding matrix in the kth subcarrier, where Nt is the number of transmit antennas, and Ns  is the number of streams. It is given in 15.3.9.2.1.1 for OL SU-MIMO, 15.3.9.2.1.2 for CL SU-MIMO, and Error: Reference source not found for MU-MIMO.

##### Mapping to data carrier

For each of the antenna jused for transmission, the block of complex-valued symbols yj,k(i)j=0,1,⋯,NT−1,k=0,1,⋯,NF−1,i=0,1,⋯,Msymstream−1 shall be mapped in sequence  starting with yj,0(0) to subcarrier k,l in the LRUs assigned for transmission and not used for pilots. The mapping to subcarrierk,l on antenna j not reserved for other purposes shall be in increasing order of first the frequency index k over the assigned LRUs and then the symbol index l, starting with the first symbol in a subframe. The sequence of yj,ki shall be in increasing order of first the index k over the NF subcarriers and then the index i.

      - Transmission for Data Channels

##### Single-user MIMO

          - OL Single-user MIMO

Open-loop STC rate-1 transmission are defined as Transmit Diversity modes. The other open-loop Single-user MIMO modes are defined as Spatial Multiplexing modes.

The dimensions of the vectors and matrices for OL SU-MIMO are shown in the following table:

Table 1 – Matrix dimensions for DL OL SU-MIMO modes

| NT | Rate | M | NS | NF |
| --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 1 | 1 |
| 2 | 1 | 2 | 2 | 2 |
| 4 | 1 | 1 | 1 | 1 |
| 4 | 1 | 2 | 2 | 2 |
| 8 | 1 | 1 | 1 | 1 |
| 8 | 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 2 | 1 |
| [4] | [2] | [4] | [4] | [2] |
| 8 | 2 | 2 | 2 | 1 |
| [8] | [2] | [4] | [4] | [2] |
| 4 | 3 | 3 | 3 | 1 |
| [4] | [3] | [6] | [4] | [2] |
| 8 | 3 | 3 | 3 | 1 |
| [8] | [3] | [6] | [4] | [2] |
| 4 | 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 4 | 1 |

            - Transmit Diversity

For the transmit diversity modes with M=1, the Sxi is

zi=xi.

Then the output of beamformer is

yi=Wkzi

where Wk is a NT × 1 vector., the values of Wkare TBD.

For the transmit diversity modes with M=2, the Sxi is

zi=s1i−s2is2is1i

the output of beamformer is

yi=Wkzi

where Wkis a NT × 2 vector, the values of Wkare TBD.

            - Spatial Multiplexing

For the rate-R spatial multiplexing modes, the input and the output of MIMO encoder is represented by an R  1 vector

zi=xi=s1is2i⋮sRi

The output of the beamformer is

yi=Wkzi

where Wk is a NT × R matrix. For NT=2, R is 2, and for NT=4, Rcan be 2, 3 and 4. The values of Wkare TBD.

          - CL SU-MIMO

            - Precoding for CL SU-MIMO

The input and the output of MIMO encoder is represented by an R  1 vector

zi=xi=s1is2i⋮sRi

The output of the precoder is denoted by a matrix Nt  1 vector

yi=Wkzi

The values of the precoding matrix Wkshall be selected among the precoder elements in the codebook configured in ABS and AMS. The configured codebook shall be selected from Table defined in 15.3.9.2.1.2.2.2..

For TDD sounding based precoding, the value of Wk is derived from the MS sounding feedback.

            - Feedback for single-user MIMO

              - CQI feedback

(TBD)

              - CSI feedback

                - Standard Mode

Based codebooks are defined for the CSI feedback of close-loop MIMO, whose codeword is sufficient for the base station to determine a new precoder. The codebooks for 2 Tx transmission with 3-bit feedback index and that for 4 Tx transmission with 4-bit feedback index are listed in Table 2 and Table 3, respectively.

Table 2 – Base codebook for 2Tx transmission

| Base Codebook Index | Number of Stream (NS) |  |
| --- | --- | --- |
|  | 1 | 2 |
| 0 | [TBD] | [TBD] |
| 1 | [TBD] | [TBD] |
| 2 | [TBD] | [TBD] |
| 3 | [TBD] | [TBD] |
| 4 | [TBD] | [TBD] |
| 5 | [TBD] | [TBD] |
| 6 | [TBD] | [TBD] |
| 7 | [TBD] | [TBD] |

Table 3 – 4bit Base codebook for 4Tx transmission

| Base Codebook Index | Number of Stream (NS) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| 0 | [TBD] | [TBD] | [TBD] | [TBD] |
| 1 | [TBD] | [TBD] | [TBD] | [TBD] |
| 2 | [TBD] | [TBD] | [TBD] | [TBD] |
| 3 | [TBD] | [TBD] | [TBD] | [TBD] |
| 4 | [TBD] | [TBD] | [TBD] | [TBD] |
| 5 | [TBD] | [TBD] | [TBD] | [TBD] |
| 6 | [TBD] | [TBD] | [TBD] | [TBD] |
| 7 | [TBD] | [TBD] | [TBD] | [TBD] |
| 8 | [TBD] | [TBD] | [TBD] | [TBD] |
| 9 | [TBD] | [TBD] | [TBD] | [TBD] |
| 10 | [TBD] | [TBD] | [TBD] | [TBD] |
| 11 | [TBD] | [TBD] | [TBD] | [TBD] |
| 12 | [TBD] | [TBD] | [TBD] | [TBD] |
| 13 | [TBD] | [TBD] | [TBD] | [TBD] |
| 14 | [TBD] | [TBD] | [TBD] | [TBD] |
| 15 | [TBD] | [TBD] | [TBD] | [TBD] |

The codebooks in Table xxx, and Table xxx are produced by generating expressions one per table. These tables are optimized for generation efficiency and storage memory.

6bit base codebook for one stream and 4 Transmit antennas, V(4,1,6), are generated as follows:

vk=uk     Buk0.7=φ

where

φ		is an empty set

Bukα=Pu∈GNT,1:1−uHuk21−α2

GNT,1 	denotes grassmann manifold, it is the set of all one-dimension subspaces in CNT

Pu 		denotes the line generated by a vector u, and αis a threshold.

gl=0,ej2π256l,ej4π256l,ej6π256lT

ui=uie−jψi, for i=2,⋯,26,

ui=HsQi−1zHHs⋅u1  for i=2,⋯,26,

Where

Hs=I,                  s=e1

wherew=s−e1 and e1  is [1 ,0 ⋯,0], p=2wHw, I is the N×N identity matrix, H  	denotes the conjugate transpose operation,

Qiz=diagej⋅2π2L⋅i,ej⋅90π2L⋅i,ej⋅44π2L⋅i,ej⋅98π2L⋅i,

u1=1Nt1,ej⋅2πNt,⋯,ej⋅2πNt⋅Nt−1T,

s=1.3954−j0.0738,0.0206+j0.4326,−0.1658−j0.5445,0.5487−j0.1599T,

ψiis the phase of the first entry of ui.

Table 4 – 6bit Base codebook for 4Tx transmission

| Number of Tx (Nt) | Number of Stream (NS) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| 4 | V(4,1,6) | HE(V(4,1,6),2) | HE(V(4,1,6),3) | H(V(4,1,6)) |

In the table, HE(vN,M) generates an N by M matrix from a unit N vector, VN, by taking the last M columns of HvN as shown in Equation (xxx)

HEvN,M=HvN:,N−M+1:N                     (xxx)

                - Differential Mode

(TBD)

                - Adaptive Mode

(TBD)

##### MU-MIMO

          - Precoding for MU-MIMO

Multi-user MIMO schemes are used to enable a resource allocation to joint transmission data to two or more MSs. The input and the output of MIMO encoder is represented by an R  1 vector

zi=xi=s1is2i⋮sRi

The output of the precoder is denoted by a matrix Nt  1 vector

yi=Wkzi

The values of the precoding matrix Wkshall be derived from the elements in the codebook configured in ABS and AMS. The configured codebook is defined in 15.3.9.2.1.2.2.2..

For TDD sounding based precoding, the value of Wk is derived from the MS sounding feedback.

          - Feedback for MU-MIMO

            - CQI feedback

(TBD)

            - CSI feedback

              - Standard Mode

For codebook based precoding, the codebooks defined in 15.3.9.2.1.2.2.2.1 are used for the standard mode CSI feedback.

              - Differential Mode

The codebooks defined in 15.3.9.2.1.2.2.2.2 are used for the differential mode CSI feedback.

              - Adaptive Mode

The codebooks defined in 15.3.9.2.1.2.2.2.3 are used for the adaptive mode CSI feedback.

      - Transmission for Control Channels

##### Transmission for Broadcast Control Channel

(TBD)

##### Transmission for Unicast Control Channel

(TBD)

      - Advanced Features

##### Multi-BS MIMO

[TBD]

##### MIMO for Multi-cast Broadcast Services

[TBD]

-------------------------------  Text End  ---------------------------------------------------