| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the MIMO Section of the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Yinggang Du, Hongjie Si, Yang Tang, Xin Chang, Qi Li, Yinwei Zhao, Ping Yu, Jianmin Lu Huawei Technologies | duyinggang@huawei.com |
| Re: | 802.16m amendment working document, IEEE 802.16m-08/053r1 |  |
| Abstract | The contribution proposes the text for the MIMO section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the MIMO

Section of the IEEE 802.16m Amendment

Yinggang Du, Hongjie Si, Yang Tang, Xin Chang

Qi Li, Yinwei Zhao, Ping Yu, Jianmin Lu

Huawei Technologies

- Introduction

The contribution proposes text for the DL MIMO and UL MIMO sections to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r6, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

Insert the following text into section 15 of IEEE 802.16m-008/050:

- Advanced Air Interface

  - Physical layer

    - DL MIMO Transmission Scheme

      - DL MIMO Architecture and Data Processing

The architecture of downlink MIMO on the transmitter side is shown in the Figure 1.

In SU-MIMO, only one user is scheduled in one Resource Unit (RU). In MU-MIMO, multiple users can be scheduled in one RU. When vertical encoding (SCW) is utilized, there is only one encoder block (one “layer”). When horizontal encoding (MCW) is utilized, there are multiple encoders (multiple “layers”). A “layer” is defined as a coding / modulation path fed to the MIMO encoder as an input, and a “stream” is defined as each output of the MIMO encoder that is passed to the beamformer / precoder.

User

1

data

User

2

data

User P

data

User i

data

MIMO

Encoder

Beamformer

/Precoder

IFFT

OFDM Symbol

Construction

IFFT

IFFT

Precoding Vector

/Matrix

Encoder

Encoder

Encoder

Encoder

Encoder

Encoder

Layer

control

Scheduler

Resource

Mapping

Feedback

CQI

CSI

ACK / NAK

Mode / Rank / Link Adaptation

Figure 1 DL MIMO Architecture

- The encoder block contains the channel encoder, interleaver, rate-matcher, and modulator for each layer.

- The Resource Mapping block maps the modulated symbols to the corresponding time-frequency resources in the allocated resource units (RUs).

- The MIMO encoder block maps L (≥1) layers onto NS (≥L) streams, which are fed to the Beamformer/Precoder block.

- The Beamformer/Precoder block maps streams to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

- The OFDM symbol construction block maps antenna-specific data to the OFDM symbol.

- The feedback block contains feedback information such as CQI and CSI from the AMS.

- The scheduler block will schedule users to resource units and decide their MCS level, MIMO parameters (MIMO mode, rank).  This block is responsible for making a number of decisions with regards to each resource allocation, including:

- Allocation type: Whether the allocation should be transmitted with a distributed or localized allocation

- Single-user (SU) versus multi-user (MU) MIMO: Whether the resource allocation should support a single user or more than one user

- MIMO Mode: Which open-loop (OL) or closed-loop (CL) transmission scheme should be used for the user(s) assigned to the resource allocation.

- User grouping: For MU-MIMO, which users should be transmitted on the Resource Unit (RU)

- Rank: For the spatial multiplexing modes in SU-MIMO, the number of streams to be used for the user allocated to the Resource Unit (RU).

- MCS level per layer: The modulation and coding rate to be used on each layer.

- Boosting: The power boosting values to be used on the data and pilot subcarriers.

- Band selection: If localized resource allocation is used, where in the frequency band should the localized allocation be placed.

##### Antenna Configuration

The ABS employs a minimum of two transmit antennas. The AMS employs a minimum of two receive antennas. The antenna configurations are (NT, NR) = (2, 2), (4, 2), (4, 4), (8, 2), (8, 4), and (8,8) where NT denotes the number of ABS transmit antennas and NR denotes the number of AMS receive antennas.

##### Layer to Stream Mapping

For open-loop spatial multiplexing and closed-loop SU-MIMO, the number of streams isNs≤min(NT,NR), where NS is no more than 8. For open-loop transmit diversity modes, NS depends on the STC schemes employed by the MIMO encoder. MU-MIMO can have up to 2 streams with 2 Tx antennas, and up to 4 streams for 4 Tx antennas and 8 Tx antennas.

For SU-MIMO, spatial multiplexing MIMO mode employs vertical encoding (SCW) and horizontal encoding (MCW), and transmit diversity MIMO mode employs vertical encoding (SCW). For MU-MIMO, horizontal encoding (MCW) is employed at the base-station while only one stream is transmitted to each mobile station.

##### Stream to Antenna Mapping

The stream to antenna mapping depends on the MIMO scheme used.  The mapping can be defined using the following equation

y=P×S(x), Equation 1

where y is the output of the precoder/beamformer, P is a pre-coding matrix, S(x) is an STC matrix, and x is the input layer vector.

##### Resource mapping

The MIMO mode permutation for various MIMO schemes is supported in either Distributed or Localized resource mapping.

##### Signaling support for MIMO

###### Signaling support for SU MIMO

In the downlink closed-loop SU-MIMO, the precoding matrix shall be signalled via explicit signaling when common demodulated pilots are used.

###### Signaling support for MU MIMO

In the downlink MU-MIMO, the precoding matrix shall be signaled via explicit signaling when common demodulation pilots are used.

#### Transmission for Data Channels

##### Single-user MIMO

Both open-loop single-user MIMO and closed-loop single-user MIMO can be used for the antenna configurations specified in Section 15.5.12.1.1. For open-loop single-user MIMO, both spatial multiplexing and transmit diversity schemes can be used. For closed-loop single-user MIMO, codebook based precoding can be used for both TDD and FDD systems. For closed-loop single-user MIMO, sounding based precoding can be used for TDD systems.

As described in section 15.3.8.1, the overall structure of MIMO processing has two parts. The first part is the MIMO encoder and second part is the precoder. The MIMO encoder is a batch processor that operates on M input symbols at a time. The input to the MIMO encoder is represented by an M  1 vector

x=s1s2⋮sM, Equation 2

where si is the i-th input symbol within a batch. The output of the MIMO encoder is an NS  NF MIMO STC matrix z = S(x), which serves as the input to the precoder. The output of the MIMO encoder is multiplied by NT  NS precoder, P. The output of the precoder is denoted by a matrix NT  NF matrix

y=P×z=y1,1y1,2⋯y1,NFy2,1y2,2⋯y2,NF⋮⋮⋱⋮yNT,1yNT,2⋯yNT,NF, Equation 3

where yj,k is the output symbol to be transmitted via the j-th physical antenna on the k-th subcarrier. NF is the number of subcarriers used to transmit the MIMO signals derived from the input vector x. For open-loop SU-MIMO, the rate of a mode is defined as R=MNF.

###### Open-loop SU-MIMO

A number of antenna configurations and transmission rates are supported in open-loop SU-MIMO. Among them, 2Tx, 4Tx, and 8Tx antennas with rate 1 transmission are defined as Transmit Diversity modes. The operation of these modes is specified in Section 15.5.12.2.1.1.1. The other modes, including 2Tx, 4Tx, and 8Tx antennas with rate 2 transmission, 4Tx and 8Tx antennas with rate 3 transmission, 4Tx and 8Tx antennas with rate 4 transmission, and 8Tx antennas with transmission up to rate 8, are defined as Spatial Multiplexing modes. The operation of these modes is specified in Section 15.5.12.2.1.1.2. The dimensions of the vectors and matrices for open-loop SU-MIMO are shown in the following table:

Table 1 Matrix dimensions for DL open-loop SU-MIMO modes

| NT | Rate | M | NS | NF |
| --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 1 | 1 |
| 2 | 1 | 2 | 2 | 2 |
| 4 | 1 | 1 | 1 | 1 |
| 4 | 1 | 2 | 2 | 2 |
| 8 | 1 | 1 | 1 | 1 |
| 8 | 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 2 | 1 |
| 4 | 2 | 4 | 4 | 2 |
| 8 | 2 | 2 | 2 | 1 |
| 8 | 2 | 4 | 4 | 2 |
| 4 | 3 | 3 | 3 | 1 |
| 4 | 3 | 6 | 4 | 2 |
| 8 | 3 | 3 | 3 | 1 |
| 8 | 3 | 6 | 4 | 2 |
| 4 | 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 4 | 1 |

On a given subcarrier k, the precoding matrix P can be defined using the following equation:

P(k) = W(k), Equation 4

W(k) is an NT  NS matrix, where NT is the number of transmit antennas and NS is the numbers of streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u (TBD) subcarriers, and/or v (TBD) OFDM symbols. The matrix W(k) can be identity matrix for 2Tx rate-2 and 4Tx rate-4.

            - Transmit Diversity

The following transmit diversity modes are supported for open-loop single-user MIMO:

- 2Tx rate-1: For M = 2,SFBC, and for M = 1, a rank-1 precoder

- 4Tx rate-1: For M = 2,SFBC with precoder, and for M = 1, a rank-1 precoder

- 8Tx rate-1: For M = 2,SFBC with precoder, and for M = 1, a rank-1 precoder

For the transmit diversity modes with M=1, the input to MIMO encoder is x=s1, and the output of the MIMO encoder is a scalar, z=x. Then the output of MIMO encoder is multiplied by NT × 1 matrix W, where W is described in section 15.5.12.2.1.1

For the transmit diversity modes with M=2, the input to the MIMO encoder is represented a 2 × 1 vector.

, Equation 5

The MIMO encoder generates the SFBC matrix.

, Equation 6

Then the output of the MIMO encoder is multiplied by NT × 2 matrix W, where W is described in section 15.5.12.2.1.1

            - Spatial Multiplexing

The following spatial multiplexing modes are supported for open-loop single-user MIMO:

- Rate-2 spatial multiplexing modes:

  - 2Tx rate-2: rate 2 SM with precoding

  - 4Tx rate-2: rate 2 SM with precoding

  - 4Tx rate-2: Double SFBC with precoding

  - 8Tx rate-2: rate 2 SM with precoding

  - 8Tx rate-2: Double SFBC with precoding

- Rate-3 spatial multiplexing modes:

  - 4Tx rate-3: rate 3 SM with precoding

  - 4Tx rate-3: Hybrid SM and SFBC with precoding

  - 8Tx rate-3: rate 3 SM with precoding

  - 8Tx rate-3: Hybrid SM and SFBC with precoding

- Rate-4 spatial multiplexing modes:

  - 4Tx rate-4: rate 4 SM with precoding

  - 8Tx rate-4: rate 4 SM with precoding

For the rate-R SM with precoding modes, the input and the output of MIMO encoder is represented by an R  1 vector

, Equation 7

Then the output of the MIMO encoder is multiplied by NT × R matrix W, where W is described in section 15.5.12.2.1.1.

For the Double SFBC with precoding modes, the input and the output of MIMO encoder is represented by a 4  1 vector and a 4  2 matrix respectively.

x=s1s2s3s4, Equation 8

z=s1−s2s2s1s3−s4s4s3, Equation 9

Then the output of the MIMO encoder is multiplied by NT × 4 matrix W, where W is described in section 15.5.12.2.1.1. W can be a set of 3 antenna circulation matrices:

W=1000010000100001,1000001001000001,1000001000010100, Equation 10

For the rate-3 Hybrid SM and SFBC with precoding modes, the input and the output of MIMO encoder is represented by 6  1 vector and a 4  2 matrix

x=s1s2s3s4s5s6, Equation 11

z=s1−s2s2s1s3s5s4s6, Equation 12

Then the output of the MIMO encoder is multiplied by NT × 4 matrix W, where W is described in section 15.5.12.2.1.1. W can be a set of 6 antenna circulation matrices::

W=1000010000100001,1000001001000001,1000001000010100 001010000100000100101000000101000010000110000100 Equation 13

###### Closed-loop SU-MIMO

##### Multi-user MIMO

#### Transmission for Control Channel

#### Advanced Features

##### Multi-BS MIMO

##### MIMO for Multi-cast Broadcast Services

Open-loop spatial multiplexing schemes as described in Section 15.5.12.2.1.1.2 are used for MBS. The MBS data shall contain up to two codewords. The transmission format for multi-codeword transmission shall include a primary codeword and a secondary codeword. The primary and secondary codewords may use different modulation and coding and/or power offsets in order to enable efficient interference cancellation at the mobile station. The stream to antenna mapping for MBS multi-codeword transmission is according to section 15.3.8.1.3, where the precoding matrix P is selected from a predefined unitary codebook, and changes every u (TBD) subcarriers, and/or v (TBD) OFDM symbols.

    - UL MIMO Transmission Scheme

      - UL MIMO Architecture and Data Processing

The architecture of uplink MIMO on the transmitter side is shown in the Figure 2.

In SU-MIMO, only one user is scheduled in one Resource Unit (RU). In MU-MIMO, multiple users can be scheduled in one RU. When vertical encoding (SCW) is utilized, there is only one encoder block (one “layer”). When horizontal encoding (MCW) is utilized, there are multiple encoders (multiple “layers”). A “layer” is defined as a coding / modulation path fed to the MIMO encoder as an input, and a “stream” is defined as each output of the MIMO encoder that is passed to the beamformer / precoder.

MIMO

Encoder

Beamformer

/Precoder

IFFT

OFDM Symbol

Construction

IFFT

IFFT

Precoding Matrix

Encoder

Encoder

Resource Allocation

MCS / Packet Size

ACK / NAK

MIMO Mode / Rank /PMI

Resource

Mapping

User

Data

Figure 2 Uplink MIMO Architecture

- The encoder block contains the channel encoder, interleaver, rate-matcher, and modulator for each layer.

- The Resource Mapping block maps the modulated symbols to the corresponding time-frequency resources in the allocated resource units (RUs).

- The MIMO encoder block maps L (≥1) layers onto NS (≥L) streams, which are fed to the Beamformer/Precoder block.

- The precoding block maps streams to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

- The OFDM symbol construction block maps antenna-specific data to the OFDM symbol.

- When only one transmit antenna is used, the codeword to stream mapping, MIMO encoding and precoder are removed in Figure 2.

- The ABS will schedule users to resource blocks and decides their MCS level, MIMO parameters (MIMO mode, rank). PMI may be calculated at the ABS or AMS.

- Decisions with regards to each resource allocation include:

- Allocation type: Whether the allocation in question should be transmitted with a distributed or localized allocation

- Single-user (SU) versus multi-user (MU) MIMO: Whether the resource allocation should support a single user or more than one user

- MIMO Mode: Which open-loop (OL) or closed-loop (CL) transmission scheme should be used for the user(s) assigned to the resource allocation.

- User grouping: For MU-MIMO, which users are allocated to the resource allocation

- Rank: For the spatial multiplexing modes in SU-MIMO, the number of streams to be used for the user allocated to the resource allocation.

- MCS level per layer: The modulation and coding rate to be used on each layer.

- Boosting: The power boosting values to be used on the data and pilot subcarriers.

- Band selection: If localized resource allocation is used, the frequency band location.

##### Antenna Configuration

The antenna configurations are denoted by (NT, NR) where NT denotes the number of AMS transmit antennas and NR denotes the number of ABS receive antennas. The supported antenna configurations are NT = 1, 2, or 4 and NR ≥ 2.

##### Layer to Stream Mapping

For open-loop spatial multiplexing and closed-loop SU-MIMO, the number of streams is Ns≤min(NT,NR). For open-loop transmit diversity modes, NS depends on the STC schemes employed by the MIMO encoder and its value is specified in 15.5.12.2.2.1. For SU-MIMO and MU-MIMO, both Vertical encoding (SCW) and Horizontal encoding (MCW) can be employed.

The layer to stream mapping depends on the MIMO scheme used. The mapping can be defined using the following equation:

z = S(x), Equation 14

where z is the output of the MIMO encoder, S(x) is an STC matrix, and x is the input layer vector.

##### Stream to Antenna Mapping

The stream to antenna mapping depends on the MIMO scheme used.  The mapping can be defined using the following equation

y=P×z, Equation 15

where P is a pre-coding matrixand z is the output of the MIMO encoder.

##### Resource mapping

The MIMO mode permutation for various MIMO schemes is supported in either Distributed or Localized resource mapping.

##### Signaling support for MIMO

#### Transmission for Data Channels

##### Single-user MIMO

Both open-loop single-user MIMO and closed-loop single-user MIMO are supported for the antenna configurations specified in Section 15.5.12.1.1. For open-loop single-user MIMO, both spatial multiplexing and transmit diversity schemes are supported. For closed-loop single-user MIMO, codebook based precoding can be used for both TDD and FDD systems. For closed-loop single-user MIMO, downlink pilot based precoding can be used for TDD systems.

As described in section 15.5.12.1, the overall structure of MIMO processing has two parts. The first part is the MIMO encoder and second part is the precoder. The MIMO encoder is a batch processor that operates on M input symbols at a time. The input to the MIMO encoder is represented by an M  1 vector

x=s1s2⋮sM, Equation 16

where si is the i-th input symbol within a batch. The output of the MIMO encoder is an NS  NF MIMO STC matrix z = S(x), which serves as the input to the precoder. The output of the MIMO encoder is multiplied by NT  NS precoder, P. The output of the precoder is denoted by a matrix NT  NF matrix

y=P×z=y1,1y1,2⋯y1,NFy2,1y2,2⋯y2,NF⋮⋮⋱⋮yNT,1yNT,2⋯yNT,NF, Equation 17

where yj,k is the output symbol to be transmitted via the j-th physical antenna on the k-th subcarrier. NF is the number of subcarriers or symbols used to transmit the MIMO signals derived from the input vector x. For open-loop SU-MIMO, the rate of a mode is defined as R=MNF.

##### Open-loop SU-MIMO

A number of antenna configurations and transmission rates are supported in uplink open-loop SU-MIMO. Among them, 2Tx and 4Tx antennas with rate 1 transmission are defined as Transmit Diversity modes. The operation of these modes is specified in Section 15.5.12.2.2.1. The other modes, including 2Tx and 4Tx antennas with rate 2 transmission, 4Tx antennas with rate 3 transmission, and 4Tx antennas with rate 4 transmission, are defined as Spatial Multiplexing modes. The operation of these modes is specified in Section 15.5.12.2.2.2. The dimensions of the vectors and matrices for open-loop SU-MIMO are shown in the following table:

Table 2 Matrix dimensions for UL open-loop SU-MIMO modes

| NT | Rate | M | NS | NF |
| --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 1 | 1 |
| 2 | 1 | 2 | 2 | 2 |
| 4 | 1 | 1 | 1 | 1 |
| 4 | 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 2 | 1 |
| 4 | 2 | 4 | 4 | 2 |
| 4 | 3 | 3 | 3 | 1 |
| 4 | 3 | 6 | 4 | 2 |
| 4 | 4 | 4 | 4 | 1 |

On a given subcarrier k, the precoding matrix P can be defined using the following equation:

P(k) = W(k), Equation 18

W(k) is an NT  NS matrix, where NT is the number of transmit antennas and NS is the number of streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u subcarriers, and/or v OFDM symbols. The matrix W(k) can be identity matrix for 2Tx rate-2 and 4Tx rate-4 spatial multiplexing case.

###### Transmit Diversity

The following transmit diversity modes are supported for open-loop single-user MIMO:

- 2Tx rate-1: For M = 2, SFBC, and for M = 1, a rank-1 precoder

- 4Tx rate-1: For M = 2, SFBC with precoder, and for M = 1, a rank-1 precoder

For the transmit diversity modes with M=1, the input to MIMO encoder is x=s1, and the output of the MIMO encoder is a scalar, z=x. Then the output of MIMO encoder is multiplied by NT × 1 matrix W, where W is described in section 15.3.12.1.1.1.

For the transmit diversity modes with M= 2, the input to the MIMO encoder is represented a 2  1 vector

, Equation 19

The MIMO encoder generates the SFBC matrix.

, Equation 20

Then the output of the MIMO encoder is multiplied by NT × 2 matrix W, where W is described in section 15.3.12.1.1.1.

###### Spatial Multiplexing

The following spatial multiplexing modes are supported for open-loop single-user MIMO:

- Rate-2 spatial multiplexing modes:

  - 2Tx rate-2: rate 2 SM with precoding

  - 4Tx rate-2: rate 2 SM with precoding

  - 4Tx rate-2: Double SFBC with precoding

- Rate-3 spatial multiplexing modes:

  - 4Tx rate-3: rate 3 SM with precoding

  - 4Tx rate-3: Hybrid SM and SFBC with precoding

- Rate-4 spatial multiplexing modes:

  - 4Tx rate-4: rate 4 SM with precoding

For the rate-R SM with precoding modes, the input and the output of MIMO encoder is represented by an R  1 vector

, Equation 21

Then the output of the MIMO encoder is multiplied by NT × R matrix W, where W is described in section 15.3.12.2.1.1.

For the Double SFBC with precoding modes, the input and the output of MIMO encoder is represented by a 4  1 vector and a 4  2 matrix respectively.

x=s1s2s3s4, Equation 22

z=s1−s2s2s1s3−s4s4s3, Equation 23

Then the output of the MIMO encoder is multiplied by NT × 4 matrix W, where W is described in section 15.3.12.2.1.1.

For the rate-3 Hybrid SM and SFBC with precoding modes, the input and the output of MIMO encoder is represented by 6  1 vector and a 4  2 matrix

x=s1s2s3s4s5s6, Equation 24

z=s1−s2s2s1s3s5s4s6, Equation 25

Then the output of the MIMO encoder is multiplied by NT × 4 matrix W, where W is described in section 15.5.12.2.1.1.

##### Closed-loop SU-MIMO

#### Multi-user MIMO

-------------------------------  Text End  ---------------------------------------------------