| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of DL MIMO Transmission Scheme Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics | +82-31-450-1931 wbong@lge.com |
| Re: | “802.16m amendment text”: IEEE 802.16m-08/xxx, “Call for Contributions on Project 802.16m Draft Amendment Content”. Target topic: “MIMO”. |  |
| Abstract | The contribution proposes the text of DL MIMO Transmission Scheme section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of DL MIMO Transmission Scheme Section for the IEEE 802.16m Amendment

Wookbong Lee, Bin-Chul Ihm

LG Electronics

- Introduction

The contribution proposes the text of DL MIMO Transmission Scheme section to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D7 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- Modifications to the SDD text

The text proposed in this contribution is based on subclause 11.9 in the IEEE 802.16m SDD [3]. The modifications to the SDD text are summarized below:

- Changed terminologies so that same terminology can be reused as in [1]. The changes are as follows:

Input symbol to MIMO encoder: x to s

Output symbol of the MIMO encoder: z to x

Rate to STC rate

MIMO precoding matrix: P to W

Output of the precoding block: y to z

Number of stream: NS to Mt

Number of transmit antenna: NT to Nt

- Modified DL MIMO Architecture according to baseline IEEE 802.16m amendment text [5]

- Defining details of OL SU-MIMO scheme based on results of contribution [6]

- Make precoding matrix sections according to subclause 11.9 as well as subclause 20.2

- In the IEEE 802.16m SDD [3], the MS is not required to know the antenna configuration prior to decoding the PBCH. And the minimum number of BS transmit antenna is 2. So, 2Tx OL-SU-MIMO scheme shall be used for BCH transmission. We suggest using SFBC for this transmission.

- References

[1] IEEE P802.16 Rev2/D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r6, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE C802.16m-08/1339r3, “Proposed Text of Frame Structure Section for the IEEE 802.16m Amendment”

[6] IEEE C802.16m-09/0019, “Performance comparison of IEEE 802.16m OL-SU-MIMO”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- Definitions

Insert the following at the end of section 3:

3.xx layer: An information path fed to the MIMO encoder as an input

3.xx stream: Each information path encoded by the MIMO encoder that is passed to the beamformer/precoder

3.xx rank: For the spatial multiplexing modes in SU-MIMO, the number of streams to be used for the user allocated to the Resource Unit (RU)

# Abbreviations and acronyms

Insert the following at the end of section 4:

CL			closed-loop

MU			multi-user

OL			open-loop

PMI			precoding matrix index

SU			single-user

Insert a new section 15:

- Advanced Air Interface

  - Physical layer

### Introduction

### OFDMA symbol description, symbol parameters and transmitted signal

### Frame Structure

### DL Physical Structure

### UL Physical Structure

### DL Control Structure

### UL Control Structure

### Channel coding and HARQ

### DL MIMO Transmission Scheme

      - DL MIMO Architecture and Data Processing

The architecture of DL MIMO on the transmitter side is shown in the Figure 1.

In SU-MIMO, only one user is scheduled in one Resource Unit (RU). In MU-MIMO, multiple users can be scheduled in one RU.

If vertical encoding is utilized, there is only one layer. If horizontal encoding is utilized, there are multiple layers.

Figure 1 DL MIMO Architecture

The FEC block contains the channel encoder, interleaver, and rate-matcher for each layer.

The Mod block contains the modulator for each layer.

The Segment by LRU maps the modulated symbols to the corresponding time-frequency resources in the allocated resource units (RUs).

The MIMO encoder block maps L (≥1) layers onto Mt (≥L) streams, which are fed to the Beamformer/Precoder block.

The Beamformer/Precoder block maps streams to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

The LRU to Data burst mapping block maps antenna-specific data to the OFDM symbol.

The feedback block contains feedback information such as CQI and CSI from the MS.

The scheduler block schedules MSs to resource units and decides their MCS level, MIMO parameters (MIMO mode, rank), allocation type, and pilot boosting value. This information shall be transmitted via USCCH.

##### Layer to Stream Mapping

The layer to stream mapping depends on the MIMO scheme used.

The mapping is defined using the following equation

x=S(s)                                                                            (15.3.1)

Where x is the output of the MIMO encoder, S(s) is an STC matrix, and s is the input layer vector.

For SU-MIMO, Vertical encoding (SCW) is employed. Details are defined in . In this case, L is equal to 1.

For MU-MIMO, Horizontal encoding (MCW) is employed at the BS while only one stream is transmitted to each MS. Details are defined in . In this case, L is equal to Mt.

##### Stream to Antenna Mapping

The stream to antenna mapping depends on the MIMO scheme used.

The mapping is defined using the following equation

(15.3.2)

Where z is the output of the precoder/beamformer, and W is an Nt  Mt precoding matrix, where Nt is the number of transmit antennas.

Details for each MIMO scheme are defined in 3.3.1.2.1.1 for OL SU-MIMO, 3.3.1.2.1.2 for CL SU-MIMO, 3.3.1.2.1.3 for OL MU-MIMO, and 3.3.1.2.1.4 for CL MU-MIMO.

      - Transmission for Data Channels

##### SU-MIMO

The MIMO encoder is a batch processor that operates on M input symbols at a time. The input to the MIMO encoder is represented by an M  1 vector

(15.3.3)

Where si is the i-th input symbol within a batch. The output of the MIMO encoder is an Mt  NF MIMO STC matrix x = S(s), which serves as the input to the precoder.

(15.3.4)

Where xs,k is the s-th stream output on the k-th subcarrier. Note NF is the number of subcarriers used to transmit the MIMO signals. For OL SU-MIMO, the rate of a mode is defined as STC Rate=M / NF.

The output of the MIMO encoder is multiplied by an Nt  Mt precoder, W. The output of the precoder is denoted by a matrix Nt  NF matrix

(15.3.5)

Where zj,k is the output symbol to be transmitted via the j-th physical antenna on the k-th subcarrier.

          - OL SU-MIMO

Open-loop STC rate-1 transmission are defined as Transmit Diversity modes. The other open-loop SU-MIMO modes are defined as Spatial Multiplexing modes.

The dimensions of the vectors and matrices for OL SU-MIMO are shown in the following table:

Table 1 – Matrix dimensions for DL OL SU-MIMO modes

| Nt | STC Rate | M | Mt | NF |
| --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 1 | 1 |
| 2 | 1 | 2 | 2 | 2 |
| 4 | 1 | 1 | 1 | 1 |
| 4 | 1 | 2 | 2 | 2 |
| 8 | 1 | 1 | 1 | 1 |
| 8 | 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 2 | 1 |
| 8 | 2 | 2 | 2 | 1 |
| 4 | 3 | 3 | 3 | 1 |
| 8 | 3 | 3 | 3 | 1 |
| 4 | 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 4 | 1 |
| 8 | 5 | 5 | 5 | 1 |
| 8 | 6 | 6 | 6 | 1 |
| 8 | 7 | 7 | 7 | 1 |
| 8 | 8 | 8 | 8 | 1 |

On a given subcarrier k, the precoding matrix W(k) is an Nt  Mt matrix. The matrix W(k) is selected from a predefined unitary codebook defined in 3.3.1.2.1.1.1, and changes every subband. A subband consists of N1 contiguous PRUs. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix.

The pilots shall be precoded with the same precoder, W(k).

            - OL SU-MIMO Precoding Matrix

[TBD]

            - Transmit Diversity

For the transmit diversity modes with M=1, the input to MIMO encoder is s=s1, and the output of the MIMO encoder is a scalar, x=s. Then the output of MIMO encoder is multiplied by Nt × 1 matrix W, where W is described in 3.3.1.2.1.1.1.

For the transmit diversity modes with M=2, the input to the MIMO encoder is represented a 2 × 1 vector.

(15.3.6)

The MIMO encoder generates the SFBC matrix.

(15.3.7)

Then the output of the MIMO encoder is multiplied by Nt × 2 matrix W, where W is described in 3.3.1.2.1.1.1.

            - Spatial Multiplexing

In this mode, M shall be equal to STC Rate and Mt.

For the STC Rate-R spatial multiplexing modes, the input and the output of MIMO encoder is represented by an R  1 vector

(15.3.8)

Then the output of the MIMO encoder is multiplied by Nt × Mt matrix W, where W is described in 3.3.1.2.1.1.1.

          - CL SU-MIMO

In this mode, M shall be equal to STC Rate and Mt.

For the STC Rate-R spatial multiplexing modes, the input and the output of MIMO encoder is represented by an R  1 vector

(15.3.9)

The output of the MIMO encoder is multiplied by a precoder, W. The output of the precoder is denoted by a matrix Nt  1 vector

(15.3.10)

Where zj is the output symbol to be transmitted via the j-th physical antenna.

On a given subcarrier k, the precoding matrix W(k) is an Nt  Mt matrix.

For codebook based precoding, the value of W(k) shall be derived from the precoder element defined in  3.3.1.2.1.2.1.

For TDD sounding based precoding, the value of W(k) is derived from the MS sounding feedback.

The pilots shall be precoded with the same precoder, W(k).

            - CL SU-MIMO Precoding Matrix

For codebook based precoding, three different modes are supported.

              - Standard Mode

[TBD]

              - Adaptive Mode

[TBD]

              - Differential Mode

[TBD]

##### MU-MIMO

In this mode, M shall be equal to STC Rate and Mt.

If the number of multiplexed MS is M, then the input and output of the MIMO encoder is represented by an M  1 vector

(15.3.11)

Where si is the i-th input symbol within a batch. The i-th input symbol is corresponding to the i-th MS data.

The output of the MIMO encoder is multiplied by a precoder, W. The output of the precoder is denoted by a matrix Nt  1 vector

(15.3.12)

Where zj is the output symbol to be transmitted via the j-th physical antenna.

          - OL MU-MIMO

On a given subcarrier k, the precoding matrix W(k) is an Nt  Mt matrix. The matrix W(k) is selected from a predefined unitary codebook defined in 3.3.1.2.1.1.1, and changes every subband.

The pilots shall be precoded with the same precoder, W(k).

          - CL MU-MIMO

On a given subcarrier k, the precoding matrix W(k) is an Nt  Mt matrix.

For codebook based precoding, the value of W(k) shall be derived from the precoder element defined in 3.3.1.2.1.4.1.

For TDD sounding based precoding, the value of W(k) is derived from the MS sounding feedback for sounding based precoding.

The pilots shall be precoded with the same precoder, W(k).

            - CL MU-MIMO Precoding Matrix

[TBD]

      - Transmission for Control Channels

##### Transmission for Broadcast Control Channel

The input to the MIMO encoder is represented a 2 × 1 vector.

(15.3.13)

The MIMO encoder generates the SFBC matrix. And the signal skips the Beamformer/Precoder block.

(15.3.14)

Pilot pattern A, defined in [DL PHY section], shall be used for BCH transmission.

##### Transmission for Unicast Control Channel

Transmit diversity mode with M=2 shall be used for transmission for USCCH, where transmit diversity mode with M=2 defined in 3.3.1.2.1.1.2. Pilot pattern A, defined in [DL PHY section], shall be used for USCCH transmission.

      - Advanced Features

##### Multi-BS MIMO

[TBD]

##### MIMO for Multi-cast Broadcast Services

[TBD]

-------------------------------  Text End  ---------------------------------------------------