| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on Channel Coding |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Seunghyun Kang, Sukwoo Lee LG Electronics | Voice: +82-31-450-1918 E-mail: sh_kang@lge.com, sugoo@lge.com |
| Re: | “802.16m amendment text”: IEEE 802.16m-08/053r1, “Call for Contributions on Project 802.16m Draft Amendment Content” for the topic of “Channel Coding” |  |
| Abstract | Proposal of amendment text on Channel Coding |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m amendment text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text on Channel coding

Seunghyun Kang, Sukwoo Lee

LG Electronics

# Introduction

This contribution proposes the text of channel coding section to be discussed and adopted in the 802.16m amendment. The proposed text is developed such that it can be readily combined with IEEE P802.16 Rev2/D8 [1].

# Modification from the SDD text

The proposed text in this contribution is based on sub clause 11.13.1 in the IEEE 802.16m SDD [2]. For the consistency with IEEE P802.16Rev2/D8, the terminologies used in the SDD are cleaned up using the definitions in Section 2.1. Also, the high level outline of the proposed text is provided in Section 2.2 with the detailed subsections for the Convolutional Turbo Codes (CTC).

# Notation

The following are the definitions used in the proposed amendment text.

NBurst	Burst size

CRCBurst	Burst CRC size

NInfo	Information block size

δ		Block size resolution depending on the ranges of information block size or (NBurst + CRCBurst)

NPadding	Number of padding bits

NFEC	Number of FEC blocks (after burst partitioning)

CRCFEC	FEC block CRC size

NEP	FEC block size (before encoding)

NEP,i	FEC block size for i-th FEC block, where i = 0, …, NFEC -1

NMaxEP	Maximum FEC block size

NBaseEP	Base FEC block size for FEC block segmentation

NDataTone	Number of data subcarriers

NDataTone,i	Number of data subcarriers for i-th FEC block, where i = 0, …, NFEC -1

NRU	Number of resource units allocated

NDataTonePerRU, j	Number of data tones in j-th RU, where j =0, …, NRU -1

NTotalDataTone 	Total number of data tones in NRU RUs allocated for a data burst

M		Modulation order 2, 4 and 6 (QPSK, 16QAM and 64QAM)

R		Code rate

RMC	Mother code rate (for convolutional turbo coding scheme RMC=1/3)

RCB	Lowest code rate of subpacket without repetition

NCB	Size of circular buffer, where NCB=NEP/RCB

# High level outline of the channel coding section

The following is the high level outline of the channel coding section.

15.13.1	  Channel coding

15.13.1.1	  Block diagram

15.13.1.2	  Padding bit insertion and information block size

15.13.1.3	  Burst CRC encoding

15.13.1.4	  Partition into FEC blocks

15.13.1.4.1	  FEC block CRC encoding

15.13.1.5	  FEC encoding

15.13.1.5.1	  Convolutional turbo codes (CTCs)

15.13.1.5.1.1	  CTC encoder

15.13.1.5.1.2	  Determination of CTC circular states

15.13.1.5.1.3	  CTC inner interleaver

15.13.1.5.1.4	  CTC concatenation rule

15.13.1.5.1.4.1  FEC block segmentation

15.13.1.5.1.4.2  Tone(Data subcarrier) allocation

15.13.1.5.1.5	  Subpacket generation (or Bit selection and repetition)

15.13.1.5.1.5.1  Bit separation

15.13.1.5.1.5.2  Subblock interleaving

15.13.1.5.1.5.3  Bit grouping

15.13.1.5.1.5.4  Bit selection

15.13.1.6	  Modulation

15.13.1.6	  Collection

# References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[3] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

# Proposed Amendment Text on 15.13.1 Channel coding

-----------------------------------------------------------Start of the Text---------------------------------------------------------

# Channel coding

# Block diagram

Channel coding procedures include one padding(see 15.13.1.2), burst CRC encoding (see 15.13.1.3), burst partitioning (see 15.13.1.4), FEC CRC encoding (see 15.13.1.4.1), FEC encoding (see 15.13.1.5), bit selection & repetition (see 15.13.1.5.1.5), modulation (see 15.13.1.6) and collection (see 15.13.1.7).

Figure xx1 Channel coding block diagram

# Padding bit insertion and information block size

A data burst is the basic unit of the channel coding, modulation and HARQ transmission. When the amount of data burst bits and burst CRC bits (and FEC block CRC bits if they are required) does not fit exactly an information block size which is the input of FEC encoder, padding of ‘1’ shall be added at the end of the data burst. Note that the information block comprises data burst bits, burst CRC bits (and FEC block CRC bits) and padding bits. Table xx1 specifies the information block size resolutions and their block size ranges. The information block sizes are regularly increased with the block size resolutions.

Table xx1 Information block size resolutions

| Information block size range | Resolution |
| --- | --- |
| 48 ~ 512 | 8 |
| 512 ~ 1024 | 16 |
| 1024 ~ 2048 | 32 |
| 2048 ~ | 64 |

When the total amount of NBurst data burst bits and CRCBurst burst CRC bits exceeds the maximum FEC block size NMaxEP (NMaxEP = 4800 bits for the case of convolutional turbo coding scheme in 15.13.1.5.1), the data burst including burst CRC is partitioned into a number of smaller FEC blocks, and an FEC block CRC is appended to each FEC block before FEC encoding. Considering an FEC block CRC of CRCFEC bits, the number of FEC blocks NFEC is given by Equation (xx1).

If (NBurst+CRCBurst)≤NMaxEP,NFEC=1Otherwise,NFEC=NBurst+CRCBurstNMaxEP−CRCFEC								(xx1)

Depending on the number of FEC blocks NFEC, the information block size NInfo is given by Equation (xx2).

If NFEC=1,NInfo=NBurst+CRCBurstδ⋅δOtherwise, NInfo=NBurst+CRCBurst+NFEC⋅CRCFECδ⋅δ 					(xx2)

Note that δ is the block size resolution in Table xx1 and it shall be specified by using (NBurst + CRCBurst) instead of the information block size for chasing its range.

Especially, for the case of convolutional turbo coding scheme in 15.13.1.5.1, the information block size NInfo shall be given by Equation (xx3).

If NFEC=1,If NBurst+CRCBurst<48NInfo=48Otherwise, NInfo=NBurst+CRCBurstδ⋅δIf NInfo mod 7 =0, NInfo=NInfo+δ Otherwise, NInfo=NBurst+CRCBurst+NFEC⋅CRCFECδ⋅δ 					(xx3)

When the amount of data burst including a burst CRC and FEC block CRCs does not fit exactly the amount of information bits, padding of ones shall be added to the end of the data burst, and the amount of padding bits NPadding is given by Equation (xx4).

If NFEC=1,NPadding=NInfo−(NBurst+CRCBurst)Otherwise, NPadding=NInfo−(NBurst+CRCBurst+NFEC⋅CRCFEC) 				(xx4)

The order of padded data burst which is input into the burst CRC encoding block is

b0, b1, ⋯, bNBurst−1NBurst Data burst bits, p0, p1,⋯ , pNPadding−1NPadding padding bits .

# Burst CRC encoding

A burst CRC is appended to the data burst including padding bits before the burst is further processed by the burst partitioning. The burst CRC is calculated based on all the bits in the burst. The size of a burst CRC CRCBurst is 16 bits. CRC 16-CCITT, as defined in ITU-T Recommendation X.25, shall be included at the end of the data burst.

After burst CRC encoding, the order of data burst including a burst CRC and padding bits, which is input into the burst partitioning block, is

b0, b1, ⋯, bNBurst−1NBurst Data burst bits,p0, p1, ⋯ , pNPadding−1NPadding padding bits, c0, c1, ⋯, cCRCBurst−1CRCBurst Burst CRC bits .

# Partition into FEC blocks

After burst CRC encoding, if the amount of data burst bits including burst CRC bits exceeds the maximum FEC block size of 4800 bits, it is partitioned into NFEC FEC blocks, each of which is encoded separately after FEC CRC encoding in 15.13.1.4.1. For i-th FEC block, the FEC block size NEP,i is specified by the FEC block segmentation rule for each channel coding scheme. The result of the burst partitioning is depicted in Figure xx2.

Figure xx2 The result of burst partition

Note that the FEC block size NEP,i includes an FEC block CRC. The relationship between the information block size NInfo and the FEC block size NEP,i is as follows.

NInfo=i=0(NFEC−1)NEP,i

The input order of the burst partitioning is

Burst partitioning input=[b0, b1, ⋯, bNBurst−1,p0, p1, ⋯ , pNPadding−1,c0, c1, ⋯, cCRCBurst−1 ]= [(f0,0, ⋯ , f0,(NEP,0−CRCFEC−1)),⋯, (fi,0, ⋯ , fi,(NEP,i−CRCFEC−1)), ⋯, (f(NFEC−1),0, ⋯ , f(NFEC−1),(NEP,(NFEC−1)−CRCFEC−1))].

After FEC CRC encoding, the order of the information block is

i0, i1, ⋯ , iNInfo−1=f0,0, ⋯ , f0,(NEP,0−CRCFEC−1), c0,0, ⋯ ,c0,(CRCFEC−1)Input of 0th FEC Block, ⋯, fi,0, ⋯ , fi,(NEP,i−CRCFEC−1), ci,0, ⋯ ,ci,(CRCFEC−1),Input of ith FEC Block⋯, f(NFEC−1),0, ⋯ , f(NFEC−1),(NEP,(NFEC−1)−CRCFEC−1), c(NFEC−1),0, ⋯ ,c(NFEC−1),(CRCFEC−1)Input of (NFEC−1)th FEC Block .

For convolutional turbo coding scheme, the FEC block segmentation rule is specified in 15.13.1.5.1.4.1.

[For other channel coding schemes like CC and LDPC, the FEC block segmentation rules are FFS.]

# FEC block CRC encoding

[TBD]

# FEC encoding

[Other coding schemes like CC and LDPC are FFS.]

# Convolutional turbo codes (CTCs)

# CTC encoder

The CTC encoder, including its constituent encoder, is depicted in Figure xx3. It uses a double binary Circular Recursive Systematic Convolutional code. The bits of the data to be encoded are alternately fed to A and B, starting with the MSB of the first byte being fed to A. The encoder is fed by blocks of k bits or N couples (k = 2×N bits).

The polynomials defining the connections are described in octal and symbol notations as follows:

- For the feedback branch: 0xB, equivalently 1 + D + D3 (in symbolic notation)

- For the Y parity bit: 0xD, equivalently 1 + D2 + D3

- For the W parity bit: 0x9, equivalently 1 + D3

Figure xx3 CTC encoder

First, the encoder (after initialization by the circulation state Sc1, see 15.13.1.5.1.2) is fed the sequence in the natural order (position 1) with the incremental address i = 0 … N-1. This first encoding is called C1 encoding. Then the encoder (after initialization by the circulation state Sc2, see 15.13.1.5.1.2) is fed by the interleaved sequence (switch in position 2) with incremental address j = 0 … N-1. This second encoding is called C2 encoding.

The order in which the encoded bit shall be fed into the subpacket generation block (15.13.1.5.1.5) is:

A,B,Y1,Y2,W1,W2=

A0,A1,⋯,AN−1,B0,B1,⋯,BN−1,Y1,0,Y1,1,⋯,Y1,N−1,Y2,0,Y2,1,⋯,Y2,N−1W1,0,W1,1,⋯,W1,N−1,W2,0,W2,1,⋯,W2,N−1

# Determination of CTC circular states

The state of encoder is denoted S (0 ≤ S ≤ 7) with S =4s1+2s2+s3 (see Figure xx3). The circulation states Sc1 and Sc2 are determined by the following operations:

- Initialize the encoder with state 0. Encode the sequence in the natural order for the determination of Sc1 or in the interleaved order for determination of Sc2. In both cases the final state of the encoder is S0N-1.

- According to the length N of the sequence, use Table xx2 to find Sc1 or Sc2.

Table xx2 Circulation state lookup table (Sc)

| Nmod7 | S0N–1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 0 | 6 | 4 | 2 | 7 | 1 | 3 | 5 |
| 2 | 0 | 3 | 7 | 4 | 5 | 6 | 2 | 1 |
| 3 | 0 | 5 | 3 | 6 | 2 | 7 | 1 | 4 |
| 4 | 0 | 4 | 1 | 5 | 6 | 2 | 7 | 3 |
| 5 | 0 | 2 | 5 | 7 | 1 | 3 | 4 | 6 |
| 6 | 0 | 7 | 6 | 1 | 3 | 4 | 5 | 2 |

# CTC inner interleaver

CTC inner interleaver requires the parameters P0, P1, P2 and P3 shown in Table xx3.

Table xx3 CTC inner interleaver parameters

| NEP | N | P0 | P1 | P2 | P3 | NEP | N | P0 | P1 | P2 | P3 | NEP | N | P0 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48 | 24 | 5 | 0 | 0 | 0 | 488 | 244 | 17 | 140 | 92 | 220 | 1696 | 848 | 37 | 722 | 136 | 698 |
| 64 | 32 | 11 | 12 | 0 | 12 | 496 | 248 | 17 | 194 | 0 | 58 | 1728 | 864 | 31 | 176 | 152 | 516 |
| 72 | 36 | 11 | 18 | 0 | 18 | 512 | 256 | 19 | 64 | 52 | 124 | 1760 | 880 | 23 | 646 | 484 | 850 |
| 80 | 40 | 7 | 4 | 32 | 36 | 528 | 264 | 17 | 36 | 196 | 100 | 1824 | 912 | 41 | 774 | 548 | 898 |
| 88 | 44 | 13 | 36 | 36 | 32 | 544 | 272 | 19 | 222 | 248 | 134 | 1856 | 928 | 47 | 576 | 212 | 728 |
| 96 | 48 | 13 | 24 | 0 | 24 | 576 | 288 | 23 | 76 | 220 | 92 | 1888 | 944 | 39 | 342 | 596 | 798 |
| 104 | 52 | 7 | 4 | 8 | 48 | 592 | 296 | 23 | 96 | 172 | 36 | 1920 | 960 | 43 | 318 | 556 | 778 |
| 120 | 60 | 11 | 30 | 0 | 34 | 608 | 304 | 23 | 288 | 244 | 140 | 1952 | 976 | 35 | 94 | 144 | 686 |
| 128 | 64 | 13 | 46 | 44 | 30 | 624 | 312 | 23 | 286 | 220 | 70 | 1984 | 992 | 43 | 84 | 488 | 92 |
| 136 | 68 | 13 | 58 | 4 | 58 | 640 | 320 | 23 | 84 | 296 | 236 | 2048 | 1024 | 31 | 2 | 332 | 622 |
| 144 | 72 | 11 | 6 | 0 | 6 | 656 | 328 | 23 | 24 | 300 | 52 | 2112 | 1056 | 43 | 96 | 720 | 980 |
| 152 | 76 | 11 | 38 | 12 | 74 | 688 | 344 | 19 | 320 | 4 | 324 | 2176 | 1088 | 37 | 428 | 976 | 28 |
| 160 | 80 | 13 | 68 | 76 | 64 | 704 | 352 | 19 | 52 | 76 | 256 | 2304 | 1152 | 41 | 16 | 484 | 888 |
| 176 | 88 | 17 | 52 | 68 | 32 | 720 | 360 | 23 | 130 | 156 | 238 | 2368 | 1184 | 47 | 228 | 440 | 724 |
| 184 | 92 | 13 | 2 | 0 | 2 | 736 | 368 | 29 | 126 | 208 | 270 | 2432 | 1216 | 43 | 452 | 888 | 96 |
| 192 | 96 | 7 | 58 | 48 | 10 | 752 | 376 | 23 | 26 | 24 | 230 | 2496 | 1248 | 43 | 0 | 208 | 528 |
| 200 | 100 | 11 | 76 | 0 | 24 | 768 | 384 | 29 | 252 | 0 | 8 | 2560 | 1280 | 53 | 264 | 488 | 824 |
| 208 | 104 | 11 | 10 | 32 | 42 | 800 | 400 | 23 | 150 | 216 | 150 | 2624 | 1312 | 47 | 378 | 1092 | 1250 |
| 216 | 108 | 11 | 54 | 56 | 2 | 816 | 408 | 29 | 342 | 132 | 42 | 2752 | 1376 | 37 | 430 | 880 | 970 |
| 232 | 116 | 11 | 70 | 60 | 58 | 832 | 416 | 23 | 354 | 164 | 198 | 2816 | 1408 | 31 | 624 | 704 | 400 |
| 240 | 120 | 13 | 60 | 0 | 60 | 848 | 424 | 29 | 234 | 388 | 82 | 2880 | 1440 | 43 | 720 | 360 | 540 |
| 248 | 124 | 13 | 6 | 84 | 46 | 864 | 432 | 29 | 216 | 272 | 180 | 2944 | 1472 | 41 | 338 | 650 | 646 |
| 256 | 128 | 11 | 64 | 8 | 8 | 880 | 440 | 29 | 410 | 328 | 218 | 3008 | 1504 | 43 | 916 | 1136 | 912 |
| 264 | 132 | 13 | 72 | 68 | 8 | 912 | 456 | 29 | 14 | 264 | 94 | 3072 | 1536 | 53 | 184 | 824 | 1368 |
| 272 | 136 | 13 | 82 | 44 | 38 | 928 | 464 | 23 | 446 | 392 | 186 | 3200 | 1600 | 43 | 1382 | 632 | 1086 |
| 288 | 144 | 17 | 74 | 72 | 2 | 944 | 472 | 23 | 356 | 68 | 332 | 3264 | 1632 | 49 | 142 | 828 | 1354 |
| 296 | 148 | 13 | 0 | 84 | 64 | 960 | 480 | 53 | 62 | 12 | 2 | 3328 | 1664 | 37 | 258 | 28 | 1522 |
| 304 | 152 | 13 | 130 | 112 | 46 | 976 | 488 | 31 | 134 | 124 | 366 | 3392 | 1696 | 51 | 460 | 56 | 1608 |
| 312 | 156 | 11 | 32 | 124 | 108 | 992 | 496 | 29 | 358 | 480 | 362 | 3456 | 1728 | 43 | 170 | 920 | 1518 |
| 320 | 160 | 17 | 84 | 108 | 132 | 1024 | 512 | 29 | 320 | 236 | 324 | 3520 | 1760 | 57 | 776 | 1232 | 1012 |
| 328 | 164 | 17 | 148 | 160 | 76 | 1056 | 528 | 31 | 86 | 380 | 250 | 3648 | 1824 | 49 | 132 | 720 | 276 |
| 344 | 172 | 17 | 160 | 116 | 52 | 1088 | 544 | 31 | 432 | 216 | 524 | 3712 | 1856 | 41 | 1328 | 772 | 1036 |
| 352 | 176 | 17 | 106 | 56 | 50 | 1152 | 576 | 31 | 534 | 372 | 246 | 3776 | 1888 | 53 | 772 | 256 | 408 |
| 360 | 180 | 17 | 40 | 132 | 128 | 1184 | 592 | 31 | 2 | 568 | 94 | 3840 | 1920 | 53 | 92 | 1124 | 476 |
| 368 | 184 | 19 | 88 | 0 | 172 | 1216 | 608 | 31 | 368 | 584 | 524 | 3904 | 1952 | 51 | 664 | 200 | 64 |
| 376 | 188 | 13 | 110 | 92 | 14 | 1248 | 624 | 31 | 88 | 404 | 608 | 3968 | 1984 | 57 | 1296 | 760 | 1360 |
| 384 | 192 | 11 | 96 | 48 | 144 | 1280 | 640 | 29 | 152 | 8 | 24 | 4096 | 2048 | 55 | 148 | 808 | 308 |
| 400 | 200 | 19 | 142 | 0 | 142 | 1312 | 656 | 31 | 214 | 160 | 506 | 4160 | 2080 | 79 | 214 | 308 | 262 |
| 408 | 204 | 23 | 42 | 20 | 46 | 1376 | 688 | 31 | 218 | 20 | 670 | 4224 | 2112 | 59 | 14 | 668 | 1474 |
| 416 | 208 | 17 | 102 | 132 | 178 | 1408 | 704 | 29 | 474 | 584 | 118 | 4288 | 2144 | 57 | 662 | 1516 | 42 |
| 424 | 212 | 17 | 138 | 56 | 18 | 1440 | 720 | 41 | 288 | 556 | 672 | 4352 | 2176 | 59 | 2052 | 712 | 1804 |
| 432 | 216 | 17 | 126 | 92 | 74 | 1472 | 736 | 29 | 164 | 424 | 76 | 4416 | 2208 | 59 | 1342 | 1968 | 1562 |
| 440 | 220 | 19 | 48 | 20 | 144 | 1504 | 752 | 31 | 254 | 416 | 474 | 4544 | 2272 | 65 | 1380 | 1068 | 1036 |
| 456 | 228 | 17 | 184 | 0 | 48 | 1536 | 768 | 31 | 34 | 564 | 710 | 4608 | 2304 | 67 | 954 | 1140 | 1566 |
| 464 | 232 | 17 | 62 | 52 | 114 | 1600 | 800 | 31 | 454 | 216 | 234 | 4672 | 2336 | 67 | 410 | 1020 | 114 |
| 472 | 236 | 19 | 40 | 104 | 28 | 1632 | 816 | 29 | 334 | 564 | 66 | 4736 | 2368 | 59 | 2 | 956 | 458 |
| 480 | 240 | 13 | 120 | 60 | 180 | 1664 | 832 | 31 | 156 | 568 | 56 | 4800 | 2400 | 53 | 66 | 24 | 2 |

The two-step interleaver shall be performed as follows:

Step 1: Switch alternate couples

Let the sequence u0=[(A0,B0),(A1,B1),(A2,B2),(A3,B3),⋯,(AN−1,BN−1)] be the input to

first encoding C1

for i=0 …N - 1

If ( i mode 2), let (Ai,Bi)→(Bi,Ai)( i.e., switch the couple)

This step gives a sequence u1=[(A0,B0),(B1,A1),(A2,B2),(B3,A3),⋯,

(BN−1,AN−1))] = [u1(0),u1(1),u1(2),u1(3),⋯,u1(N−1)].

Step 2: P(j)

The function P(j) provides the address of the couple of the sequence u1 that shall be mapped

onto the address j of the interleaved sequence (i.e.. u2(j) = u1 (P(j))).

for j = 0…N - 1

switch (j mod 4):

case 0: P(i)=(P0⋅j+1) mod N

case 1: P(i)=(P0⋅j+1+N2+P1) mod N

case 2: P(i)=(P0⋅j+1+P2) mod N

case 3: P(i)=(P0⋅j+1+N2+P3) mod N

This step gives a sequence u2=[u1(P(0)),u1(P(1)),u1(P(2)),u1(P(3)),⋯,u1(P(N−1))]

=[(BP(0),AP(0)),(AP(1),BP(1)),(BP(2),AP(2)),(AP(3),BP(3)),⋯,(AP(N−1),BP(N−1))]. Sequence

u2 is the input to the second encoding C2.

# CTC concatenation rule

The CTC concatenation rule is depicted in Figure xx4.

Figure xx4 CTC concatenation rule block diagram

After burst CRC encoding, the data burst including padding bits and a burst CRC is partitioned into NFEC FEC blocks when the amount of input bits of the CTC concatenation block exceeds the maximum FEC block size of 4800. Note that the amount of input bits of the CTC concatenation block is NInfo-(NFEC·CRCFEC).

The FEC block segmentation rule in 15.13.1.5.1.4.1 segments the information block into NFEC FEC blocks, each size of which is NEP,i, as the input of CTC encoders. Note that an FEC block size NEP,i includes an FEC block CRC.

When the order of input bits of FEC block segmentation block is

Iinput of FEC block  segmentation=[b0, b1, ⋯, bNBurst−1,p0, p1, ⋯ , pNPadding−1,c0, c1, ⋯, cCRCBurst−1 ]= [(f0,0, ⋯ , f0,(NEP,0−CRCFEC−1)),⋯, (fi,0, ⋯ , fi,(NEP,i−CRCFEC−1)), ⋯ , (f(NFEC−1),0, ⋯ , f(NFEC−1),(NEP,(NFEC−1)−CRCFEC−1))]

, the order of inputs of CTC encoders after the FEC CRC encoding is as follows.

Input of 0th CTC encoder =[f0,0, ⋯ , f0,(NEP,0−CRCFEC−1), c0,0, ⋯ ,c0,CRCFEC−1] ⋮Input of ith CTC encoder =[fi,0, ⋯ , fi,(NEP,i−CRCFEC−1), ci,0, ⋯ ,ci,CRCFEC−1] ⋮Input of (NFEC−1)th CTC encoder =[f(NFEC−1),0, ⋯ , f(NFEC−1),(NEP,(NFEC−1)−CRCFEC−1), c(NFEC−1),0, ⋯ ,c(NFEC−1),CRCFEC−1]

The CTC encoder for i-th FEC block produces mother code rate codeword, of which the amount is 3·NEP,i for each FEC block. After CTC encoding, the subpacket generation (or bit selection and repetition) in 15.13.1.5.1.5 adapts the number of coded bits to the size of the resource allocation (in QAM symbol) for each FEC block. Note that a unit for loading a QAM symbol is a data tone (or data subcarrier). When there are NFEC FEC blocks in a data burst, the total amount of data tones for the transmission of a data burst shall also be segmented into NFEC FEC blocks. For i-th FEC block the number of data tones NDataTone,i is specified by the tone allocation rule in 15.13.1.5.1.4.2. The subpacket generation (or bit selection and repetition) adapts coded bits to M·NDataTone,i for each FEC block ,where M means the modulation order.

# FEC block segmentation

When the information block size exceeds the maximum FEC block size of 4800 bits, it is partitioned into NFEC FEC blocks, each of which is encoded separately. For i-th FEC block, the FEC block size NEP,i is specified by the following FEC block segmentation rule. Note that the resolution value δ is 64 as shown in Table xx1.

FEC block segmentation rule:

- First, the base FEC block size NBaseEP is given by the followings.

NBaseEP=NInfoNFEC⋅δ⋅δIf  NBaseEPmod 7 =0,NBaseEP=NBaseEP−δ

- The parameters α,Δ, k and m are specified by the followings.

If  NBaseEPδmod 7 =6,α=2⋅δOtherwise,α=δ

Δ=NInfo−NFEC⋅NBaseEP

k=Δα

m=Δmodα

- Initialization of NEP,i , where i=0,…, NFEC -1

If  i < k,NEP,i=NBaseEP+αOtherwise,NEP,i=NBaseEP

- If m = δ, modify NEP,i as follows

If k =0NEP,0=NEP,0+2⋅δ NEP,(NFEC−1)=NEP,(NFEC−1)−δOtherwise,NEP,0=NEP,0+δ

# Tone (Data Subcarrier) allocation

For the transmission of a data burst, a transmission format shall be specified, and a transmission format comprises modulation order M, code rate R and the number of Resource Units (RUs) NRU. After FEC encoding of an information block, subpacket generation (or bit selection and repetition) in 15.13.1.5.1.5 adapts the number of coded bits to the size of the resource allocation (in QAM symbol) which may vary depending on the LRU and subframe types. Table xx4 specifies the numbers of data tones in an RU.

Table xx4 Number of data tones per an RU

|  | Number of Tx streams | Number of pilot tones | NDataTonePerRU |
| --- | --- | --- | --- |
| Type-1 subframe with 5 OFDMA symbols (18 by 5) | 2 Tx | 10 | 80 |
|  | 4 Tx | 16 | 74 |
| Type-1 subframe (18 by 6) | 2 Tx | 12 | 92 |
|  | 4 Tx | 16 | 88 |
| Type-2 subframe (18 by 7) | 2 Tx | 14 | 112 |
|  | 4 Tx | 16 | 110 |

Note that an LRU may include control information. When NControlTonePerRU denotes the number of tones used for the control information, the number of data tones shall be given by subtracting NControlTonePerRU from NDataTonePerRU specified in Table xx4.

For NRU RUs, the total number of data tones NTotalDataTone which shall be used for loading the same amount of QAM symbols after modulation (15.13.1.6) and collection (15.13.1.6) is specified by Equation (xx5).

NTotalDataTone=j=0NRU−1NDataTonePerRU,j									(xx5)

When there are NFEC FEC blocks, the total number of data tones shall also be partitioned into NFEC FEC blocks. For i-th FEC block, the number of data tone NDataTone,i is specified by the following tone allocation rule.

Tone allocation rule:

- First, the number of base data tones for i-th FEC block NBaseTone,i is given by the following equation.

NBaseTone,i=NEP,iM⋅R

- The parametersΔ, k and m are specified by the following equations.

Δ=NTotalDataTone−i=0NFEC−1NBaseTone,i

k=ΔNFEC

m=ΔmodNFEC

- The number of data tones NDataTone,i is given as follows, where i=0… NFEC -1.

If  i < m,If Δ > 0,NDataTone,i=NBaseTone,i+k+1Otherwise,NDataTone,i=NBaseTone,i+k−1Otherwise,NDataTone,i=NBaseTone,i+k

# Subpacket generation (or Bit selection and repetition)

Proposed FEC structure punctures the mother codeword to generate a subpacket with various coding rates. The subpacket is also used as HARQ packet transmission. Figure xx5 shows the block diagram of subpacket generation. 1/3 CTC encoded codeword goes through interleaving block and the puncturing is performed. Figure xx6 shows block diagram of the interleaving block. The puncturing is performed to select the consecutive interleaved bit sequence that starts at any point of whole codeword. For the first transmission, the subpacket is generated to select the consecutive interleaved bit sequence that starts from the first bit of the systematic part of the mother codeword. The length of the subpacket is chosen according to the needed coding rate reflecting the channel condition. The first subpacket can also be used as a codeword with the needed coding rate for a burst where HARQ is not applied.

Figure xx5 Block diagram of subpacket generation

Figure xx6 Block diagram of the interleaving scheme

# Bit separation

All of the encoded bits shall be demultiplexed into six subblocks denoted A, B, Y1, Y2, W1, and W2. The encoder output bits shall be sequentially distributed into six subblocks with the first N encoder output bits going to the A subblock, the second N encoder output going to the B subblock, the third N to the Y1 subblock, the fourth N to the Y2 subblock, the fifth N to the W1 subblock, the sixth N to the W2 subblock.

# Subblock interleaving

The six subblocks shall be interleaved separately. The interleaving is performed by the unit of bits. The sequence of interleaver output bits for each subblock shall be generated by the procedure described below. The entire subblock of bits to be interleaved is written into an array at addresses from 0 to the number of the bits minus one (N-1), and the interleaved bits are read out in a permuted order with the i-th bit being read from an address, ADi (i = 0…N-1), as follows:

  - Determine the subblock interleaver parameters, m and J. Table xx5 gives three parameters.

  - Initialize i and k to 0.

  - Form a tentative output address Tk according to the formula:

Tk=2m(k mod J) +BROm(kJ)

where BROm(y) indicates the bit-reversed m-bit value of y (i.e., BRO3(6) = 3).

  - If Tk is less than N ADi = Tk and increment i and k by 1. Otherwise, discard Tk and increment k only.

  - Repeat steps 3) and 4) until all N interleaver output addresses are obtained.

The parameters for the subblock interleavers are specified in Table xx5.

Table xx5 Parameters for the subblock interleavers

| NEP | N | m | J | NEP | N | m | J | NEP | N | m | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48 | 24 | 3 | 3 | 488 | 244 | 7 | 2 | 1696 | 848 | 8 | 4 |
| 64 | 32 | 4 | 3 | 496 | 248 | 7 | 2 | 1728 | 864 | 8 | 4 |
| 72 | 36 | 4 | 3 | 512 | 256 | 7 | 2 | 1760 | 880 | 8 | 4 |
| 80 | 40 | 4 | 3 | 528 | 264 | 7 | 3 | 1824 | 912 | 8 | 4 |
| 88 | 44 | 4 | 3 | 544 | 272 | 7 | 3 | 1856 | 928 | 8 | 4 |
| 96 | 48 | 4 | 3 | 576 | 288 | 7 | 3 | 1888 | 944 | 8 | 4 |
| 104 | 52 | 5 | 3 | 592 | 296 | 7 | 3 | 1920 | 960 | 9 | 2 |
| 120 | 60 | 5 | 3 | 608 | 304 | 7 | 3 | 1952 | 976 | 9 | 2 |
| 128 | 64 | 5 | 3 | 624 | 312 | 7 | 3 | 1984 | 992 | 9 | 2 |
| 136 | 68 | 5 | 3 | 640 | 320 | 7 | 3 | 2048 | 1024 | 9 | 2 |
| 144 | 72 | 5 | 3 | 656 | 328 | 7 | 3 | 2112 | 1056 | 9 | 3 |
| 152 | 76 | 5 | 3 | 688 | 344 | 7 | 3 | 2176 | 1088 | 9 | 3 |
| 160 | 80 | 5 | 3 | 704 | 352 | 7 | 3 | 2304 | 1152 | 9 | 3 |
| 176 | 88 | 5 | 3 | 720 | 360 | 7 | 3 | 2368 | 1184 | 9 | 3 |
| 184 | 92 | 5 | 3 | 736 | 368 | 7 | 3 | 2432 | 1216 | 9 | 3 |
| 192 | 96 | 5 | 3 | 752 | 376 | 7 | 3 | 2496 | 1248 | 9 | 3 |
| 200 | 100 | 6 | 2 | 768 | 384 | 7 | 3 | 2560 | 1280 | 9 | 3 |
| 208 | 104 | 6 | 2 | 800 | 400 | 7 | 4 | 2624 | 1312 | 9 | 3 |
| 216 | 108 | 6 | 2 | 816 | 408 | 7 | 4 | 2752 | 1376 | 9 | 3 |
| 232 | 116 | 6 | 2 | 832 | 416 | 7 | 4 | 2816 | 1408 | 9 | 3 |
| 240 | 120 | 6 | 2 | 848 | 424 | 7 | 4 | 2880 | 1440 | 9 | 3 |
| 248 | 124 | 6 | 2 | 864 | 432 | 7 | 4 | 2944 | 1472 | 9 | 3 |
| 256 | 128 | 6 | 2 | 880 | 440 | 7 | 4 | 3008 | 1504 | 9 | 3 |
| 264 | 132 | 6 | 3 | 912 | 456 | 8 | 2 | 3072 | 1536 | 9 | 3 |
| 272 | 136 | 6 | 3 | 928 | 464 | 8 | 2 | 3200 | 1600 | 9 | 4 |
| 288 | 144 | 6 | 3 | 944 | 472 | 8 | 2 | 3264 | 1632 | 9 | 4 |
| 296 | 148 | 6 | 3 | 960 | 480 | 8 | 2 | 3328 | 1664 | 9 | 4 |
| 304 | 152 | 6 | 3 | 976 | 488 | 8 | 2 | 3392 | 1696 | 9 | 4 |
| 312 | 156 | 6 | 3 | 992 | 496 | 8 | 2 | 3456 | 1728 | 9 | 4 |
| 320 | 160 | 6 | 3 | 1024 | 512 | 8 | 2 | 3520 | 1760 | 9 | 4 |
| 328 | 164 | 6 | 3 | 1056 | 528 | 8 | 3 | 3648 | 1824 | 9 | 4 |
| 344 | 172 | 6 | 3 | 1088 | 544 | 8 | 3 | 3712 | 1856 | 9 | 4 |
| 352 | 176 | 6 | 3 | 1152 | 576 | 8 | 3 | 3776 | 1888 | 9 | 4 |
| 360 | 180 | 6 | 3 | 1184 | 592 | 8 | 3 | 3840 | 1920 | 10 | 2 |
| 368 | 184 | 6 | 3 | 1216 | 608 | 8 | 3 | 3904 | 1952 | 10 | 2 |
| 376 | 188 | 6 | 3 | 1248 | 624 | 8 | 3 | 3968 | 1984 | 10 | 2 |
| 384 | 192 | 6 | 3 | 1280 | 640 | 8 | 3 | 4096 | 2048 | 10 | 2 |
| 400 | 200 | 6 | 4 | 1312 | 656 | 8 | 3 | 4160 | 2080 | 10 | 3 |
| 408 | 204 | 6 | 4 | 1376 | 688 | 8 | 3 | 4224 | 2112 | 10 | 3 |
| 416 | 208 | 6 | 4 | 1408 | 704 | 8 | 3 | 4288 | 2144 | 10 | 3 |
| 424 | 212 | 6 | 4 | 1440 | 720 | 8 | 3 | 4352 | 2176 | 10 | 3 |
| 432 | 216 | 6 | 4 | 1472 | 736 | 8 | 3 | 4416 | 2208 | 10 | 3 |
| 440 | 220 | 6 | 4 | 1504 | 752 | 8 | 3 | 4544 | 2272 | 10 | 3 |
| 456 | 228 | 6 | 4 | 1536 | 768 | 8 | 3 | 4608 | 2304 | 10 | 3 |
| 464 | 232 | 6 | 4 | 1600 | 800 | 8 | 4 | 4672 | 2336 | 10 | 3 |
| 472 | 236 | 6 | 4 | 1632 | 816 | 8 | 4 | 4736 | 2368 | 10 | 3 |
| 480 | 240 | 7 | 2 | 1664 | 832 | 8 | 4 | 4800 | 2400 | 10 | 3 |

# Bit grouping

The channel interleaver output sequence shall consist of the interleaved A and B subblock sequence, followed by a bit-by-bit multiplexed sequence of the interleaved Y1 and Y2 subblock sequences, followed by a bit-by-bit multiplexed sequence of the interleaved W1 and W2 subblock sequences. The bit-by-bit multiplexed sequence of interleaved Y1 and Y2 subblock sequences shall consist of the first output bit from the Y1 subblock interleaver, the first output bit from the Y2 subblock interleaver, the second output bit from the Y1 subblock interleaver, the second output bit from the Y2 subblock interleaver, etc, The bit-by-bit multiplexed sequence of interleaved W1 and W2 subblock sequences shall consist of the first output bit from the W1 subblock interleaver, the first output bit from the W2 subblock interleaver, the second output bit from the W1 subblock interleaver, the second output bit from the W2 subblock interleaver, etc. Figure xx6 shows the interleaving scheme.

# Bit selection

Lastly, bit selection is performed to generate the subpacket. The puncturing and repetition block is referred as bits selection in the viewpoint of subpacket generation. Mother Code Bits, the total number of information and parity bits generated by FEC encoder, are considered as a maximum size of circular buffer. The bits in a subpacket are formed by selecting specific sequences of bits from the interleaved CTC encoder output sequence in the circular buffer. In case that the size of the circular buffer NCB is smaller than the number of Mother Code Bits, the first NCB bits of Mother Code Bits are considered as selected bits. Repetition is performed when the number of transmitted bits is larger than the number of selected bits. The selection of coded bits is done cyclically over the buffer. The resulting subpacket sequence is a binary sequence of bits for the modulator.

Let

k	be the subpacket index when HARQ is enabled. k = 0 for the first transmission and increases by one for the next subpacket. k = 0 when H-ARQ is not used. When there are more than one FEC block in a burst, the subpacket index for each FEC block shall be the same.

NEP     	be the number of bits in the encoder packet (before CTC encoding).

NEP,i     	be the number of bits in the encoder packet for i-th FEC block, where i = 0 …NFEC -1 (before CTC encoding).

RCB		be the lowest code rate of subpacket without repetition.

NCB		be the size of circular buffer, where NCB=NEP/RCB

NDataTone		be the number of data tones for the transmission of a data bust.

NDataTone,i	be the number of data subcarriers for i-th FEC block, where i = 0 …NFEC -1.

Mk	be the modulation order for the k-th subpacket (Mk = 2 for QPSK, 4 for 16-QAM, and 6 for 64-QAM).

SPIDk		be the subpacket ID for the k-th subpacket, (for the first subpacket, SPIDk=0 = 0).

For NRU RUs, the total number of data tones NTotalDataTone is specified in Equation (xx5). For the case that there is only one FEC block in a burst, the number of data tones NDataTone is equal to NTotalDataTone, and it is used for the subpacket generation. In addition, when there are more than one FEC block in a burst, total number of data tones NTotalDataTone is partitioned into NFEC FEC blocks by using the tone allocation rule. For i-th FEC block, the number of data tone NDataTone,i is specified, and the NDataTone,i is used for the subpacket generation instead of NDataTone for i-th FEC block (See also 15.13.1.5.1.4.2).

Let the scrambled and selected bits be numbered from zero with the 0-th bit being the first bit in the sequence. Then, the index of the j-th bit for the k-th subpacket shall be:

Sk,j = ( Fk +j ) mod NCB										(xx6)

where

j = 0 … Lk-1

Lk= NDataTone · Mk

Fk=(SPIDk · Lk) mod NCB

The above bit selection makes the following possible.

- The first transmission includes the systematic part of the mother code. Thus, it can be used as the codeword for a burst where the HARQ is not applied or when Chase HARQ is applied.

- The location of the subpacket can be determined by the SPID itself without the knowledge of previous subpacket. It is very important property for IR HARQ retransmission.

# Modulation

Modulation constellations of QPSK, 16 QAM, and 64 QAM are supported as depicted in Figure xx7. QAM symbols are collected from each FEC block and mapped to the input of the MIMO encoder.

Figure xx7 QPSK, 16-QAM, and 64-QAM constellations

# Collection

After data modulation of the subpackets, there are NDataTone,i QAM symbols for each FEC block, where i = 1 … NFEC -1. The QAM symbols of FEC blocks shall be serially collected from 0-th FEC block to (NFEC-1)-th FEC block.

------------------------------------------------------------End of the Text---------------------------------------------------------