| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | FEC block segmentation rule for IEEE 802.16m CTC |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Seunghyun Kang, Sukwoo Lee LG Electronics | Voice: +82-31-450-1918 E-mail: sh_kang@lge.com, sugoo@lge.com |
| Re: | “802.16m amendment text”: IEEE 802.16m-08/053r1, “Call for Contributions on Project 802.16m Draft Amendment Content” for the topic of “Channel Coding” |  |
| Abstract | This contribution proposes the FEC block segmentation rule for IEEE 802.16m CTC. |  |
| Purpose | To be discussed by TGm for use in the IEEE 802.16m amendment text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

FEC block segmentation rule for IEEE 802.16m CTC

Seunghyun Kang, Sukwoo Lee

LG Electronics

# Introduction

According to the current SDD [2], when the size of a data burst including a burst CRC exceeds the maximum FEC block size of 4800 bits, it is partitioned into a number of FEC blocks, each of which is encoded separately. In this contribution, we propose the FEC block segmentation rule for IEEE 802.16m CTC (Convolutional Turbo Codes) which performs the burst partitioning into FEC blocks.

# FEC block segmentation

# General concept of FEC block segmentation

For NBurst bits data burst, the number of FEC blocks NFEC and the information block size NInfo have been specified by considering CRCFEC bits FEC block CRC and the block size resolutions in [3]. Also, NPadding bits of ‘1’ are padded at the end of the data burst to fit the total amount of input bits of FEC encoders into the information block size NInfo. After ‘1’ padding and burst CRC attachment, the order of the data burst which is input into the FEC block segmentation (or burst partition) block is

b0, b1, ⋯, bNBurst−1NBurst Data burst bits,p0, p1, ⋯ , pNPadding−1NPadding padding bits, c0, c1, ⋯, cCRCBurst−1CRCBurst Burst CRC bits .

The FEC block segmentation block partitions the information block size NInfo into NFEC FEC blocks, each size of which is NEP,i where i = 0, …, NFEC -1. Note that the FEC block size NEP,i includes an FEC block CRC. The information block size NInfo and FEC block sizes shall be

NInfo=i=0(NFEC−1)NEP,i.

In other words, the data burst including padding bits and burst CRC bits is segmented into NFEC FEC blocks, each of which contains (NEP,i - CRCFEC) bits of data burst as follows.

For (NEP,i - CRCFEC) bits of data burst, FEC block CRC encoding is performed. The result of the FEC block segmentation and FEC block CRC encoding is depicted in Figure 1.

Figure 1 ─ Result of FEC block segmentation and FEC block CRC encoding

Also, the order of each CTC encoder input bits is as follows.

Order of 0th CTC encoder input bits=[f0,0, ⋯ , f0,(NEP,0−CRCFEC−1), c0,0, ⋯ ,c0,CRCFEC−1] ⋮Order of ith CTC encoder input bits =[fi,0, ⋯ , fi,(NEP,i−CRCFEC−1), ci,0, ⋯ ,ci,CRCFEC−1] ⋮Order of (NFEC−1)th CTC encoder input bits=[f(NFEC−1),0, ⋯ , f(NFEC−1),(NEP,(NFEC−1)−CRCFEC−1), c(NFEC−1),0, ⋯ ,c(NFEC−1),CRCFEC−1]

# FEC block segmentation rule for IEEE 802.16m CTC

When the information block size exceeds the maximum FEC block size of 4800 bits, it is partitioned into NFEC FEC blocks, each of which is encoded separately. For i-th FEC block, the FEC block size NEP,i is specified by the following rules. Note that the resolution value δ = 64. According to the following FEC block segmentation rule for IEEE 802.16m CTC, an information block size NInfo is partitioned into NFEC FEC blocks which can have 1 ~ 3 kinds of CTC block sizes in [3] even though it is multiple of 7.

FEC block segmentation rule:

- First, the base FEC block size NBaseEP is given by the followings.

NBaseEP=NInfoNFEC⋅δ⋅δIf  NBaseEPmod 7 =0,NBaseEP=NBaseEP−δ

- The parameters α,Δ, k and m are specified by the followings.

If  NBaseEPδmod 7 =6,α=2⋅δOtherwise,α=δ

Δ=NInfo−NFEC⋅NBaseEP

k=Δα

m=Δmodα

- Initialization of NEP,i , where i=0… NFEC -1

If  i < k,NEP,i=NBaseEP+αOtherwise,NEP,i=NBaseEP

- If m = δ, modify NEP,i as follows

If k =0NEP,0=NEP,0+2⋅δ NEP,(NFEC−1)=NEP,(NFEC−1)−δOtherwise,NEP,0=NEP,0+δ

# Examples of FEC block segmentation

With the CTC block sizes in [3], the examples of FEC block segmentation are provided with different burst sizes in table 2. In the examples, the amount of an FEC block CRC CRCFEC is assumed to be 16 bits.

Table 1 ─ Examples of FEC block segmentation

| NBurst + CRCBurst | NInfo | NPadding | NFEC | NEP,0 | NEP,1 | NEP,2 | NEP,3 | NEP,4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4856 | 4928 | 40 | 2 | 2496 | 2432 |  |  |  |
| 10808 | 10880 | 24 | 3 | 3712 | 3648 | 3520 |  |  |
| 16496 | 16576 | 16 | 4 | 4160 | 4160 | 4160 | 4096 |  |
| 23920 | 24000 | 0 | 5 | 4800 | 4800 | 4800 | 4800 | 4800 |

# References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[3] IEEE 802.16m-09/0312, “Proposed Amendment Text on Channel Coding”