| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Tone allocation rule for IEEE 802.16m CTC |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Seunghyun Kang, Sukwoo Lee LG Electronics | Voice: +82-31-450-1918 E-mail: sh_kang@lge.com, sugoo@lge.com |
| Re: | “802.16m amendment text”: IEEE 802.16m-08/053r1, “Call for Contributions on Project 802.16m Draft Amendment Content” for the topic of “Channel Coding” |  |
| Abstract | This contribution proposes the tone allocation rule for IEEE 802.16m CTC. |  |
| Purpose | To be discussed by TGm for use in the IEEE 802.16m amendment text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Tone allocation rule for IEEE 802.16m CTC

Seunghyun Kang, Sukwoo Lee

LG Electronics

# Introduction

According to the current SDD [2], when the size of a data burst including a burst CRC exceeds the maximum FEC block size of 4800 bits, it is partitioned into a number of FEC blocks, each of which is encoded separately. Also, for each FEC block, the bit selection scheme after CTC encoding adapts the number of coded bits to the size of the resource allocation in QAM symbols. However, since a number of resources are allocated only for the data burst, they shall be segmented and re-allocated for each FEC block. In this contribution, we propose the tone allocation rule, for IEEE 802.16m CTC (Convolutional Turbo Codes), which segments and re-allocates the resources into the FEC blocks.

# Tone allocation rule

For the transmission of a data burst, a transmission format shall be specified, and transmission format comprises modulation order M, code rate R and the number of RUs (Resource Units). In order to segment and re-allocate the resources, the total number of data tones NTotalDataTone in the NRU RUs shall be specified first. Also, after modulation and collection, NTotalDataTone data tones shall be used for loading the same amount of QAM symbols. Since the number data tones per an RU may vary depending on the LRU and subframe type, tones NTotalDataTone is specified by Equation (1) where NDataTonePerRU, j is the number of data tones in j-th RU (, where j =0, …, NRU -1).

NTotalDataTone=j=0NRU−1NDataTonePerRU,j							(1)

When a data burst is partitioned into NFEC FEC blocks, the total number of data tones shall also be partitioned into NFEC FEC blocks. For i-th FEC block, the number of data tone NDataTone,i is specified by the following tone allocation rule.

Tone allocation rule:

- First, the number of base data tones for i-th FEC block NBaseTone,i is given by the following equation.

NBaseTone,i=NEP,iM⋅R

- The parametersΔ, k and m are specified by the following equations.

Δ=NTotalDataTone−i=0NFEC−1NBaseTone,i

k=ΔNFEC

m=ΔmodNFEC

- The number of data tones NDataTone,i is given by the following rule , where i=0… NFEC -1.

If  i < m,If Δ > 0,NDataTone,i=NBaseTone,i+k+1Otherwise,NDataTone,i=NBaseTone,i+k−1Otherwise,NDataTone,i=NBaseTone,i+k

# Example of tone allocation

Let’s assume that the information block size NInfo is 5056 and it is segmented into two FEC blocks with the CTC block sizes NEP,0 = 2560and NEP,1 =2496. Also, the transmission format is assumed that the modulation order M is 6 and the transmission code rate R is 0.8. Table 1 shows the result of the proposed tone allocation rules while increasing the total number of tones from 1042 to 1053. In the table, Ri means the code rate of i-th FEC block, and it is give as follows.

Ri=NEP,iM⋅NDataTone,i

As shown in the table, the proposed tone allocation rule makes good matches between the transmission code rate 0.8 and the code rate of each FEC block after bit selection.

Table 1 – The results of tone allocation

| NTotalDataTone | NDataTone,0 | NDataTone,1 | R0 | R1 |
| --- | --- | --- | --- | --- |
| 1042 | 528 | 514 | 0.808 | 0.809 |
| 1043 | 528 | 515 | 0.808 | 0.808 |
| 1044 | 529 | 515 | 0.807 | 0.808 |
| 1045 | 529 | 516 | 0.807 | 0.806 |
| 1046 | 530 | 516 | 0.805 | 0.806 |
| 1047 | 530 | 517 | 0.805 | 0.805 |
| 1048 | 531 | 517 | 0.804 | 0.805 |
| 1049 | 531 | 518 | 0.804 | 0.803 |
| 1050 | 532 | 518 | 0.802 | 0.803 |
| 1051 | 532 | 519 | 0.802 | 0.802 |
| 1052 | 533 | 519 | 0.801 | 0.802 |
| 1053 | 533 | 520 | 0.801 | 0.800 |

# References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[3] IEEE 802.16m-09/0312, “Proposed Amendment Text on Channel Coding”