| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications on section 15.3.5.3.1 and 15.3.5.3.2 of 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | HanGyu Cho, Jinsoo Choi, Seung-Hyun Kang, Jin Sam Kwak LG Electronic Inc. | Voice: +82-31-450-1857 E-mail:{hgcho,emptylie,sh_kang, samji}@lge.com |
| Re: | IEEE C802.16m-08/053 “Call for Comments and Contributions on Project 802.16m Amendment Working Document ” |  |
| Abstract | The contribution proposes modifications on IEEE 802.16m amendment working document IEEE 80216m-08_050. |  |
| Purpose | To be incorporated into the IEEE 802.16 amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications on section 15.3.5.3.1 and 15.3.5.3.2 of 802.16m Amendment

HanGyu Cho, Jinsoo Choi, Seung-Hyun Kang, Jin Sam Kwak

LG Electronics

# Introduction

# This contribution proposes modifications on section 15.3.5.3.1 and 15.3.5.3.2 of 802.16m Amendment.

# Text proposal for inclusion in the 802.16m Amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

        - CRU/DRU allocation

The partition between CRUs and DRUs is done on a sector specific basis. CRU/DRU allocation is signaled in two step process.

There are four possible pre-configured allocations, called PCA, signaled in two bits:

00 --  All PRUFPis are allocated to DRUs
01 --  All PRUFPis are allocated to CRUs
10 --  All subband PRUs are allocated to CRUs and all miniband PRUs are allocated to DRUs
11 --  The mapping is signaled explicitly

When PCA!=11, the number of CRUs in the frequency partition i, denoted by LCRU,FPi., is given by

LCRU,FPi=0                              when PCA=00LSB,FPi+LMB,FPi       when PCA=01LSB,FPi                      when PCA=10

Then, the mapping of PRUFPi to CRUFPi is given by

CRUFPi[j]=PRUFPi[j]         for0≤i<FPCT,    and       0≤j<LCRU,FPi       when PCA!=11

When explicit mapping is indicated (PCA=11), additional 4-bit (TBD) CRU allocation size (CAS) field is sent in the BCH for each allocated frequency partition. CASi indicates the number of allocated subband-based CRUs for partition FPi in the unit of subband size. And additional 2-bit PRU-based CRU count (PCC) is sent in the BCH for each allocated frequency partition. PCCi indicates the number of allocated PRU-based CRUs for partition FPi in the unit of subband size.

The number of CRUs in each frequency partition is denoted by LCRU,FPi., where

- LCRU,FPi = CASi*N1 + PCCi  for 0 ≤ i < FPCT when PCA=11

The mapping of PRUFPi to CRUFPi is given by

CRUFPi[j]=PRUFPi[j]      for 0≤j<CASi⋅N1PRUFPi[k+CASi⋅N1]      for CASi⋅N1≤j<LCRU,FPi        0≤i<FPCT,    when PCA=11where k=s[j−CASi⋅N1].  is the CRU/DRU-allocation sequence (TBD), and 0≤s[j]<FPSi⋅N2−CASi⋅N1.

- The number of DRUs in each frequency partition is denoted by LDRU,FPi, where

LDRU,FPi = FPSi *N2- LCRU,FPi for 0 ≤ i < FPCT

The mapping of PRUFPi to DRUFPi is given by

DRUFPi[j]=PRUFPi[j+LCRU,FPi]       when PCA!=11PRUFPi[k+CASi⋅N1]       when PCA=11         for 0≤i<FPCT,    and      0≤j<LDRU,FPiwhere k=s[j+PCCi].

[Editor’s note: following option is proposed in 1466r1…]

where

s[j]=j⋅DSP+j⋅GCD(FPSi⋅N−LSB,FPi,DSP)FPSi⋅N−LSB,FPi+OSPmod(FPSi⋅N−LSB,FPi)

where

DSP=(Cell IDmod(FPSi⋅N−LSB,FPi−1))+1

OSP=Cell ID(FPSi⋅N−LSB,FPi−1)

------------------------------------------------------------End of the Text---------------------------------------------------------