| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to the DL ACK Channel Design for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Hyungho Park, Jinsoo Choi, Sukwoo Lee, Bin-chul Lim LG Electronics | Voice : +82-31-450-1392 E-mail: {hyunghopark, emptylie, sugoo, bcihm}@lge.com |
| Re: | IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document”. Target topic: “DL PHY control structure, especially mapping” |  |
| Abstract | The contribution proposes the text of DL ACK channel design to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to the DL ACK Channel Design 
for the IEEE 802.16m Amendment

Hyungho Park, Jinsoo Choi, Sukwoo Lee, Bin-chul Ihm

LG Electronics.

- Introduction

This contribution includes the proposed text for the DL ACK channel design in the IEEE 802.16m Amendment. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2] and the IEEE P802.16 Rev2/D7 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4]. In this contribution, amendment text including several illustrations is proposed for the section in which 16m members have enough consensus.

- Modifications from the SDD and key descriptions

- We proposed the transmission format of DL ACK channel to AAIF HARQ ACK/NACK transmission [5]. For DL ACK channel transmission format,

- We proposed the mapping of DL ACK channel index to AAIF UL resource unit [5].

- We proposed the structure of DL ACK channel structure according to the DL to UL AAIF frame ratio [5]

- Rationale/Motivation of new proposals

- Transmission format of DL ACK channel

- In current SDD text [2], it is described that DL ACK channel is separated from other control block for resource allocation and power control. Since ACK to NACK/NACK to ACK feedback error results in delayed latency and UL resource collision [1], enhanced transmission requirement comparing to other user specific control channels should be required in design of DL ACK channel.

- DL ACK channel index to AAIF resource unit mapping

- Since AAIF UL HARQ was determined as synchronous HARQ operation in the aspect of signaling overhead, HARQ functionality is performed without any control signaling. In this regard, DL ACK channel index should be known to AMS without any signaling. In addition, ACK channel index mapping to AAIF UL resource unit should be consistent of AAIF HARQ operation

- DL ACK channel structure,

- According to the system BWs described in [2], the total number of logical resource units allocated in AAIF uplink subframe is variable. Therefore, system BW should be considered in mapping of UL resource unit to DL ACK channel.

- References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE C802.16m-09/0329, “DL ACK channel for IEEE 802.16m”

- Text proposal for the 802.16m amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

## 15 Advanced Air Interface

## 15.3 Physical Layer

## 15.3.7 DL Control Structure

## 15.3.7.2 Transmission of DL Control Information

## 15.3.7.2.3 Unicast Service Control Channels (USCCH)

## 15.3.7.2.3.2 Transmission of USCCH

## 15.3.7.2.3.2.x Transmission for HARQ ACK/NACK information

HARQ ACK/NACK information for AAIF uplink data transmission is carried by DL ACK channel which is separated from control blocks for other user specific control information.

The amount of DL ACK channel depends on scheduling granularity of UL data resource assignment to a AMS, the number of data streams in AAIF UL MU-MIMO, and DL to UL ratio. The mechanism to reduce the amount of DL ACK channel resource is FFS.

## 15.3.7.2.3.2.x.1 Multiplexing and location

Within a subframe, DL ACK channel is FDM with other control blocks and data.

In the AAIF TDD mode, DL ACK channel is located in DL subframe M+k of a frame, where k means the predetermined value of ACK/NACK delay to UL data transmission in UL subframe M. According to the DL to UL ratio, k shall be determined as shown in Table 15.3.7.2.3.2.1-1.

Table 15.3.7.2.3.2.x.1-1: The ACK/NACK delay k according to UL to DL frame configuration

| - UL/DL Configuration | - subframe M |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 |
| - 4:4 | - 4 | - 4 | - 4 | - 4 |  |  |  |  |
| - 3:5 | - 4(3) | - 4(3) | - 4(3) |  |  |  |  |  |
| - 2:6 | - 4(2) | - 4(2) |  |  |  |  |  |  |
| - TBD |  |  |  |  |  |  |  |  |

In the FDD mode, the fixed value of k is FFS.

## 15.3.7.2.3.2.x.2 Transmission format

For transmission of HARQ ACK/NACK information, Figure 15.3.7.2.3.2.x.2-1 illustrates the overall transmission structure.

Figure 15.3.7.2.3.2.x.2-1: HARQ ACK/NACK transmission structure

First, repeated ACK/NACK bits are mapped by QPSK modulation and allocated in time-frequency domain by the corresponding data resource mapping with distributed LRUs.

After resource mapping, the MIMO encoder is applied as following. The input to the MIMO encoder is represented to a 2 × 1 vector.

Equation 15.3.7.2.3.2.x.2-1

The MIMO encoder generates the SFBC matrix.

Equation 15.3.7.2.3.2.x.2-2

Then, the output of the MIMO encoder is multiplied by Nt × 2 matrix W [where Nt and W will be defined in DL MIMO transmission scheme section 15.3.8.x]. The Pilot pattern A defined in DL PHY Section [15.3.5.x] shall be used for HARQ ACK/NACK transmission.

## 15.3.7.2.3.2.x.3 Resource allocation

The mapping of DL ACK channel to AAIF UL data transmission is linked to logical channel index on CRU and DRU assigned by UL control block for UL data resource allocation. Figure 15.3.7.2.3.2.x.3-1 illustrates the linking rule of DL ACK channel to AAIF UL resource unit to be transmitted. Each CRU/DRU is assigned in a unit of LRU. The index of DL ACK Channel is mapped to that of LRU. From mapping LRU index to DL ACK channel index, a AMS is able to implicitly acquire ACK/NACK information transmitted by ABS. The total number of DL ACK channel index is the same as that of LRU.

Figure 15.3.7.2.3.2.x.3-1: Linking method of DL ACK channel index to UL resource unit index

For resource allocation of HARQ ACK/NACK information, DL ACK channel is allocated to the distributed LRUs. Multiplexed ACK/NACK bits are assigned to the distributed LRUs by the transmission format described in 15.3.7.2.3.2.2. Resource elements for DL ACK channel are allocated in a unit of 1/2 distributed LRU. DL ACK channel shall be multiplexed with other user specific control channels (Resource allocation, Power control). According to the system BW 5MHz, 10MHz and 20MHz, the amount of resource elements required for DL ACK channel is 1/2 LRU, 1 LRU and 2 LRU, respectively.

-----------------------------------------------------------End of the Text---------------------------------------------------------