| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of DL MIMO Transmission Scheme Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-05 |  |
| Source(s) | Wooram Shin, Choong Il Yeh, Young Seog Song, Byung-Jae Kwak, Jihyung Kim, Min Sik Seo, Dong Seung Kwon ETRI Daejeon, 305-700, Korea | Voice: +82-42-860-6687 E-mail: w.shin@etri.re.kr (Woorm Shin) ciyeh@etri.re.kr (Choong Il Yeh) dskwon@etri.re.kr (Dong Seung Kwon) |
| Re: | IEEE 802.16m-08/053r1, “Call for Comments and Contributions on Project 802.16m Amendment Working Document”. Target topic: “DL MIMO”. |  |
| Abstract | The contribution proposes the text of DL MIMO Transmission Scheme section to be included in the IEEE 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of DL MIMO Transmission Scheme Section for the IEEE 802.16m Amendment

Wooram Shin, Choong Il Yeh, Young Seog Song,
Byung-Jae Kwak, Jihyung Kim, Min Sik Seo, Dong Seung Kwon

ETRI

- Introduction

The contribution proposes the text of DL MIMO Transmission Scheme section to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- Modifications to the SDD text

The text proposed in this contribution is based on subclause 11.8 in the IEEE 802.16m SDD [3]. The modifications to the SDD text are summarized as follows:

- Reorganizing the subclauses in [3] and adding some subclauses

- In order to reuse the terminologies and notations in [1], changing the some of them in [3] as follows:

  - Rate to STC rate

  - An input layer vector to MIMO encoder: x to s

  - An output vector of the MIMO encoder: z to x

  - A MIMO precoding matrix: P to W

  - An output vector of the precoded/beamformer block: y to z

  - Number of transmit antenna: NT to Nt

  - Number of transmit antenna: NR to Nr

  - Number of stream: NS to Mt

- Adding an opportunistic MU-MIMO mode

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r6, “IEEE 802.16m System Requirements”

[3] IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- Definitions

Insert the followings at the end of section 3:

xx. layer: A coded modulated information path fed to the MIMO encoder as an input

xx. stream: Each output of the MIMO encoder that is passed to the beamformer/precoder

xx. rank: The number of streams allocated to each resource unit

# Abbreviations and Acronyms

Insert the followings at the end of section 4:

ABS			advanced base station

AMS			advanced mobile station

CL			closed-loop

CSI			channel state information

MCW		multiple-codeword

MU			multi-user

OL			open-loop

PMI			precoding matrix index

RU			resource unit

SCW			single-codeword

SFBC		space frequency block code

SU			single-user

Insert DL MIMO Transmission Scheme section as follows:

- Advanced Air Interface

  - Physical layer

### Introduction

### OFDMA symbol description, symbol parameters and transmitted signal

### Frame Structure

### DL Physical Structure

### UL Physical Structure

### DL Control Structure

### UL Control Structure

### Channel coding and HARQ

### DL MIMO Transmission Scheme

      - DL MIMO Architecture and Data Processing

The architecture of DL MIMO on the transmitter side is shown in the Figure 1.

In SU-MIMO, only one user is scheduled in one RU. In MU-MIMO, multiple users are scheduled in one RU.

If vertical encoding (SCW) is utilized, there is only one layer. If horizontal encoding (MCW) is utilized, there are multiple layers.

User 1

data

User 2

data

User P

tailthe input to the precoder. d MIMO mode switching./d scheme used. tted to each AMS.data

User i

data

MIMO

Encoder

Beamformer

/Precoder

IFFT

OFDM Symbol

Construction

IFFT

IFFT

Precoding Vector

/Matrix

Encoder

Encoder

Layer

control

Scheduler

Resource

Mapping

Feedback

CQI

CSI

ACK / NAK

Mode / Rank / Link Adaptation

Encoder

Encoder

IFFT

Figure 1 – DL MIMO Architecture

The encoder block contains the channel encoder, interleaver, rate-matcher, and modulator for each layer.

The resource mapping block maps the coded modulated symbols to the corresponding time-frequency resources in the allocated RUs.

The MIMO encoder block maps L (≥1) layers onto Mt (≥L) streams, which are fed to the Beamformer/Precoder block.

The beamformer/precoder block maps streams to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

The OFDM symbol construction block maps antenna-specific data to the OFDM symbol.

The feedback block contains feedback information such as CQI and CSI from the AMS.

The scheduler block schedules AMSs to resource units and decides the followings:

- Allocation type: Whether the allocation should be transmitted with a distributed or localized allocation

- SU versus MU: Whether the resource allocation should support a single user or more than one user

- MIMO mode: Which OL or CL transmission scheme should be used for the user(s) assigned to the resource allocation

- User grouping: For MU-MIMO, which users should be transmitted on the RU

- Rank: The number of streams allocated to each RU

- MCS level per layer: The modulation and code rate to be used on each layer

- Boosting: The power boosting values to be used on the data and pilot subcarriers

- Band selection: If localized resource allocation is used, where in the frequency band should the localized allocation be placed

##### Antenna Configuration

The ABS employs a minimum of two transmit antennas. The AMS employs a minimum of two receive antennas. The antenna configurations are (Nt, Nr) = (2, 2), (4, 2), (4, 4), (8, 2), (8, 4), and (8, 8) where Nt denotes the number of ABS transmit antennas and Nr denotes the number of AMS receive antennas.

##### Layer to Stream Mapping

The layer to stream mapping depends on the MIMO scheme used. For SU-MIMO, the number of streams is Mt≤min(Nt, Nr), where Mt is no more than 8. The mapping is defined using the following equation

(15.3.1)

where x is the output of the MIMO encoder, S(s) is an Mt  NF STC matrix, and s is the input layer vector.

For SU-MIMO, SCW is employed. For MU-MIMO, MCW is employed at the ABS while only one stream is transmitted to each AMS.

##### Stream to Antenna Mapping

The stream to antenna mapping depends on the MIMO scheme used. The mapping is defined using the following equation

(15.3.2)

where z is the output of the precoder/beamformer and W is an Nt  Mt precoding matrix.

      - Signaling support for MIMO

##### Signaling support for SU MIMO

For CL SU-MIMO, the precoding matrix shall be signaled via explicit signaling if common demodulated pilots are used, or via dedicated pilots.

##### Signaling support for MU MIMO

For MU-MIMO, the precoding matrix shall be signaled via explicit signaling if common demodulation pilots are used, or via dedicated pilots.

      - Transmission for Data Channels

##### SU-MIMO

Single-user MIMO schemes are used to improve per-link performance.

The MIMO encoder is a batch processor that operates on M input symbols at a time. The input to the MIMO encoder is represented by an M  1 vector

(15.3.3)

where si is the i-th input symbol within a batch. The output of the MIMO encoder is an Mt  NF STC matrix x = S(s), which serves as the input to the precoder.

(15.3.4)

where xs,k is the s-th stream output on the k-th subcarrier. Note NF is the number of subcarriers used to transmit the MIMO signals derived from the input vector s. For OL SU-MIMO, the STC rate of a mode is defined as R = M / NF. The output of the MIMO encoder is multiplied by an Nt  Mt precoder, W. The output of the precoder, z, is denoted by a matrix Nt  NF matrix

(15.3.5)

where zj,k is the output symbol to be transmitted via the j-th physical antenna on the k-th subcarrier.

          - OL SU-MIMO

A number of antenna configurations and transmission rates are supported in OL SU-MIMO. STC rate 1 transmission modes are defined as Transmit Diversity modes. The other modes with STC rate more than one are defined as Spatial Multiplexing modes. The dimensions of the vectors and matrices for OL SU-MIMO are shown in the following table:

Table 1 – Matrix dimensions for DL OL SU-MIMO modes

| Nt | R | M | Mt | NF |
| --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 1 | 1 |
| 2 | 1 | 2 | 2 | 2 |
| 4 | 1 | 1 | 1 | 1 |
| 4 | 1 | 2 | 2 | 2 |
| 8 | 1 | 1 | 1 | 1 |
| 8 | 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 2 | 1 |
| 8 | 2 | 2 | 2 | 1 |
| 4 | 3 | 3 | 3 | 1 |
| 8 | 3 | 3 | 3 | 1 |
| 4 | 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 4 | 1 |
| 8 | 5 | 5 | 5 | 1 |
| 8 | 6 | 6 | 6 | 1 |
| 8 | 7 | 7 | 7 | 1 |
| 8 | 8 | 8 | 8 | 1 |

The output of the STC encoder on k-th subcarrier is precoded by multiplying with an Nt  Mt precoding matrix W(k). The matrix W(k) is selected from a predefined codebook defined in 3.3.1.3.1.3, and changes every u PRUs.

            - Transmit Diversity

For the transmit diversity modes with M=1, the input to MIMO encoder is s = s1, and the output of the MIMO encoder is a scalar, z = s. Then the output of MIMO encoder is multiplied by Nt × 1 matrix W.

For the transmit diversity modes with M=2, the input to the MIMO encoder is represented by a 2 × 1 vector.

(15.3.6)

The MIMO encoder generates the SFBC matrix.

(15.3.7)

Then the output of the MIMO encoder is multiplied by an Nt × 2 matrix W.

            - Spatial Multiplexing

For the STC rate-R spatial multiplexing modes, the input and the output of MIMO encoder is represented by an R  1 vector

(15.3.8)

Then the output of the MIMO encoder is multiplied by Nt × Mt matrix W.

            - Feedback for OL SU-MIMO

For OL SU-MIMO, an AMS may feedback rank and CQI to assist the ABS’s decision of rank adaptation and MIMO mode switching.

          - CL SU-MIMO

For the STC rate-R spatial multiplexing modes, the input and the output of MIMO encoder is represented by an R  1 vector

(15.3.9)

Then the output of the MIMO encoder is multiplied by a precoder matrix W. For codebook based precoding in FDD and TDD systems, W is obtained from the PMI fed back from an AMS, which is defined in 3.3.1.3.1.3. For sounding based precoding in TDD systems, W is derived from the UL sounding channel.

            - Feedback for CL SU-MIMO

In FDD and TDD systems, an AMS may feedback some of the following information in CL SU-MIMO mode:

- Rank (Wideband or sub-band)

- Sub-band selection

- CQI (Wideband or sub-band, per layer)

- PMI (Wideband or sub-band for serving cell and/or neighboring cell)

- Long-term CSI

For codebook based precoding, three different feedback modes for the PMI are supported:

- The standard mode: The PMI feedback from an AMS shall represent an entry of the base codebook. It shall be sufficient for the ABS to determine a new precoder.

- The adaptive mode: The PMI feedback from an AMS shall represent an entry of the transformed base codebook according to long term channel information.

- The differential mode: the feedback from an AMS provides a differential knowledge of the short-term channel information. This feedback represents information that is used along with other feedback information known at the ABS for determining a new precoder.

Mobile station shall support the standard and adaptive mode and may support the differential mode.

The feedback information may be transmitted via a physical layer control channel or via a higher layer signaling message.

In TDD systems, an AMS may transmit a sounding signal on the uplink.

          - Codebook for SU-MIMO

The base codebook is an IEEE 802.16e-based and/or DFT-based codebook. [The details are TBD.]

##### MU-MIMO

MU-MIMO schemes are used to boost system throughput.

For MU-MIMO, the MIMO encoder performs multiplexing each layer from Nu AMSs

(15.3.10)

where si is the i-th input symbol within a batch from the i-th AMS and Nu is up to four. The output of the MIMO encoder is multiplied by an Nt  Nu precoder, W. The output of the precoder, z, is denoted by a matrix Nt  1 matrix

(15.3.11)

where zj is the output symbol to be transmitted via the j-th physical antenna.

          - Opportunistic MU-MIMO

The output of the STC encoder on k-th subcarrier is precoded by multiplying with an Nt  Nu precoding matrix . The matrix  is selected from a predefined codebook defined in the following

(15.3.12)

where  can be randomly selected over . Note that  changes every u frames (or super-frames). The matrix WF(k) is derived from the PMI fed back from an AMS, which is defined in 3.3.1.3.1.7.

          - CL MU-MIMO

For codebook based precoding in FDD and TDD systems, W is derived from the PMI fed back from an AMS, which is defined in 3.3.1.3.1.7. For sounding based precoding in TDD systems, W is derived from the UL sounding channel.

          - Feedback for MU-MIMO

In FDD and TDD systems, an AMS may feedback some of the following information in MU-MIMO mode:

- Sub-band selection

- CQI (Wideband or sub-band, per layer)

- PMI (Wideband or sub-band for serving cell and/or neighboring cell)

- Long-term CSI

For CQI feedback, the AMS measures the DL pilot channel reference signal, or the DL dedicated pilots in the allocated resource unit, computes the channel quality information (CQI), and reports the CQI on the UL feedback channel. Both wideband CQI and subband CQI may be transmitted by an AMS. Wideband CQI is the average CQI of a wide frequency band. In contrast, sub-band CQI is the CQI of a localized sub-band. The CQI is calculated at the AMS assuming that the interfering users are scheduled by the serving ABS using precoders orthogonal to each other and orthogonal to the reported PMI.

Channel state information feedback may be employed for MU-MIMO. Codebook-based feedback is supported in FDD and TDD systems. Sounding-based feedback is supported in TDD systems.

For codebook based precoding, three different feedback modes for the PMI are supported:

- The standard mode: the PMI feedback from an AMS shall represent an entry of the base codebook. It shall be sufficient for the ABS to determine a new precoder.

- The adaptive mode: The PMI feedback from an AMS shall represent an entry of the transformed base codebook according to long term channel information.

- The differential mode: the feedback from an AMS provides a differential knowledge of the short-term channel information. This feedback represents information that is used along with other feedback information known at the ABS for determining a new precoder.

AMS shall support the standard and adaptive mode and may support the differential mode.

An enhanced UL sounding channel is used to feedback CSI-related information by the AMS to facilitate vendor-specific adaptive CL MIMO precoding.  For sounding-based precoding, the enhanced UL sounding channel can be configured to carry a known pilot signal from one or more AMS antennas to enable the ABS to compute its precoding/beamforming weights by leveraging TDD reciprocity. The sounding waveform can be configured to occupy portions of the frequency bandwidth in a manner similar to the sounding waveform used in the legacy system. To facilitate analog-feedback-based precoding, the enhanced UL sounding channel can be configured to carry unquantized CSI-related information (e.g., an unquantized encoding of the DL spatial covariance matrix or an unquantized encoding of the eigenvectors of the DL spatial covariance matrix). The unquanitized CSI-related information can be specific to a particular specified portion of the band (narrowband feedback) or specific to the entire bandwidth (wideband feedback).

          - Codebook for MU-MIMO

The unified codebook for SU and MU is employed. The MU-MIMO codebook contains subsets of the unified codebook (including full set) to support both unitary and non-unitary precoding. The ABS indicates which codebook subset (including full set) will be used explicitly or implicitly. [Details are TBD.]

          - Unification with SU-MIMO

Predefined and flexible adaptation between SU-MIMO and MU-MIMO are supported. The adaptation between SU MIMO rank 1 and MU MIMO is dynamic by using the same feedback information. The adaptation between feedback for SU MIMO rank 2 (or more) and feedback for MU MIMO is semi-static.

##### Rank and Mode Adaptation

To support the numerous radio environments for IEEE 802.16m systems, both MIMO mode and rank adaptation are supported. ABSs and AMSs may adaptively switch between DL MIMO techniques depending on parameters such as antenna configurations and channel conditions. Parameters selected for mode adaptation may have slowly or fast varying dynamics. By switching between DL MIMO techniques, an IEEE 802.16m system can dynamically optimize throughput or coverage for a specific radio environment.

The adaptation of MIMO modes is related with the system load, the channel information, AMS speed, and average CINR. Switching between SU-MIMO and MU-MIMO is also supported.

Both dynamic and semi-static adaptation mechanisms are supported. For dynamic adaptation, the mode/rank may be changed frame by frame. For semi-static adaptation, AMS may request adaptation. The decision of rank and mode adaptation is made by the ABS. The adaptation occurs slowly, and feedback overhead is less.

      - Transmission for Control Channels

##### Transmission for Broadcast Control Channel

An OL SU-MIMO technique that provides diversity gain is used for BCH.

##### Transmission for Unicast Service Control Channel

An OL SU-MIMO technique that provides diversity and/or beamforming gain is used for USCCH.

      - Advanced Features

##### Multi-BS MIMO

Multi-BS MIMO techniques are supported for improving sector throughput and cell-edge throughput through multi-BS collaborative precoding, network coordinated beamforming, or inter-cell interference nulling. Both open-loop and closed-loop multi-BS MIMO techniques can be considered. For closed-loop multi-BS MIMO, CSI feedback via codebook based feedback or sounding channel will be used. The feedback information may be shared by neighboring ABSs via network interface. Mode adaptation between single-BS MIMO and multi-BS MIMO is utilized.

##### MIMO for Multi-cast Broadcast Services

OL spatial multiplexing schemes as described in Section are used for MBS. No CL-MIMO scheme is supported in MBS.

-------------------------------  Text End  ---------------------------------------------------