| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of DL MIMO Transmission Scheme Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Guangjie Li, Qinghua Li, Hongming Zheng, Yang-seok Choi, Senjie Zhang, Shanshan Zheng, Feng Zhou, Ayelet Doron, Tolis Papathanassiou, Minnie Ho, Yuval Lomnitz Intel Corporation. | +86-10-82611515 Guangjie.li@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “802.16m amendment working document”: IEEE 802.16m-08/053r1, “Call for Contributions for P802.16m Amendment Text Proposals”. Target topic: “DL MIMO”. |  |
| Abstract | The contribution proposes the text of DL MIMO Transmission Scheme section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text of DL MIMO Transmission Scheme Section for the IEEE 802.16m Amendment

Guangjie Li, Qinghua Li, Hongming Zheng, Yang-seok Choi, Senjie Zhang,

Shanshan Zheng, Feng Zhou, Ayelet Doron, Tolis Papathanassiou, Minnie Ho, Yuval Lomnitz

Intel Corporation

# Overview

This contribution proposes text for DL-MIMO transmission schemes for the P802.16m Amendment draft working document, based on the proposals outlined by the TGm SDD. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D7 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

# Modifications to the SDD text

The text proposed in this contribution is based on subclause 11.9 in the IEEE 802.16m SDD [3]. The modifications to the SDD text are summarized below:

- Antenna configuration: change it to only define the transmit antenna configuration

- Add x to be the MIMO output data stream

- Change precoding matrix P to W.

- Delete the empty “Pilots” section

- Merge the two sections “feedback for SU and feedback for MU MIMO” to one section to avoid duplication

- Number of stream: NS to Mt

- Input symbol to MIMO encoder: x to s

- Output symbol of the MIMO encoder: z to x

- Output of the precoding block: y to z

- Delete the common pilot related description

- Add precoder to 2x2 SFBC

# References

[1] IEEE P802.16 Rev2/D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r6, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

# Proposed Text

----------------------------------------------------------------------------

[Please adopt the following text as indicated in the respective sections]

3. Definitions

[Add the following definitions:]

3.xxx: Resource Unit (RU): A granular unit in frequency and time, described by the number of OFDMA subcarriers and OFDMA symbols.

3.xxx: Single User MIMO (SU-MIMO): A MIMO transmission scheme in which a single MS is scheduled in one RU.

3.xxx: Multi-User MIMO (MU-MIMO): A MIMO transmission scheme in which multiple MSs are scheduled in a one RU, by virtue of spatial separation of the transimitted signals.

3.xxx: Layer: Information path fed to the MIMO encoder as an input

3.xxx: Stream: Information path encoded by the MIMO encoder that is passed to the beamformer/precoder

3.xxx: Rank: For the spatial multiplexing modes in SU-MIMO, the number of streams to be used for a user allocated to an RU

3.xxx: Single Code Word (SCW): In MIMO transmission, multiple MIMO stream are encoded by one encoder block(one “layer”).

4. Abbreviations and acronyms

[Add the following abbreviations in the alphabetical sequence:]

RU		Resource Unit

SCW		Single Code Word

SU-MIMO 	Single User Multiple Input Multiple Output

MU-MIMO     Multi-User Multiple Input Multiple Output

15.3.9 DL MIMO Transmission Scheme

15.3.9.1 DL MIMO Architecture and Data Processing

The architecture of downlink MIMO at the transmitter side is shown in the Figure 1.

In SU-MIMO, only one user is scheduled in one Resource Unit (RU). In MU-MIMO, multiple users can be scheduled in one RU.

If vertical encoding is utilized, there is only one encoder block (one “layer”). If horizontal encoding is utilized, there are multiple encoders (multiple “layers”). A “layer” is defined as a coding and modulation path fed to the MIMO encoder as an input. An stream is defined as an output of the MIMO encoder which is passed to the beamformer / precoder.

User

1

data

User

2

data

User P

data

User i

data

MIMO

Encoder

Beamformer

/Precoder

IFFT

OFDM Symbol

Construction

IFFT

IFFT

Precoding Vector

/Matrix

Encoder

Encoder

Encoder

Encoder

Encoder

Encoder

Layer

control

Scheduler

Resource

Mapping

Feedback

CQI

CSI

ACK / NACK

Mode / Rank / Link Adaptation

Figure 1 MIMO Architecture

The encoder block contains the channel encoder, interleaver, rate-matcher, and modulator for each layer.

The Resource Mapping block maps the modulated symbols to the corresponding time-frequency resources in the allocated resource units (RUs).

The MIMO encoder block maps L (≥1) layers onto Mt (≥L) streams, which are fed to the Beamformer/Precoder block.  For SU MIMO, only one MIMO encoder exists at each resource block (vertical encoding at transmit side), and for MU MIMO, multiple MIMO encoders exist at each resource block (horizontal encoding at transmit side)

The Beamformer/Precoder block maps stream(s) to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

The OFDM symbol construction block maps antenna-specific data to the OFDM symbol.

The feedback block contains feedback information such as CQI and CSI from the AMS.

The scheduler block will schedule users to RUs and is responsible for making a number of decisions with regards to each resource allocation, including:

- Allocation type: Whether the allocation is distributed or localized allocation

- Single-user (SU) versus multi-user (MU) MIMO: Whether the RU should support a single user or more than one user

- MIMO Mode: Which open-loop (OL) or closed-loop (CL) transmission scheme should be used for the user(s) assigned to the RU.

- User grouping: For MU-MIMO, which users should be transmitted in the RU.

- Rank: For the spatial multiplexing modes in SU-MIMO, the number of streams to be used for the user allocated to the Resource Unit (RU).

- MCS level per layer: The modulation and coding scheme to be used at each layer.

- Boosting: The power boosting values to be used on the data and pilot subcarriers.

- Band selection: If localized resource allocation is used, the position of the localized RU in the frequency band

15.3.9.2	Antenna Configuration

The ABS employs a minimum of two transmit antennas. The AMS employs a minimum of two receive antennas. The transmit antenna configurations are Nt = 2, 4 or 8, where Nt denotes the number of ABS transmit antennas.

15.3.9.3	Layer to Stream Mapping

The layer to stream mapping maps the data layer(s) into streams, which depends on the MIMO scheme used.

The mapping equation is

x=S(s)            Equation 1

where x is the output of the MIMO encoder, S(s) is an STC matrix, and s is the input layer vector.

For open-loop SU MIMO and closed-loop SU-MIMO, the number of streams is Mt≤min(Nt,NR), where Mt is no more than 8. For open-loop transmit diversity modes, NS depends on the STC schemes employed by the MIMO encoder. MU-MIMO can have up to 2 streams with 2 Tx antennas, and up to 4 streams for 4 Tx antennas and 8 Tx antennas.

For SU-MIMO, the spatial multiplexing MIMO mode employs vertical encoding. For SU-MIMO, transmit diversity MIMO mode employs vertical encoding.  For MU-MIMO, horizontal encoding is employed at the base-station while only one stream is transmitted to each AMS.

15.3.9.4   Stream to Antenna Mapping

The stream to antenna mapping depends on the MIMO scheme used.  The mapping can be defined using the following equation

z=W×x, Equation 2

where z is the output of the precoder/beamformer, W is a pre-coding matrix, and x is the input layer vector.

15.3.9.5   Resource mapping

The MIMO mode permutation operation is supported in either Distributed or Localized resource mapping.

The scheduler block in ABS decides whether distributed or localized resource mapping is used according to system information, user feedback etc.

15.3.9.6 DL Signaling support for MIMO

15.3.9.6.1 DL Signaling support for SU MIMO

For CL SU MIMO, The set codebook of (subset) information is signaled via explicit signaling to AMS.

The subset is TBD

For OL SU MIMO, the fixed precoder information is signaled via explicit signaling to AMS if midamble is used for measurement.

The information is the codebook set,.

The details of the signaling is TBD

15.3.9.6.2 DL Signaling support for MU MIMO

For CL MU MIMO, The set of codebook (subset) information is signaled via explicit signaling to AMS. The subset is defined in 15.3.x.x.x.

For OL MU MIMO, the fixed precoder information shall be signaled via explicit signaling to AMS when midamble is used for measurement.

The information is the codebook set.

The format of the signaling is TBD.

15.3.9.7 Single-user MIMO Data Transmission

Both OLSU-MIMO and CL SU MIMO are supported.

For OLSU-MIMO, both spatial multiplexing and transmit diversity schemes are supported.

For CL SU MIMO, codebook based precoding is supported for both TDD and FDD systems.

As described in section 15.3.9.1, the overall structure of the SU MIMO processing has two parts. The first part is the MIMO encoder and second part is the precoder.

The MIMO encoder is a batch processor that operates on M input symbols at a time. The input to the MIMO encoder is represented by an M  1 vector

s=s1s2⋮sM, Equation 3

where si is the i-th input symbol within a batch.

The output of the MIMO encoder is an Mt  NF MIMO STC matrix x = S(s), which serves as the input to the precoder. Mt is the number of streams.

, Equation 4

The output of the MIMO encoder is multiplied by the Nt  Mt precoder, W. The output of the precoder is denoted by a matrix the NT  NF matrix

Equation 5

where zj,k is the output symbol to be transmitted via the j-th physical antenna on the k-th subcarrier.

Note NF is the number of subcarriers used to transmit the MIMO signals derived from the input vector x. For open-loop SU-MIMO, the rate of a mode is defined as R=MNF. For OL SU-MIMO spatial multiplexing and CLSU-MMO, NF = 1.

15.3.9.7.1Open-loop SU-MIMO

In OL SU MIMO, 2Tx, 4Tx, and 8Tx antennas with rate 1 transmission are defined as Transmit Diversity modes. The other modes, including 2Tx, 4Tx, and 8Tx antennas with rate 2 transmission, 4Tx and 8Tx antennas with rate 3 transmission, 4Tx and 8Tx antennas with rate 4 transmission, and 8Tx antennas with transmission up to rate 8 are defined as Spatial Multiplexing modes.

The operation of these modes is specified in Section 15.3.9.7.1.1 and 15.3.9.7.1.2. The dimensions of the vectors and matrices for OL SU-MIMO are shown in the following table:

| Nt | R | M | Mt | NF |
| --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 1 | 1 |
| 2 | 1 | 2 | 2 | 2 |
| 4 | 1 | 1 | 1 | 1 |
| 4 | 1 | 2 | 2 | 2 |
| 8 | 1 | 1 | 1 | 1 |
| 8 | 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 2 | 1 |
| 8 | 2 | 2 | 2 | 1 |
| 4 | 3 | 3 | 3 | 1 |
| 8 | 3 | 3 | 3 | 1 |
| 4 | 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 4 | 1 |

Table 1 Matrix dimensions for open-loop SU-MIMO modes

Where Nt is the number of transmit antenna, R is the Rate of MIMO schemes, M is number of symbols input to MIMO encoder at a time, Mt is the number of stream, and NF is the number of subcarriers used to transmit the MIMO.

Precoding matrix W(k) is an Nt  Mt  matrix. The W(k) is defined in 15.3.9.7.1.3

The pilots shall be precoded with the same precoder W(k).

15.3.9.7.1.1 Transmit Diversity

The following transmit diversity modes are supported for OL SU MIMO:

- 2Tx rate-1: For M = 2,SFBC with precoder, and for M = 1, a rank-1 precoder

- 4Tx rate-1: For M = 2,SFBC with precoder, and for M = 1, a rank-1 precoder

- 8Tx rate-1: For M = 2,SFBC with precoder, and for M = 1, a rank-1 precoder

For the transmit diversity modes with M=1, the input to MIMO encoder is s=s1, and the output of the MIMO encoder is a scalar x=S(s). Then the output of the MIMO encoder is multiplied by the Nt × 1 matrix W, where W is described in section 15.3.9.7.1.3

For the transmit diversity modes with M=2, the input to the MIMO encoder is represented a 2 × 1 vector.

x=s1s2, Equation 6

The MIMO encoder generates the SFBC matrix.

x=s1−s2s2s1, Equation 7

Then the output of the MIMO encoder is multiplied by the Nt × 2 matrix W, where W is described in section 15.3.9.7.1.3

15.3.9.7.1.2 Spatial Multiplexing

For the rate-R spatial multiplexing modes, the input and the output of MIMO encoder is represented by an R  1 vector

x=s=s1s2⋮sR, Equation 8

In spatial multiplex mode, the rate R equals to M and Mt

Then the output of the MIMO encoder is multiplied by the Nt × R (Nt ×Mt ) matrix W where W is described in Section 15.3.9.7.4.

15.3.9.7.1.3 OL SU-MIMO Precoding Matrix

The precoding matrix W is selected from the codebook (or subset). The codebook is defined in 15.x.x.x.x,

The W is fixed over time.

W is selected from a codeword set which is the base codebook (or subset of the base codebook). The definition of subset is TBD.

The base codebook V(Nt Nt, NB) is used for OL SU MIMO, which is the highest rank of the codebook (Nt Tx antenna and Nt rank) and NB is the number of codebook bits, which is TBD.

W is selected from the first Mt column of the base codebook to construct the Nt ×Mt codebook set, Vs.

W cyclically changes with the logical index of subband in an allocation. A subband consists of Nx contiguous PRUs. The parameter Nx is signaled by ABS.

W starts with the first index of codebook set Vs, and incrementally (cyclically) changes the index of codebook set Vs for subands. Within one subband, the W is the same for all subcarriers.

The equation for W at the k-th subband in an allocation is

W(k) = Vs(i), i = ( k-1 mod Nc) + 1.  k=1,2,3,4.., Nc is the size of codebook set (number of precoding matrix within the set), and i is the index of matrix within Vs.

The used codebook set Vs shall be signaled to AMS for the channel quality estimation when midamble is used for channel measurement. The format of signaling is defined in 15.3.9.6.

15.3.9.7.2 Closed-loop SU-MIMO

15.3.9.7.2.1 Precoding technique

In both FDD and TDD systems, unitary codebook based precoding is supported.

In TDD systems, sounding based precoding is supported.

CL SU-MIMO uses spatial multiplex mode, in which M shall be equal to STC rate R and Mt.

For the STC rate-R spatial multiplexing modes, the input and the output of MIMO encoder is represented by an R  1 vector

x=s=s1s2⋮sR , Equation 9

The output of the MIMO encoder is multiplied by a precoder, Nt × R (Nt ×Mt ) matrix W. The precoder W is derived from the feedback of AMS.

For codebook-based precoding, there are 3 modes: base mode, adaptive mode and differential mode.

15.3.9.7.2.1.1 Base mode

In base mode, AMS feeds back the index of the precoding codeword, and ABS computes the W according to the index. Both ABS and AMS shall use the same codebook for correct operation. The codebook is defined in 15.x.x.x.x.

15.3.9.7.2.1.2 Adaptive mode

In adaptive mode, both ABS and AMS transform the base codebook to a transformed codebook using the correlation matrix. The transformation takes the form

Vi=orthR Vi , Equation 10

where Vi and Vi is the i-th codeword of the original codebook and the transformed codebook, respectively; orthX converts the input matrix (or vector) X to an orthogonal matrix with orthogonal column(s) that span the same subspace as X’s columns. R is the Nt × Nt transmit covariance matrix. The correlation matrix R contains the averaged directions for precoding.

After obtaining the transformed codebook, both AMS and ABS shall use the transformed codebook for the feedback and precoding process.

The measurement and feedback of R is defined in 15.x.x.x. and 15.x.xxx.

15.3.9.7.2.1.3 Differential mode (optional)

In the differential mode, an initial step and tracking steps comprise one period and the differential mode runs for multiple periods.

In the initial step, base codebook or transformed codebook shall be used for AMS feedback and ABS precoding. In the tracking steps, AMS shall send a differential feedback index for Dt to ABS and ABS shall compute the precoding matrix Wt that is for the t-th step as

Wt=Mt−1 Dt, 	Equation 11

where Mt−1 is a square, unitary matrix for t>1. Dt shall be computed from the differential matrix Dt that is

Dt=MHt−1 Wt, 	Equation 12

where Wt is the unquantized precoding matrix for the t-th step. Dt is quantized by a differential codebook VdNt,Mt,NB,θd defined in 15.x.x.x. The computation of the square matrix Mt−1 shall be the same at both AMS and ABS.

Define the Householder matrix Ωx of vector x as

Ωx=I−2w2wwH       forw, x>0I                                  otherwise	Equation 13

where w=e−jθxx−e1; θ is the phase of the first entry of x; e1=10⋯0T. For Mt=1, Wt−1 is an NT by 1 vector and

Mt−1=ΩWt−1.	Equation 14

For NS=2, Wt−1 is first converted to a unitary matrix with a zero at the lower left corner as

Wt−1=Wt−111Ωa11 	Equation 15

where a=Wt−1NT,2Wt−1NT,1H; Wt−1i,j is the entry of Wt−1 on the i-th row and j-th column. Wt−1 is decomposed by a series of Householder transformations as

Wt−1=Ωb11Ωcejθ1ejθ2, 	Equation 16

where b and c  are NT−NS+1 by 1 unit vectors f; b=Wt−11,1⋮Wt−1NT−1,1The square matrix Mt−1 shall be computed as

Mt−1=Ωb11Ωc. 	Equation 17

For Mt=3 and Nt=4, the beamforming feedback shall be for the norm vector of the subspace spanned by the ideal beamforming matrix.

For TDD sounding based precoding, the value of W is derived from the MS sounding feedback. The sounding channel is defined in 15.x.x.x

The pilots shall be precoded with the same precoder W.

15.3.9.9 Multi-user MIMO

Multi-user MIMO schemes are used to enable a resource allocation to communicate data to two or more AMSs.

Multi-user transmission with one stream per user is supported for MU-MIMO. MU-MIMO includes the MIMO configuration of 2Tx antennas to support up to 2 users, and 4Tx or 8Tx antennas to support up to 4 users.

Both OL MU-MIMO and CL MU-MIMO are supported.

15.3.9.9.1 Precoding technique

Up to four AMSs can be assigned to one resource allocation.

The source data s from M AMSs are multiplexed at MIMO encoder, the equation is

x=s=s1s2⋮sM , Equation 18

where si is the i-th input symbol, and it corresponds to the i-th AMS data in the resource unit.

The output of the MIMO encoder is multiplied by a Nt × M precoder W. The precoder W is derived from the feedback of AMS.

For OL MU MIMO, the precoding matrix is predefined unitary matrix and fixed over time.

For CL MU MIMO, the precoding matrix is derived from the feedback of AMSs by codebook-based or sounding-based feedback.

For codebook based precoding, AMS feedbacks the index of codebook (and the index of corresponding column), and ABS derives the W according to the feedback. Both ABS and AMS shall use the same codebook for correct operation. The codebook is defined in 15.x.x.x.x.

For codebook based precoding, there are 3 modes: base mode, adaptive mode and differential mode. The definition of the 3 mode are defined in 15.3.9.7.2.1.

For TDD sounding based precoding, the value of W is derived from the MS sounding feedback.

The pilots shall be precoded with the same precoder W.

        - OL MU-MIMO

W is Nt × M  matrix for each subcarriers. The matrix W is a predefined unitary codebook selected from codebook defined in 15.x.x.x.x, and changes every subband.

W is selected from the based codebook V(Nt,M, NB).

W cyclically changes with the logical index of subband in an allocation. A subband consists of Nx contiguous PRUs. The parameter Nx is signaled by ABS.

W starts with the first index of codebook, and incrementally (cyclically) changes the index of codebook for subands. Within one subband, the W is the same for all subcarriers.

The equation for W at k-th subband in an allocation is

W(k) = V(i) where  i = ( k-1 mod Nc) + 1.  k=1,2,3,4.., Nc is the codebook size (number of matrix within the codebook), and i is the index of matrix within V.

AMS feedbacks CQI of the preferred index of streams in subbands and corresponding stream indicator to indicate which stream is preferred in the subband. ABS will pair the AMS to RUs according to the CQI information, and use the predefined matrix W for transmission.

The CQI measurement and feedback for OL MU MIMO is defined in 15.x.x.x

        - CL MU-MIMO

W is Nt × M  matrix for each subcarriers.

For codebook based precoding, W shall be derived from the codebook index feedback from multiple AMS.

CL MU-MIMO use the rank 1 codebook V(Nt,1, NB) for both AMS and ABS, and each AMS feedbacks the index of preferred vector in a certain subband and corresponding CQI.

When ABS pairs multiple AMSs in the subband, it shall construct the initial precoding matrix Wp.

Wp=[v1v2⋯vM], where vi is the preferred vector from i-th AMS.

The orthogonal property of columns of Wp shall be checked by ABS.

If the columns are orthogonal to each other, the precoding matrix W = Wp, and the feedbacked CQI is used for MCS estimation.

If the columns are not orthogonal to each other, zero forcing method is used, and the CQI shall be adjusted according to the output of zero forcing. The processing is as following.

- Zero-forcing of the combined matrix Wp

Wx=Wp(WpHWp)−1;  , Equation 19

- Normalize the output of zero-forcing, and obtain the precoding matrix with normalized transmission power

W=Wx.(P(Wx))−12,  , Equation 20

where P(X)=i=1N1j=1N2(Xi,j)2N2,  N1 and N2 is the size of row and column of matrix X respectively.

- Adjust the CQIs from the information of the precoding weight and feedbacked codebook pairs.

,CQI1CQI2⋮CQIn=CQI1CQI2⋮CQIn⋅diag(WpHWWHWp)  Equation 21

where CQIi is the derived from feedback of CQI from i-th AMS, and CQIi is the adjusted CQI after zero-forcing.

The content of the CQI used in the zero-forcing adjustment shall be based on linear effective SINR value derived from the feedbacked CQI, which is defined in 15.x.x.x

The precoding W and the adjusted CQI are used for transmission and MCS estimation.

The CQI measurement and PMI measurement for CL MU MIMO is defined in 15.x.x.x.

For TDD sounding based precoding, W is derived from sounding feedback of multiple AMS.

15.3.9.9.5 MU CQI adjustment for non-full user transmission.

In 15.x.x.x, MU MIMO CQI estimation is based on the multi-user unitary precoding. If the CQI/PMI feedback is missing in a certain subchannel, non-full user transmission will be used, in which scenario the CQI shall be adjusted to avoid the mismatch.

CQIi=MtNk⋅CQIi,  Equation 22

where Nk is the paired users in one subband, and Mt is the maximum number of user in one subband. CQI measurement in AMS side assumes Mt users transmit simultaneously.

15.3.9.9.4 Unification with SU-MIMO

For both OL and CL MU MIMO, if ABS can not pair two or multiple AMSs in one RU because of the miss of CQI feedback, SU-MIMO shall be used based on the same feedback (rank 1 SU CL-MIMO), and CQI shall be adjusted because there is not inter-user cross interference in this case, Nk=1. This is a special case of MU MIMO.

The equation of the adjustment of CQI is

CQIi=Mt⋅CQIi, Equation 23

The adaptation between feedback for SU MIMO rank 2 (or more) and feedback for MU MIMO is semi-static.

15.3.9.10 Feedback for DL MIMO

Various feedback information from AMS is utilized to DL MIMO transmission, including rank, CQI, PMI etc.

15.3.9.10.1 Rank

In both OL SU and CL SU MIMO, rank information shall be feedbacked from AMS to ABS every Nx frame. [Nx is TBD]

The number of feedback bits for rank adaptation is ceil(log2(Nt)).

ABS shall adjust the precoding matrix W according to the feedback of rank.

MU MIMO mode doesn’t require the feedback of rank.

Rank feedback is carried by primary CQICH or secondary CQICH.

15.3.9.10.2 Sub-band selection

When frequency selective scheduling is enabled, AMS shall feedback the index of the best M preferred sub-band/stream. [M is TBD].

The feedback bits for sub-band selection is ceil(log2(nchoosek(N,M))), where N is the number of total subbands/streams.

Sub-band/stream selection is carried by secondary CQICH.

15.3.9.10.3 CQI (Wideband or sub-band, per layer)

15.3.9.10.3.1 Wideband CQI

There is one average CQI for whole band, and it shall be carried by primary CQICH or secondary CQICH.

15.3.9.10.3.2 Sub-band CQI

In OL and CL SU MIMO, AMS reports the CQI of the preferred subbands. There is one CQI report for one subband.

The sub-band CQI is feedbacked together with sub-band selection and carried by secondary CQICH.

15.3.9.10.3.3 Sub-band per layer CQI

In OL and CL MU MIMO, AMS reports the CQI of the preferred layers in certain subbands.

For OL MU MIMO, the feedback includes CQI and the index of the preferred layer

For CL MU MIMO, the feedback includes CQI and the index of the preferred layer and the corresponding codebook index.

The sub-band per layer CQI is feedbacked together with sub-band selection and carried by secondary CQICH

15.3.9.10.4 PMI

PMI is the index from the codebook (base codebook, transformed codebook or differential codebook).

There is one PMI for one subband. The number of PMI feedback bits depends on the codebook size.

The column vector of base codebook is written as

vn=v1nv2nv3nv4n+i v5nv6nv7nv8n=αnv1znv2znv3znv4zn+i βnv5znv6znv7znv8zn,  for  n=1,2,Nt

where the scaling factors αn and βn can vary across the codewords .

We will set vkzns as integers for rank 1. The superscript notation z is for integral quantities.

The quantization and the computation of the scaling factor αn (or βn) are as follows. Given a real vector of the real part (or imaginary part), the approximation is

a1a2a3a4≈cz1z2z3z4,

where c=maxiaiInteger(maxiai).  The remaining numbers can be scaled by 1c and quantized to integers.

15.3.9.10.4.1 Base codebook for 2Tx antenna (3 bits)

15.3.9.10.4.1.1 Rate 1  V[2 1 3]

| Index | Value |
| --- | --- |
| A(:,:,1) | alpha(:,1) = [0.7071]; belta(:,1) = [0]; |
|  | [1.0000, 1.0000]; |
| A(:,:,2) | alpha(:,1) = [0.7071]; belta(:,1) = [0.7071]; |
|  | [1.0000, 0+1.0000j]; |
| A(:,:,3) | alpha(:,3) = [0.2412]; belta(:,3) = [0.4938]; |
|  | [-3.0000, 2.0000-1.0000i]; |
| A(:,:,4) | alpha(:,4) = [0.6619]; belta(:,4) = [0.3516]; |
|  | [-1.0000, -1.0000+1.0000i]; |
| A(:,:,5) | alpha(:,5) = [0.2342]; belta(:,5) = [0.5355]; |
|  | [-3.0000, 2.0000+1.0000i]; |
| A(:,:,6) | alpha(:,6) = [0.1768]; belta(:,6) = [0.6843]; |
|  | [-4.0000, -1.0000+1.0000i]; |
| A(:,:,7) | alpha(:,7) = [0.7069]; belta(:,7) = [0.0245]; |
|  | [-1.0000, 1.0000+1.0000i]; |
| A(:,:,8) | alpha(:,8) = [0.2378]; belta(:,8) = [0.5147]; |
|  | [-3.0000, -2.0000-1.0000i]; |

15.3.9.10.4.1.1 Rate 2  V[2 2 3]

The 3bits 2x2 codebook is generated from V[2,1,3]. One operation Hv is employed to construct the second column for Rank 2 case.

Hv=I,v=e1I−pwwH,otherwise,

Where

v is the vector from V[2,1,3]

w=v−e1ande1=[10...0];

p=2wHw

Iis the 2x2 indentity matrix;

Hdenotes the conjugate transpose operation

V[2,2,3]=HV[2,1,3]

15.3.9.10.4.2 Base codebook for 4Tx antenna

15.3.9.10.4.2.1  4 bits

15.3.9.10.4.2.1.1 Rate 1  V[4 1 4]

| Index | Value |
| --- | --- |
| A(:,:,1) | alpha(:,1) = [0.5000]; belta(:,1) = [0.0000]; |
|  | [1.0000, 1.0000, 1.0000, 1.0000]; |
| A(:,:,2) | alpha(:,2) = [0.5000]; belta(:,2) = [0.5000]; |
|  | [1.0000, 0.0000-1.0000i, -1.0000, 0.0000-1.0000i]; |
| A(:,:,3) | alpha(:,3) = [0.5000]; belta(:,3) = [0.0000]; |
|  | [1.0000, -1.0000, 1.0000, -1.0000]; |
| A(:,:,4) | alpha(:,4) = [0.5000]; belta(:,4) = [0.5000]; |
|  | [1.0000, 0.0000+1.0000i, -1.0000, 0.0000-1.0000i]; |
| A(:,:,5) | alpha(:,5) = [0.2858]; belta(:,5) = [0.0396]; |
|  | [-1.0000, -3.0000-7.0000i, 1.0000, 0.0000+4.0000i]; |
| A(:,:,6) | alpha(:,6) = [0.2344]; belta(:,6) = [0.0967]; |
|  | [-2.0000, 0.0000+5.0000i, 2.0000-2.0000i, -1.0000-5.0000i]; |
| A(:,:,7) | alpha(:,7) = [0.0392]; belta(:,7) = [0.0909]; |
|  | [-12.0000, -12.0000+2.0000i, -8.0000+5.0000i, -3.0000+5.0000i]; |
| A(:,:,8) | alpha(:,8) = [0.0242]; belta(:,8) = [0.2547]; |
|  | [-21.0000, -14.0000-1.0000i, -6.0000-2.0000i, 6.0000-2.0000i]; |
| A(:,:,9) | alpha(:,9) = [0.1735]; belta(:,9) = [0.0375]; |
|  | [-1.0000, 0.0000-4.0000i, 2.0000+7.0000i, 4.0000+14.0000i]; |
| A(:,:,10) | alpha(:,10) = [0.0837]; belta(:,10) = [0.1835]; |
|  | [-6.0000, 4.0000-2.0000i, 0.0000+3.0000i, -3.0000-2.0000i]; |
| A(:,:,11) | alpha(:,11) = [0.0435]; belta(:,11) = [0.1409]; |
|  | [-21.0000, 7.0000-1.0000i, 3.0000, 3.0000+1.0000i]; |
| A(:,:,12) | alpha(:,12) = [0.1851]; belta(:,12) = [0.0790]; |
|  | [-1.0000, -3.0000, -3.0000+3.0000i, 2.0000-5.0000i]; |
| A(:,:,13) | alpha(:,13) = [0.5626]; belta(:,13) = [0.0832]; |
|  | [-1.0000, 0.0000+7.0000i, -1.0000, 0.0000+2.0000i]; |
| A(:,:,14) | alpha(:,14) = [0.0046]; belta(:,14) = [0.1659]; |
|  | [-105.0000, 70.0000+2.0000i, -42.0000-3.0000i, -30.0000+3.0000i]; |
| A(:,:,15) | alpha(:,15) = [0.0225]; belta(:,15) = [0.1411]; |
|  | [-21.0000, 21.0000-2.0000i, 6.0000-3.0000i, -14.0000-3.0000i]; |
| A(:,:,16) | alpha(:,16) = [0.2537]; belta(:,16) = [0.1039]; |
|  | [-2.0000, -1.0000+4.0000i, 1.0000+4.0000i, 2.0000-1.0000i]; |

15.3.9.10.4.2.1.2 Rate 2  V[4 2 4]

| Index | Value |
| --- | --- |
| A(:,:,1) | alpha(:,1) = [0.5000, 0.5000]; belta(:,1) = [0.0000, 0.0000]; |
|  | [1.0000, 1.0000, 1.0000, 1.0000; -1.0000, -1.0000, 1.0000, 1.0000]; |
| A(:,:,2) | alpha(:,2) = [0.5000, 0.5000]; belta(:,2) = [0.5000, 0.5000]; |
|  | [1.0000, 0.0000+1.0000i, 1.0000, 0.0000+1.0000i; -1.0000, 0.0000-1.0000i, 1.0000, 0.0000+1.0000i]; |
| A(:,:,3) | alpha(:,3) = [0.5000, 0.5000]; belta(:,3) = [0.0000, 0.5000]; |
|  | [1.0000, 1.0000, 1.0000, 1.0000; -1.0000, 0.0000-1.0000i, 1.0000, 0.0000+1.0000i]; |
| A(:,:,4) | alpha(:,4) = [0.5000, 0.5000]; belta(:,4) = [0.5000, 0.0000]; |
|  | [1.0000, 0.0000+1.0000i, 1.0000, 0.0000+1.0000i; -1.0000, -1.0000, 1.0000, 1.0000]; |
| A(:,:,5) | alpha(:,5) = [0.0426, 0.0214]; belta(:,5) = [0.0858, 0.1054]; |
|  | [-8.0000, 8.0000+2.0000i, -3.0000+7.0000i, 0.0000+7.0000i; 28.0000, -7.0000, 8.0000-2.0000i, 0.0000+7.0000i]; |
| A(:,:,6) | alpha(:,6) = [0.0965, 0.0671]; belta(:,6) = [0.2640, 0.0500]; |
|  | [-3.0000, 6.0000+2.0000i, -3.0000+1.0000i, 4.0000; 12.0000, 4.0000+7.0000i, 3.0000+3.0000i, -4.0000+3.0000i]; |
| A(:,:,7) | alpha(:,7) = [0.1134, 0.0063]; belta(:,7) = [0.2524, 0.0139]; |
|  | [-3.0000, 2.0000+1.0000i, 6.0000-2.0000i, 2.0000; 105.0000, -30.0000-28.0000i, 70.0000-21.0000i, -42.0000+12.0000i]; |
| A(:,:,8) | alpha(:,8) = [0.0401, 0.0079]; belta(:,8) = [0.1716, 0.0419]; |
|  | [-14.0000, 7.0000+2.0000i, -14.0000-2.0000i, 4.0000+1.0000i; 56.0000, -21.0000-3.0000i, -24.0000-4.0000i, 84.0000+12.0000i]; |
| A(:,:,9) | alpha(:,9) = [0.0377, 0.2408]; belta(:,9) = [0.2174, 0.0530]; |
|  | [-24.0000, -3.0000, -4.0000-1.0000i, 6.0000+1.0000i; 1.0000, -3.0000-3.0000i, 0.0000+3.0000i, 2.0000+7.0000i]; |
| A(:,:,10) | alpha(:,10) = [0.0464, 0.0306]; belta(:,10) = [0.0946, 0.1132]; |
|  | [-14.0000, 4.0000+6.0000i, 7.0000+3.0000i, 0.0000+2.0000i; 14.0000, 0.0000+5.0000i, -6.0000, 21.0000+2.0000i]; |
| A(:,:,11) | alpha(:,11) = [0.0810, 0.0318]; belta(:,11) = [0.1382, 0.0288]; |
|  | [3.0000, 2.0000-6.0000i, -2.0000+1.0000i, 4.0000+2.0000i; -6.0000, -10.0000-5.0000i, 15.0000+8.0000i, -15.0000+20.0000i]; |
| A(:,:,12) | alpha(:,12) = [0.0052, 0.0757]; belta(:,12) = [0.2444, 0.1471]; |
|  | [28.0000, -168.0000-1.0000i, -21.0000-1.0000i, 24.0000-1.0000i; -4.0000, 2.0000, 3.0000-5.0000i, -6.0000-2.0000i]; |
| A(:,:,13) | alpha(:,13) = [0.0327, 0.0282]; belta(:,13) = [0.2242, 0.0385]; |
|  | [-14.0000, 4.0000+1.0000i, -4.0000-2.0000i, -7.0000+3.0000i; 14.0000, 21.0000+6.0000i, 7.0000-15.0000i, 6.0000-5.0000i]; |
| A(:,:,14) | alpha(:,14) = [0.0953, 0.0821]; belta(:,14) = [0.6876, 0.1860]; |
|  | [-6.0000, -2.0000, 3.0000+1.0000i, 3.0000; 4.0000, -6.0000+3.0000i, -3.0000, 6.0000-1.0000i]; |
| A(:,:,15) | alpha(:,15) = [0.5681, 0.0507]; belta(:,15) = [0.0199, 0.0302]; |
|  | [-1.0000, 0.0000-28.0000i, 0.0000+8.0000i, 1.0000-7.0000i; 14.0000, 0.0000-10.0000i, -4.0000+15.0000i, 7.0000-6.0000i]; |
| A(:,:,16) | alpha(:,16) = [0.0111, 0.1245]; belta(:,16) = [0.0673, 0.0214]; |
|  | [-21.0000, 0.0000-4.0000i, 15.0000-12.0000i, 35.0000-3.0000i; 2.0000, -1.0000-5.0000i, -3.0000+8.0000i, 6.0000+20.0000i]; |

15.3.9.10.4.2.1.3 Rate 3  V[4 1 4]

15.3.9.10.4.2.1.4 Rate 4  V[4 1 4]

15.3.9.10.4.2.2  6 bits

[TBD]

15.3.9.10.4.3  Differential codebook

[TBD]

15.3.9.10.5 Long-term CSI

Correlation matrix R shall be feedbacked to support adaptive mode of codebook based precoding and long-term beamforming.

The R is feedbacked every Nx superframe (Nx is TBD) and one R is valid for whole band.

The measured correlation matrix has the format of

R=r11r12conjr12r22 (NT=2)          R=r11r12r13r14conjr12r22r23r24conjr13conjr23r33r34conjr14conjr24conjr34r44  (NT=4)

where the diagonal entries are positive and the other nonzero entries are complex. Because of the symmetric, only upper triangular elements shall be feedbacked after quantization.

The R matrix is normalized by the maximum element (amplitude), and then quantized to reduce the feedback overhead.

The equation of normalization is

R=Rmax(abs(ri,j)),  (i,j=1:Nt)

The normalized diagonal elements are quantized by 1 bit , and the normalized complex elements  are quantized by 4 bits

The equation for quantization is

q=a*exp(j∗b∗2π)

a=[0.6 0.9] and  b=0 for diagonal entries

| Diagonal Entries | a | b | q |
| --- | --- | --- | --- |
| q1 | 0.6 | 0 | 0.6000 |
| q2 | 0.9 | 0 | 0.9000 |

a=[0.1 0.5]  and b=[0 1/8 1/4 3/8 1/2 5/8 3/4 7/8] for non-diagonal upper trangular entries.

| Non-Diagonal Entries | a | b | q |
| --- | --- | --- | --- |
| q1 | 0.1 | 0 | 0.1000 |
| q2 | 0.1 | 1/8 | 0.0707 + 0.0707i |
| q3 | 0.1 | 1/4 | 0.0000 + 0.1000i |
| q4 | 0.1 | 3/8 | -0.0707 + 0.0707i |
| q5 | 0.1 | 1/2 | -0.1000 + 0.0000i |
| q6 | 0.1 | 5/8 | -0.0707 - 0.0707i |
| q7 | 0.1 | 3/4 | -0.0000 - 0.1000i |
| q8 | 0.1 | 7/8 | 0.0707 - 0.0707i |
| q9 | 0.5 | 0 | 0.5000 |
| q10 | 0.5 | 1/8 | 0.3536 + 0.3536i |
| q11 | 0.5 | 1/4 | 0.0000 + 0.5000i |
| q12 | 0.5 | 3/8 | -0.3536 + 0.3536i |
| q13 | 0.5 | 1/2 | -0.5000 + 0.0000i |
| q14 | 0.5 | 5/8 | -0.3536 - 0.3536i |
| q15 | 0.5 | 3/4 | -0.0000 - 0.5000i |
| q16 | 0.5 | 7/8 | 0.3536 - 0.3536i |

The total overhead is 6 bits for 2 transmit antenna and 28 bits for 4 transmit antenna.

15.3.9.11 Format of feedback

15.3.9.11.1 Feedback for rank

There is one rank feedback for whole band.

The format is

| Number of Bits | ceil(log2(Mt)) |
| --- | --- |
| Content | Rank |

Mt is the maximum number of stream supported in the MIMO mode.

15.3.9.11.2 Wideband CQI feedback for OL SU-MIMO

There is one CQI feedback for whole band.

The format is

| Number of Bits | 4 |
| --- | --- |
| Content | CQIW (Wideband CQI) |

15.3.9.11.3 Subband CQI feedback for OL SU-MIMO

There is one CQI feedback in one subband.

The format is

| Number of Bits | ceil(log2(nchoosek(N,M))) (bits) | 2 (bits) | 2 (bits) | 2 (bits) | 2 (bits) |
| --- | --- | --- | --- | --- | --- |
| Content | Subband selection | CQIM | … | CQI2 | CQI1 |

Where M is the parameter of Best-M feedback, and indicates the number of preferred CQI to feedback. N is the total subband in the allocation. The value of M is TBD

The filed of “subband selection” is coded by combinational index (the detail is TBD)

2 bits differential CQI over the wideband CQI is used, and the differential CQI is always positive value.

The final CQI is derived from CQIi=CQIW+CQIi (dB)

15.3.9.11.4 Subband CQI/PMI feedback for CL SU MIMO

There are one PMI and one CQI feedback in one subband.

The format is

| Number of Bits | ceil(log2(nchoosek(N,M))) (bits) | 2 (bits) | 2 (bits) | 2 (bits) | 2 (bits) | Nc (bits) | Nc (bits) | Nc (bits) | Nc (bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Content | Subband selection | CQIM | … | CQI2 | CQI1 | PMIM | … | PMI2 | PMI1 |

Where, M is the parameter of Best-M feedback, and indicates the number of preferred CQI to feedback. N is the total subband in the allocation. The value of M is TBD

The filed of “subband selection” is coded by combinational index (the detail is TBD)

2 bits differential CQI over the wideband CQI is used, and the differential CQI is always positive value.

The final CQI is derived from CQIi=CQIW+CQIi (dB)

Nc is the codebook size.

15.3.9.11.5 Feedback for OL MU MIMO

There are one CQI feedback for one spatial stream in one subband, and corresponding stream indicator.

The format is

| Number of Bits | ceil(log2(nchoosek(N,M))) (bits) | 2 (bits) | 2 (bits) | 2 (bits) | 2 (bits) | Nd (bits) | Nd (bits) | Nd (bits) | Nd (bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Content | Subband selection | CQIM | … | CQI2 | CQI1 | SIM | … | SI2 | SI1 |

Where, M is the parameter of Best-M feedback, and indicates the number of preferred CQI to feedback. N is the total subband in the allocation. The value of M is TBD

The filed of “subband selection” is coded by combinational index (the detail is TBD)

Nd = log2(Mt), where Mt is the total number of stream supported, and Nd is used to indicate which stream is the preferred for the AMS.

SIi is the stream indicator for the i-th subband.

2 bits differential CQI over the wideband CQI is used, and the differential CQI is always positive value.

The final CQI is derived from CQIi=CQIW+CQIi (dB)

15.3.9.11.6 Feedback for CL MU MIMO

There is one CQI and PMI feedback for one spatial stream in one subband

The format is

| Number of Bits | ceil(log2(nchoosek(N,M))) (bits) | 2 (bits) | 2 (bits) | 2 (bits) | 2 (bits) | Nc (bits) | Nc (bits) | Nc (bits) | Nc (bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Content | Subband selection | CQIM | … | CQI2 | CQI1 | PMIM | … | PMI2 | PMI1 |

Where, M is the parameter of Best-M feedback, and indicates the number of preferred CQI to feedback. N is the number of total subband in the allocation. The value of M is TBD

The filed of “subband selection” is coded by combinational index (the detail is TBD)

2 bits differential CQI over the wideband CQI is used, and the differential CQI is always positive value.

The final CQI is derived from CQIi=CQIW+CQIi (dB)

Nc is the codebook size.

15.3.9.11 MIMO measurement

AMS shall measure various MIMO parameters and feedback to ABS, such as rank, CQI, PMI.

For MIMO measurement, the midamble is used for CL MIMO modes.  Either midamble or dedicated pilot is used for OL MIMO modes. Midamble is defined in 15.3.x.x.x.x.

The channel state information shall be first measured.

15.3.9.11.1 rank measurement

In OL and CL SU MIMO, AMS measures the CQI of all the supported rank every Nx frame (Nx is TBD), and decide which rank is preferred.

15.3.9.11.2 CQI measurement

The CQI is measured based on the effective channel according to channel H and precoding vector W.

The content of CQI is TBD.

The CQI measurement equation is

[TBD]

15.3.9.11.2.1 CQI measure for OL SU MIMO

The predefined precoding matrix shall be derived by the DL control signaling, and the corresponding CQI is estimated based on the channel and the precoding matrix. The average CQI across multiple stream (number of rank) is obtained and feedbacked.

15.3.9.11.2.2 CQI measure for OL MU MIMO

The predefined precoding matrix shall be derived by the DL control signaling, and the corresponding CQI of each layer is estimated based on the channel and the precoding matrix. The CQI of best layer shall be feedbacked together with the index of the layer.

15.3.9.11.2.3 CQI measure for CL SU MIMO

The PMI shall be first decided, and the corresponding CQI is estimated on the channel and assumed precoding matrix derived from selected PMI. The average CQI across multiple streams (number of rank) is obtained and feedbacked.

15.3.9.11.2.4 CQI measure for CL MU MIMO

[TBD]( adjustment is required for non-unitary MU MIMO]

15.3.9.11.3 PMI measurement

For CL MIMO, in one subband, based on the channel information, AMS compares all the entry of the codebook and selects the best entry of the base or transformed codebook in terms of capacity or throughput.

15.3.9.11.4 Long-term CSI measurement

During some time period and whole band, the correlation matrix is measured as

R=E(HijHHij)

where R is the Nt by Nt transmit covariance matrix. The correlated channel matrix is Hij in the i-th OFDM symbol period and j-th subcarriers.

R matrix is updated every Nx super frame (Nx is TBD)

15.3.9.12 Rank and Mode Adaptation

To support the numerous radio environments for IEEE 802.16m systems, both MIMO mode and rank adaptation are supported. ABSs and AMSs shall adaptively switch between DL MIMO techniques depending on parameters such as antenna configurations and channel conditions etc.

Both dynamic and semi-static adaptation mechanisms are supported in 16m. For dynamic adaptation, the mode/rank may be changed frame by frame. For semi-static adaptation, AMS may request adaptation. The decision of rank and mode adaptation is made by the ABS. The adaptation occurs slowly, and feedback overhead is less.

15.3.9.12.1 Adaptation between SU MIMO and MU MIMO

SU MIMO (both OL and CL SU MIMO) is used to improve per link performance, while MU MIMO is used to improve system performance.

When persistent scheduling (voip, etc), number of user is small and system load is low, SU MIM shall be used.

When the number of user is large and system load is high, MU MIMO shall be used.

ABS shall monitor the user traffic and system information, and decide whether SU and MU MIMO mode to be used for a certain allocation.

The adaptation between SU and MU MIMO is semi-static. The adaptation requires re-allocation RU.

15.3.9.12.2 Adaptation between OL MIMO and CL MIMO.

OL MIMO requires less feedback and robust to channel environment, especially high speed.

When channel changes slowly, the channel state information can be used by CL SU MIMO to improve performance.

The adaptation between SU and MU MIMO is semi-static. The adaptation requires re-allocation RU.

15.3.9.12.3 Rank Adaptation for SU MIMO

Rank adaptation for SU MIMO is flexible adaptation.

AMS decide the preferred rank and feedback every Nx frame. ABS decides which rank to use according to the feedback and change the rank at the boundary of every Nx frame.

15.3.9.11 Transmission for Control Channel

15.3.9.11.1 Transmission for Broadcast Control Channel

TBD

15.3.9.11.2 Transmission for Unicast Control Channel

TBD

-------------------------------  Text End  ---------------------------------------------------