| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modifications on Signaling Protocol of DL Subchannelization for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-01-07 |  |
| Source(s) | Taeyoung Kim, Jeongho Park, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics | E-mail: ty33.kim@samsung.com |
| Re: | “802.16m amendment working document”: IEEE 802.16m-08/053r1, “Call for Contribution on Project 802.16m Draft Amendment Content”. |  |
| Abstract | The contribution proposes modifications on IEEE 802.16m amendment working document IEEE 80216m-08_050. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modifications on Signaling Protocol of DL Subchannelization for IEEE 802.16m Amendment

Taeyoung Kim, Jeongho Park, Jaeweon Cho, Hokyu Choi, Heewon Kang

Samsung Electronics

# Introduction

In this contribution, the signaling protocol for DL subchannelization in the current Amendment working document [1] is summarized and the revised signaling protocol is proposed to reduce the number of indication bits and effectively indicate the several resource types. Based on the modified signaling protocol, some equations and description for DL subchannelization is added in the current Amendment working document.

# Current signaling protocol in Amendment working document [1]

In the current Amendment working document [1], many kinds of signaling protocols for DL subchannelization are defined as shown in Table 1. Using these signaling indication fields, the allocation of subband, frequency partitioning for FFR operation and DRU/CRU allocation are performed. The assumptions considered here are as follows:

- The number of frequency partitions is 4.

- The size of frequency partitions, FPi (i0) is equal.

- The number of subbands allocated to FPi (i0) is equal.

- The indication fields on CRU allocation size for all frequency partitions are necessary.

- There are 4 possible pre-configured allocations.

Table 1 Signaling protocol defined in current Amendment document

|  | Field |  | Bits | Description | Unit | Container |
| --- | --- | --- | --- | --- | --- | --- |
| Multi-cell | Subband Allocation Count (SAC) |  | 5 (TBD) | # of PRUs allocated to subbands, LSB | SB | (P- /S-) BCH |
|  | Freq. Partition Configuration (FPC) |  | 12 |  |  | (P- /S-) BCH |
|  |  | Freq. Partition Count (FPCT) | 2 | # of freq. partitions (1~4) | - |  |
|  |  | Freq. Partition Size (FPS) | 6 | # of PRUs allocated to FPi, i>0 | MB |  |
|  |  | Freq. Partition Subband Count (FPSC) | 4 | # of subbands allocated to FPi, i>0 | SB |  |
| Cell-specific | Pre-configured allocation signal |  | 2 | Four possible pre-configured allocation 00 : only DRUs 01 : only CRUs 10 : Subband « CRU, miniband « DRU 11 : Explicitly signaled | - | (P- /S-) BCH |
|  | CRU allocation size per freq. partition (CAS) |  | 4 (4) (TBD) | # of allocated CRUs for freq. partition FPi Different CRU/DRU ratio per freq. partition | SB | (P- /S-) BCH |

With the signaling protocol shown in Table 1, the total number of indication bits is 35(= 5+12+2+44). However, the signaling protocol defined in Amendment document has a couple of unnecessary indication fields and is not efficient to apply the miniband based CRU allocation. The miniband based CRU can be assigned only in the restricted configuration. Therefore, the degree of freedom to use the miniband based CRU is very limit.

# Proposed signaling protocol

In order to reduce the number of indication bits for DL subchannelization, the concrete and feasible operation scenario is considered. In addition, the unnecessary assumptions defined in above section are removed, that is, the last assumption above causes the increase of the number of indication bits. The remained assumptions are still considered.

To efficiently allocate the miniband based CRU, the additional information field is considered for each frequency partition. Using two indication fields on subband and miniband based CRU allocation, all of possible resource allocations can be utilized without any restriction. In Table 2, the revised signaling protocol for DL subchannelization is proposed.

Table 2 Proposed signaling protocol for DL subchannelization

|  | Field | Bits | Description | Unit | Container |
| --- | --- | --- | --- | --- | --- |
| Multi-cell | Subband Allocation Count (SAC) | 4 | # of PRUs allocated to subbands | SB | S-BCH |
|  | Freq. Partition Configuration (FPC) | [4] | Freq. partition configuration set | - | S-BCH |
|  | Freq. Partition Subband Count (FPSC) | 3 | # of subbands allocated to FPi, i>0 | SB | S-BCH |
| Cell-specific | Subband based CRU allocation size for FP0 (CASSB,0) | 4 | # of allocated subband based CRUs for FP0 | SB | P-BCH |
|  | Miniband based CRU allocation size for FP0 (CASMB,0) | [6] | # of allocated miniband based CRUs for FP0 | MB | P-BCH |
|  | Subband based CRU allocation size for FPi (CASSB,i) | 3 (3) | # of allocated subband based CRUs for freq. partitions besides FP0 | SB | P-BCH |

- Subband Allocation Count (SAC) : 4 bits

Subband allocation count (SAC) field indicates the number of subbands allocated. This information is applied for all system bandwidths (e.g. 5~20MHz). For 20MHz bandwidth, the total number of PRUs is 96, which means the total number of subbands is 24. But, it is not necessary to indicate the all cases. As shown in Table 3, the number of bits required for SAC indication field can be reduced by mapping.

Table 3 Example of mapping between SAC and the number of subbands allocated

| SAC (4bits) | # of subbands allocated | SAC (4bits) | # of subbands allocated |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 10 |
| 1 | 1 | 9 | 12 |
| 2 | 2 | 10 | 14 |
| 3 | 3 | 11 | 16 |
| 4 | 4 | 12 | 18 |
| 5 | 5 | 13 | 20 |
| 6 | 6 | 14 | 22 |
| 7 | 8 | 15 | 24 |

- Frequency Partition Configuration (FPC) : [4] bits

Frequency partition configuration field give the information about the frequency partitioning such as the number of frequency partitions and the size of each frequency partition. In the signaling protocol defined in Amendment working document, the explicit fields are given to divide the frequency partition, so the number of required indication bits for frequency partitioning is unnecessarily large. As shown in Table 4, it is available to give the information on frequency partition configuration with only small number of indication bits using the predetermined frequency partition set.

Table 4 Example of mapping between FPC and frequency partitioning

| FPC ([4] bits) | Freq. Partitioning (FP0:FP1:FP2:FP3) | Example (10MHz, 48 PRUs) |
| --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 48 : 0 : 0 : 0 |
| 1 | 5 : 1 : 1 : 1 | 30 : 6 : 6 : 6 |
| 2 | 7 : 3 : 3 : 3 | 21 : 9 : 9 : 9 |
| 3 | 1 : 1 : 1 : 1 | 12 : 12 : 12 : 12 |
| 4 | 0 : 1 : 1 : 1 | 16 : 16 : 16 : 0 |
| 5 – 15 | Reserved |  |

- Frequency Partition Subband Count (FPSC) : 3 bits

This is about the number of subbands for frequency partitions besides FP0. The maximum size of frequency partitions except FP0 is at least one third of total frequency band. So, the number of indication bits for FPSC field can be reduced compared to the case of FP0.

- CRU allocation size (Subbnad/Miniband-based)

With two indication fields (e.g. CASSB,i and CASMB,i), the subband and/or miniband based CRU allocation can be performed without any restriction to use, as shown in Figure 1. According to the frequency partition used, the number of indication bits can be different because of the maximum size of frequency partition. By applying this allocation method, the use of miniband based CRU is simple.

Figure 1 Illustration of subband/miniband based CRU allocation in FPi

With the signaling protocol shown in Table 2, the total number of indication bits is 30 (= 4+4+3+4+6+33). Compared to the previous signaling protocol, the proposed signaling provides the facility to use the miniband based CRU, while the size of indication bits is almost same.

# Proposed Amendment Text

Adopt the following text changes for [1]:

Original Amendment text is shown in black. New proposed text is in blue. Removed Amendment text is shown in strike out red text.

------------------------------------------- Text Start ---------------------------------------------------------

### 15.3.5.2.1.	Subband partitioning

[Page 24, Line 6]

The number of subbands reserved is denoted by KSB. The number of PRUs allocated to subbands is denoted by LSB, where LSB = N1* KSB. A 54-bit (TBD) field called Subband Allocation Count (SAC) field determines the value of KSB.

### 15.3.5.2.3.	Frequency partitioning

[Page 26, Line 61]

The frequency partition configuration is transmitted in the BCH in a 12 [4]-bit called the Frequency Partition Configuration (FPC). Frequency Partition Count (FPCT) defines the number of frequency partitions. Frequency Partition Size (FPS) defines the number of PRUs allocated to FPi, i>0 in the number of minibands (N2). FPCT and FPS can be inferred from FPC.

The FPC consists of a Frequency Partition Count (FPCT), Frequency Partition Size (FPS) and Frequency Partition Subband Count (FPSC).  The first two bits carry the FPCT that defines the number of frequency partitions (1 to 4).  The following 6 bits carry the FPS that defines the number of PRUs allocated to FPi, i>0 in the number of minibands (N2). The remaining 43 bits carry FPSC that define the number of subbands allocated to FPi, i>0.

### 15.3.5.3.1.	CRU/DRU allocation

[Page 28, Line 64]

The partition between CRUs and DRUs is done on a sector specific basis. DRU allocation is signaled in two step process.

There are four possible pre-configured allocations signaled in two bits:

00 --  All PRUFPis are allocated to DRUs

01 --  All PRUFPis are allocated to CRUs

10 --  All subband PRUs are allocated to CRUs and all miniband PRUs are allocated to DRUs

11 --  The mapping is signaled explicitly

When explicit mapping is indicated, additional For partition FP0, 4-bit (TBD) subband CRU allocation size (CASSB,0) field and [6]-bit miniband CRU allocation size (CASMB,0) field is are sent in the BCH for each allocated frequency partition. For the remained FPi, 3-bit subband CRU allocation size (CASSB,i) field only is sent in the BCH. CASi CASSB,i and CASMB,i indicated the number of allocated subband and miniband CRUs for partition FPi in a unit of subband and miniband size, respectively.

The number of subband and miniband CRUs in each frequency partition is are denoted by LCRU,FPi. and , where

LCRU,FPi = CASi*N1 for 0 ≤ i < FPCT

for 0  i < FPCT

for 0  i < FPCT

The number of DRUs in each frequency partition is denoted by LDRU,FPi, where LDRU,FPi = FPSi - CASi·N1  for 0 ≤ i < FPCT

The mappings of PRUFPiFPi to

CRUFPi[j]=PRUFPi[j],for0≤i<FPCT,and0≤j<LCRU,FPi

DRUFPi[j]=PRUFPi[j+LCRU,FPi],for0≤i<FPCT,and0≤j<LDRU,FPi

for 0  i < FPCT and 0  j <

for 0  i < FPCT and 0  j <

,			for 0  i < FPCT and 0  j <

[where  for 0  j <, and PermSeq is the sequence of length+and is determined by SEED=(IDcell*1357351) mod 220.  for 0  j <, where PermSeqC is to sequentially map the index of the remainders of the PRUs after allocating the miniband CRUs to j.]

[Page 29, Line 32]

### 15.3.5.3.2.	Secondary permutation

The miniband CRUs may be permuted on a sector specific basis by a secondary permutation. Permutation of the secondary permutation will be signaled by a 1-bit secondary permutation field in the BCH.

The secondary permutation will be governed by Equation (179):

TBD (eq X)

------------------------------------------- Text End -----------------------------------------------------------

# Reference

[1] IEEE 802.16m-08/050, “IEEE 802.16m Amendment Working Document”