| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Amendment Text Proposal for transmission format of MIMO HARQ |  |
| Date Submitted | 2009-1-7 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII, Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | 802.16m amendment working document - Target topic: “11.13 Channel Coding and HARQ”. |  |
| Abstract | We propose the transmission format of MIMO HARQ. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Amendment Text Proposal for transmission format of MIMO HARQ

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corportion

# Introduction

In this contribution, we propose the MIMO HARQ transmission format to improve performance gain by adopting the dynamic mapping of modulated symbols to spatial streams in order to change or randomize channel of data symbols for every re-transmission[1].

# Proposed MIMO HARQ mechanism

In the Open-loop SU-MIMO transmission for both DL and UL, the MIMO subpacket retransmission can be generated by using the MIMO-HARQ mapping of modulated symbols to spatial streams. The MIMO-HARQ mapping causes that a modulated symbol is mapped to different transmit antennas in (re-)transmissions.

The transmission rule for the MIMO-HARQ mapping of modulated symbols to spatial streams is listed in Table 1, Table 2, and Table 3.

The MS shall process the initial transmission, first retransmission, and second retransmission, etc., in the form of spatial modulation and SFBC decoding. The retransmission of FEC codeword shall use the Chase combing retransmission.

Table 1 MIMO-HARQ mapping of modulated symbols to spatial streams (2-stream case)

|  | Initial transmission | Odd transmission | Even transmission |
| --- | --- | --- | --- |
| Selected Mapping Matrix | S2 (0)=s1s2 | S2(odd)=s2s1 | S2(even)=s1s2 |

Table 2 MIMO-HARQ mapping of modulated symbols to spatial streams (3-stream case)

|  | Initial transmission | Odd transmission | Even transmission |
| --- | --- | --- | --- |
| Selected Mapping Matrix | S2(0)=s1s2s3 | S2(odd)= s3s1s2 | S2(even)= s1s2s3 |

Table 3 MIMO-HARQ mapping of modulated symbols to spatial streams (4-stream case)

|  | Initial transmission | Odd transmission | Even transmission |
| --- | --- | --- | --- |
| Selected Mapping Matrix | S2(0)=s1s2s3s4 | S2(odd)= s3s4s1s2 | S2(even)= s1s2s3s4 |

# Simulation result

Figure 1 shows the performance comparison of 16e Chase HARQ[2] and proposed Chase MIMO-HARQ. 
As a simulation result, the proposed MIMO-HARQ mapping obtains about 0.5dB gain at 10% target BLER,  over 16e Chase HARQ under low mobility environment.

Table 4 Simulation Parameters

| Parameters | Assumption |
| --- | --- |
| Bandwidth | 10 MHz |
| Number of subcarrier | 1024 |
| Frame length | 5ms |
| Channel estimation | Perfect |
| Channel code | Turbo (Rate 1/2) |
| Modulation | 16 QAM |
| MIMO configuration | Tx: 2, Rx:2 |
| Resource allocation | PUSC |
| Channel model | VEH A |
| MS mobility | 30km/h |
| Receiver type | MMSE |
| Maximum number of retransmission | 1, 2, 3 |
| Retransmission latency | 10ms |

Figure 1  Performance comparison of proposed Chase MIMO-HARQ and 16e Chase HARQ

(Re: Maximum number of retransmission, 16 QAM, Rate=1/2)

# Conclusion

This contribution proposes the MIMO-HARQ mapping which causes a modulated symbol to be mapped on different transmit antennas in (re-)transmissions. The proposed schemes can improve spatial diversity and time diversity, and thus they should be included into IEEE 802.16m amendment working document[3].

# References

[1]IEEE802.16m-08/770r1,Panasonic,“Subpacket retransmission with spatial diversity schemes in MIMO channel”

[2]IEEE Std 802.16e-2005
[3]IEEE802.16m-08/050, “IEEE 802.16m Amendment Working Document”

# Proposed Amendment Text

-------------------------------  Text Start  ---------------------------------------------------

15.3.x MIMO HARQ

In the Open-loop SU-MIMO transmission for both DL and UL, the MIMO subpacket retransmission can be generated by using the MIMO-HARQ mapping of modulated symbols to spatial streams . The transmission rule for the MIMO-HARQ mapping of modulated symbols to spatial streams is listed in Table 1, Table 2, and Table 3.

The MS shall process the initial transmission, first retransmission, and second retransmission, etc., in the form of spatial modulation and SFBC decoding. The retransmission of FEC codeword shall use the Chase combing retransmission.

Table 1 MIMO-HARQ mapping of modulated symbols to spatial streams (2-stream case)

|  | Initial transmission | Odd transmission | Even transmission |
| --- | --- | --- | --- |
| Selected Mapping Matrix | S2 (0)=s1s2 | S2(odd)=s2s1 | S2(even)=s1s2 |

Table 2 MIMO-HARQ mapping of modulated symbols to spatial streams (3-stream case)

|  | Initial transmission | Odd transmission | Even transmission |
| --- | --- | --- | --- |
| Selected Mapping Matrix | S2(0)=s1s2s3 | S2(odd)= s3s1s2 | S2(even)= s1s2s3 |

Table 3 MIMO-HARQ mapping of modulated symbols to spatial streams (4-stream case)

|  | Initial transmission | Odd transmission | Even transmission |
| --- | --- | --- | --- |
| Selected Mapping Matrix | S2(0)=s1s2s3s4 | S2(odd)= s3s4s1s2 | S2(even)= s1s2s3s4 |

-------------------------------  Text End  ---------------------------------------------------