| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Relay Station State Diagram |  |
| Date Submitted | 2009-02-26 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai*, Chun-Yen Hsu, Chih-Wei Su, Youn-Tai Lee, Yi-Ting Lin NMI/ESTI*, Institute for Information Industry Hua-Chiang Yin, Yung-Ting Lee Coiler Whai-En Chen National Ilan University Shiann-Tsong Sheu, Chih-Cheng Yang National Central University Yih Guang Jan Tamkang University | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw, lucas@iii.org.tw* |
| Re: | 802.16m AWD |  |
| Abstract | Comment on 802.16m System Description Document |  |
| Purpose | For review and discussion in the Project 802.16m Relay Rapporteur Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Relay Station State Diagram

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Chun-Yen Hsu, Chih-Wei Su, Youn-Tai Lee, Yi-Ting Lin, Hua-Chiang Yin,
Yung-Ting Lee, Whai-En Chen, Shiann-Tsong Sheu, Chih-Cheng Yang, Yih Guang Jan

III, Coiler, NIU, NCU, TKU

Introduction

This contribution (see IEEE C80216m-08_890r1) proposes text for the 16m ARS network entry based on the draft standard IEEE P802.16j.

Proposed Changes:

15 Relay

[Add following subclauses in 15:]

## 15.4.1 Relay Station State Diagram

The Figure xxx illustrates the Relay Station state transition diagram for an ARS. The diagram consists of 3 states, Initialization State, Access State, and Operational State.

Figure xxx System State Transition Diagram of IEEE802.16m Relay

## 15.4.1.1 Initialization State

In the initialization state, the ARS performs cell selection by scanning and synchronizing to an ABS A-PREAMBLE, and acquiring the system configuration information through SFH before entering Access State. During this state, if the ARS cannot properly perform the SFH information decoding and cell selection, it should return to perform scanning and DL synchronization. If the ARS successfully decodes SFH information and selects one target ABS, it transits to the Access State.

Figure yyy Initialization State Transition Diagram of IEEE802.16m Relay

## 15.4.1.2 Access State

The ARS performs network entry with the target ABS while in the Access state. Network entry is a multi step process consisting of ranging, pre-authentication capability negotiation, authentication and authorization, capability exchange, registration, neighbor station measurement, access station selection, ARS operation. ARS receives the 802.16m specific IDs as part of Access State procedures. The ARS receives its Station ID and transitions to the Operational state. Upon failure to complete any one of the steps of network entry the ARS transitions to the Initialization state.

Figure zzz Access State Transition Diagram of IEEE802.16m Relay

## 15.4.1.3 Operational State

The Operational State consists of 2 modes; Active Mode and Neighbor Measurement Mode. During Operational State, the ARS and the access station may establish additional management connections. And also, ARS can scan neighbor’s signal to reconfigure its parameters which provide robust and reliable services.

Figure uuu Operational State Transition Diagram of IEEE802.16m Relay

### 15.4.1.3.1 Active Mode

During Active Mode, the ARS and the access station perform normal operations to relay the DL/UL traffic transaction between the AMS and the access station. The ARS can perform the fast network re-entry procedures after handover; while in handover, the ARS maintains any 802.16m specific identifiers required for handover and its connections with access station. Without going through Access State, the ARS may remain in Operational State with target ABS.

### 15.4.1.3.2 Neighbor Measurement Mode

During Neighbor Measurement Mode, the ARS performs neighbor measurement or scanning operation and may be temporarily unavailable to the AMS. While in active mode, the ARS transits to Neighbor Measurement Mode implicitly, or explicitly via MAC signaling.