| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Indoor relay operation with GPS assistance |  |
| Date Submitted | 2009-02-27 |  |
| Source(s) | Yi-Hsueh Tsai*, Kanchei (Ken) Loa, Yi-Ting Lin Emerging Smart Technology Institute*/ Networks and Multimedia Institute, Institute for Information Industry | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw, lucas@iii.org.tw* |
| Re: | Change request for IEEE 802.16m System Description Document (802.16m-08/003r7) |  |
| Abstract | Comment on 802.16m System Description Document |  |
| Purpose | For review and discussion in the Project 802.16m Task Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Indoor relay operation with GPS assistance

Yi-Hsueh Tsai*, Kanchei (Ken) Loa+, Yi-Ting Lin+

Emerging Smart Technology Institute*/Networks and Multimedia Institute+, Institute for Information Industry

- Introduction

For GPS indoor usage, the traditional GPS receiver is unable to (1) demodulate navigation data from the GPS satellites and (2) find GPS signals since the receiver cannot dwell for long enough at each search location in order to achieve the required sensitivity. In order to detect highly attenuated GPS signals indoors, dwell times must be increased by two to three orders of magnitude. This can be achieved by using two primary techniques: (1) A-GPS which provides additional GPS information or (2) massively increasing the number of correlators available for acquiring the encoded GPS signals. The second option is not suggest since it will increase the hardware cost and consuming power.

A-GPS can provide an acquisition-aiding signal, which identifies satellites in view and includes Doppler prediction data as well as GPS satellite coordinates, for user terminals. A Doppler estimate and a stable frequency reference allow the search space in frequency to be collapsed to just a single frequency. But, a GPS C/A-code signal still contains 1,023 chips of possible code delay. For GPS-synchronized cellular systems (such as IS-95 (CDMA)), the aiding information can also carry delay estimated information that further narrows the search window. For non-GPS synchronized systems (such as GSM, TDMA, and W-CDMA), the GPS receiver must be able to search a substantial number of delay bins in parallel.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Insert the following text in line 29 of page 132:]

15.4.1 Indoor relay operation with GPS assistance

In order to obtain the location and the synchronization of the indoor RS, the Assisted-GPS-enabled 16m network service provides additional GPS information such as an acquisition-aiding signal, which identifies satellites in view and includes Doppler prediction data as well as GPS satellites’ coordinates for user terminals to facilitate indoor GPS operation. In addition, the aiding information can also carry delay estimated information that further narrows the search window for GPS synchronized cellular systems.

-----------------------------------------------------------End of the Text---------------------------------------------------------