| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Request for Femtocell Relay Support in IEEE 802.16m System Description Document |  |
| Date Submitted | 2008-02-27 |  |
| Source(s) | Ping-Heng KUO, Yu-Tao HSIEH, Pang-An TING ITRI | E-mail: pinghengkuo@itri.org.tw (+ 886-3-5918371) |
| Re: | Change request to the 802.16m SDD (section 17.10) |  |
| Abstract | We propose to include the feature of supporting cooperative MIMO schemes in femtocells for 802.16m by regarding an unscheduled MS as a virtual ARS. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed SDD Text for

Femtocell Cooperative Relay Support

Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting,

ITRI

Introduction

Multiple-input multiple-output (MIMO) techniques are supported by the femtocells for throughput enhancement and interference mitigation [1]. In some circumstances, due to restrictions on physical sizes, only single antenna can be equipped at the mobile stations (MS). Thus, Single-User MIMO can not be applied in these cases.

According to Section 15.1 of the current SDD [1], an ARS can jointly transmit data from a subordinate station, which can enable multiple stations to share the antennas to create virtual antenna array, so the benefits of MIMO systems such as transmit/receive diversity, multiplexing, and beamforming gains can be extracted in a distributed fashion. Along with the advantages of MIMO transmission, cooperative communications also offer many other benefits such as reductions in energy consumptions (see [2] and references therein). In femtocells, however, due to the small coverage area, it is unlikely for an ARS to be existed within a femtocell. Hence, when the MS has only one antenna and there is no ARS in proximity, cooperative relaying may not be inapplicable in femtocells.

Nevertheless, the femtocells are mainly applied in indoor environments, so the MS mobility levels are generally very low. Therefore, an unscheduled MS may be regarded as a virtual static ARS in femtocells that can assist the scheduled MS to perform cooperative relaying. Furthermore, since the distances between MS in a femtocell are usually smaller, the virtual ARS may listen to the data from the MS more easily due to insignificant path loss.

Thus, we suggest the support of cooperative relaying in femtocell by regarding a MS as a virtual ARS. The protocol, control signals, and frame structures etc are all the same as the current multi-hop relay systems for 802.16m.

Text Proposal

----------------------------------------------------------Start of text ---------------------------------------------------------

17.10 MIMO Support

Femto BS may support multi-antenna techniques for improved throughput and interference mitigation performance.

MIMO techniques can also be applied in femtocell via cooperative relaying. In particular, an unscheduled MS can be regarded as a virtual ARS, based on the same protocol for cooperative relaying as described in Section 15.1.

----------------------------------------------------------End of text ---------------------------------------------------------

References

[1] “The Draft IEEE 802.16m System Description Document”, IEEE 80216m-08_003r7.

[2] M. D. Kates and F. H. P. Fitzek, Ed., “WiMAX Evolution: Emerging Technologies and Applications (Chapter 6: Cooperative Principles in WiMAX)”, John Wiley & Sons, 2009.