| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC SDU Fragmentation and Packing scheme for 16m |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Baowei Ji, Sungjin Lee, Rakesh Taori, Jungje Son Samsung Electronics Sungcheol Chang ETRI Shashikant Maheshwari, Haihong Zheng, Adrian Boariu, Yousuf Saifullah Nokia Siemens Networks Mary Chion, Hongyun Qu ZTE | E-mail:anilag@samsung.com, yb.chang@samsung.com E-mail: scchang@etri.re.kr E-mail: shashi.maheshwari@nsn.com E-mail: mchion@zteusa.com qu.hongyun@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0008 “IEEE 802.16m Internal Documents Configuration Control Procedure (CCP) |  |
| Abstract | This contribution proposes the detail description of fragmentation and packing extended header in SDD document. |  |
| Purpose | To be discussed and accepted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC SDU Fragmentation and Packing scheme for 16m

Anil Agiwal, Youngbin Chang, Baowei Ji, Sungjin Lee, Rakesh Taori, Jungje Son

Samsung Electronics

Sungcheol Chang

ETRI

Shashikant Maheshwari, Haihong Zheng, Adrian Boariu, Yousuf Saifullah

Nokia Siemens Networks

Mary Chion, Hongyun Qu, Jerry Chow, Huiying Fang, Sean Cai

ZTE

# Introduction

This contribution is dealing with Fragmentation and Packing Extended Header (FPEH) format for single flow case. We proposed two fragmentation and packing extended header formats here.

1) MAC PDU contains transport connection payload- There is no type field, but it shall always locate on the last extended header among multiple extended headers.

2) MAC PDU contains management connection payload – FPEH needs type field.

- If management is not fragmented, then there is no FPEH.

- If management is fragmented, then it needs a FPEH including FPEH ‘type’ field.

# Usage examples of Fragmentation and Packing extended header.

1) MAC PDU contains transport connection payload

2) MAC PDU contains management connection payload

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Insert the following text in section 10.12.2.1 in IEEE 802.16m-08/003r7]

10.12.2.1 Fragmentation and packing extended header for transport connection

This fragmentation and packing extended header is shown in Figure xxx. This header shall be used when MAC PDU contains single transport connection payload. The location of this header exists after the last extended header ( i.e. extended header with ‘Last’ = ‘1’) if ‘EH’ in GMH set to ‘1’ or after the GMH if ‘EH’ in GMH set to ‘0’.

Figure xxx. Fragmentation and packing extended header format for transport connection

- SN (10 bits): Payload sequence number

- FC (2 bits): Fragmentation control bits definition is given in table yyy

- End (1 bit): If this bit set to ‘0’, another ‘Length’ and ‘End’ field are followed. If this bit set to ‘1’, reserved bits may follow for byte alignment

- Length (11bits): This field represents the length of SDU/SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 length fields are present in the header

- Rsvd: Reserved bits for byte alignment.

| FC | Meaning | Examples |
| --- | --- | --- |
| 00 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | One or Multiple Full SDUs packed in an MPDU |
| 01 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only First fragment of an SDU b) MPDU with one or more unfragmented SDUs, followed by first fragment of subsequent SDU |
| 10 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | a) MPDU with only Last fragment of an SDU b) MPDU with Last fragment of an SDU, followed by one or more unfragmented subsequent SDUs |
| 11 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only middle fragment of an SDU b) MPDU with Last fragment of an SDU, followed by zero or more unfragmented SDUs, followed by first fragment of a subsequent SDU |

Table yyy. Fragmentation control information

[Insert the following text after section 10.12.2.1 in IEEE 802.16m-08/003r7]

10.12.2.2 Fragmentation extended header for management connection

This fragmentation extended header is shown in Figure zzz. This header shall be used when MAC PDU contains single management message payload.

Figure zzz. Fragmentation extended header format for management connection

- Last (1 bit): always set to ‘1’

- TYPE(TBD): Extended header type field

- SN (8 bits): Payload sequence number

- FC (2 bits): Fragmentation control bits definition is given in table yyy

- Rsvd: Reserved bits for byte alignment.

---------------------------------------------------------End of the Text----------------------------------------------------------