| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of MC Text in SDD |  |
| Date Submitted | 2009-02-07 |  |
| Source(s) | Jaehee Cho Samsung Electronics | E-mail: jaehee1.cho@samsung.com |
| Re: | 80216m-08_003r7: SDD |  |
| Abstract | This contribution contains text modification on MC reflecting Jan IEEE 802.16m meeting |  |
| Purpose | Draft for further development of the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Support for Multi-carrier Operation

## PHY Aspects of OFDMA Multi-carrier Operation

Physical layer to support OFDMA multi-carrier operation is shown in Figure 57. A single MAC PDU or a concatenated MAC PDUs is received through the PHY SAP and they can form a FEC block called PHY PDU. The figure shows that the physical layer performs channel encoding, modulation and MIMO encoding for a PHY PDU and generates a single modulated symbol sequence. Any one of the multiple carriers (primary or secondary carriers) can deliver a modulated symbol sequence. Different modulated symbol sequences transmitted on the same or different carriers may have different MCS and MIMO schemes. Or, in case of allocation on DRU, a single modulated symbol sequence may be segmented into multiple segments where each segment can be transmitted on a different carrier. The same MCS level and MIMO scheme are used for all segments of a PHY PDU. However, different PHY PDUs transmitted on the same or different carriers may have different MCS and MIMO schemes. The physical layer performs subcarrier mapping for a modulated symbol sequence or a segment of the sequence relevant to the given carrier.

### 19.3.8 UL Power Control

Depending on the correlation between RF carriers, separate controls of UL power for different RF carriers are necessary. Thus, one or multiple power control commands for multiple carriers are supported. Although multiple power control commands are allowed, the power control commands or messages can be sent to AMS through the primary carrier. When an AMS switches to a secondary carrier in carrier switching mode, the secondary carrier can carry power control commands or messages if supported by its configuration.