| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Management Message Protection in 802.16m |  |
| Date Submitted | 2009-2-27 |  |
| Source(s) | Chengyan Feng ZTE Corporation | Email: feng.chengyan@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | TGm SDD: 10.6.5.2.1 Management Message Protection |  |
| Abstract | Propose management message protection to support the selective confidentiality protection over MAC management messages. |  |
| Purpose | For discussion and adoption in 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Management Message Protection in 802.16m

Chengyan Feng

ZTE Corporation

# 1. Introduction

IEEE 802.16m supports the selective confidentiality protection over MAC management messages. Through capability negotiation, AMS and ABS know whether the selective confidentiality protection is applied or not. If the selective confidentiality protection is activated, the negotiated keying materials and cipher suites are used to encrypt the management messages. And three levels of selective confidentiality protection over management messages in SDD have been defined. They are NO Protection, CMAC based integrity protection and AES-CCM based authenticated encryption.

In SDD, CMAC Keys are defined to protect integrity of management messages, which are CMAC_KEY_U and CMAC_KEY_D in 802.16-2005. We can extend these keys’ function either to authenticate the messages or to encrypt the messages.

# 2. Solution

In this section, we propose a method to support the selective confidentiality protection over MAC management messages. We use CMAC keys to protect management messages no matter CMAC based integrity protection is required or AES-CCM based authenticated encryption is required. Maybe we can call the CMAC keys Message Encryption Key(MEK) better.

During SBC procedure, AMS and ABS negotiate the management message protection policy: NO Protection, CMAC based integrity protection or AES-CCM based authenticated encryption.

When NO Protection is selected, the management messages are neither encrypted nor authenticated.

When CMAC based integrity protection or AES-CCM based authenticated encryption is selected, AMS and ABS use MEK to protect the management messages. When only CMAC is required, MEK is used as the CMAC key to protect intergrity of the messages. When authenticated encryption is required, MEK is used as the encryption and authentication key based on AES CCM mode.

MEK is derived locally by using the AK, the KEY_COUNT, BSID and SAID of SA concerned with control

plane/management signaling, as well as other identity parameters. MEK is divided into MEK_U and MEK_D, and MEK_U is used to protect UL management messages, while MEK_D is used to protect DL management messages.

Figure 1 Management Message Protection

# 3. Text Proposal

============================== Start of Proposed Text ===============

10.6.3.1 Key Derivation

All IEEE 802.16m security keys are either derived directly / indirectly from the MSK or generated randomly by the ABS.

The Pairwise Master Key (PMK) is derived from the MSK and then this PMK is used to derive the

Authorization Key (AK).

Some security keys are respectively derived and updated by both the ABS and the AMS.

The Authorization Key (AK) is used to derive other keys:

• Key Encryption Key (KEK)

• Transmission Encryption Key (TEK)

• Management Message Encryption Key(MEK)

After completing (re)authentication process and obtaining an AK, key agreement is performed to verify the

newly created AK and exchange other required security parameters.

KEK derivation follows procedures as defined in the WirelessMAN-OFDMA Reference system..

TEK is derived at AMS and ABS by feeding identity parameters into a key derivation function. Parameters

such as AK, Security Association ID (SAID), NONCE, KEY_COUNT, BSID, AMS MAC address can be used.

NONCE is generated by ABS and distributed to AMS. If more than one TEK is to be created for an SA, separate KEY_COUNTs are maintained for each TEK.

The MEK is derived locally by using the AK, the KEY_COUNT and SAID of SA concerned with control plane/management signaling, as well as other identity parameters.

TEK(s) and MEK(s) are derived in the following situations:

• Initial authentication

• Re-authentication

• Key update procedure for unicast connection.

• Network re-entry to new ABS.

In the last two cases, KEY_COUNT value is incremented prior derivation.

10.6.5.2 Control Plane Signaling Protection

10.6.5.2.1 Management Message Protection

IEEE 802.16m supports the selective confidentiality protection over MAC management messages. Through

capability negotiation, AMS and ABS negotiate the management message protection policy: NO Protection, CMAC based integrity protection and AES-CCM based authenticated encryption.  When AES-CCM based authenticated encryption is selected, AMS and ABS use MEK to encryt and authenticate the management messages. When only CMAC is required, MEK is used as the CMAC key to protect intergrity of the messages.

============================== End of Proposed Text ===============