| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | STID Transmission in 802.16m |  |
| Date Submitted | 2009-2-27 |  |
| Source(s) | Chengyan Feng ZTE Corporation | Email: feng.chengyan@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | TGm SDD: 10.6.2.1 AMS Privacy |  |
| Abstract | Propose Station Identifier transmission to support AMS identity privacy |  |
| Purpose | For discussion and adoption in 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

STID Transmission in 802.16m

Chengyan Feng

ZTE Corporation

# 1. Introduction

In 802.16m SDD, the STID (Station Identifier) is defined to uniquely identify the AMS within the domain of the ABS. The ABS assigns a STID to the AMS during network entry, and, in some cases, network re-entry. Each AMS registered in the network has an assigned STID. Some specific “STIDs” are reserved, for example, for broadcast, multicast, and ranging. 802.16m SDD also said that the STID is assigned during the authentication process, and the assignment message is encrypted.

However, there are some problems. First, the messages are transparent to ABS during the authentication process. What the ABS does is only to relay the messages between AMS and Authenticator. Second, only after the AMS completes the authentication successfully and establishes the security context with ABS, STID transmission can be protected by encryption. The last problem is that if the selective confidentiality protection isn’t activated during SBC procedure, the message carrying STID can’t be encrypted in this situation.

# 2. Solution

In this contribution, we propose a method to transmit STID from ABS to AMS safely. The idea is that the STID is transmitted in REG procedure after the successful initial authentication or reauthentication as the figure below. If REG-REQ/RSP messages are encrypted, STID can be carried in the REG-RSP message directly. And if REG-REQ/RSP messages are not encrypted, we can use some key to encrypt STID first and then to send. One option is to use KEK to encrypt STID in this situation.

When AMS re-entry from Idle mode, secure location Update, or HO, the ABS transmits the updated STID in RNG-RSP message. And in order to fasten the HO procedure, another method is to transmit STID during the HO preparation phase. In more detail, the updated STID is provided from T-ABS to S-ABS during HO preparation phase, and provided to AMS in MOB_BSHO-RSP message.

Figure 1 STID transmission

# 3. Text Proposal

============================== Start of Proposed Text ===============

10.6.2.1 AMS Privacy

In order to protect the mapping between the STID and the AMS MAC Address, two types of STIDs are

assigned to an AMS during network entry - temporary STID (TSTID) and (normal) STID.A TSTID is assigned during initial ranging process, and is used until the STID is allocated. The STID is assigned during REG procedure, which is after the successful authentication process, and the STID is encrypted during transmission. The TSTID is released after STID is assigned. The STID is used for all the remaining transactions.

============================== End of Proposed Text ===============